# GARBAGE MANAGEMENT PLAN

(IN ACCORDANCE WITH ANNEX V of MARPOL 73/78)

Incorporating latest amendments as per MEPC.360(79)

|  | GARBAGE MANAGEMNET PLAN | Issue No. : 00 Rev : 00 Date : |
| --- | --- | --- |
|  |  | Page 1 of 31 |

Contents

- Application	2

- Purpose	2

- Ship particulars	2

- Introduction	4

- Definitions	4

- Duties of designated garbage management personnel	9

  - Chief Officer	9

  - Designated garbage management personnel	10

  - Duties of shipboard personnel	10

- Garbage categories	10

- Collection, separation and stowage	11

  - General	11

  - Garbage separation as per categories	12

  - Marking of garbage receptacles	13

- Onboard processing methods	13

    - Plastic	18

    - Cooking Oil	20

    - Food wastes	21

    - Prohibited items	21

    - Expired medicines	22

    - COVID-19 Disposable Masks/ Protective kits	22

- Discharge to sea / port / other ship	23

- Management of cargo residues of solid bulk cargoes	24

- Waste minimization/reduction	25

  - Minimization strategies	25

- Record keeping	26

- Special items	28

- Training, education and information	30

- Exceptions	30

- Referenced forms and checklists	30

- Filing	31

1

APPLICATION

This document applies specifically to the vessel mentioned under Vessel Particulars.

2

PURPOSE

Purpose of Garbage Management Plan is to assist crew members of the vessel, contractors and visitors in achieving compliance to MARPOL Annex V to prevent pollution from ship generated wastes.

3

SHIP PARTICULARS

| Name of Ship |  | : |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Call Sign |  | : |  |  |  |
| IMO Number |  | : |  |  |  |
| Designated Garbage Management Personnel |  |  |  |  |  |
| 1. |  |  |  | Designated Person in charge (PIC) |  |
| 2. |  |  |  | Shipboard Incinerator | Location: |
| 3. |  |  |  | Grinder or Communiter | Location: |
| 4. |  |  |  | Compactor | Location: POOP DECK, WORK SHOP |
| 5. |  |  |  | Galley Collection & Segregation |  |
| 6. |  |  |  | Environment Officer (Chief Engineer) |  |
| 7. |  |  |  | Process : | Location: |
| 8. |  |  |  | Process : | Location: |
| 9. |  |  |  | Equipment : | Location: |
| 10. |  |  |  | Equipment : | Location: |

| Ship’s Equipment for Processing Garbage |  |  |
| --- | --- | --- |
| 1. |  |  |
| 2. |  |  |
| 3. |  |  |

## Storage

The Designated garbage storage or processing area located in

| Garbage Category | No of Storage Bins | Total Storage Capacity M3 | Garbage Category | No of Storage Bins | Total Storage Capacity M3 |
| --- | --- | --- | --- | --- | --- |

Recycling / Reuse: The Designated garbage storage or processing area located in

| 1. | ( m3) | 2. | ( m3) |
| --- | --- | --- | --- |

Other collection areas: Refer Poster GP01 for specific colour coding of receptacles.

| Areas | No of Bins | Garbage Category |
| --- | --- | --- |

| Areas | No of Bins | Garbage Category |
| --- | --- | --- |

4

INTRODUCTION

MARPOL Annex V and Polar Code (as adopted by IMO Resolution MEPC.264-68) prohibit the discharge of all types of garbage into the sea unless explicitly permitted under the Annex.

In accordance with regulation 10 of Annex V of the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 (MARPOL), a record is to be kept of each discharge operation or completed incineration. This includes discharges into the sea, to reception facilities, or to other ships, as well as the accidental loss of garbage.

Discharge is restricted to food wastes and solid bulk cargo residues which are not harmful to marine environment (Non-HME), animal carcasses, and identified cleaning agents and additives and cargo residues entrained in wash water which are not harmful to the marine environment.

Caution: MARPOL Annex V prohibits discharge of all types of garbage into the sea unless explicitly permitted under the Annex. No Food waste is permitted to be discharged to sea whilst the vessel is at anchorage/ Port or drifting. Flag approval will be necessary for such, should accumulation be a cause of health and hygiene concern. Ref Sec 9.5.3 for guidance.

It is recommended that ships use port reception facilities as the primary means of discharge for all garbage.

5

DEFINITIONS

Animal carcasses

Animal carcasses means the bodies of any animals that are carried on board as cargo and that die or are euthanized during the voyage.

## Arctic Waters (Reference shall be made to Polar Code as amended)

Arctic waters means those waters which are located north of a line from the latitude 58º00΄.0 N and longitude 042º00΄.0 W to latitude 64º37΄.0 N, longitude 035º27΄.0 W and thence by a rhumb line to latitude 67º03΄.9 N, longitude 026º33΄.4 W and thence by a rhumb line to the latitude 70º49΄.56 N and longitude 008º59΄.61 W (Sørkapp, Jan Mayen) and by the southern shore of Jan Mayen to 73º31'.6 N and 019º01'.0 E by the Island of Bjørnøya, and thence by a great circle line to the latitude 68º38΄.29 N and longitude 043º23΄.08 E (Cap Kanin Nos) and hence by the northern shore of the Asian Continent eastward to the Bering Strait and thence from the Bering Strait westward to latitude 60º N as far as Il'pyrskiy and following the 60th North parallel eastward as far as and including Etolin Strait and thence by the northern shore of the North American continent as far south as latitude 60º N and thence eastward along

parallel of latitude 60º N, to longitude 056º37΄.1 W and thence to the latitude 58º00΄.0 N, longitude 042º00΄.0 W

## Cargo residues

Cargo residues means the remnants of any cargo which are not covered by other Annexes to the present Convention and which remain on the deck or in holds following loading or unloading, including loading and unloading excess or spillage, whether in wet or dry condition or entrained in wash water but does not include cargo dust remaining on the deck after sweeping or dust on the external surfaces of the ship.

On log carrier vessels, bark which is the outermost layers of stems and roots of woody plants in dry or wet form is to be treated as cargo residues and discharged in accordance with criteria for cargo residues (HME/ Non-HME).

## Cooking oil

Cooking oil means any type of edible oil or animal fat used or intended to be used for the preparation or cooking of food, but does not include the food itself that is prepared using these oils.

## Domestic wastes

Domestic wastes mean all types of wastes not covered by other Annexes that are generated in the accommodation spaces on board the ship.

Domestic wastes do not include grey water. The COVID-19 related PPE are a part of Domestic wastes and should be stored in separate bins as “Domestic Bins”

## Dishwater

Dishwater means the residue from the manual or automatic washing of dishes and cooking utensils which have been pre-cleaned to the extent that any food particles adhering to them would not normally interfere with the operation of automatic dishwashers.

## En route

En route means that the ship is underway at sea on a course or courses, including deviation from the shortest direct route, which as far as practicable for navigational purposes, will cause any discharge to be spread over as great an area of the sea as is reasonable and practicable. Company policy defines En-Route as speed of 4-6 Knots.

## E-Waste

E-waste means electrical and electronic equipment used for the normal operation of the ship or in the accommodation spaces, including all components, sub-assemblies and consumables which are part of the equipment at the time of discarding, with the presence of material potentially hazardous to human health and/or the environment.

## Fast Ice

Fast ice means sea ice which forms and remains fast along the coast, where it is attached to the shore, to an ice wall, to an ice front, between shoals or grounded icebergs.

## Fishing gear

Fishing gear means any physical device or part thereof or combination of items that may be placed on or in the water or on the seabed with the intended purpose of capturing, or controlling for subsequent capture or harvesting, marine or freshwater organisms.

## Fixed or floating platforms

Fixed or floating platforms means fixed or floating structures located at sea which are engaged in the exploration, exploitation or associated offshore processing of sea-bed mineral resources.

## Food wastes

Food wastes mean any spoiled or unspoiled food substances and include fruits, vegetables, dairy products, poultry, meat products and food scraps generated aboard ship.

## Garbage

Garbage means all kinds of food wastes, domestic wastes and operational wastes, all plastics, cargo residues, cooking oil, fishing gear, and animal carcasses generated during the normal operation of the ship and liable to be disposed of continuously or periodically except those substances which are defined or listed in other Annexes to the present Convention. Garbage does not include fresh fish and parts thereof generated because of fishing activities undertaken during the voyage, or as a result of aquaculture activities which involve the transport of fish including shellfish for placement in the aquaculture facility and the transport of harvested fish including shellfish from such facilities to shore for processing.

## Other Annexes to the present convention include:

Annex I - Oil

Annex II - Noxious Liquid Substances

Annex III – Harmful Substances in Packaged Form Annex IV – Sewage

Annex VI – Air Emissions

Releasing small quantities of food into the sea for the specific purpose of fish feeding in connection with fishing or tourist operations should not be considered a discharge of garbage in the context of Annex V.

## Grey water

Grey water means drainage from dishwater, shower, laundry, bath and washbasin drains. It does not include drainage from toilets, urinals, hospitals, and animal spaces, as defined in regulation 1.3 of MARPOL Annex IV (sewage), and it does not include drainage from cargo spaces. Grey water is not considered garbage in the context of Annex V.

## Harmful to Marine Environment (HME)

For the purpose of this Annex, cargo residues are considered to be harmful to the marine environment (HME) if they are residues of solid bulk cargoes which are classified according to the criteria of the United Nations Globally Harmonized System of Classification and Labelling of Chemicals (GHS).

Solid bulk cargoes other than harmful to marine environment are classified as non-harmful to marine environment (NON-HME).

## Ice shelf

Ice-shelf means a floating ice sheet of considerable thickness, showing 2 to 50 m or more above sea-level, attached to the coast.

## Incinerator ashes

Incinerator ashes mean ash and clinkers resulting from shipboard incinerators used for the incineration of garbage.

## IMSBC Code

It means the International Maritime Solid Bulk Cargoes Code.

## Nearest land

The term "from the nearest land" means from the baseline from which the territorial sea of the territory in question is established in accordance with international law, except that, for the purposes of the present Annex, ''from the nearest land'' off the north-eastern coast of Australia shall mean from a line drawn from a point on the coast of Australia in:

- latitude 11°00΄ S, longitude 142°08΄ E

- to a point in latitude 10°35΄ S, longitude 141°55΄ E,

- thence to a point latitude 10°00΄ S, longitude 142°00΄ E,

- thence to a point latitude 09°10΄ S, longitude 143°52΄ E,

- thence to a point latitude 09°00΄ S, longitude 144°30΄ E,

- thence to a point latitude 10°41΄ S, longitude 145°00΄ E,

- thence to a point latitude 13°00΄ S, longitude 145°00΄ E,

- thence to a point latitude 15°00΄ S, longitude 146°00΄ E,

- thence to a point latitude 17°30΄ S, longitude 147°00΄ E,

- thence to a point latitude 21°00΄ S, longitude 152°55΄ E,

- thence to a point latitude 24°30΄ S, longitude 154°00΄ E,

- thence to a point on the coast of Australia in latitude 24°42΄ S, longitude 153°15΄ E.

## Operational wastes

Operational wastes mean all solid wastes (including slurries) not covered by other Annexes that are collected on board during normal maintenance or operations of a ship, or used for cargo stowage and handling. Operational wastes also include cleaning agents and additives contained in cargo hold and external wash water. Operational wastes does not include grey water, bilge water, or other similar discharges essential to the operation of a ship, taking into account the guidelines developed by the Organization.

Ash and clinkers from shipboard incinerators and coal-burning boilers should be considered as operational wastes.

The definition of "operational wastes" (regulation 1.12 of MARPOL Annex V) excludes grey water, bilge water, or other similar discharges essential to the operation of a ship. "Other similar discharges" essential to the operation of a ship include, but are not limited to the following:

- Boiler / economizer blowdown;

- Boat engine wet exhaust;

- Chain locker effluent;

- Controllable pitch propeller and thruster hydraulic fluid and other oil to sea interfaces (e.g. thruster bearings, stabilizers, rudder bearings, etc.);

- Distillation / reverse osmosis brine;

- Elevator pit effluent;

- Firemain systems water;

- Freshwater lay-up;

- Gas turbine washwater;

- Motor gasoline and compensating discharge;

- Machinery wastewater;

- Pool, spa water and recreational waters;

- Sonar dome discharge; and

- Welldeck discharges.

## Plastic

Plastic means a solid material which contains as an essential ingredient one or more high molecular mass polymers and which is formed (shaped) during either manufacture of the polymer or the fabrication into a finished product by heat and/or pressure. Plastics have material properties ranging from hard and brittle to soft and elastic. For the purposes of this annex, "all plastics" means all garbage that consists of or includes plastic in any form, including synthetic ropes, synthetic fishing nets, plastic garbage bags and incinerator ashes from plastic products.

## Polar Code

International Code for Ships Operating in Polar Waters, consisting of an introduction, part I-A and part II-A and parts I-B and II-B, as adopted by resolutions MSC.385(94) and MEPC.264(68), as may be amended)

## Polar Waters

Polar Waters means Arctic waters and / or Antarctic area.

## Recycling

Recycling means the activity of segregating and recovering components and materials for reprocessing.

## Reuse

Reuse means the activity of recovering components and materials for further use without reprocessing.

## Annex-V Special area

Special area means a sea area where for recognized technical reasons in relation to its oceanographic and ecological condition and to the character of its traffic the adoption of special mandatory methods for the prevention of sea pollution by garbage is required.

For the purposes of this Annex the special areas are the Mediterranean Sea area, the Baltic Sea area, the Black Sea area, the Red Sea area, the Gulfs area, the North Sea area, the Antarctic area and the Wider Caribbean Region, which are defined as follows:

- The Mediterranean Sea area means the Mediterranean Sea proper including the gulfs and seas therein with the boundary between the Mediterranean and the Black Sea constituted by the 41º N parallel and bounded to the west by the Straits of Gibraltar at the meridian 5°36΄ W.

- The Baltic Sea area means the Baltic Sea proper with the Gulf of Bothnia and the Gulf of Finland and the entrance to the Baltic Sea bounded by the parallel of the Skaw in the Skagerrak at 57º 44.8΄ N.

- The Black Sea area means the Black Sea proper with the boundary between the Mediterranean and the Black Sea constituted by the parallel 41º N.

- The Red Sea area means the Red Sea proper including the Gulfs of Suez and Aqaba bounded at the south by the rhumb line between Ras si Ane (12º 28.5΄ N, 43º 19.6΄ E) and Husn Murad (12º 40.4΄ N, 43º 30.2΄ E).

- The Gulfs area means the sea area located north-west of the rhumb line between Ras al Hadd (22º 30΄ N, 59º 48΄ E) and Ras al Fasteh (25º 04΄ N, 61º 25΄ E).

- The North Sea area means the North Sea proper including seas therein with the boundary between:

- the North Sea southwards of latitude 62º N and eastwards of longitude 4º W;

- the Skagerrak, the southern limit of which is determined east of the Skaw by latitude 57º

44.8΄ N; and

- the English Channel and its approaches eastwards of longitude 5º W and northwards of latitude 48º 30΄ N.

- The Wider Caribbean Region means the Gulf of Mexico and Caribbean Sea proper including the bays and seas therein and that portion of the Atlantic Ocean within the boundary constituted by the 30° N parallel from Florida eastward to 77°30΄ W meridian, thence a rhumb line to the intersection of 20° N parallel and 59° W meridian, thence a rhumb line to the intersection of 7°20' N parallel and 50° W meridian, thence a rhumb line drawn southwesterly to the eastern boundary of French Guiana.

- The Antarctic area means the sea area south of latitude 60º S.

DUTIES OF DESIGNATED GARBAGE MANAGEMENT PERSONNEL

6

## Chief Officer

Chief Officer is designated as Person In-Charge (PIC) and is responsible to:

    - Implement Garbage management Plan.

    - Update ship specific information in Section 3.0

    - Make entries in Garbage Record Books Part I and Part II as applicable. CO may delegate Incineration entries for 2E, however the overall responsibility of factual entry in GRB lies with Chief Officer.

    - Conduct training & familiarization of Shipboard personnel to facilitate compliance to the Garbage management Plan.

    - Liaise with Shore authorities for port reception facilities.

    - Ensure posters GP01/GP02/GP03/GP04, 05A/05B, 06 are displayed as per company’s instructions.

    - Review garbage management plan and ship specific measures and provide appropriate feedback to master and company to take necessary actions.

    - Ensure each Crew cabin and Public space has two bins rightly colour coded, one for Plastics and one for Domestic wastes.

    - To Train, advise and instruct Galley crew on confirming with CO and/ or Bridge prior disposing any food wastes into sea

    - To Train, advise and instruct all crew on correct garbage segregation and ensuring none familiarising on items prohibited for incineration

## Designated garbage management personnel

PIC shall record names of crew members designated as garbage management personnel in section 3.0.

The designated garbage management personnel responsible for a Ship Specific Equipment or Process, shall ensure:

    - Operation and maintenance of the Equipment is carried out as per Manufacturer’s instructions.

    - Copies of Manufacturer’s instruction to be kept in the safe custody of the person responsible for the equipment for ready and quick reference when required.

    - Processing of Garbage waste is carried out as per the Garbage management plan and in compliance with MARPOL Annex V and Polar Code

    - Reporting each operation with Time and Quantity processed.

    - Reporting any inconsistency noted.

## Duties of shipboard personnel

Shipboard personnel shall all times co-operate with the designated garbage management personnel and person in charge to ensure minimization, collection, separation and processing of garbage is appropriate and efficient in all areas of the ship.

Shipboard personnel shall be fully informed about the requirements of the MARPOL Annex V and Polar Code with regards to Garbage or seek education or training from the PIC.

If in any doubt with regards to the processing or discharge of garbage, express advice from the designated management personnel or PIC shall be sought.

GARBAGE CATEGORIES

7

In accordance with MARPOL Annex V, as amended, shipboard garbage is categorized as per below:

A - Plastics

B - Food Wastes1

C - Domestic Waste D - Cooking Oil2

E - Incinerator ashes

F - Operational Wastes G – Animal carcasses H – Fishing Gear

I – E-Waste

J – Cargo Residues (non-HME) 3

Note:

- If food waste is being disposed overboard, there must be strict separation in the galley.

Food and beverage packs must be separated prior disposal. For Example – Tea bags, which must be opened after use and contents to be separated for disposal in food waste while wrapper in domestic waste or plastic waste as applicable.

- Refer to special measure for disposal of cooking oil.

- Applicable only to ships carrying solid bulk cargoes.

K – Cargo Residues (HME) 3

8

COLLECTION, SEPARATION AND STOWAGE

## General

It is expected that garbage is separated in accordance with categories into designated receptacles as per Poster GP01.

Distinctively marked colour coded garbage receptacles (As per poster GP01) shall be provided at various locations on-board.

Receptacles on board must be of non-combustible material and provided with lid/ cover as appropriate.

Receptacles should be placed in appropriate spaces throughout the ship (e.g. the engine-room, mess rooms, smoke rooms, laundries, crew cabins, CCR, ECR, galley, and other living or working spaces) and all crew members and visitors should be trained as per poster GP01 for proper separation.

Chief Officer shall update ship specific arrangements of receptacles in section 3.0 Garbage should be stored in a manner which avoids health and safety hazards.

Enough storage space shall be provided and checked by the designated Garbage Management Personnel

Where storage space is limited, installation of compactors or incinerators are to be considered.

To the extent possible, all processed and unprocessed garbage stored for any length of time should be in tight, securely covered containers in order to prevent the unintentional discharge of stored garbage.

Food wastes and other garbage to be returned to port and which may carry diseases or pests should be stored in tightly covered containers and be kept separate from garbage which does not contain such food wastes. Food wastes must not be stored in refrigerated provision rooms.

Quarantine arrangements in some countries may require double bagging of this type of waste.

Cleaning and disinfecting are both preventative and remedial pest control methods that should be applied regularly in garbage storage areas.

The designated Garbage Management Personnel must ensure that there is no build-up of gas from the waste in the storage area.

All processed and unprocessed garbage should be in tight, securely covered, non-combustible containers with no openings in the sides or bottom.

The storage locations for garbage should be carefully selected to ensure that the garbage presents no potential hazard to adjacent spaces.

## Garbage separation as per categories

| Garbage Category | Examples | Colour Code | Sea Discharge | Incinerate | Recycle / Reuse On-board | Reception Facilities |
| --- | --- | --- | --- | --- | --- | --- |
| A. Plastics Categories as PVC, PCB, PS | Synthetic ropes, Styrofoam or other similar packing material. | Red | Refer Poster GP02: Statutory requirements for disposal of garbage | No | No | Yes |
| A. Plastics- Othe categories as PETE HDPE, LDPE | Eg: Garbage bags, bubble wraps, cling wrap, packaging, drinking bottles, squeezable tubes & bottles etc | Red |  | Yes* | No | Yes |
| B. Food Wastes | Spoiled or unspoiled food, fruits, vegetables, dairy products, poultry, meat products, food scraps | Blue |  | Yes* * | No | Yes |
| C. Domestic Wastes (I) | Eg Paper, cardboard, corrugated board, wood, textiles, Disposable COVID-19 Masks/ PPE (Sep bin) | Green |  | Yes3 | Yes3 | Yes |
| C. Domestic Wastes (II) | Glass, aluminium cans, metal | Green |  | No | Yes3 | Yes |
| D. Cooking Oil | Edible Oil | Yellow |  | Yes3 | Yes3 | Yes |
| E. Incinerator Ashes | Garbage generated ash or clinkers | Grey |  | No | No | Yes |
| F. Operational Wastes (I) | Cleaning agents and additives contained in cargo hold wash- water and in deck and external surfaces wash-water | Black |  | No | No | Yes |
| F. Operational Wastes (II) | Oily rags, other Wastes (Item 4 below) | Black |  | Yes3 | No | Yes |
| I. E-Waste | Computer, printers, batteries, CD DVD, fluorescent lights, PCB, Ink cartridges | White |  | No | Yes3 | Yes |
| J. Cargo Residues5 (Non HME) | Solid bulk cargo residues NO hazardous to marine environmen as per shipper’s declaration an Appendix 1 of MARPOL Annex V | Light Blue |  | No | No | Yes |
| K. Cargo Residues5 (HME) | Solid bulk cargo residues hazardous to marine environmen as per shipper’s declaration and Appendix 1 of MARPOL Annex V | Orange |  | No | No | Yes |

T

* Incinerator Type approval must allow Plastics & Master to Ref GP A2, Poster 006. Refer to sec 9.1 for more details

** Incinerator operations are prohibited within Port Limits and ECA areas. In cases of prolonged anchorage and high garbage accumulation (Including Food Wastes), Master to refer to GP A1 for guidance and approach office (TSI/MTSI) for landing arrangements. For Incineration use in port, Port state approval and office approval is mandatory.

Note: Domestic and Operational wastes have two parts due to the nature of the wastes with regards to possible disposal methods.

## Marking of garbage receptacles

    - All garbage receptacles are required to be colour coded as well as stencilled with the ‘garbage category’.

    - Colour coding would include either colouring of the complete receptacle in the designated colour or at least Two (2) distinctive bands of 5 inches + cover of the receptacle.

    - Reference to be made to sections in this for requirements on incineration and recycle, re- use.

    - Operational waste hazardous in nature such as aerosol cans, smoke detectors, Medicine wastes; expired pyrotechnics, are to be stored in separate receptacle of black colour marked as “Hazardous wastes”. The hazardous waste shall be disposed to port facilities as Cat-F (Operational waste) and Expired medicines as Cat-C (Domestic Wastes)

    - Receptacles for CAT J and CAT K are only required for bulk carrier vessels carrying solid bulk cargo

    - Category G: Animal carcass and category H: Fishing net has not been included. If required, a separate receptacle shall be designated as per ship specific requirement for storage of such waste. Crew shall be trained in this regard.

9

ONBOARD PROCESSING METHODS

## General

Use of compactors, incinerators, comminuters, and other such devices have a number of advantages, such as, reducing shipboard space requirements for storing garbage, and making it easier to discharge garbage at port reception facilities.

The designated Garbage Management Personnel responsible for a specific equipment operation

/ process shall carry out or arrange the processing of the garbage as directed by the PIC. The garbage shall be processed according to the manufacturer’s / machinery instructions. A copy of the machinery instructions should be placed for easy reference.

The processing of garbage shall occur as and when necessary. Once the garbage has been processed it shall be returned to the storage area for final disposal. Appended flow chart provides guidance on shipboard garbage management.

Sea discharge

## Grinding or comminnution

A wide variety of food waste grinders is available on the market and most modern ships' galleys have the equipment needed to produce slurry of food particles and water that washes easily through the required 25 mm screen. Output ranges from 10 to 250 litres per minute. The discharge from shipboard comminuters should be directed into an appropriately constructed holding tank when the vessel is operating within an area where discharge is prohibited or alternatively prohibited from use. A warning notice is to be posted highlighting the same. Type approval certificate must eb available onboard and posted near the comminuter.

Although unprocessed food wastes may be discharged beyond 12 nautical miles, it is recommended that comminuters be used as they hasten assimilation into the marine environment. Because food wastes comminuted with plastics cannot be discharged into the sea, all plastic materials need to be removed before food wastes are placed into a comminuter or grinder.

## Compaction

Most garbage can be compacted to some degree; the exceptions include unground plastics, fibre and paper board, bulky cargo containers and thick metal items. Pressurized containers should not be compacted or shredded without the use of specialized equipment designed for this purpose because they present an explosion hazard in standard compactors.

Compaction reduces the volume of garbage. In most cases, the output from a compactor is a block of material which facilitates the shipboard storage of garbage and its discharging of the material in a port facility. It should be taken into account that the output from a compactor might be subject to quarantine, sanitary or health requirements or other requirements from the port reception facilities and advice from local authorities should be sought on any standards or requirements which are additional to those set by the Organization.

A compactor should be installed in a compartment with adequate room for operating and maintaining the unit and storing garbage to be processed.

Compactors have options including sanitizing, deodorizing, adjustable compaction ratios, bagging in plastic or paper, boxing in cardboard (with or without plastic or wax paper lining), baling, etc. Compacted materials should be stored appropriately. While metal and plastic bales can get wet, paper and cardboard bales should be kept dry.

## Compaction options for shipboard-generated garbage

| Examples of garbage |  | Special handling by vessel personnel before compaction | Compaction characteristics |  |  |  |  | Onboard storage space |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rate alteration | of | Retainment of compacted form | Density compacted form | of |  |

| Examples of garbage |  | Special handling by vessel personnel before compaction | Compaction characteristics |  |  |  |  | Onboard storage space |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rate alteration | of | Retainment of compacted form | Density compacted form | of |  |

Incineration

Shipboard incineration shall not take place inside Port Limits and ECA areas.

Incineration conducted in a shipboard incinerator can significantly reduce the need to store garbage on board the ship. Shipboard incinerators should be designed, constructed, operated and maintained in accordance with the IMO Standard Specification for Shipboard Incinerators. MARPOL Annex VI requires shipboard incinerators installed after 1 January 2000 to be type approved as per Resolution MEPC.244(66) – 2014 Standard Specification for Shipboard Incinerators as well as meet specific air pollution criteria. Incinerators should only be used to incinerate materials that are specified by the incinerator manufacturer. Incinerator should be always operated as per makers Instructions with all alarms, safety trips, door-interlocks functional.

Incinerator Operation in Port Limits and ECA areas (including anchorages) is prohibited or and shall be subject to applicable local Port state rules and regulations which the Master must confirm with the local agent prior arrival. It should be noted that special rules on incineration under domestic law may apply in some ports and may exist in some special areas. Master must communicate with agents to get approval for use of incinerator when vessel is in Port limits and at anchorage. Same applies to ECA area. The incineration of sewage or sludge in the main or Aux boilers is always prohibited.

## Batteries, aerosol cans, fluorescent tubes, chemical waste, old paint, PCB, PVC, Medical wastes and other Hazardous wastes must never be incinerated.

Ash and clinkers from shipboard incinerators is considered as operational waste and, therefore, as garbage and not permitted for discharge into the sea.

Each operator & engineer should be competent & trained for use of incinerator for on-board garbage & sludge incineration and don appropriate PPE as per sms. The Training should be carried out by Chief Engineer on safe & correct use of Incinerator as per Makers Instructions as well on categories of garbage which are prohibited for incineration as per Marpol Annex V and company procedure (Listed in Poster GP06 & this Manual). Maker specific Incinerator procedure must be made and followed onboard for safe incineration of garbage including food wastes.

Some of the disadvantages of incinerators may include the possible hazardous nature of the ash or vapour, dirty operation, excessive labour required for charging, stoking and ash removal.

On-board incineration of garbage may reduce the volume of garbage subject to quarantine requirements in some countries. However, incinerator ash may still be subject to local quarantine, sanitary or health requirements. Advice should be sought from local authorities regarding requirements that are in addition to MARPOL. For example, higher temperatures and more complete combustion may be required to effectively destroy organisms that present a risk.

Caution: Following precautions must be additionally followed for safe incineration:

- Prohibited items as listed in MARPOL Annex VI Regulation 16.2 and Company Poster GP06 must never be incinerated

-Excessive charge of garbage must not be done as this may lead to blow back as well as serious injury to personnel. Makers instruction on correct charge of garbage must be followed.

-All safety alarms, trips and door interlocks of the incinerator must always be operational and never be bypassed.

-Incinerator Maintenance as per Makers instructions and PMS must always be followed and refractory maintained in good condition.

-On Incinerator system fuel feed must be used for incineration. No manual feed of gas Oil or MDO to be used for aiding incineration as this can cause Flashback fires and injury to operator.

-Incinerator not to be operated in Port Limits and ECA areas. For Incineration in Port Limits, Master must have Port state and Office approval.

- Appropriate PPE must always be donned for Incineration operation.

## Poster 043 for safe incineration must be referred to & followed.

Note: Each operator of the on-board garbage incinerator should be trained and familiarised by Chief Engineer in the use of the Incinerator and the method, types and charge of garbage that can be incinerated.

## Incineration options for shipboard-generated garbage

| Examples of garbage | Special handling by vessel personnel before incineration | Combustibility | Reduction of volume | Residual | Exhaust | Onboard storage space |
| --- | --- | --- | --- | --- | --- | --- |

| Examples garbage | of | Special handling by vessel personnel before incineration |  | Combustibility | Reduction of volume | Residual | Exhaust |  | Onboard storage space |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

** Ref Sec 9.1.1 and GP A2 Plastic Incineration- Additional measures.

## Special incineration measures adopted by company

Note: Incinerator must always be operated as per makers instructions with all alarms, trips and safety interlocks functional & tested. Chief Engineer should ensure proper training provided to incinerator operators on safe operation of incinerator, guidance on correct amount & method of garbage dosage, precautions for safe start, cooling down and safe ash removal. Certified engineers to operate and supervise the incinerator operations. RA for Incineration as in Ocean Manager Library to be referred to for guidance on safe incineration. In case of doubt, Master to approach company.

Warning: External feed of gas oil or similar to aid faster combustion of the garbage, is strictly prohibited as there is risk of severe personal injury due to flash back.

Caution: Use of Incinerator in Port Limits and Emission Control Areas (ECA) is strictly prohibited unless approval taken from port state and office.

Cautions as mentioned in below boxes and in Sec 9.4 must always be adhered to, when incinerating garbage including Food wastes and Plastics. These cautions apply to all sub sections of Section 9.1.

# Plastic

Primary means of disposal must be to shore reception facilities. Use compactor (if provided), to compact plastic to minimise the storing space. In circumstances due to vessel’s trade pattern & operational requirements whereby Plastic has accumulated and could not be landed due to port restrictions as well as accumulation during sailing/ anchorages and/ or any other operational requirements etc, only below mentioned plastic waste (Excluding PVC, PCB, PS) may be incinerated subject to compliance to all of the following:

- The plastics are compacted.

- The vessel must be in open Sea and not in port Limits and/or in ECA area.

- Incinerators must have Type approval certification in accordance with MEPC 244(66) (built Jan’2000 onwards) indicating the design is capable of incinerating plastic as well as indicating quantity of plastic to be charged with other garbage and designed to operate with a combustion temperature in the range of 850ºC-1200ºC.

- Safe Incineration specific procedure to be made in line with Makers instructions for burning plastics and followed onboard. C/E is responsible for enforcing safe incineration operation procedure for Plastic Incineration and to be reviewed/ approved by office (VM/FGM). Such approved procedure will be guidance for future such incinerations.

- Master/CE to carry out a training on safe incineration procedure (as approved by FG), including Safe incineration Poster 43 and Makers instruction. Training to record in Form SAF26

- Poster GP06 must be referred to each time prior such operation for ensuring no prohibited items are for incineration.

- Correct records must be maintained in GRB for all categories of garbage as incinerated.

Note: Ashes generated from plastic waste incineration to be landed as plastics.

- Any doubts on plastic material whether containing PVC, PCB or PS etc must be landed and not incinerated. In case of doubts Master to clarify with office.

- Should other garbage be dosed along with the Plastics, amount of garbage dosed in the Incinerator must be as per Makers instruction (i.e. qty, temperature, loading door, interlocks etc)

- Only Plastics mentioned in below table to be incinerated.

- PPE must always be donned.

- All measures as in “Poster 43” for safe incineration must be additionally followed.

- Besides compliance to all of the above, GP A2- Additional guidance on Plastic incineration, must be followed.

Note: Incinerator specific procedure shall be made by Chief Engineer for Plastics incineration and same to be reviewed and approved by TSI/TM.

Warning: Polyvinyl Chloride (PVC), Polychlorinated Biphenyl (PCB) & Polystyrene (PS) incineration is strictly prohibited.

# Cooking Oil

  - Disposal of used cooking oil overboard is strictly prohibited.

  - Transfer of used Cooking Oil in any tank in Engine Room such as waste oil / sludge / bilge

/Overflow tanks is prohibited.

  - Transfer of used Cooking Oil in any cargo tanks such as Slop tank, drip tray drain tank is prohibited.

  - Used cooking Oil shall be collected in dedicated drums colour coded in YELLOW and stored in protected space inside accommodation (preferably galley in dedicated corner) and accounted for collection.

  - Used cooking oil in small quantities shall be soaked in cotton condemned rags and then be incinerated together with other shipboard waste which is permitted for incineration. Crew handling such must be under guidance of Chief Officer and 2E.

  - Make entry in Garbage Record Book (GRB –Part I) under category D for incineration of cooking oil.

  - All other garbage incinerated together with used cooking oil shall be recorded separately which includes rags (Eg Rags will be Cat-F)

  - If not Incinerated, used cooking Oil shall be landed ashore in the circumstances where shipboard incinerator has malfunctioned. Receipt shall be obtained for disposal ashore.

  - Incinerator use as per Makers instruction & follow safe incineration procedure

    - Food wastes

Food waste must be landed to shore facilities, if collection is in port stay or anchorage or stay in Annex V Special area. Any food waste generated during sailing can be disposed in open sea in accordance with MARPOL Annex V Reg 6 when vessel is more than 12 N-miles from base line of nearest port.

      - The criteria for food waste disposal at Sea is as follows:

  - The vessel is En-Route with min 4-6 Knots speed and not drifting/ circling

  - The vessel is at least 12 N-miles off nearest base line of nearest port. If vessel is in Annex- V special area, the Food waste must be comminuted and discharged > than 12 N-miles and that the comminuted food waste must pass through a screen with mesh no larger than 25 mm.

      - In circumstances due to vessel’s trade pattern & operational requirements leading to high food waste accumulation due to extended anchorage or any other reason, one or all following options are to be explored by Master:

  - Vessels to preferably have a dedicated Deep Freezer for storing food-wastes. Should there be no such deep freezer, Master to liaise with VM/FG to procure same. Stored food-wastes in Freezer to be subsequently disposed after sailing out in open seas and/ or landed.

Note: Food-wastes must never be stored in Provision Fridge rooms (Meat/Fish room etc).

  - Should food waste accumulation exceed available safe storage means, Master to communicate with Agents and TSI for landing arrangements ashore. (TSI/TM to Communicate with operators/ owners on plan for such landings).

  - One of the options for food waste disposal is to approach Flag for approval on Route exemption criteria (i.e. vessel in anchorage or drifting) under Marpol Annex V Reg 7.2.1. Master to forward office (TSI/TM ) on health & hygiene concerns due to accumulated food wastes with photos for request to flag and await approval. (Ref sec 16 below).

Note: Permission for incineration provided by Port state and office will be valid for that anchorage stay only. Ref to Poster GP 06 for prohibited items, “Poster 43”, Sec 9.1 & subsections for guidance on safe incineration and Makers Instruction.

For use of incinerator in Port (for sludge and/ or garbage incineration including food wastes), Master to approach port state and office (TSI/TM) for approval for incinerator use.

Remember: Absence of documented guidance doesn’t mean approval. In case of any doubt, Master to contact office.

# Prohibited items

As per MARPOL and company requirements, following items are prohibited from incineration.

# ** Note: Ref Sec 9.1.1 and GP A2.

    - Expired medicines

These are stored as domestic wastes category.

  - Primary means of disposal of medicines (noncontrolled) and medical equipment

    - returned to the supplier where possible; or

    - sent to an approved shore-side contractor for disposal.

  - If disposal for above is not possible, expired medicines (noncontrolled) may be incinerated only at Open sea, subject to all of below:

    - Approval taken from TD .

    - Incinerator Maker confirms capability of handling expired medicines.

    - As far as practicable, there should be evidence of Port refusal to accept expired medicines.

    - Incinerator operated as per Makers Instructions with all alarms and trips functional and safety interlocks in place (ref Poster 43 for safe incineration)

Caution:For Incineration of expired narcotics (Some ports may refuse to accept), Incinerator Makers to confirm capability to incinerate and specific Flag state instructions for such to be followed and recorded in log/ GRB. (Normally 2 officers are required to be a part of unpacking, incineration and logging in GRB)

Records of such incinerated medicines shall enter in GRB Part 1 under code C and copy of such entry be kept as part of the medicine register.

# COVID-19 Disposable Masks/ Protective kits

The management of garbage generated by COVID-19 related masks/ protective kits are to be done in following manner:

  - These are classified as domestic wastes and to be managed likewise. These COVID-19 related masks/ PPE must be stored in separate bins with a sealed cover and marked distinctly “COVID-19 related masks/PPE”.

  - The collected used COVID-19 masks and/or related PPE can be sealed in BIO- Hazard bags and landed ashore. Appropriate GRB entry to be made and a comment made in “Remarks Column”

  - The Collected used COVID-19 Masks and/ or related PPE can be incinerated onboard and recorded likewise in GRB with a comment in “remarks column”

10

DISCHARGE TO SEA / PORT / OTHER SHIP

## General

MARPOL Annex V and poster GP 02 shall always be complied with.

Warning: If local regulations are more stringent, then local regulations shall be complied with.

Caution: Although discharge into the sea of limited types of garbage is permitted under MARPOL Annex V and Polar Code, discharge of garbage to port reception facilities should be primary means of disposal.

To ensure timely transfer of large quantities of ship-generated garbage to port reception facilities, it is essential to decide well in advance for garbage reception. At the same time, discharge needs should be identified in order to decide for garbage requiring special handling or other necessary arrangements. Advice should be provided to the port of the type of garbage to be discharged and whether it is separated and the estimated amounts.

The port may have special discharge requirements for food wastes and related garbage which may carry certain disease or pest organisms, dunnage, batteries, medicines, outdated pyrotechnics or unusually large, heavy, or odorous derelict fishing gear, etc.

Poster GP02 provides guidance on statutory requirements for disposal of garbage and is required to be displayed in common garbage collection locations.

## Following notes particularly apply for discharge criteria:

  - When garbage is mixed with or contaminated by other harmful substances prohibited from discharge or having different discharge requirements, the more stringent requirements shall apply.

  - Comminuted or ground food wastes must be able to pass through a screen with mesh no larger than 25 mm.

  - The discharge of introduced avian products in the Antarctic area is not permitted unless incinerated, autoclaved or otherwise treated to be made sterile.

  - Offshore platforms located 12 nm from nearest land and associated ships include all fixed or floating platforms engaged in exploration or exploitation or associated processing of seabed mineral resources, and all ships alongside or within 500 m of such platforms.

  - Cargo residues mean only those cargo residues that cannot be recovered using commonly available methods for unloading.

  - Cargo residues, cleaning agents or additives, contained in hold washing water must NOT be Harmful to the Marine Environment (HME) as established based on criteria in Appendix 1 of MARPOL Annex V and declared by the shipper.

A cleaning agent or additive is considered not harmful to the marine environment if it:

  - Is not a "harmful substance" in accordance with the criteria in United Nations Globally Harmonized System of Classification and Labelling of Chemicals (GHS) MARPOL Annex V and Annex III; and

  - It does not contain any components which are known to be carcinogenic, mutagenic or reprotoxic (CMR).

## Port reception facilities

It should be noted that, due to differences in port reception procedures and additional treatment among ports, port reception facilities may require the separation on board of:

  - Food wastes;

  - Cooking oil (animal derived products and by-products because of risk of animal diseases);

  - Plastics;

  - Domestic waste, operational waste and recyclable or reusable material;

  - Special items like medical waste, outdated pyrotechnics and fumigation remnants;

  - Animal wastes, including used bedding from the transport of live animals (due to risk of disease) but excluding drainage from spaces containing living animals; and

  - Cargo residues.

  - E-waste such as electronic cards, gadgets, equipment, computers, printer cartridges, etc.

Vessels should inform their flag State of ports in foreign countries Party to Annex V which do not have adequate port reception facilities for garbage. This can provide a basis for advising responsible governments of possible problems and calling the Organization's attention to possible issues of compliance. A specimen reporting format is reproduced in MEPC Circulars published by IMO on regular basis along with the procedure for submitting and handling such reports. Masters can be provided a copy of the reporting format upon request to MTSI incharge.

MANAGEMENT OF CARGO RESIDUES OF SOLID BULK CARGOES

11

Cargo residues are included in the definition of garbage within the meaning of regulation 1.9 of MARPOL Annex V and may be discharged in accordance with regulations 4.1.3 and 6.1.2 and paragraph 5.2.1.5 of part II-A of the Polar Code. However, cargo material contained in the cargo hold bilge water should not be treated as cargo residues if the cargo material is not harmful to the marine environment and the bilge water is discharged from a loaded hold through the ship's fixed piping bilge drainage system.

Cargo residues that are harmful to the marine environment may require special handling not normally provided by reception facilities. Ports and terminals receiving such cargoes should have adequate reception facilities for all relevant residues, including when contained in wash water.

Solid bulk cargoes, as defined in regulation VI/1-1.2 of the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, other than grain, shall be classified in accordance with appendix I of MARPOL Annex V, and declared by the shipper as to whether or not they are harmful to the marine environment. For ships engaged on international voyages, such a declaration should be included in the information required in section 4.2.3 of the IMSBC Code.

Masters, in coordination with terminal personnel should consider cargo loading, unloading and onboard handling practices in order to minimize production of cargo residues. Cargo residues

are created through inefficiencies in loading, unloading, on-board handling. Options that should be considered to decrease the amount of such garbage include the following:

  - 1Ensuring ships are suitable to carry the intended cargo and suitable for unloading the same cargo using conventional unloading methods;

  - 2Unloading cargo as efficiently as possible, utilizing all appropriate safety precautions to prevent injury or ship and equipment damage and to avoid or minimize cargo residues;

  - Minimizing spillage of the cargo during transfer operations by carefully controlling cargo transfer operations, both on board and from dockside. This should include effective measures to enable immediate communications between relevant ship and shore-based personnel during the transfer operations and when feasible, enclosure of conveyance devices such as conveyor belts. Since this spillage typically occurs in port, it should be completely cleaned up immediately following the loading and unloading event and handled as cargo; delivering it into the intended cargo space or into the appropriate unloading holding area.

When the master, based on the information received from the relevant port authorities, determines that there are no adequate reception facilities at either the port of departure or the port of destination in the case where both ports are situated within the same special area or Arctic waters, the condition under regulation 6.1.2.5 of MARPOL Annex V or paragraph 5.2.1.5 of part II-A of the Polar Code should be considered satisfied.

MARPOL Annex V, regulation 6.1.2, also applies when the "port of departure" and the "next port of destination" are the same port. To discharge cargo hold wash water in this situation, the ship must be En-route and the discharge must take place not less than 12 nm from the nearest land.

12

WASTE MINIMIZATION/REDUCTION

Note: Masters, Garbage Management Officer (CO) and crew to make all endeavours to control and reduce garbage generation onboard to a minimum.

## Minimization strategies

All vessels shall consider recycling or reusing garbage by adopting any one of the methods:

    - Landing your waste to a recycling facility via an agent or a voluntary port service.

    - Return the waste or part to the original manufacturer or his / her representative.

## Reuse on board

Some practical ways of reusing on-board:

    - Waste copy paper on-board shall be separated and re-used for printing of draft documents so that both sides are used.

    - Waste papers can be reused on board as scratch paper and sometimes cut into smaller sizes for quick writing notes on the chart table.

    - Packaging material can be sometimes reused to off land instruments or equipment for service.

    - Glass jars and Ice cream containers can be washed and reused for storing.

    - Pallets can be reused as dunnage to prevent metal to metal contacts where required.

    - Stock reusable items instead of disposable items. (E.g., cups, utensils, towels, etc.).

    - Re-use cleaning items (rags, etc.).

## Reduce if possible.

    - Do not print emails if not essential.

    - Print both sides of paper prior discarding.

    - Batteries

    - Cartridges

    - Spares parts

    - Return plastic packing/ clingwrap where possible with the vendors.

    - Reduce plastic volume by installing/using compactors.

    - Reduce SUP bottles (Single use plastics) by installing water purifiers and using Company’s metal water flasks for use.

    - Recommend changeover to/ Use energy saving bulbs/lights instead of CFL’s.

    - Avoid Food wastage which leads to food waste garbage generation.

RECORD KEEPING

13

All vessels shall maintain Garbage Record Book Part I to record activities for following categories of garbage:

- Plastics

- Food wastes

- Domestic wastes

- Cooking oil

- Incinerator ashes

- Operational wastes

- Animal carcass

- Fishing gear

- E-waste

Additionally, vessels carrying solid bulk cargo shall maintain Garbage Record Book Part II

to record activities for following categories of cargo residues

- Cargo residues (non-Harmful to the Marine Environment)

- Cargo residues (Harmful to the Marine Environment)

The Garbage disposal entry must be also made in Data collection system Garbage Log and any shore landing receipts uploaded in the Data collection system platform.

Entries in the Garbage Record Book shall be made on each of the following occasions:

## When garbage is discharged to a reception facility ashore or to other ships:

    - Date and time of discharge

    - Port or facility, or name of ship

    - Categories of garbage discharged

    - Estimated amount discharged for each category in cubic meters

    - Signature of officer in charge of the operation.

## When garbage is incinerated (Additionally Refer Sec 9.1):

    - Date and time of start and stop of incineration

    - Position of the ship (latitude and longitude) at the start and stop of incineration

    - Categories of garbage incinerated.

    - Estimated amount incinerated in cubic metres

    - Signature of the officer in charge of the operation.

## When garbage is discharged into the sea in accordance with regulations 4, 5 or 6 of MARPOL Annex V or chapter 5 of part II-A of the Polar Code

    - Date and time of discharge

Note: Additionally for Garbage record book Part II - For cargo residue discharges, include discharge start and stop positions.

Position of the ship (latitude and longitude).

    - Category of garbage discharged

    - Estimated amount discharged for each category in cubic metres

    - Signature of the officer in charge of the operation

Accidental or other exceptional discharges or loss of garbage into the sea, including in accordance with regulation 7 of Annex V of MARPOL: Every such instance of accidental garbage discharge to sea must be reported to TSI/MTSI within 24 Hrs of the incident and due guidance taken for GRB entry, Flag notification.

    - Date and time of occurrence

    - Port or position of the ship a time of occurrence (latitude, longitude and water depth if known)

    - Categories of garbage discharged or lost

    - Estimated amounts for each category in cubic metres

Note: Chief Officer must ensure to obtain official receipts or certificates from the operator of the reception facilities, which includes barges and trucks, specifying the estimated amount and category of garbage landed.

The receipts or certificates must be kept together with the Garbage Record Books Part I or Part II as applicable.

Period of retention is 2 years from the last date of entry in garbage record book.

The reason for the discharge or loss and general remarks.

Master must ensure to request office for supply of new garbage record book in timely manner.

## Amount of garbage

The amount of garbage on board should be estimated in cubic meters, separately according to category. The Garbage Record Book contains many references to estimated amount of garbage. It is recognized that the accuracy of estimating amounts of garbage is left to interpretation.

Volume estimates will differ before and after processing. Some processing procedures may not allow for a usable estimate of volume, e.g., the continuous processing of food waste. Such factors should be taken into consideration when making and interpreting entries made in a record

14

SPECIAL ITEMS

## Asbestos

    - Any installation of materials that contain asbestos, is prohibited, for all ships. If any discover is made of asbestos, the TSI/MTSI/TD must be contacted for due process to follow up including informing Flag on same.

## Batteries

    - Store used batteries in receptacle dedicated for “E-Wastes” for disposal at Port Reception facilities.

Warning: Batteries are explosive and must never be incinerated

## Chemicals (Used/ Expired/ Contaminated)

    - Cleaning chemicals in engine room and deck, which have been used for various cleaning & maintenance activities on-board ship, undergoes dilution & mixing with other solvents (Lube oil / Fuel Oil / Water). In these process, chemical properties are expended thus rendering used chemicals to be safe for safely transfer into a waste oil tank for evaporation OR collected in plastic drums-sealed for shore landing. Following to be done wrt used chemicals Or Expired chemicals:

      - Month end log and inventory of chemicals shall be updated to reflect consumed chemicals.

      - Empty Chemical drums to be landed ashore as garbage.

      - Expired Or contaminated cleaning chemicals are to be Stored in sealed drums for disposal to port facilities

      - Used chemicals can be safely transferred to waste oil tank for evaporation OR landing

      - Chemicals must never be mixed and stored in dedicated locker with MSDS in vicinity. Where there are rags or material contaminated with these chemicals:

    - Wash, dry and recycle if possible, Or

    - Burn if not recyclable or soaked in chemicals and only after a wash to get rid of residual chemicals, Or

    - Store for disposal at port facilities if unsafe to burn.

Caution: Expired, Unused or contaminated chemicals should never be incinerated and should be landed to shore reception facilities.

## Hazardous wastes/ materials

    - Operational waste hazardous in nature such as aerosol cans, smoke detectors, Medical wastes; expired pyrotechnics, expired-Used chemicals (as applicable) etc are to be stored in separate receptacle of black colour marked in white as “Hazardous wastes”. The receptacle shall be stored at locations where such hazardous waste is expected to be generated such as:

      - Engine room for collecting Smoke/Fire detectors and similar

      - Hospitals for Medicinal wastes (Hazardous)

      - Bridge for Expired pyrotechnics) and similar.

      - Expired Narcotics should be segregated and kept under the charge of Master for disposal to shore or as per the guidance issued by the vessels Flag.

** These Hazardous wastes must be stored on-board and disposed only to port reception facilities.

Medicines and Medicine Containers: Store all medicines and its packaging for disposal to port facilities. Log down any narcotic drug disposal in the Official Logbook or Medical Log. No Incineration of Expired Medicines/ Narcotics to eb done without Office approval. Note: Regarding Covid-19 related masks, PPE, refer to sec 9.1.5

## E-wastes

Wastes generated on-board such as Computer, printers, batteries, CD-DVD, fluorescent lights, bulbs, PCB, Ink cartridges etc are to be stored in E-waste receptacles. The receptacle to be white colour and stencilled in red as “E-wastes”. This should be stored on-board and disposed only to Port reception facility. E-Wastes should never be Incinerated.

## Oil impregnated waste (Oily rags)

    - Store oil impregnated waste carefully and only for short periods as the waste can be self- heating and self-igniting.

    - Burn oil impregnated waste (e.g. rags, lining, etc.), which are capable of being burnt, in the incinerator.

    - Store oil impregnated waste (e.g. scale, etc.) for disposal at port facility.

## Operational Wastes such as Under Piston space dry sludge/ Waste Oil Tank/Incinerator tank Dry sludge after cleaning

    - All Dry sludge collected from Under piston space can be either Landed ashore or Incinerated. In either case to be recorded as “Operational waste-Category F”

    - Any dry sludge (Un-Pumpable) which is collected during cleaning of the Waste Oil Incinerators service/ Sett tanks or any of Oily waste tanks must be disposed as follows:

      - If the dry sludge collected in drums, an entry under “I” must be made in ORB part-1 stating xxM3 of dry sludge collected during incinerator tank cleaning

      - Dry sludge if incinerated, such must be entered only in GRB under Category F- Operational waste

      - If the dry sludge is landed, such must be entered as landing in GRB under “F”

TRAINING, EDUCATION AND INFORMATION

15

Chief Officer is responsible for Training & familiarisation of Shipboard personnel to facilitate compliance to the Garbage management Plan.

This can be achieved by conducting frequent and regular training sessions such as (e.g. placards, posters, brochures, etc.), photographs, DVDs, audio and video tapes, and films as well as from synopses of training programmes, seminars and formal curricula.

The placards and posters wrt Garbage Management Plan must be placed in prominent places where crew will be working and living, and in areas where bins are placed for collection of garbage. These places include the engine-room, mess rooms, smoke rooms, laundries, crew cabins, CCR, ECR, BCR, galley, and other living or working spaces). All crew must be made familiar with Posters GP01/ GP02 for correct Garbage segregation, storage, bin colours and appropriate disposals.

Technical information on ship specific garbage management methods such as minimization, recovery, recycling, reuse, incineration, compaction, separation, sorting and sanitation system, packaging and provisioning methods etc to be made available to all crew in easily accessible manner.

Office shall include various case studies and industrial feedback in HSSEQ campaigns and lessons learnt shall be shared with all crew for effective implementation.

EXCEPTIONS

16

Ref MEPC.201(62) Annex V Reg 7.1, Regulations 3 (General prohibition on discharge of garbage into the sea), 4 (Discharge of garbage outside special areas), 5 (Special requirements for discharge of garbage from fixed or floating platforms) and 6 (Discharge of garbage within special areas) of MARPOL Annex V shall not apply to:

    - The discharge of garbage from a ship necessary for the purpose of securing the safety of a ship and those on board or saving life at sea; or

    - The accidental loss of garbage resulting from damage to a ship or its equipment, provided that all reasonable precautions have been taken before and after the occurrence of the damage, to prevent or minimize the accidental loss; or

    - The accidental loss of fishing gear from a ship provided that all reasonable precautions have been taken to prevent such loss; or

    - The discharge of fishing gear from a ship for the protection of the marine environment or for the safety of that ship or its crew.

** Ref MEPC.201(62) Annex V Reg 7.2.1 states on Exception of En-Route The en route requirements of regulations 4 and 6 shall not apply to the discharge of food wastes where it is clear the retention on board of these food wastes presents an imminenthealth risk to the people on board. For such cases of exception, TSI/MTSI AND Flag approval must be taken before exercising such exception.

REFERENCED FORMS AND CHECKLISTS

17

Poster GP02 - Statutory Requirements for Disposal of Garbage Poster GP01 - Segregation and Management of Garbage onboard

Poster GP03- Discharge of Garbage Restricted Poster GP04 - Discharge of Garbage Restricted Poster GP05 - Garbage Management Plan

Poster GP06 - Materials Prohibited for Incineration