Mooring System Management Plan

Contents

INTRODUCTION	2

PART A - GENERAL SHIP'S DETAILS	4

A1 - SHIP DETAILS	4

A2 – OWNERS / OPERATORS	5

A3 – RECORD OF CHANGES	5

PART B - MOORING EQUIPMENT DESIGN PHILOSOPHY	6

B1 - DESIGN CONSIDERATIONS	6

MOORING ARRANGEMENT PLAN	7

B2 - MOORING FORCE CALCULATION	8

STANDARD ENVIRONMENTAL CRITERIA	8

B3 - MOORING RESTRAINT CALCULATIONS	8

B4 - DESIGN LOADS	8

B5 - STANDARD MOORING PATTERN ASSUMPTIONS & CONSIDERATIONS FOR REDUNDANCY	9

B6 - INITIAL MOORING LINE SELECTION	10

B7 - SERVICE LIFE EXPECTATIONS	12

B8 - ALTERNATE MOORING PATTERNS	14

B9 - LIMITATIONS AND EXCLUSIONS	14

EQUIPMENT AND FITTING LINE-UP WITH OPERATIONAL CONSIDERATIONS:	15

PART C - DETAILED LIST OF MOORING EQUIPMENT	17

PART D - INSPECTION, MAINTENANCE AND RETIREMENT STRATEGIES / PRINCIPLES	18

PART E - RISK AND CHANGE MANAGEMENT, SAFETY OF PERSONNEL AND HUMAN FACTORS	19

PART F - RECORDS AND DOCUMENTATION	22

PART G - MOORING SYSTEM MANAGEMENT PLAN REGISTER	23

APPENDIX I – MOORING ARRANGEMENT PLAN	24

APPENDIX II – CALCULATIONS	24

# INTRODUCTION

Applies to:

- All Vessels

Requirement

VIQ 9.2: Each ship should be provided with a Mooring System Management Plan (MEG 1.9).

This plan ensures that the ship’s mooring system is properly designed and effectively operated. This is based on a range of fundamental mooring principles.

Many mooring system components may be critical to the ship therefore it is recommended that the necessary information is readily available and kept as part of the ships management system. This will ensure that crew members have a complete understanding of the mooring system.

Purpose

This plan is to help operators ensure the mooring system is:

- Inspected

- Maintained and

- Operated in accordance with the original design basis.

The information in the plan is available to anyone on the ship that needs to review it.

This plan uses a goal-based approach, typically used by the IMO, to give a consistent approach and methodology. Through this process, a series of goals and functional requirements are provided that will enable ship operators to align with the process.

Objective

The objective for the MSMP is to ensure that all assessed risks are effectively managed through the mooring system design and operation. It aims to ensure that, during mooring operations, no harm comes to ship or terminal colleagues, the ship or the terminal / facility. It also aims to ensure that the mooring system meets applicable regulations, codes and recommended practice.

All stakeholders should ensure that the MSMP is appropriately created. Ship operators and ship builders should collaborate from the earliest stage in the ship’s design to ensure risks are reduced through mooring design and risk reviews.

A structure for an MSMP is proposed in this section and contains details of items that may be ship or operator specific (e.g. parts of the operator’s SMS) and guidance on items that should be retained in a Mooring System Management Plan Register (MSMPR) that stays with the ship throughout its life cycle.

Structure

The MSMP information will be outlined in individual parts that each cover a key component of the ship's mooring system.

Each part will establish:

- The overall goal for that section.

- Functional requirements to achieve the goal.

- Where detailed information may be found to provide additional guidance or clarify to assist in achieving the goal.

Collectively the parts will achieve the objective of the MSMP through establishing;

- Records of mooring equipment, including all permanent and loose items and its 'as built' design basis, to be retained by the ship throughout its life cycle.

- Information required to be maintained as an up-to-date record of the ship's mooring equipment technical status.

- Operating procedures and risk assessments that ensure the safety and occupational health and well being of the ship's colleagues when using mooring equipment or operating in the mooring workspace.

It is intended to be a complementary component of the SMS, which facilitates updates. Internal and external compliance verifications, induction and training, communication with manufacturers and transition to a new ship operator. The contents of the MSMP may include information in both electronic and hard copy.

The format of the MSMP may therefore vary from a complete standalone approach to an integrated system that uses links and/or references to existing systems.

However, in all cases it should be easy for the mooring system users to access the MSMP information, whether that be a physical location (e.g. documented manual) or a virtual location such as an online library.

# PART A - GENERAL SHIP'S DETAILS

Purpose:

To maintain a detailed, continuous, and up-to-date record of the ship's ownership history.

## A1 - SHIP DETAILS

| Ship’s Name |  |
| --- | --- |
| IMO Number |  |
| Call Sign |  |
| INMARSAT No. |  |
| Ship Type |  |
| Port of Registry |  |
| Flag / Class |  |
| Deadweight |  |
| Maximum Draught |  |
| Date of Delivery |  |
| Shipyard Name / Location |  |
| Ship Design MBL ( tonnes)* |  |

*Ship Design MBL.   The minimum breaking load of new, dry mooring lines for which the ship’s mooring system is designed, to meet OCIMF standard environmental criteria restraint requirements.  The Ship Design MBL is the core parameter against which all the other components of a ship’s mooring system are sized and designed with defined tolerances.

## A2 – OWNERS / OPERATORS

|  | Name, Address, Contact Details ** | Date From ** |
| --- | --- | --- |
| Owner |  |  |
| Commercial Operator * |  |  |
| Bareboat Owners * |  |  |
| ISM Manager |  |  |
| Other * |  |  |

*If Applicable

**If any changes, complete a new page.   Record of all changes in section A3

## A3 – RECORD OF CHANGES

| Date | Section | Change | Change made by |
| --- | --- | --- | --- |
| dd/mm/yyyy | A2 | Example: ISM Manager change from V.Ships Asia to V.Ships India | Name / Rank / Date |

## PART B - MOORING EQUIPMENT DESIGN PHILOSOPHY

## B1 - DESIGN CONSIDERATIONS

To achieve the ship's optimal mooring pattern including information on the siting of mooring winches and fairleads to provide for direct leads and minimal lines across open decks.

### MOORING ARRANGEMENT PLAN

Keep a copy of the Mooring Arrangement Plan here.

## B2 - MOORING FORCE CALCULATION

Mooring force calculations against the standard environmental criteria

The standard environmental criteria is in MEG 3.2.2

A ship at a jetty , or sea island, is held at the berth by a combination of mooring lines and breasting dolphins that resist the environmental forces applied to the ship. There are multiple mooring lines that connect from several locations on the ship to several mooring points on the jetty or sea island. The ship’s mooring lines and the breasting dolphins together may be considered as a complete mooring system, such that when a force is applied to the ship, the ship will move a small but determinable amount. By determining the amount of movement of the ship, the forces in the mooring lines and breasting dolphins can be calculated using the principle of static equilibrium. However, it may not be assumed that the system geometry remains fixed under the application of forces and the solution must also account for any elongation of each mooring line due to stiffness of the components.

### STANDARD ENVIRONMENTAL CRITERIA

60 knot wind (defined below) from any direction simultaneously with:

- 3 knots current at 0 degrees or 180 degrees

or

- 2 knots current at 10 degrees of 170 degrees

or

- 0.75 knots form the direction of maximum beam loading

## B3 - MOORING RESTRAINT CALCULATIONS

Once the environmental forces acting on the ship are known, the strength and number of mooring lines required to balance them, can be calculated.

To calculate mooring restraint, the following must be used:

- The three-dimensional co-ordinates of all ship and terminal mooring points (know or assumed)

- The stiffness of the mooring lines and compression of fenders. When calculating mooring line stiffness, the full length of line from winch-to-shore bollard must be used.

- The structural characteristics of mooring and breasting dolphins may need to be accounted for if these structures are flexible.

Computer simulations / calculations are the most effective way to test the mooring system of an existing or planned ship at a terminal known to have unusual environmental conditions or mooring layout.

## B4 - DESIGN LOADS

Design loads, safety factors and strength for required mooring lines and fixed equipment.

Design Load: The design load on a fitting given by multiplying the mooring ship design Minimum Breaking Load (MBL) by the Geometric Factors (GF).

- The ship design MBL is calculated as described in MEG (4) Section 3. The mooring system design is based on this value.

- When the ship is moored, the mooring lines are held on the primary brake of the mooring winch with the drum disconnected form the drive and gear box.

- In the event of mooring loads increasing due to environmental or other factors, the mooring brake is designed to render at 60% of the ship design MBL to avoid the mooring line breaking and, ultimately, damage to the fitting and hull structure.

- Should the ship design MBL be reached, for whatever reason, the mooring line should fail before structural damage takes place as line failure is generally seen as a less catastrophic event than a structural failure of the hull structure or fittings.

Geometric Factor

The actual load on the fitting form the mooring line is a function of the geometry of the contact between the line and fitting. The Geometric Factor (GF) can be directly related to the angle through which the mooring line is deflected in its passage through or over the fitting. If the wrap angle is defined as Ø, then the theoretical GP is

GF = 2sin(Ø/2)

The product of the hsip design MBL and the GF gives the resulting load acting on the fitting and on which the fitting design is based. This is described here as the Design Basis Load (DBL).

IACS requirements and industry standards take account of the GF during the design of the bollard.

## B5 - STANDARD MOORING PATTERN ASSUMPTIONS & CONSIDERATIONS FOR REDUNDANCY

Assumptions on the standard mooring pattern and considerations for redundancy provisions, including sub-optimal line distribution to cover unpredicted events (e.g. storm surges, shore mooring hooks out of service).

Safety factors generally account for uncertainties such as additional dynamic loads, normal wear or corrosion of fittings or equipment, small material or welding defects, locked-in weld stress and for the uncertainties in the design calculation model used. However, some industry standards include margins for normal wear and corrosion. Additional allowance in a general safety factor will add to the safety margin. Winch brake rendering (slippage) should not be relied on to instigate a reduction in the DBL to a value below GFx ship design MBL.

Brake rendering at specific loads cannot be guaranteed as brake settings are not precise and winches may inadvertently be left with gears engaged. Brake rending is an additional safety mechanism to protect the integrity of the mooring system against unexpected failure when the ship is moored at a terminal or to another ship.

The capacity of the foundations and supporting deck structure to any fitting must be considered when rating the capacity of any fitting. The strength of the supporting structure and the connection of the fitting to it should be equal to, or greater than, the marked SWL of the fitting itself, so that any fitting failure does not result in damage to the hull structure.

Mooring Pattern

The term mooring pattern refers to the geometric arrangement of mooring lines between the ship and the berth. Industry has previously standardized on the concept of a generic mooring pattern taking into account standard environmental criteria. The generic mooring pattern is mainly applicable to a multi-directional environment and to the design of a ship's mooring system.

For ships regularly trading to terminals with a directional environment, a site- specific pattern (such as one including head and stern lines and/or extra breast and spring lines) may be more effective. Consideration may be given to the provision of additional or higher capacity mooring equipment.

The most efficient line lead for resisting any given environmental load is a line orientated in the same direction of the load. This would imply that, theoretically, mooring lines should all be orientated in the direction of the environmental forces and be attached at such a longitudinal location on the ship that the resultant load and restraint act through one location. Such a system would, however, be impractical since it has no flexibility to accommodate the different environmental load directions and mooring point locations encountered at various terminals.

For general applications, the mooring pattern must be able to cope with environmental forces from any direction. This can best be approached by splitting these forces into a longitudinal and a transverse component and then calculating how to most effectively resist them. It follows that some lines should be in a longitudinal direction (spring lines) and some lines in a transverse direction (breast lines). This is the guiding principle for an effective mooring pattern for general application, although the locations of the actual fittings at the terminal will not always allow it to be put into practice.

## B6 - INITIAL MOORING LINE SELECTION

Initial mooring line selection inputs, including assumptions, methodology and supplier(s) and information on the agreed rope commissioning process.

Example:

The terms ship design MBL, Line Design Break Force (LDBF), Working Load Limit (WLL) and Design Basis Load (DBL) are used throughout in MEG4 and are defined below. Using these definitions will help to ensure a clear and consistent link between tested and rated line performance, operational loads and calculated safety margins.

Strength requirements are established during the line type selection process at the ship design stage.

Ship design Minimum Breaking Load (ship design MBL)

The MBL of new, dry mooring lines for which a ship's mooring system is designed, which meets standard environmental criteria restraint requirements. The ship design MBL is the core parameter against which all the other components of a ship's mooring system are sized and designed, with defined tolerances.

Line Design Break Force (LDBF)

The minimum force at which a new, dry, spliced mooring line will break when tested according to appendix B of MEG4. This is for all mooring line and tail materials except those manufactured from nylon, which is tested wet and spliced. This value is declared by the manufacturer on each line's mooring line certificate (see appendix B of MEG4) and is stated on a manufacturer's line data sheet. As outlined in appendix B, when selecting lines, the LDBF of a line shall be 100%-105% of the ship design MBL.

The LDBF for nylon (polyamide) mooring lines should be specified as break tested wet, because nylon lines change strength characteristics once exposed to water and generally do not fully dry to their original construction state.

Working Load Limit (WLL)

The maximum load that a mooring line should be subjected to in operational service, calculated from the standard environmental criteria. The WLL is expressed as a percentage of ship design MBL and should be used as a limiting value in both ship design and operational mooring analyses. During operation, the WLL should not be exceeded.

In the same way that SWL is a limit for fixed equipment, the WLL value is used as a limit with the standard environmental criteria and mooring layout when designing mooring systems in establishing mooring system designs. Steel wire ropes have a WLL of 55% of the ship design MBL and all other cordage (synthetic) has a WLL of 22% of the ship design MBL.

## B7 - SERVICE LIFE EXPECTATIONS

Determines the mooring line service life expectancy including supporting evidence used to determine criterion. This can be achieved through various means;

- OEM guidance and recommendations ( )

- References form other operating companies on similar trades ( )

- Empirical data from similar fleets ( )

- Historical data related ot that type of rope, ship and service etc

Initial mooring line selection inputs, including assumptions, methodology and supplier(s) and information on the agreed rope commissioning process.

General

Steel wire ropes are constructed of steel wire strands that are twisted (or laid) into rope form. Products can be made and finished with slightly different processes to improve line quality and performance. Steel wire ropes are used on a wide variety of ship types and sizes.

Steel wire ropes have a high strength-to-diameter ratio and excellent resistance to abrasion and other forms of mechanical damage. Steel wire ropes require routine maintenance to prevent corrosion and are significantly heavier than synthetic lines, which can introduce handling challenges for ship personnel.

Lines made with Independent Wire Rope Core (IWRC) are strongly recommended over fibre core steel wire ropes. An IWRC steel wire rope has much greater resistance to crushing, higher LDBF for a given diameter and greater strength retention when bent.

Material

High quality, high tensile, drawn galvanized wire is recommended for wire mooring lines. Other steel materials and processing methods are used in the manufacture of steel wires, but are not included in this section. If using other materials, consideration should be given to intended use, compatibility with interfacing equipment and maintenance.

Other steel materials and processing methods may include high carbon steel, ungalvanized steel, stainless steel, aluminum/zinc coatings and hot dip galvanizing.

Factors influencing performance

The key factors influencing performance of steel wire mooring lines are strength, bend fatigue, tensile fatigue, crushing and deformation, corrosion and abrasion, and temperature. These factors are explained in more detail below, with guidance on how to address each issue provided where feasible.

Strength

Tensile strength of new steel wire mooring lines (i.e. LDBF) should be specified in accordance with the design guidance in section one. It is recommended to use wire with minimum tensile strengths ranging from 1,770 to 2,160N/mm2. Pre- formed, galvanized drawn wire with these tensile grades is now widely available and is recommended in place of ungalvanized wire grades.

Bend fatigue and D/d ratio

As with all rope, steel wire will lose strength when bent over a radius. This is a major factor in the design of shipboard equipment for steel wire rope since items such as winch drums and fairleads should have an adequate diameter or surface radius. The recommended minimum values listed in section 5.2.3 are based on lines using the recommended IWRC. A fiber core rope will lose more strength at a given bend ratio than an IWRC rope. It is therefore recommended that fiber cored steel wire ropes are not used.

A minimum bend ratio (D/d) of 15 is recommended.

Fatigue and abrasion

Fatigue in wire mooring lines is mainly caused by the wires being bent over a winch drum, sheaves or deck fairleads. A wire constructed with big outer wire strands, e.g. a 6 x 19 construction, will have less resistance to bending fatigue than a wire with smaller wire strands, e.g. a 6 x 36 construction.

Abrasion to wire can be caused when it runs over deck fittings or when it rubs against itself when spooled on and off the winch drum.

Usage and care

The following gives general guidance on steel wire rope usage and care in service. More detailed information can be found in EN 12385-3, ISO 4309 and in the Wire Rope Technical Board's Wire Rope User's Manual. Further advice should also be obtained from the line manufacturer.

Storage

A clean, well-ventilated, dry, under cover location such as a rope locker is preferred. If outdoor storage is unavoidable, the rope should be covered with waterproof material to protect it from the sun and weather. The rope should be stored in a location where it is not likely to be affected by salt water contact, steam, chemical fumes or other corrosive agents, and protected in such a manner that it will not be exposed to any accidental damage.

During long periods of storage, the reel or winch should be rotated periodically, particularly in warm environments, to prevent lubricant from leaking out of the mooring line and/or migrating to other areas of the mooring line. It should be ensured that the rope does not make any direct contact with the deck and that there is a flow of air under the reel. Consideration should be given to supporting a reel on a simple A-frame or cradle securely located in the rope locker or on deck.

Ropes in storage should be examined periodically and, when necessary, a suitable dressing that is compatible with the manufacturing lubricant should be applied. The rope supplier should be contacted for guidance on the type of dressings available and the methods of application. After the dressing has been applied, the rope should be re-wrapped unless otherwise specified by the manufacturer.

Steel wire ropes installed on winches should be covered with waterproof tarpaulins or other suitable coverings when not in use.

Dynamic loading

A dynamic load (often referred to as a shock load) is a sudden high load that is transferred into a line that has previously been under a low or static load. This can cause a peak load that is higher than the recommended load limit or, in extreme cases, the line's residual break load, which can cause the line to fail. Repeated dynamic loading can cause significant strength loss in the line. This can be difficult to detect in normal conditions and can lead the line to fail some time later, even when operating below the maximum load limits.

Dynamic loads exceeding the WLL should always be recorded and advice should be sought from the line manufacturer on whether they can continue to be used.

Crushing

Crushing damage to wire can be caused by high pressure on winches with more than one layer of line or by the wire being bent on deck fittings at high load. Wire should be installed on the winch drum with back tension according to the manufacturer's guidelines and should be uniformly spooled. It is recommended that IWRC ropes are used instead of fibre core ropes because they have a higher crush resistance.

Corrosion

Corrosion on steel wire mooring lines will increase the rate of fatigue of the line and should be avoided. More details on proper line maintenance and corrosion prevention is provided in section 5.5.6.3.

Temperature

For most applications, steel wires can usually be used in temperatures as low as -40°C and as high as +90°C, although wire lubricants and terminations such as sockets and ferrules may have limitations on use at extreme temperatures. Further advice should be obtained from the line manufacturer.

Specifications and procurement

There are currently no international standards for the specification of steel wire ropes specifically for use as ship mooring lines. 6 x 36 class wire is suitable for most mooring line applications. Purchasers should consult with suppliers to make sure the lines meet their required performance criteria and that testing is appropriate for the intended use. Appendix B provides a framework that may be used by operators during the line selection.

Steel wire rope products are covered by many national and international standards. The following standards concern steel wire ropes that use the recommended materials and constructions noted above.

## B8 - ALTERNATE MOORING PATTERNS

Alternate mooring patterns must meet the standard environmental criteria assessment and designed-in options when the optimal mooring pattern is unachievable in some real-world scenarios (e.g. hooks, dolphins or mooring winches out of service, breast lines not in an optimal perpendicular lead, etc.).

## B9 - LIMITATIONS AND EXCLUSIONS

Limitation and exclusions for initial mooring philosophy, and provisions for modifications.

It is not practical to design all ships for the worst possible operating environment, where the standard environmental criteria are exceeded, the ship should either leave the berth, get continuous tug assistance or arrange for additional mooring restraint.

Ships and berths are not normally designed for conditions above the standard environmental criteria. When conditions exceed the standard environmental criteria, additional shore moorings may be provided to augment the ship mooring system. To receive these moorings on board, it is recommended that ships are provided with enough closed fairleads, bitts and warping drums to serve 1.5 times the standard number of mooring lines. The additional closed fairleads will usually be located next to the fairleads for the standard mooring lines. This allows shore lines to be brought aboard using the same winches as the standard mooring lines. The strength of the additional fairleads and bitts should be based on the ship design MBL of the ship's standard mooring lines.

Some ships (such as those that are externally framed) may have the mooring winch and drums mounted at a higher level than the deck on which the fairleads and bitts are sited. This multi-level arrangement is not recommended because, should secondary mooring lines from shore be required, the line may pass over the top of the bitt with a geometry that will not allow an effective stopper to be used in the securing process.

However, if the winch must be mounted at such a height, additional higher-level bitts and fairleads should be fitted, to allow the mooring line to be held correctly at an appropriate position to stopper off, make fast and lead to shore effectively and safely. Horizontal side bars attached to the normal deck mounted bitts, or cruciform bitts should not be fitted or used. These attachments to the bitts are not covered by construction standards and the SWL cannot be known. There is a risk that mooring lines can slip off these fittings as the tension in the line changes.

The requirements set out for mooring arrangements and layouts may be combined during the design stage to reduce the complexity of the mooring arrangement. For example, the location of shipside fairleads may be slightly adjusted so that one fairlead or set of bitts can serve several requirements.

At the same time, all possible line leads for the various requirements should be considered without introducing potential problems from:

- Large changes of line angle.

- Inappropriate D/d ratios.

These factors may mean that additional separate fittings need to be provided, such as the installation of separate fittings for each mooring line type (steel wire rope or synthetic fiber). The strength of the fittings should be designed for the highest possible load that the fitting may experience.

## EQUIPMENT AND FITTING LINE-UP WITH OPERATIONAL CONSIDERATIONS:

Mooring lines should lead from winch drums directly to the shipside fairlead. If the use of pedestal fairleads cannot be avoided, the change of angle for the line should be minimal and the winch controls should be located to minimize snap- back risk to the operator.

Equally, mooring fittings need adequate clearance for safe operations. Winches have to be arranged to provide an adequate fleet angle for the drum. This is the maximum angle the mooring line deviates from a direction perpendicular to the drum axis and is necessary to allow each turn to sit flush next to the previous turn and for each layer to lie cleanly on top of the previous layer. The following are basic guidelines.

The minimum distance between a fairlead and bitts should be 1.8m to provide adequate space for the application of mooring line stoppers.

The minimum distance between a winch drum and the nearest fairlead (deck edge or pedestal) should be such that the fleet angle does not exceed 1.5 degrees. This equates to approximately 19 times the drum width, if the mooring line rests in one position in the fairlead, e.g. in a pedestal fairlead. If the mooring line position in a fairlead is variable, e.g. in a deck edge closed fairlead, the distance should be increased to meet the 1.5 degrees requirement in any position. If the design results in fleet angles in excess of 1.5 degrees, powered spooling assistance should be considered.

In the case of split drum winches, only the tension part of the drum should be considered in establishing maximum fleet angles. This is because an unloaded mooring line may be paid out or heaved in directly from any part of either the tension or storage drum during the beginning and end of mooring operations.

In the interest of safe and efficient mooring / unmooring operations, it is recommended that all mooring lines should be deployed from drums and consideration given to the provision of winches that have an individual drive for each drum without the need for clutching and de-clutching. This will eliminate the sometimes-difficult task of clutching and de-clutching drums from a common drive shaft in combination with setting and releasing drum brakes. While it may be commonplace to have a winch motor drive more than one winch drum, more than two should be avoided because of the impact on redundancy and the complexity of the mooring arrangement in both normal and emergency operating modes.

## PART C - DETAILED LIST OF MOORING EQUIPMENT

Purpose:

To provide detailed information on all of the ship's mooring equipment

Functional Requirements:

Documents should contain information that details the ship specific mooring capabilities and limitations for the following equipment where fitted.

C1 - Permanent Fittings - mooring fittings, rollers, fairleads etc.. Refer to Mooring Arrangement Plan in Appendix I

C2 - Permanent machinery installations (winch motors / drives etc)

Documents should contain information that contains the ship-specific mooring capabilities and limitations for the following equipment, where fitted.

C3 - Loose equipment (mooring lines, tails, pennant, joining shackles etc)

C4 - Critical and specialist equipment (e.g. winch brake testing equipment) including tools to undertake maintenance and repairs of this equipment.

C5 - Performance standards / requirements for above equipment

C6 - Details of ship structure and under-deck strengthening

C7 - Location of all permanent / fixed equipment

This should, as a minimum, include winches, their direct leads and any alternative arrangements. this arrangement plan can be combined with records for Part B.

- Location of hazardous mooring zones including higher risk areas, protective locations, optimal viewing and sight lines for supervisors and other human element considerations included in Part E.

- For ease of use, consideration should be given to marking the plan with basic functional information; e.g. ship design MBL, winch brake holding capacity, mooring line WLL/LDBF, SWL of mooring bitts, minimum yield load for bow mooring equipment, location of extra strength bitts for tugs and towing etc.

- Any changes to mooring equipment

## PART D - INSPECTION, MAINTENANCE AND RETIREMENT STRATEGIES / PRINCIPLES

Purpose:

To provide detailed information for inspecting and maintaining loose and permanent mooring equipment as well as strategies to test, retire and replace equipment and interface with the OEM.

Information

Information should, as a minimum, cover all items of loose of permanent mooring equipment. the information should complement that held in the ship's Planned Maintenance Schedule and SMS.

Note: Use links, wherever possible, either to information in this plan or in the V.MS

D1 - Mandatory and Recommended Survey Requirements for all equipment

D2 and D3 - Identification of Critical and Specialist Equipment

D4 - Mooring Line Management Plan

- See - Mooring Line Management Plan

- See – OPS 7 - Mooring Equipment Maintenance

D5 - Records of Certificates and winch break test records

Note: The Chief Officer is responsible for maintaining Mooring Line Management Plan, certificates and winch brake test records (OPS	9 - Winch Break Test Certificate)

## PART E - RISK AND CHANGE MANAGEMENT, SAFETY OF PERSONNEL AND HUMAN FACTORS

Purpose:

To provide detailed information on the requirements for identification and management of hazards and risks arising from the mooring system.

Mooring an unmooring operations, including tug line handling, are dangerous operations. It is important that everyone concerned is fully aware of the hazards and take appropriate precautions to prevent accidents.

Should issues arise in meeting full compliance with the MSMP, (for example lines or mooring fittings do not concur with ship design MBL base line criteria), hazards that arise shall be risk assessed and controlled to ensure the safety and occupational health and well being of colleagues using the equipment. Individual assessments shall be made for each case.

Functional Requirements:

Information should be readily available covering health, safety and requirements that ensure, as a minimum, the safety, health and well being of all personnel in or around the mooring workspace.

Requirements should also cover personnel handling or operating loose or permanent mooring equipment used or deployed by the ship and methods to ensure safe mooring operation, including:

E1 - Assessment of Safety

Good ship mooring management requires a thorough knowledge of mooring principles as well as knowledge about the safe operation of the equipment on board. Mooring and unmooring operations, including the handling of tugs lines and anchoring, are potentially dangerous operations. Such operations must be carried out under the instructions of a responsible and experienced officer who has direct contact with the bridge.

Proper precautions must be observed at all times so as to prevent accidents. The following are some simple but important notes of guidance:

- Always use appropriate Personal Protective Equipment including safety helmets, safety shoes and gloves. Safety goggles are to be worn during anchoring.

- Stand clear of all lines when under load. Do not stand in ‘snap back zones’.

- Watch your feet. Do not step into a coil or loop.

- Always ensure that a winch operator can see the person in charge.

- Do not leave winches or windlasses running unattended.

- Ensure all equipment is operational and well maintained.

- Mooring areas are to be free from obstructions and litter and must be adequately illuminated at night.

- Sufficient personnel shall be kept on board to monitor and safely tend moorings as required by the circumstances of the port.

When making synthetic fibre ropes fast to bitts, use two round turns (but no more) around the leading post of the bitts before “figure of eighting”. This method allows better control of the rope, is easy to use and is safer. It also prevents an effective reduction in SWL caused by the compressive forces imposed by “figure of eighting”.

Working areas around the mooring winches, bitts and rollers should have a clearly defined non-slip surface.

Extreme care must be exercised when applying the brakes and at no time should the brake torque, as determined by the brake holding test, be exceeded. This will ensure that the brakes will render at the correct load.

The publications “Effective Mooring (3rd Edition)” and “COSWP” should be consulted for more comprehensive guidance on safe mooring. Tankers shall be familiar with all requirements contained in the latest edition of OCIMF – Mooring Equipment Guidelines (MEG4).

E2 - Critical Equipment and Control

Mooring Equipment Protection from failure (integrity and redundancy)

Mooring equipment provision is provided within the “as built” specification, and generally cannot be changed subsequently, a risk assessment shall be developed and maintained within Ship Sure to assess the vessel’s specific arrangements for equipment protection. The assessment must consider the following:-

- Redundancy of Equipment - Where power source is a single hydraulic unit what alternatives are provided in case of failure? I.e. crossover from forward/aft systems, spare motor / other components, which are not duplicated. In the latter case, this must be identified as critical equipment and consequential spares carried.

- Supply Protections i.e. electrical cut-offs, overload trips etc.

- Maintenance and Testing Regime - Power source protection form water or hydraulic leaks, i.e. spray shields protect electrical equipment and personnel, flange shields at pipe flanges etc. Any such guards are regularly inspected to be completed and in place. This assessment should be conducted by the Chief Engineer to correctly identify and control all risks from pressurised piping.

- Monitoring of Spaces Containing Mooring Equipment Power Supplies - Subject to design / build or subsequent Owner’s approval, enclosed spaces which contain mooring equipment power supplies should be fitted with gas/fire detection systems and/or extinguishing systems. Where fitted, this equipment is to be regularly tested in accordance with the manufacturer’s instructions. Where not fitted, compensatory measures are to be put in place such as regular rounds and inspection during use, control of ventilation to ensure space remains gas free.

Mooring equipment and fittings

- The purpose of these procedures is to ensure compliance with relevant industry guidelines. In this respect the prime considerations are –

- Mooring equipment and support structures are maintained in good condition in accordance with the PMS, SWL’s are clearly and permanently marked.

- The MBL of ropes made fast on fittings shall not exceed the SWL of the fitting.

- Mooring equipment provided is reviewed in relation to lessons learnt from incidents / near miss reports and third party inspections, consideration given to modification of equipment and operational practices.

And for vessels entering management –

- Permanent mooring fittings are reviewed for compliance with recommended design criteria.

- Permanent mooring fittings are inspected for possible damage or modification from original design and mooring layout plan.

E3 - Manning and Training

E4 - Human Factors and Risk Management

- See – Section Anchors and Anchoring in Element 6

- See - Mooring Safety in Element 6

- See -  Snap-Back Zones in Element 6

E5 - Mooring Operations Plans and Procedures

See - Mooring Plan

E6 - Change Management

See - Element – 7 -  Management of Change (MOC)

## PART F - RECORDS AND DOCUMENTATION

Purpose:

To provide records of detailed maintenance information on the ship’s mooring equipment system, and requirements for documentation management. This may also include upgrades or modifications to mooring equipment. This information forms the historical record of the mooring equipment which should be transferred with the ship.

Functional Requirements:

Documentation covering equipment and operational requirements that ensure, as a minimum, the safety, health and well being of all personnel in or around the mooring workspace shall be available.

- F1 OEM books, operating and maintenance guides

- F2 Records of all associated management of change reviews

## PART G - MOORING SYSTEM MANAGEMENT PLAN REGISTER

Purpose:

To provide records to be retained by the ship throughout its lifecycle from original design to disposal. This information is to be easily transferable if the ship's ownership changes.

G1 Mooring System Management Plan Register

The MSMPF is to be available to all ship’s staff, and others, authorised to review or monitor the equipment status.  All stakeholders have a responsibility in collaborating to ensure the MSMPR is properly created.

Record of Activities

| Date | Occurrence | Action | Comment | Initials |
| --- | --- | --- | --- | --- |

## APPENDIX I – MOORING ARRANGEMENT PLAN

Keep a copy of the Mooring Arrangement Plan here.

## APPENDIX II – CALCULATIONS

Wind Loads on the Fully-Loaded Tanker (Unit: tonne) – Size 6

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw | 38.03 | 32.96 | 14.71 | 2.03 | -16.74 | -37.02 | -48.68 | -48.68 | 180 |
| FYAw | 0.00 | 56.01 | 83.24 | 81.68 | 60.68 | 29.56 | 0.00 | 83.24 | 60 |
| FYFw | 0.00 | 10.89 | 26.45 | 38.12 | 36.56 | 18.67 | 0.00 | 38.12 | 90 |

Current Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc | 14.40 | -0.36 | -0.08 | 0.00 | -14.00 |
| FYAc | 0.00 | 87.368 | 39.77 | 32.21 | 0.00 |
| FYFc | 0.00 | 17.89 | 35.40 | 66.21 | 0.00 |

Wind Loads on the Fully-Loaded Tanker (Unit: tonne) – Size 6

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw |  |  |  |  |  |  |  |  |  |
| FYAw |  |  |  |  |  |  |  |  |  |
| FYFw |  |  |  |  |  |  |  |  |  |

Current Loads on the Fully-Ballast Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc |  |  |  |  |  |
| FYAc |  |  |  |  |  |
| FYFc |  |  |  |  |  |

Wind Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw |  |  |  |  |  |  |  |  |  |
| FYAw |  |  |  |  |  |  |  |  |  |
| FYFw |  |  |  |  |  |  |  |  |  |

Current Loads on the Fully-Ballast Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc |  |  |  |  |  |
| FYAc |  |  |  |  |  |
| FYFc |  |  |  |  |  |

Wind Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw |  |  |  |  |  |  |  |  |  |
| FYAw |  |  |  |  |  |  |  |  |  |
| FYFw |  |  |  |  |  |  |  |  |  |

Current Loads on the Fully-Ballast Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc |  |  |  |  |  |
| FYAc |  |  |  |  |  |
| FYFc |  |  |  |  |  |

Wind Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw |  |  |  |  |  |  |  |  |  |
| FYAw |  |  |  |  |  |  |  |  |  |
| FYFw |  |  |  |  |  |  |  |  |  |

Current Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc |  |  |  |  |  |
| FYAc |  |  |  |  |  |
| FYFc |  |  |  |  |  |

Wind Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Wind angle of attack (degree) |  |  |  |  |  |  | Max. force | Angle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0° | 30° | 60° | 90° | 120° | 150° | 180° |  |  |
| FXw |  |  |  |  |  |  |  |  |  |
| FYAw |  |  |  |  |  |  |  |  |  |
| FYFw |  |  |  |  |  |  |  |  |  |

Current Loads on the Fully-Loaded Tanker (Unit: tonne)

|  | Current angle of attack / velocity (degree/knots) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0/3 | 10/2 | 90/0.75 | 170/2 | 180/3 |
| FXc |  |  |  |  |  |
| FYAc |  |  |  |  |  |
| FYFc |  |  |  |  |  |