RECOVERY OF PERSON FROM WATER – PLANS AND PROCEDURE

Contents

1. INTRODUCTION	2

1.1 GENERAL	2

1.2 PURPOSE	2

1.3 OBJECTIVE	2

1.4 APPLICABILITY AND SCOPE	2

1.5 REQUIREMENTS REGULATIONS AND GUIDELINES	2

1.5.1 SOLAS III “REGULATION 17-1 RECOVERY OF PERSONS FROM THE WATER”	2

1.6 RELATION TO THE INTERNATIONAL SAFETY MANAGEMENT (ISM) CODE	3

2.   PRINCIPLES OF OPERATION	4

2.1 GENERAL	4

2.2   NECESSITY OF PLANNING	4

2.3 RECOVERY OPERATION FROM WATER	4

2.4   SAFETY PRECAUTIONS	4

3.  ROLES AND RESPONSIBILITIES	5

3.1 RESPONSIBILITY OF MASTER	5

3.2 DUTIES OF THE CREW	5

4.   COMPETENCE AND FAMILIARIZATION	6

4.1 GENERAL	6

4.2 EXERCISE RECORD FOR RECOVERING OPERATION	6

4.3 EXERCISE RECORD FOR RECOVERING OPERATION	7

4.4 EXERCISE RECORD FOR RECOVERING OPERATION	8

ANNEX I - SHIP-SPECIFIC INFORMATION	9

RECOVERY EQUIPMENT	9

SHIP-PROCEDURES SPECIFICALLY USED ON BOARD THIS VESSEL.	11

ANNEX 2 - SHIP PROCEDURES AND CHECKLIST	12

CHECK LISTS	13

ATTACHMENTS	15

APPENDICES	15

# 1. INTRODUCTION

## 1.1 GENERAL

This document is developed based on “GUIDELINES FOR THE DEVELOPMENT FOR PLANS AND PROCEDURES FOR RECOVERY OF PERSONS FROM THE WATER (MSC.1/Circ.1447)”, as set out in Appendix 1 to this document.

Risk assessment with equipment intended to be used onboard, taking into account the anticipated conditions and ship-specific characteristics, were carried out as per Annex 1 to this document.

The Guide to recovery techniques (MSC.1/Circ.1182: attached as appendix 2) provides a number of examples of how certain types of equipment can be used to recover persons from the water, to be referred for facilitating the procedures.

In  addition,  following  related  documents  were  also  considered  when  developing  this document:

1.	MSC.1/Circ.1182 “GUIDE TO RECOVERY TECHNIQUES” (Appendix 2 of this document),

2.	MSC.1/Circ.1185/Rev.1” GUIDE FOR COLD WATER SURVIVAL” (Appendix 3 of this document),

3.	MSC/Circ.810   “RECOMMENDATION ON MEANS OF RESCUE ON RO-RO PASSENGER SHIPS” (Appendix 4 of this document).

## 1.2 PURPOSE

The purpose of this document is to provide guidance for the master and other crew members on board the ship regarding procedures for recovering persons from water.

This document is to be kept onboard and maintained in accordance with SOLAS regulation III/17-1 and in order to achieve its purpose, crews on board should be familiar with this document.

## 1.3 OBJECTIVE

This  document  aims  for  the  effective  recovery  and  rescue  of  persons  from  water  with reducing the risk to shipboard personnel involved in recover operation.

The recovery plans and procedures should facilitate the transfer of persons from the water to the ship while minimizing the risk of injury from impact with the   ships side or other structures, including the recovery appliances itself.

## 1.4 APPLICABILITY AND SCOPE

This can be used as a guidance in case of responding to any distress signal where recovery operations are required and also when preparing for the rescue operations for recovering persons from the sea.

## 1.5 REQUIREMENTS REGULATIONS AND GUIDELINES

### 1.5.1 SOLAS III “REGULATION 17-1 RECOVERY OF PERSONS FROM THE WATER”

All ships shall have ship-specific plans and procedures for recovery of persons from the water, taking into account the guidelines developed by the Organization. The plans and procedures shall identify the equipment intended to be used for recovery purposes and measures to be taken to minimize the risk to shipboard personnel involved in recovery operations. Ships constructed before 1 July 2014 shall comply with this requirement by the first periodical or renewal safety equipment survey of the ship to be carried out after 1 July 2014 whichever comes first.

1.5.2 “GUIDELINES FOR THE DEVELOPMENT OF PLANS AND PROCEDURES FOR RECOVERY OF PERSONS FROM THE WATER (MSC.1/Circ.1447)”

MSC.1/Circ.1447 insists to refer the following documents for reference:

1. MSC.1/Circ.1182 “GUIDE TO RECOVERY TECHNIQUES” (Appendix 2 of this document),

2. MSC.1/Circ.1185/Rev.1“GUIDE FOR COLD WATER SURVIVAL” (Appendix 3 of this document).

## 1.6 RELATION TO THE INTERNATIONAL SAFETY MANAGEMENT (ISM) CODE

This document should be considered as a part of the emergency preparedness plan required by paragraph 8 of Part A of the International Safety Management (ISM) Code.

# 2.   PRINCIPLES OF OPERATION

## 2.1 GENERAL

The initiation or continuation of recovery operations should be at the discretion of the master of the recovering ships, in accordance with the provisions of SOLAS regulation III/17-1.

Life-saving and other equipment carried on board may be used to recover persons from the water, even though this may require using such equipment in unconventional ways.

Annex 1/Annex 2  to  this  document  to  be  referred  for  the  information  and  procedures specifically used onboard this vessel.

## 2.2   NECESSITY OF PLANNING

During voyages, there can be situations when the crew suddenly engages to recover people in distress at sea.    This might be a person overboard from the same ship, a fellow crew member, or a passenger or the ship might be responding to someone else’s emergency; for example a ship abandoned because of flooding, fire or a ditched aircraft.

In such cases, crew may have to prepare, with little or no notice, to recover a person or persons. Whoever they are, their lives may be in your hands.

In many areas of the world, especially when out of range of shore-based search and rescue (SAR) facilities, your ship may be the first, or the only, rescue unit to reach them. Even if you are joined by specialized units, you will still have a vital role to play, especially in a major incident. If you are required to recover people in distress, it is your capability and your ship that matters. You may have to find a unique solution to a unique lifesaving problem. To ensure that you can respond safely and effectively, you need to think about the general issues beforehand.

## 2.3 RECOVERY OPERATION FROM WATER

While undergoing the recovery operations, ship’s crew have to refer the procedures stipulated in Annex 2 to this document.

Illumination is necessary for the recovery operation from water. Source of illumination and power (where required) should be available for the area where the recovery operation is conducted.

## 2.4   SAFETY PRECAUTIONS

Recovery operations should be conducted at a position clear of the ship's propellers and, as far as practicable, within the ship's parallel mid-body section.

If carried, dedicated recovery equipment should be clearly marked with the maximum number of persons it can accommodate, based on a weight of 82.5 kg per person.

Master should take necessary precautions as described in Annex 2 to this document.

# 3.  ROLES AND RESPONSIBILITIES

## 3.1 RESPONSIBILITY OF MASTER

The initiation or continuation of recovery operations should be at the discretion of the master of the recovering ships, in accordance with the provisions of SOLAS regulation III/17-1.

The use of ship's rescue craft must be for the master to decide, depending on the particular circumstances of the incident.  Allowable circumstances are referred to Annex 1/Annex 2 to this document.

There will be times when recovery cannot be attempted or completed without unduly endangering the ship, her crew or those needing recovery. Only the assisting ship’s master can decide when this is the case.

Master  has  to develop procedures and  should  carry out  drills  for  emergency  actions  of recovery.

## 3.2 DUTIES OF THE CREW

The various tasks involved are defined and assigned to particular personnel onboard, like who will be required for the recovery process; who will manage the ship in the meantime etc.

# 4.   COMPETENCE AND FAMILIARIZATION

## 4.1 GENERAL

Drills-SOLAS, Chapter III, Regulation19-should ensure that crew’s competence and familiarization with the plans, procedures and equipment for recovery of persons from the water. Such exercise may be conducted in conjunction with routine man-overboard drills (SAF24) and in this case records to be maintained in relevant documents.

## 4.2 EXERCISE RECORD FOR RECOVERING OPERATION

| Date | Name | Rank | Signature | Remarks | Verified by |
| --- | --- | --- | --- | --- | --- |

## 4.3 EXERCISE RECORD FOR RECOVERING OPERATION

| Date | Name | Rank | Signature | Remarks | Verified by |
| --- | --- | --- | --- | --- | --- |

## 4.4 EXERCISE RECORD FOR RECOVERING OPERATION

| Date | Name | Rank | Signature | Remarks | Verified by |
| --- | --- | --- | --- | --- | --- |

# ANNEX I - SHIP-SPECIFIC INFORMATION

| Name |  |
| --- | --- |
| Call Sign |  |
| Flag |  |
| IMO Number |  |
| Type of ship |  |
| Class Society / Notation |  |
| GRT |  |
| Hull type |  |
| Breadth |  |
| Depth |  |
| Main Engine (Type/Output) |  |
| Propeller |  |
| Thruster(s) |  |
| Height (keel to highest point) |  |

## RECOVERY EQUIPMENT

The following rescue boat(s) is/are available onboard

| Item | Location |
| --- | --- |

The following climbing equipment is available on-board

| Item | Location |
| --- | --- |

The following Communication equipment is available on board

| Item | Location |
| --- | --- |

The following lifting equipment is available on board

| Item | Location |
| --- | --- |

To the extent practicable, recovery procedures should provide for recovery of persons in a horizontal or near-horizontal (“deck-chair”) position. Recovery in a vertical position should be avoided whenever possible as it risks cardiac arrest in hypothermic casualties (refer to the Guide for cold water survival (MSC.1/Circ.1185/Rev.1)).

## SHIP-PROCEDURES SPECIFICALLY USED ON BOARD THIS VESSEL.

| Master | To muster on the Bridge, co-ordinate the rescue operation. |
| --- | --- |
| Chief Officer | Muster as per - EMERGENCY INSTRUCTION CARD, In charge of lowering the rescue crafts and of all equipment used for the rescue party. |
| Chief Engineer | To muster in the Engine Room - - EMERGENCY INSTRUCTION CARD |
| 2nd Officer | Standby on the Bridge - - EMERGENCY INSTRUCTION CARD |
| 2nd Engineer | Standby in the Engine Room - - EMERGENCY INSTRUCTION CARD |
| Stretcher Party | Crew No: To muster at the designated area - - EMERGENCY INSTRUCTION CARD |
| Bosun | Lowering the boat, ladders as instructed by the Chief Officer - - EMERGENCY INSTRUCTION CARD |

# ANNEX 2 - SHIP PROCEDURES AND CHECKLIST

Annex 2  to  this  document  specify  the  anticipated  conditions  under  which  a  recovery operation may be conducted without causing undue hazard to the ship and the ship's crew, taking into account, but not limited to:

Ship Specific Details Below

| 1. | Factors contributing to the maneuverability of the ship | - Rudder - Bow Thruster |
| --- | --- | --- |
| 2. | General freeboard of the ship, | - Load - Ballast |
| 3. | Points on the ship to which casualties may be recovered, |  |
| 4. | Characteristics and limitations of equipment intended to be used for recovery operations, |  |
| 5. | Available crew and personal protective equipment (PPE), |  |
| 6. | Other factors which may be considered during rescue operation, | - Wind Force - Direction - Spray - Period of Waves - Swell |
| 7. | Safety Of Navigation. | - Traffic density - Proximity of hazards to navigation |

To the extent practicable, recovery procedures should provide for recovery of persons in a horizontal or near-horizontal (“deck-chair”) position. Recovery in a vertical position should be avoided whenever possible as it risks cardiac arrest in hypothermic casualties (refer to the Guide for cold water survival (MSC.1/Circ.1185/Rev.1)).

Ship-Procedures specifically used on board this vessel.

| Master | To muster on the Bridge, co-ordinate the rescue operation. |
| --- | --- |
| Chief Officer | Muster as per - EMERGENCY INSTRUCTION CARD, In charge of lowering the rescue crafts and of all equipment used for the rescue party. |
| Chief Engineer | To muster in the Engine Room - - EMERGENCY INSTRUCTION CARD |
| 2nd Officer | Standby on the Bridge - - EMERGENCY INSTRUCTION CARD |
| 2nd Engineer | Standby in the Engine Room - - EMERGENCY INSTRUCTION CARD |
| Stretcher Party | Crew No: To muster at the designated area - - EMERGENCY INSTRUCTION CARD |
| Bosun | Lowering the boat, ladders as instructed by the Chief Officer - - EMERGENCY INSTRUCTION CARD |

## CHECK LISTS

- Planning and Preparation

| PREPARE your means of recovery before you arrive at the scene | i.e from the water or from a craft |
| --- | --- |
| PREPARE yourself and your crew before you arrive at the scene. Everyone should know their duties and stick to them as much as possible; |  |
| PREPARE on-board communications, so that lookouts and the recovery team will be able to communicate readily with the Bridge team; | i.e. Handheld or hand free radio |
| BE READY to receive craft and/or people alongside, with boat ropes rigged & other equipment ready to hand |  |
| DECIDE: Who will be required for the recovery process |  |
| What buoyancy aids you going to use, lifebuoys, and lifejackets? |  |
| Detection aid that you going to use: - Reflective material, - Lights, - SART, - EPIRB. |  |

- Approaching Scene

| DETERMINE what will be the most significant factor in creating a lee for the casualty (wind, sea or swell) |  |
| --- | --- |
| ASSESS navigational hazards on scene |  |
| DECIDE on which side you want to make the lee, bearing in mind your own ship’s manoeuvring characteristics; |  |
| CONSIDER running by the casualty first, if time permits, to help you make your assessment; |  |
| CONSIDER stopping well short of the casualty during the final approach, to get the way off your vessel and to assess the effects of wind, sea and swell when stopped/at slow speeds; |  |
| PRIORITY: who should be recover first (injured or people in the water should take priority) |  |

- Bringing People to the Side of the Ship

| CONSIDER the main danger i.e running over and/or crushing the target |  |
| --- | --- |
| CONSIDER that it is possible to over-compensate for the risk, so that the survival craft/person will be missed because too far away. |  |
| DETERMINE that both your ship & target are likely to be affected by wind, sea state, and currents. |  |
| DETERMINE on the severity of the risk of those in distress (can they be left where they are until more suitable help arrives). |  |
| ASSESS on scene weather condition. |  |
| CONSIDER the capability of the rescue craft : |  |
| - The launch & recovery equipment. |  |
| - The experience of the rescue craft’s crew. |  |
| - the availability of PPE for the rescue craft’s crew. |  |
| - The effectiveness of communications between the rescue craft& recovery ship. |  |
| - The proximity of navigational hazards to the rescue crafts. |  |
| - The rescue craft’s ability to navigate, whether independently or conned from the ship, so as to avoid hazard and to locate the person (s) in distress. |  |
| DETERMINE the manoeuvrability of the recovering ship. |  |
| DETERMINE the proximity of navigational hazards: limiting your ability to manoeuvre to provide alternative help to those in distress. |  |

- Bringing People on Board the Ship

| This will depend on: The prevailing weather & sea condition. |  |
| --- | --- |
| The condition of the people to be recovered. |  |
| The size & design of your ship. |  |
| The equipment available and |  |
| The competency of those using it |  |
| ASSESS the best point of entry into the ship & consider : |  |
| Where can the ladder or other climbing devices be rigged? | Ex. Aft port side shell door |
| Are there any low freeboard areas, can they be safely accessed? |  |
| Are there any hull openings? |  |
| Is there belting along the ship side? |  |
| Pilot ladders & lifts or the accommodation ladder. |  |
| CONSIDER the lifting devices as: Cranes, gantries, derricks |  |
| Davits, |  |
| Windlass, winches. |  |
| A purpose-build or improvised rescue basket is usually better then straps and loops. |  |

- Immediate Care of People Recovered

| DETERMINE the location where the survivors will be located on the ship |  |
| --- | --- |
| DECIDE who will take them there (name the stretcher party) |  |
| DECIDE what you are going to do with the dead. |  |

|  |  | DATE: |
| --- | --- | --- |

# APPENDICES

- Appendix 1 - MSC1/Circ. 1447

- Appendix 2 - MSC1/Circ.1182

- Appendix 3 - MSC1/Circ.1185

- Appendix 4 - MSC/Circ.810