Ship’s Name:

| Restricted areas means “spaces that are essential to the operation, control or safety of the vessel.” These areas are to be identified and included in this table and also marked on the copy of the GA plan in appendix 5. Cross each box relevant to the security level in force. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | Deck |  | LEVEL 1 |  | LEVEL 2 | LEVEL 3 | Remarks |
| An area encompassing Wheelhouse Top, Navigation Bridge, Funnel, the Officers and Crews Accommodation, Galley, Mess Rooms, Gymnasium, Laundry, Internal Lockers, Conference & Ships Offices |  |  |  |  |  |  |  | Spaces requiring Daily/Constant access. At Level 1 to be locked when unattended in port. Level 2 to be locked when unattended. Level 3 to be locked positively. Compass deck area cannot be locked, therefore to be monitored/patrolled at frequent intervals. Locking to be secured that can allow emergency escape from Accommodation/E.R. and also allow emergency party access in an emergency. At level 2+3 one agreed access point to Accommodation/E.R. only to be unlocked. Security control by Duty Personnel, with checks and patrol made by watchmen once per watch at level 1 (6 in 24 hours) and twice per watch at level 2 (12 in 24 hours). Patrols to be double /extra manned at level 3 if considered necessary with continuous patrolling of all areas. Authorised personnel are – Deck & Engine Officers + designated ratings + authorised visitors. |
| Locked |  |  |  | -- |  | X | X |  |
| Unlocked |  |  |  | X |  | -- | -- |  |
| Keys Held By |  |  |  | Master / CO / CE / OOW |  |  |  |  |
| Security Control / Check |  |  |  | X | X |  | X |  |
| Restricted areas means “spaces that are essential to the operation, control or safety of the vessel.” These areas are to be identified and included in this table and also marked on the copy of the GA plan in appendix 5. Cross each box relevant to the security level in force. |  |  |  |  |  |  |  |  |
| Location | Deck |  | LEVEL 1 |  | LEVEL 2 |  | LEVEL 3 | Remarks |
|  |  |  |  |  |  |  |  | Spaces requiring access in Emergencies. At Level 1 – to be unlocked, Level 2 to be locked when unattended in port. Level 3 to be locked. Keys to be easily accessible in emergency. *May be locked by plastic pull tag for easy monitoring during patrol. **Hospital to be locked at all times, keys to be readily available on the bridge. Security patrols at level 1 once per watch (6 in 24 hours) and twice per watch at level 2 (12 in 24 hours). Patrols to be double/extra manned at level 3 if considered necessary with continuous patrolling of areas. Authorised personnel are – Deck & Engine Officers + ratings + authorised visitors. |
| Locked |  |  | -- |  | X (in Port)* |  | X* |  |
| Unlocked |  |  | X |  | X (at Sea) |  | -- |  |
| Keys Held By |  |  | Master / CO / CE / OOW |  |  |  |  |  |
| Security Control / Check |  |  | X |  | X |  | X |  |

Security control is undertaken by OOW and/or Deck Watchman unless otherwise stated

| Restricted areas means “spaces that are essential to the operation, control or safety of the vessel.” These areas are to be identified and included in this table and also marked on the copy of the GA plan in appendix 5. Cross each box relevant to the security level in force. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | Deck | LEVEL 1 | LEVEL 2 | LEVEL 3 | Remarks |
|  |  |  |  |  | Spaces not normally in daily use. At Level 1 to be locked when unattended in port. Level 2 to be locked when unattended. Level 3 to be locked positively. *Locking to be secured that can allow emergency escape from Funnel Casing and also allow emergency party access in an emergency. Security patrols at level 1 once per watch (6 in 24 hours) and twice per watch at level 2 (12 in 24 hours). Patrols to be double/extra manned at level 3 if considered necessary with continuous patrolling of areas. Authorised personnel are – Deck & Engine Officers + ratings + authorised visitors. |
| Locked |  | -- | X | X |  |
| Unlocked |  | X | -- | -- |  |
| Keys Held By |  | Master / CO / CE / OOW |  |  |  |
| Security Control / Check |  | X | X | X |  |

Security control is undertaken by OOW and/or Deck Watchman unless otherwise stated