Ship’s Name:

Access to Vessel

All access points to the vessel are to be listed in the table and information included for in port, at anchor and at sea. Cross each box relevant to the security level in force.  Each door or entry point requires an identification number. The copy of the GA plan in appendix 5 is to be marked up accordingly with the number included.

| Access to vessel | No | Location | Level 1 |  | Level 2 |  | Level 3 |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  | Yes | No | Yes | No | Yes | No |  |
| Mooring Lines Monitor visually Monitor by cameras Use of alarms and sensors Use of security lighting Man access points Lock, secure access points Checked at intervals by security patrol Checked regularly by security patrol |  |  | X X X | X X X X X | X X X X | X X X X | X X X X | X X X X | Mooring arrangements and lines are set up and controlled from Forecastle and Upper Deck Aft. The lines are monitored and checked as part of the deck rounds for each watch (at least once every 4 hours) Rat guards can be used as deterrent. |
| Anchor chains Monitor visually Monitor by cameras Use of alarms and sensors Use of security lighting Man access points Lock, secure access points Checked at intervals by security patrol Checked regularly by security patrol |  |  | X X X | X X X X X | X X X X | X X X X | X X X X | X X X X | Cover plate on hawse pipe to be secured at levels 2 and 3. Anchor wash may be used at levels 2 and 3 if port permits. If used as part of mooring then rounds as per above. At anchor checked at least once per watch with more frequent patrols at levels 2 and 3. |

| Access to vessel | No | Location | Level 1 |  | Level 2 |  | Level 3 |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  | Yes | No | Yes | No | Yes | No |  |
| Pilot ladder/Jacobs ladder Secured in board when not in use Manned when in use Checked at intervals by security patrol Checked regularly by security patrol Only used with Masters authority or for emergency |  |  | X X X X X |  | X X X X X |  | X X X X X |  | Manned and monitored when in use. Secured inboard when not in use. At level 3 the ladders may be used as a means of evacuation with Master’s permission |
| Gangways Monitor visually Monitor by cameras Use of alarms and sensors Use of security lighting Man access points Lock, secure access points Checked at intervals by security patrol Checked regularly by security patrol |  |  | X X X X X X X | X | X X X X X X X | X | X X X X X X X | X | Only one gangway used at a time. Visitors pass system to be in use. 100% ID checks and positive identification. Only approved persons to be allowed on-board. At level 3 gangways to be hoisted unless authorised persons require to board. |
| Ramps Monitor visually Monitor by cameras Use of alarms and sensors Use of security lighting Man access points Lock, secure access points Checked at intervals by security patrol Checked regularly by security patrol |  |  | X X X X X X X | X | X X X X X X X | X | X X X X X X X | X | Only one gangway used at a time. Visitors pass system to be in use. 100% ID checks and positive identification. Only approved persons / cargoes to be allowed on-board. At level 3 Ramps to be hoisted . |

| Access to vessel | No | Location | Level 1 |  | Level 2 |  | Level 3 |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  | Yes | No | Yes | No | Yes | No |  |
| Cranes (ship) Manned when in use Secured inboard when not in use Not to be used |  |  | X X | X | X X | X | X X X |  | Manned and monitored when in use. Secured inboard when not in use. At level 3 no provisions to be loaded. |
| Hoists Manned when in use Secured inboard when not in use Not to be used |  |  | X X X |  | X X X |  | X X X |  | At level 3 operations to be avoided. For emergency use during abandon ship or evacuation only. |
| Hoses (Bunkers, etc) Monitor visually Monitor by cameras Use of security lighting Manned when in use Checked at intervals by security patrol Checked regularly by security patrol Not to be used |  |  | X X X X X | X | X X X X X | X | X X X X X X | X | Manned and monitored when in use. Secured inboard when not in use. At level 3 Bunker operations and / or fresh water supply to be avoided. |
| Cargo hoses Monitor visually Monitor by cameras Use of security lighting Manned when in use Checked at intervals by security patrol Checked regularly by security patrol Not to be used |  |  | X X X X X | X | X X X X X | X | X X X X X X | X | Manned and monitored when in use. Secured inboard when not in use. At level 3 cargo operations to be avoided. |

| Access to vessel | No | Location | Level 1 |  | Level 2 |  | Level 3 |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  | Yes | No | Yes | No | Yes | No |  |
| Liferaft’s & lifeboats embarkation ladders Secured in board when not in use Manned when in use Checked at intervals by security patrol Checked regularly by security patrol Only used with Masters authority for emergency |  |  | X X X X X |  | X X X X X |  | X X X X X |  | For emergency use during abandon ship or evacuation only. |
| Access doors, sidescu t t les, w in dow s an d port s Locked |  |  |  |  |  |  |  |  |  |