| SHIPBOARD OIL POLLUTION PREVENTION PLAN (SOPEP) VESSEL’S NAME: Note: The latest Revision Number of this Manual is the Revision Number indicated on this “TITLE PAGE”. |  |  |
| --- | --- | --- |

#### REVISION SHEET

pages

When it becomes necessary to revise this manual revision will be made by the issue of the new relevant	/ section.

- These revised pages will bear a new revision number that must be entered on this revision sheet and initialed by the Master or responsible manager after insertion into this book

| Document Number | Section/page | Revision Number | Date issued | Date Inserted | Manager’s or Master’s Initials |
| --- | --- | --- | --- | --- | --- |

ACKNOWLEDGEMENT SHEETS

This manual should be read by the Master and by all other officers, and this page should be signed by each officer as confirmation * Subsequently, all officers joining this vessel must read this manual and sign as confirmation

| Name | Rank | Date | Signed |
| --- | --- | --- | --- |

* Any questions concerning this manual should be directed to the Designated Person Ashore.

#### ACKNOWLEDGEMENT SHEET

(CONTINUED)

| Name | Rank | Date | Signed |
| --- | --- | --- | --- |

#### ACKNOWLEDGEMENT SHEET

(CONTINUED)

| Name | Rank | Date | Signed |
| --- | --- | --- | --- |

(CONTINUED)

| Name | Rank | Date | Signed |
| --- | --- | --- | --- |

| SR.NO |  | Particulars | No. of pages | Revision Number | Date |
| --- | --- | --- | --- | --- | --- |
| 0.1 |  | COVER PAGE | 1 |  |  |
| 0.2 |  | Revision Sheet | 1 |  |  |
| 0.3 |  | Acknowledgement Sheet | 4 |  |  |
| 0.4 |  | Index | 3 |  |  |
| 1.0 |  | INTRODUCTION | 4 |  |  |
| 1.1 |  | Preamble |  |  |  |
| 1.2 |  | Which plan |  |  |  |
| 1.3 |  | Summary Flow Chart |  |  |  |
| 2.0 |  | REPORTING REQUIREMENTS | 11 |  |  |
| 2.1 |  | When to report |  |  |  |
| 2.2 |  | Information required |  |  |  |
|  | 2.2.1 | Format & content of reports |  |  |  |
|  | 2.2.2 | Example reports |  |  |  |
| 2.3 |  | Whom to contact |  |  |  |
| 3.0 |  | STEPS TO CONTROL DISCHARGE | 34 |  |  |
| 3.1 |  | Master’s Responsibilities and Authority |  |  |  |
| 3.2 |  | Pollution - General |  |  |  |
|  | 3.2.1 | Collection and Disposal of Residues |  |  |  |
|  | 3.2.2 | Operational Spills |  |  |  |
|  | 3.2.3 | Pipeline Leakage |  |  |  |
|  | 3.2.4 | Tank Overflow During Cargo or Bunker Operations |  |  |  |
|  | 3.2.5 | Hull Leakage |  |  |  |
| 3.3 |  | Spills Resulting from Casualties |  |  |  |
|  | 3.3.1 | Grounding |  |  |  |
|  | 3.3.2 | Fire/Explosion |  |  |  |
|  | 3.3.3 | Collision with Fixed or Moving Object |  |  |  |
|  | 3.3.4 | Hull Failure |  |  |  |
|  | 3.3.5 | Excessive List |  |  |  |
|  | 3.3.6 | Containment System Failure |  |  |  |
|  | 3.3.7 | Submerged/Foundered |  |  |  |
|  | 3.3.8 | Wrecked or Stranded |  |  |  |
|  | 3.3.9 | Hazardous Vapor release |  |  |  |
|  | 3.3.10 | Dangerous Reaction of Cargo |  |  |  |
|  | 3.3.11 | Other Dangerous Cargo release |  |  |  |
|  | 3.3.12 | Loss of Tank Environmental control |  |  |  |
|  | 3.3.13 | Cargo Contamination Yielding a Hazardous condition |  |  |  |
| 3.4 |  | Stability and Stress Conditions |  |  |  |
| 3.5 |  | Lightering |  |  |  |
| 3.6 |  | Plans, Drawings and Ship Specific Details |  |  |  |
|  | 3.6.1 | Shipboard Spill Response Resources |  |  |  |
| 3.7 |  | Mitigating Activities |  |  |  |
| 4.0 |  | PROCEDURES AND POINT OF CONTACT FOR CO-ORDINATION | 4 |  |  |
| 4.1 |  | National and Local Co-ordination |  |  |  |
| 4.2 |  | Identities of National and Local Authorities |  |  |  |
| 4.3 |  | Responsibilities |  |  |  |
|  | 4.3.1 | Coastal State Response to Pollution Incident |  |  |  |
|  | 4.3.2 | The Vessel Response to a Pollution |  |  |  |

|  |  | Incident |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4.4 |  | Planning Standards |  |  |  |
| 5.0 |  | ADDITIONAL NON-MANDATORY PROVISIONS | 8 |  |  |
| 5.1 |  | Ship Specific Information |  |  |  |
| 5.2 |  | On Board Response Equipment |  |  |  |
| 5.3 |  | Legal Assistance |  |  |  |
| 5.4 |  | Public and Media Affairs |  |  |  |
| 5.5 |  | Recordkeeping |  |  |  |
| 5.6 |  | Plan Review |  |  |  |
| 5.7 |  | Training, Drills and Exercises |  |  |  |
| 5.8 |  | Crew Safety and Welfare |  |  |  |
| 5.9 |  | General Emergency Actions |  |  |  |
|  | 5.9.1 | Steering Failure / Emergency Steering / Technical Failure |  |  |  |
|  | 5.9.2 | Shifting of Cargo |  |  |  |
| Appendix I |  | Coastal State Contact Information |  |  |  |
| Appendix II |  | Emergency Port Contact Sheet | 1 |  |  |
| Appendix III |  | Ship Interest Contact Details | 3 |  |  |
| Appendix IV |  | Ship’s Particulars | 1 |  |  |
| Appendix V |  | Contingency Checklists | 26 |  |  |
|  | 1 | Pipeline Leakage and Tank Overflow |  |  |  |
|  | 2 | Hull Leakage/Failure |  |  |  |
|  | 3 | Grounding, Wrecked or Stranded/Major Flooding |  |  |  |
|  | 4 | Fire Explosion |  |  |  |
|  | 5 | Fire in Accommodation |  |  |  |
|  | 6 | Cargo Fire on Deck |  |  |  |
|  | 7 | Fire on Deck |  |  |  |
|  | 8 | Fire in Pumproom |  |  |  |
|  | 9 | Collision |  |  |  |
|  | 10 | Excessive List |  |  |  |
|  | 11 | Containment System Failure |  |  |  |
|  | 12 | Sea Pollution |  |  |  |
|  | 13 | Bunker Spills |  |  |  |
|  | 14 | Submerged/Foundered |  |  |  |
|  | 15 | Hazardous Vapour Release |  |  |  |
|  | 16 | Emergency Lightering |  |  |  |
|  | 17 | Steering Failure |  |  |  |
|  | 18 | Electrical Power Failure |  |  |  |
|  | 19 | Technical Breakdown |  |  |  |
|  | 20 | Cargo Shift |  |  |  |
|  | 21 | Gyro Compass Malfunction |  |  |  |
| Appendix VI |  | Lloyds Open Form (LOF) | 2 |  |  |
| Appendix VII |  | Caution Note | 1 |  |  |
| Appendix VIII |  | General | 1 |  |  |

# INTRODUCTION

This Plan is written in accordance with the requirements of Regulation 37 of Annex I to the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 relating thereto.

The purpose of the Plan is to provide guidance to the Master and Officers on board the ship with respect to the steps to take when a pollution incident occurs or is likely to occur.

The Plan contains all the information and operational instructions required by the Guidelines. The appendices contain names, telephone, telex numbers, etc. of all contacts referenced in the Plan as well as other reference material.

This plan has been approved by the Administration and, except as provided below, no alteration shall be made to any part of it without prior approval of the Administration

Changes to Section 5 and the appendices will not be required to be approved by the Administration. The appendices should be maintained up to date by the owners, operators and managers.

All Bunker and Ballast disposition information is located on the bridge.

#### PREAMBLE

This Plan is written to assist personnel both on board and ashore in dealing with an unexpected discharge of oil or other noxious liquid substance (NLS). Its primary purpose is to set in motion the necessary actions required to stop or minimize the discharge and mitigate its effects. Effective planning ensures that the necessary actions are taken in a structured, logical, safe and timely manner.

The plan goes beyond providing for operational spills and guidance is contained within the plan to assist the Master in meeting the demands of a catastrophic discharge, should the ship become involved in one.

The need for a predetermined and properly structured plan is clear when the pressures and multiple tasks facing personnel confronted with an emergency situation are considered. The plan aims to minimize the risk of confusion, mistakes and failure to advise key people. Without the plan, delays may be incurred, and time wasted, time during which a situation may well worsen. As a consequence, the ship and its personnel may be exposed to increasing hazards and greater environmental damage occurs.

Although the Plan is designated as a ship-specific tool it must also be considered as an additional instruction and as a link to shore-based plans. With this the Plans allow an efficient co-ordination between the ship and shore-based Authorizations in mitigating the effects of an oil pollution incident,

For the plan to accomplish its purpose it must be:

Realistic, practical and easy to use

Understood by Shipboard Management Team and the Fleet Management Cell ashore Evaluated, reviewed and updated regularly.

All staff, on board and ashore, is responsible for advising the Safety & Quality Superintendent of any shortcomings, errors or possible improvements to the plan.

The plan must be used in conjunction with other documents, drawings and manuals on board the vessel and within the management office. These should include but not be limited to:

Ship’s drawings and plans

Stability booklet and computer loadicator SOLAS Training Manual

Contingency section of Company Operating Manual

Damage stability information provided by the management office or third party provider Reference publications contained within the ships reference library.

The Master should be aware of the following proviso when implementing this plan:

Without interfering with ship owners’ liability, some coastal states consider that it is their responsibility to define techniques and means to be taken against a marine pollution incident and approve such operations which might cause further pollution

e.g. lightening. States are generally entitled to do so under the International Convention relating to Intervention on the High Seas in Cases of Oil Pollution Casualties, 1969 (1969 Intervention Convention) and the Protocol relating to Intervention on the High Seas in Cases of Pollution by Substances other than Oil, 1973 (1973 Intervention Convention).

#### WHICH PLAN

The plan required by MARPOL, Annex I will not fully meet United States regulations applicable to vessels which carry oil in bulk, either as primary or secondary cargo. Under the US Federal Water Pollution Control Act (FWPCA), as amended by the Oil Pollution Act of 1990 (OPA 90), owners or operators of such vessels in US waters must additionally: identify and ensure, through contract or other approved means, the availability of private fire fighting, salvage, lightering and clean-up resources; identify a qualified individual with full authority to implement the response plan, including the activation and funding of contracted clean-up resources; and describe training and drill procedures.

For further information regarding requirements under OPA 90, reference should be made to the OPA90

- Vessel Response Plan issued by PGSHIPS for all the PGSHIPS controlled / managed vessels expected to call in the U.S.A.

The flow diagram entitled "WHICH PLAN" below should be used to ascertain whether the vessel has to be ready to put into effect the, either the MARPOL "Shipboard Oil/Marine Pollution Emergency Plan – SOPEP/SMPEP" or the OPA 90 "Vessel Response Plan - VRP".

# WHICH PLAN?

EMERGENCY

Oil Pollution Threat

NO

Refer to ECP

YES

US Territorial Waters

YES	NO

Vessel Response Plan

Written in accordance with US OPA 1990

SOPEP / SMPEP

* Any vessel carrying oil in bulk as cargo excluding LNG and LPG carriers

Any vessel carrying oil in bulk as cargo excluding LNG and LPG carriers

#### SUMMARY FLOW CHART

This flow diagram is an outline of the course of action that shipboard personnel should follow in responding to a pollution emergency based on the guidelines published by IMO. The diagram is not exhaustive and should not be used as the sole reference for response.

DISCHARGE OF OIL

Probable or Actual

ASSESSMENT OF THE NATURE OF INCIDENT ACTIONS REQUIRED

- Alert crew members

- Spill assessment

- Identify spill source

- Personnel Protection

- Vapor Monitoring

- Evacuation

REPORTING

By Master and/or designated crew member

WHEN TO REPORT

* All probable and actual spills

ACTION TO CONTROL DISCHARGE

Measures to minimize the escape of oil and threat to the marine environment

NAVIGATIONAL MEASURES	SEAMANSHIP MEASURES

HOW TO REPORT

- By quickest means to coastal radio station

- Designated ship movement reporting station or

- Rescue Co-ordination Centre (RCC) - at sea

- By quickest available means to local authorities

WHO TO CONTACT

- Nearest coastal State

- Harbour and terminal operators (in port)

- Ship owner, PGSHIPS, P & I Club

- Commercial Manager, Charterer

- Alter course/position and/or speed

- Change of list and/or trim

- Anchoring

- Setting aground

- Initiate towage

- Assess safe haven requirements

- Weather/tide/swell forecasting

- Slick monitoring

- Record of events and communications

- Safety assessment and precautions

- Advice on priority countermeasures and preventive measures

- Damage stability and stress consideration, loadicator use

- Ballasting / deballasting

- Internal cargo/fuel transfer operations

- Emergency ship-to-ship transfers of cargo and / or bunkers

- Set up shipboard response for :

- Leak sealing

- Fire fighting

- Handling of shipboard response

- Refer to contact lists	 	equipment (if available), etc.

WHAT TO REPORT

- Initial report (Res. A851(20))

- Follow-Up reports

- Characteristics of oil spilled

- Cargo/ballast/bunker dispositions

- Weather and sea conditions

- Slick movement

- Assistance required

- Salvage

- Lightening capacity

- Mechanical equipment

- External strike team

- Chemical dispersant/degreasant

STEPS TO INITIATE EXTERNAL RESPONSE

- Refer to Coastal Port State listings for local assistance

- Refer to Ship Interest Contact list

- External clean-up resources required

- Continued monitoring of activities

# REPORTING REQUIREMENTS

#### WHEN TO REPORT

The Master must make a report without delay using the Notification Form, whenever his ship is involved in an actual or probable discharge incident. The intent of this requirement is to ensure that coastal states are informed, without delay, of any incident giving rise to pollution, or threat of pollution, of the marine environment, as well as of assistance and salvage measures, so that appropriate action may be taken.

An actual discharge is defined as:

    - A discharge above the permitted level of oil for whatever reason including those for the purpose of securing the safety of the ship or saving life at sea, or

    - A discharge during the operation of the ship of oil in excess of the quantity or instantaneous rate permitted under MARPOL 73/78.

A probable discharge depends upon many factors and when assessing the probability of a discharge the Master should consider, as a minimum, the following factors should be considered:

    - The nature of the damage, failure or breakdown of the ship, machinery or equipment

    - Ship location and proximity to land or other navigational hazards

    - Weather, tide, current and sea state

    - Traffic density

It is impracticable to lay down precise definitions of all types of situations involving probable discharge, which would warrant an obligation to report. As a general guideline the Master must make a report in cases of:

    - Damage, failure or breakdown which affects the safety of the ship. Examples of such situations are collision, grounding, fire, explosion, structural failure, flooding, cargo shifting and

    - Failure or breakdown of machinery or equipment which results in impairment of the safety of navigation. Examples of such incidents are failure or breakdown of steering gear,propulsion, electrical generating system, essential ship borne navigational aids

###### REPORTING REQUIREMENTS

HAS THERE BEEN AN INCIDENT OR HAZARDOUS OCCURRENCE?

YES	NO

IS THERE AN ACTUAL OIL SPILL?

NO REPORT REQUIRED

YES

A REPORT IS REQUIRED

IS THERE A PROBABILITY OF AN

YES

NO

INFORM DPA/ALT.DPA/ FLT SUPT.

A REPORT IS REQUIRED

NO

NO

YES

NOTIFY PORT

STATE

MEANS

D.P.A./ALTERNATE DPA / F.SUPT

(Contact details in Appendix III)

IS SHIP IN PORT

MANDATORY INITIAL REPORTING ACTION NOW COMPLETE

PREPARE FOLLOW-UP REPORTS AS PRACTICABLE

  - INFORMATION REQUIRED

    - Format and Content of Reports

The format and content of an initial report is given below. The format is consistent with the General Principles for Ship Reporting Systems and Ship Reporting Requirements, including Guidelines for Reporting Incidents Involving Dangerous Goods, Harmful Substances and/or Marine Pollutants, adopted as Resolution A.851(20) by the International Maritime Organization (IMO), and should be followed so far as possible. (Note: The reference letters in the listing below do not follow the complete alphabetical sequence as certain letters are allocated to information required for other reporting formats).

The report should contain the following information:

Letter Function	Explanation

AA.      Ship	Name of ship, call sign, GMDSS contact details & flag.

BB.    Date and time (GMT)	A 6-digit group giving day of month (first two digits), incident

hours and minutes (last four digits).

CC.      Ship's position	Giving latitude: a 4-digit group in degrees and minutes suffixed with

N (North) or S (South);

longitude: a 5-digit group in degrees and minutes suffixed with E (East) or W (West);

or

DD.	Ship's position	By true bearing (first 3 digits) and distance (stated) from

a clearly identified landmark.

EE.	True course	As a 3-digit group.

FF.	Speed at time of	In knots and tenths of a knot as a 3-digit group incident.

LL.	Route information	Details of intended track.

MM.	Radio Communications	Full details of radio stations and frequencies being guarded.

| NN. | Date & Time of next report | A 6-digit group as in BB. |
| --- | --- | --- |
| OO. | Draught | A 4-digit group giving draught in metres and centimetres. |
| PP. | Cargo Onboard | Types and quantities of cargo and bunkers on board |
|  |  | and brief details of dangerous goods if any. |
| QQ. | Defects or Damage | Brief details of defects, damage, deficiencies or other |
|  |  | limitations. These must include the condition of ship and |
|  |  | the ability to transfer cargo, ballast, or fuel. |
| RR. | Description of the | Brief details of actual pollution. This should include the pollution pollution cause, type of oil, an estimate of the quantity discharged, whether the discharge is continuing, and the |

|  |  | cause of the discharge and, if possible, an estimate of the movement of the slick. |
| --- | --- | --- |
| SS. | Weather conditions | Weather and sea condition, including wind force and direction and relevant tidal or current details. |
| TT. | Operator details | Name, address, telex, facsimile and telephone numbers of the ship's owner or representative (manager or operator of the ship, or their agents). |
| UU. | Ships particulars | Details of length, breadth, tonnage and type of ship. |
| WW. | Crew details | Total number of persons onboard. |
| XX. | Miscellaneous | To include relevant details including, as appropriate: |
|  |  | - Brief details of incident. |
|  |  | - Names of other ships involved. |
|  |  | - Action taken with regard to the discharge and movement of the ship. |
|  |  | - Assistance or salvage resources which are needed or have been requested or provided. |
|  |  | - Personnel injuries sustained if any. |
|  |  | - Whether medical assistance is required. |
|  |  | - Details of P & I Club |
| YY. | Relay | To request to relay the message to another reporting system. e.g. Amver, Ausrep, Marep etc. |
| ZZ. | End of report |  |

If no outside assistance is required, this should be clearly stated.

Reports should be transmitted by the quickest available means to the responsible authorities of the nearest coastal state or the Rescue Co-ordination Centre (RCC) via the appropriate shore radio station. If the ship is within or near to an area for which a ship reporting system has been established, reports should be transmitted to the designated shore station of that system.

The following additional information should be sent to PGSHIPS office either at the same time as the initial report or as soon as possible thereafter:

      - Further details of damage to ship and equipment.

      - Whether damage is still being sustained.

      - Assessment of fire risk and precautions taken.

      - Disposition of cargo on board and quantities involved.

      - Number (Names) of casualties.

      - Damage to other ships or property.

      - Time (GMT) assistance was requested and time (GMT) assistance expected to arrive at the scene.

      - Name of salvor and type of salvage equipment.

      - Whether further assistance is required.

      - Priority requirements for spare parts and other materials.

      - Details of outside parties advised or aware of the incident.

      - Any other important information.

      - In this context, details of bunker disposition & ballast condition would help.

After transmission of the information in an initial report, as much as possible of the information essential for the safeguarding of life and the protection of the ship and the marine environment should be reported in a supplementary report to the coastal state and the owner or operator, in order to keep them informed of the situation as the incident develops. This information should include items PP, QQ, RR, SS and XX, as appropriate.

Examples of initial reports follow, together with an example of a format which may be found suitable for direct transmission.

Please establish which means of communications will be used/monitored and if possible to establish timing of next update from the vessel. All follow up reports should include information on any significant changes in the ship's condition, the rate of release and spread of oil, weather and sea conditions and clean-up activities underway.

## Example Reports

a.	Initial Report - sent to the government of the coastal state and to the owner or operator: AA	BLACKGOLD CALL SIGN.....USFLAG…..SATCOM A/C…..

BB      291150

CC	2230N 06000E

EE	137

FF	120

LL	BOUND SINGAPORE FROM RAS TANURA

MM	BAHRAIN RADIO 2182 KHZ, VHF 16, INMARSAT NO. 888888 NN	AS REQUIRED

OO	1700

PP	CRUDE OIL ARAB LIGHT 85742 TONNES. QATAR 36764 TONNES

QQ COLLISION WITH CARGO SHIP WHITE SKY. TANK 6 PORT BREACHED SUBSEQUENT FIREMAINDECKAND ENGINE ROOM THUS UNABLE TO MANOEUVRE. FIRENOW EXTINGUISHED. UNABLE TRANSFER CARGO DUE FULL TANKS. BALLAST ANDFUEL SYSTEM INOPERATIVE.

RR ESTIMATE LOSS 2000 TONNES ARAB LIGHT CRUDE OIL FROM 6 PORT. OUTFLOW NOW STOPPED APART FROM SEA ACTION. ESTIMATE OF SLICK MOVEMENT ANDAREA NOT POSSIBLE.

SS  WEATHER FINE. WIND SE FORCE 3. SEA SLIGHT.

TT	SHIP OPERATOR BLACK GOLD SHIPPING CO., LONDON, ENGLAND TELEPHONE:	+... ... ....

FAX:	+... ... ....

UU	LENGTH 223M. BREADTH 42M. TONNAGE 127506 SDWT. TYPE OIL TANKER. WW	25

XX	TUG ABC 2 CONTRACTED TO ASSIST ETA 291600 DO NOT ANTICIPATE FURTHER ASSISTANCE REQUIRED.

The following is an example of additional information for owner or operator:

QQ TANK 6 PORT BREACHED FROM DECK TO 1 METRE ABOVE WATER. SHIP LISTED 5 DEGREES STARBOARD. BOILER OUT OF SERVICE HOWEVER ANTICIPATE WILL RESTORE TO SERVICE APPROX. 8 HOURS. GENERATORS OK. NO FURTHER DAMAGE. FIRE RISK UNCERTAIN. ALL INTACT TANKS INERT.

RR CARGO DISPOSITION ARAB LIGHT ALL WINGS PLUSONE/FOUR CENTRE QATAR TWO THREE FIVE CENTRE. ALL TANKS FULL.

XX ONE MISSING (NAME). THREE NON SERIOUS INJURIES (NAMES). DAMAGE TO COASTER WHITE SKYUNKNOWN HOWEVER SHE ADVISES NOT IN DANGER OF SINKING.

TUG ABC 2 ETA REMAINS 1600 GMT. LOF 90 AGREED. DO NOT ANTICIPATE FURTHER ASSISTANCE REQUIRED. WILL REVERT WITH SPARES/MATERIALS REQUIREMENTS.

SELF AND WHITE SKY BROADCAST VHF PAN MESSAGE, COASTAL STATE ADVISED. UNDERSTAND LOCALCOAST GUARD ARRANGING AERIAL SPRAYING.

NO OTHER INFORMATION.

b	Example of an Update Report :

Use the same alphabetical codes as for the Initial Notification Format - but then send only the relevant sections needed for an update report e.g.

aa	BLACKGOLD CALL SIGN.....USFLAG	SATCOM A/C bb		291500

cc	2230N 06000E

nn	291800

qq	As initial Report

rr	Discharge flow now stopped.

ss	weather fine. Wind se force 3. Sea slight

xx Oil clean up contractor has been contacted through P & I Club local representative and he advises that cleaning up operation shall start at (xxxx) hours with sufficient number of personnel engaged in the operation to satisfy the authorities requirements.

Regards / Master

A blank report form is shown:

| SHIPBOARD OIL/MARINE POLLUTION EMERGENCY PLAN Sample format for initial notification |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AA (ship name, call sign, flag) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BB (date and time of event, UTC) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D D H H M M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CC (position, latitude, longitude) or DD (bearing, distance from landmark) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | d d d N miles |  |  |  |
|  |  |  |  |  |  | N | S |  |  |  |  |  |  |
|  |  | d d m m |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | E | W |  |  |  |  |  |  |
| d d d m m |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EE (course) d d d |  |  |  |  |  |  |  |  |  | FF (speed, knots) kn kn 1/10 |  |  |  |
| LL (intended track) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MM (radio station(s) guarded) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NN (date and time of next report, UTC) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D D H H M M |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PP (type and quantity of cargo/bunkers on board) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| QQ (brief details of defects/deficiencies/damage) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RR (brief details of pollution, including estimate of quantity lost) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SS (brief details of weather and sea conditions) direction direction WIND SWELL speed (Beaufort) height (m) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TT (contact details of ship’s owner/operator/agent) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UU (ship size and type) Length: (m) Breadth: (m) Draught: (m) Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |

(Table cont.)

XX	(Additional information) Brief details of incident:

Need for outside assistance:

Actions being taken:

Number of crew and details of any injuries: Details of P&I Club and local correspondent: Others:

Footnote: The alphabetical reference letters in the above format are from “General principles for ship reporting systems and ship reporting systems and ship reporting requirements, including guidelines for reporting incidents involving dangerous goods, harmful substances and/or marine pollutants” adopted by the International Maritime Organization by resolution A.851(20). The letters do not follow the complete alphabetical sequence as certain letters are used to designate information required for other standard reporting formats, e.g. those used to transmit route information.

  - WHOM TO CONTACT

Priority in any incident is the safety of life, the ship and cargo, and minimizing the effects of environmental pollution. If possible, the ship must report all incidents related to safety and pollution to the following contacts:

Coastal State Contacts:

CoastalState contact numbers are provided in Appendix I. Should an incident causing or threatening oil discharge occur while at sea, the Master should refer to this Appendix and select the closest State(s) whose waters may be threatened by the incident.

In the absence of a listed contact point or if any undue delay is experienced in contacting the listed responsible authority by direct means, the Master should contact the nearest coastal radio station, designated ship movement reporting station or Rescue Co-ordination Centre (RCC) by the quickest available means.

In order to expedite response and minimize damage from an oil pollution incident, it is essential that this notification be made without delay. This process should begin with the initial report set forth herein and required by Article 8 and Protocol 1 of the convention, made in accordance with the guidelines of this section.

Ship's Interest Contacts:

During an emergency the Master shall contact DPA. (S & Q) /Alternate DPA. (S & Q) and if due any reasons unable to contact DPA, he can contact any person on the ship’s contact list (provided in Appendix III) i.e. Fleet Supt/Fleet Director/Managing Director etc. and the first person Master makes contact with, shall remain his point of contact till the time ERT (Emergency Response Team) has assembled in the office and Master advised of same.

Unless expressly stated otherwise, notification to parties identified in the MARPOL Contingency Reporting System (see diagram) will be made by the Company as indicated.

Flag Administration Contact:

The Owner shall report any oil pollution incident to the Flag Administration, the contact details for your flag administration are contained within appendix III:

Notification Sequence:

A notification tree, specifying those responsible for informing the various parties having an interest in the ship, is set out in the following diagram. In areas where local law requires the ship to respond to all aspects of a spill, the ship's Plan must be coordinated with a shoreside Plan to guarantee that all parties have been notified and to avoid duplication of report efforts.

Emergency Port Contacts:

The ship's Emergency Port Contact Sheet must be completed, as much as possible, prior to arrival at each port visited and it must be kept in Appendix II until the vessel has departed the area.

Details that are not readily available should be obtained from the port's Ship's Agent, Harbour Master or other local authorities upon arrival at the port.

Should an incident occur while in port the Master must refer to the Emergency Port Contact Sheet for information on whom to contact.

# STEPS TO CONTROL DISCHARGE

#### MASTER’S RESPONSIBILITY AND AUTHORITY

At all times the Master has full authority to act in the best interests of his crew, ship, cargo and environment.

#### The Master has full authority to engage salvage tugs.

  - POLLUTION – GENERAL

Priority Actions: This section provides general considerations that apply to a wide range of incidents as follows:

In responding to a casualty, the Master's priority will be to ensure the safety of personnel, the ship, cargo and the environment and to take action to prevent escalation of the incident.

In casualties involving volatile petroleum spills, immediate consideration should be given to measures aimed at preventing fire and explosion, such as altering course such that the ship is upwind of the slick, shutting down non-essential air intakes, etc.

If the ship is aground and cannot manoeuvre, all possible sources of ignition should be eliminated and action taken to prevent flammable vapours from entering the accommodation and engine room spaces.

When it is possible to manoeuvre, the Master, in conjunction with the appropriate shore authorities, may consider moving the ship to a more suitable location in order, for example, to facilitate emergency repair work or lightering operations, or to reduce the threat posed to any particularly sensitive shoreline areas. Such manoeuvring is usually subject to Coastal State jurisdiction.

Prior to considering remedial action, the Master will need to obtain detailed information on damage sustained by the ship. A visual inspection should be carried out and all cargo spaces, bunker tanks and other compartments sounded. Due regard should be paid to indiscriminate opening of ullage plugs, sighting ports and/or void space accesses, especially when the ship is aground, as loss of buoyancy could result.

To assist in considering remedial action, identification of relevant hull stresses and damage stability should be made by reference to the ship's emergency response service.

Having assessed the damage to the ship, the Master will be in a position to decide what action should be taken to prevent or further minimize spillage.

When bottom damage is sustained, hydrostatic balance between the tank contents and the sea will be quickly achieved, especially if the damage is severe. In such case, the time available for preventative action will often be limited.

When significant side damage is sustained entirely below waterline in way of oil tanks, cargo or bunkers will be released fairly rapidly until hydrostatic balance is achieved. The rate of release will then be governed by the geometry of the penetration and the rate at which oil in the tank is displaced by water flowing into the tank beneath the oil.

If the below-waterline damage is fairly limited and restricted to one or two compartments, or the rate of inflow of water through the breach(es) does not exceed the capacity of the ship's pumps to move cargo and slack tankage permits, for example, consideration may be given to internally transferring oil from damaged to intact tanks with the ship's equipment to stem or mitigate discharge from the damaged tanks. If the inflow through hull breaches exceeds the ship's pump capacity, submersible pumps will be required to move oil "over the top" to other tanks, to lighter or ashore.

Total above-waterline side damage in way of oil tanks will result in bunkers or cargo release for the contents of the tank above the lowest point of hull penetration. Again, if the tank involvement is limited and slack tankage permits, consideration may be given to internally transferring oil from damaged to intact tanks to secure against further discharge of oil.

Before any response is considered the nature of the cargo being carried should be considered. In particular the following elements should be assessed

Solubility of cargo Density of cargo

Water reactivity of cargo Solidification of cargo

Compatibility of cargo with contents of other tanks on board

Substantial side hull penetration extending above and below the waterline will generally result in the loss of the entire contents of the damaged tank and any tanks common with that tank as water replaces the portion of the tank contents below the waterline while the portion above flows out through the above-the-waterline penetration.

In all cases of operational spill it is essential that steps are quickly taken to:

- minimize the amount spilled

- Contain that which has been spilled

- Clean up that which has been spilled

    - Collection and disposal of residues

All vessels are provided with a spill clean up kit and it is essential that this is kept readily available for use during all operations involving the transfer of oil or other pollutants.

Steps should be taken to collect all spilled oil or NLS into containment carried on board the vessel , this containment may be oil drums or where possible slop tanks.

Oil collected into oil drums and any absorbent material used during the clean up must be disposed of ashore or, if the vessel has suitable equipment, incinerated on board.

## If the vessel is unable to dispose off residues by using on board resources alone, the Master may engage a shore based clean up company. Whenever possible the Master should confirm the arrangements with the managing office prior to contracting such a company.

    - Operational Spills

This Section covers the incidents that may occur on a vessel.

The actions to be taken are guidelines and if the Master wants to deviate from them for the safety of life, the ship, cargo or the environment, he has the full authority to do so.

If an incident occurs that is not covered by one of these examples, the Master must take appropriate action to safeguard lives, the safety of the ship and cargo, and the protection of the environment.

Additionally, the Master must take into account the following when determining whether salvage assistance may be needed or not.

      - Nearest land or hazard to navigation

      - Vessel’s set and drift

      - Location and possible time of impact with any hazard – based on vessel’s set and drift.

      - Estimated time of casualty repair and,

      - Determination of nearest capable assistance and its response time.

## Pipe Leakage

Pipe Leakage may occur during the transfer of cargo (oil or NLS) or bunkers. At all times they must be dealt with quickly and in compliance with Port and Flag State Regulations. Pipeline spills include leaks at the manifold. For tankers in the USA the Vessel Response Plan (VRP) must be followed. The following table gives guidance on actions to be taken in the event of pipe leakage and checklist No.1 contained in appendix V should be used.

Initial Duties

| Person who Discovers an Oil Spill | Duty Officer - Deck/Bridge | Duty Engineer |
| --- | --- | --- |
| 1 Raise the alarm | 1 Stop all operations and close all valves | 1 Stop all operations and close all valves . |

Emergency Duties and Actions

| Chief Officer - Spill Officer | Master | Emergency Party/Technical Party |
| --- | --- | --- |
| - Ensure efficient communications with bridge. - Mobilize Spill Squad. - Ensure that all oil operations are stopped. - Ascertain - the cause and location of the spill; - the quantity of oil spilled; - whether or not the source of the spill has been stopped; - Whether or not any discharge has gone overboard or is in imminent danger of doing so. - Direct the spill mitigation, containment, and removal and clean- up activities of the Emergency Party. - If an overboard discharge has not occurred, evaluate the likelihood of such a discharge and the necessity of mobilizing shoreside assistance as a precaution. - Take samples of the fuel oil and/or bunkers on board, and, if practicable, samples of any oil slick. | - Comply with International Regulations. - Make necessary notifications e.g.: - nearest Coastal State; - local Port Authorities; - In US - Qualified Individual; - Terminal Operators; Using the Initial Notification Form. - Inform Company. - Commence record of incident and inform crew of situation. - Continually up-date the relevant authority as soon as new information becomes known. - Safety Items to consider: - Is there danger of fire? - Is the crew safe from the toxic or other health hazards of the spill? - Will the stability conditions of the ship be affected by the spill? - Pollution items to consider: - Are there alternative tanks into which oil can be transferred? - Stop ER sea suction if oil in water from spill. | - If leakage is from on-deck piping, cargo lines or bunker hose, drain the affected section to empty or slack tanks. - If leakage at sea valves, relieve pressure in the affected lines. - If leakages from sea chest fill the space between sea chest valve and inboard block valve with water to isolate the sea chest. - Check all scuppers to ensure that they are plugged - Check that deck containment systems are adequate. - Establish portable pumps to put excess spillage into slack or slop tanks or other suitable containment equipment. - Un-pumpable spillage must be removed in liquid form so far as practicable into containers for removal to slop tanks or containers. Clingage remaining after these actions will be mopped up using sorbents, rags and/or cleaning agents as necessary. |

## Tank Overflow during Cargo or Bunker Operations

of cargo (oil or NLS) and

Tank overflow may occur during internal or external transfer operations

bunkers. At all times they must be dealt with quickly and in compliance with Port and Flag State Regulations. For tankers in the USA the Vessel Response Plan (VRP) must be followed. The following table gives guidance on actions to be taken in the event of tank overflow and checklist No.1 contained in appendix V should be used.

Initial Duties

| Person who Discovers an Oil Spill | Duty Officer - Deck/Bridge | Duty Engineer |
| --- | --- | --- |
| 1 Raise the alarm | 1 Stop all operations and close all valves | 1 Stop all operations and close all valves . |

Emergency Duties and Actions

| Chief Officer - Spill Officer |  |  |  | Master | Emergency Party/Technical Party |
| --- | --- | --- | --- | --- | --- |
| Ensure efficient communications with bridge. |  |  |  | - Comply with International Regulations. - Make necessary notifications e.g.: - nearest Coastal State; - local Port Authorities; - In US - Qualified Individual; - Terminal Operators; Using the Initial Notification Form. - Inform Company. - Commence record of incident and inform crew of situation. - Continually up-date the relevant authority as soon as new information becomes known. - Safety Items to consider: - Is there danger of fire? - Is the crew safe from the toxic or other health hazards of the spill? - Will the stability conditions of the ship be affected by the spill? - Pollution items to consider: - Are there alternative tanks into which oil can be transferred? - Stop ER sea suction if oil in water from spill. - Where transfer actions are required to reduce spillage, cargo/fuel quality and segregation concerns should be secondary | - If leakage is from on-deck piping, cargo lines or bunker hose, drain the affected section to empty or slack tanks. - If leakage at sea valves, relieve pressure in the affected lines. - If leakages from sea chest fill the space between sea chest valve and inboard block valve with water to isolate the sea chest. - Check all scuppers to ensure that they are plugged - Check that deck containment systems are adequate. - Establish portable pumps to put excess spillage into slack or slop tanks or other suitable containment equipment. - Un-pumpable spillage must be removed in liquid form so far as practicable into containers for removal to slop tanks or containers. Clingage remaining after these actions will be mopped up using sorbents, rags and/or cleaning agents as necessary. |
| - Mobilize Spill Squad. - Ensure that all oil operations are |  |  |  |  |  |
|  | stopped. |  |  |  |  |
| 3 Ascertain: - the cause and location of the spill; |  |  |  |  |  |
|  | - the quantity of oil spilled; - whether or not the source of the spill has been stopped; |  |  |  |  |
|  | - Whether or not any discharge has gone overboard or is in imminent danger of doing so. |  |  |  |  |
| 4 Direct the spill mitigation, containment, and removal and clean-up activities of |  |  |  |  |  |
| the Emergency Party. 5 If an overboard discharge has occurred, immediately call for shore side assistance. DO NOT PUT CHEMICALS INTO WATER. |  |  |  |  |  |
|  | If an overboard discharge has not occurred, evaluate the likelihood of such a discharge and the necessity of mobilizing shoreside assistance as a |  |  |  |  |
| precaution. 6 Reduce the level in the overflowing tank by opening slack tanks and draining oil into |  |  |  |  |  |
| them. 7 Take samples of the fuel oil and/or bunkers on board, and, if practicable, samples of any oil slick. |  |  |  |  |  |
| 8 | Prepare ships pumps for possible |  |  |  |  |
|  | discharge ashore |  |  |  |  |

## Hull Leakage

For tankers in the USA the

Hull leakage may occur as a result of a failure in the hull structure. At all times they must be dealt with quickly and in compliance with Port and Flag State Regulations.

Vessel Response Plan (VRP) must be followed.

The following table gives guidance on actions to be taken in the event of hull leakage and checklist No.2 contained in appendix V should be used.

In the event of a suspected hull fracture oil should be transferred out of the tanks in question or a water bottom introduced to prevent further leakage. When decisions are being made with respect to the transfer of cargo, introduction of a water bottom and subsequent repair of the hull structure consideration must be given to the stability and stresses being place on the vessel’s structure.

Initial Duties

| Person who Discovers an Oil Spill | Duty Officer - Deck/Bridge | Duty Engineer |
| --- | --- | --- |
| 1 Raise the alarm | 1 Stop all operations and close all valves | 1 Stop all operations and close all valves . |

Emergency Duties and Actions

| Chief Officer - Spill Officer | Master | Emergency Party/Technical Party |
| --- | --- | --- |
| Ensure efficient communications with bridge. - Mobilize Spill Squad. - Ensure that all oil operations are stopped. - Ascertain: - the cause and location of the spill; - the quantity of oil spilled; - whether or not the source of the spill has been stopped; - Whether or not any discharge has gone overboard or is in imminent danger of doing so. - Direct the spill mitigation, containment, and removal and clean-up activities of the Emergency Party. - If an overboard discharge has occurred, immediately call for shore side assistance. DO NOT PUT CHEMICALS INTO WATER. If an overboard discharge has not occurred, evaluate the likelihood of such a discharge and the necessity of mobilizing shoreside assistance as a precaution. - Reduce the level in the overflowing tank by opening slack tanks and draining oil into them. - Take samples of the fuel oil and/or bunkers on board, and, if practicable, samples of any oil slick. . | - Comply with International Regulations. - Make necessary notifications e.g.: - nearest Coastal State; - local Port Authorities; - In US - Qualified Individual; - Terminal Operators; Using the Initial Notification Form. - Inform Company. - Commence record of incident and inform crew of situation. - Continually up-date the relevant authority as soon as new information becomes known. - Safety Items to consider: - Is there danger of fire? - Is the crew safe from the toxic or other health hazards of the spill? - Will the stability conditions of the ship be affected by the spill? - Pollution items to consider: - Are there alternative tanks into which oil can be transferred? - Stop ER sea suction if oil in water from spill. - Where transfer actions are required to reduce spillage, cargo/fuel quality and segregation concerns should be secondary | - Stop all oil movement in vessel by stopping shoreside or vessel pumps. - Where difficulty is experienced in identifying source of spill, tank ullages should be checked against loading or estimated used/discharged ullages for unexplained changes. - Commonalty between tanks either through leakage in common bulkheads, pipelines or valves can fail, check tank levels of adjacent tanks. - If leakage is identified or suspected to be coming from the bottom shell or near the turn of the bilge shell plating, further leakage can be prevented after the hydrostatic head is reduced by pumping water into the tank to establish a water bottom buffer cushion under the remaining tank contents. - Should spillage be identified during de-ballast operations, potential leakage of cargo lines passing through ballast tanks should be suspect. - Unexplained continuing oil discharge from the ship could originate from residual oil in the condensers/heat transfer units, taken in through the sea chest during spillage or passage of a slick past the ship. Such oil can be deposited on the condenser/heat transfer unit surfaces and then gradually discharge through the overboard discharge during subsequent routine operation of the cooling system. |

  - SPILLS RESULTING FROM CASUALTIES

## Grounding

In the event of a vessel touching bottom or going aground whilst on passage the following priorities must be observed: safety of life; control of damage to the vessel and cargo; and prevention of environmental pollution. This section should also be utilized for vessels touching bottom whilst alongside and those sections relevant to vessels on passage should be considered as not applicable.

Before taking any action to re-float the vessel, Master should carefully consider their current situation and all options available to them. When assessing what actions should be taken the following should be considered:

      - Has there been any injuries on board

      - Has the ship’s hull been broached or is there any risk of damage to the hull; where possible visually inspect hull

      - Is there any pollution or risk of pollution

      - ullage ports and sounding caps should not be opened indiscriminately as loss of buoyancy could result

      - Eliminate all avoidable sources of ignition and ban all smoking on board

      - What is the nature of the seabed in the area of grounding

      - What is the extend of the grounding i.e. loss of draft

      - What is the tidal range in the area and consider the possibility that the vessel may re-float in the near future

      - When considering the use of astern engine movements be aware that the propeller wash may make the situation worse

      - Consider any loss of stability and possible further risk to the vessel on re-floating

      - When considering the possibility of discharge or transfer of ballast or cargo take into account stability and stresses being placed on the hull

If the risk of additional damage to the vessel by attempting to refloat it by its own means is assessed to be greater than by remaining aground until assistance has been obtained, the Master should try to prevent the vessel from moving from its present position by;

      - Using anchors

      - Taking in ballast in empty tanks (if possible)

Attention should be given to hull stress and damage stability information, referring to the Company emergency response team for guidance on how to obtain the necessary information.

The Master must carefully evaluate the refloating process considering the following points:

      - whether ship is damaged in such a way that it might sink, break-up or capsize after refloating,

      - whether the ship after refloating might have any manoeuvring problems and may need salvage assistance

      - suitable weather conditions for refloating

      - stability, trim and list might change after refloating

      - whether machinery, rudder or propeller are damaged due to grounding or may be damaged during refloating

      - Whether there is any latent damage to the vessel and equipment. For example crank case deflections should be taken to ascertain if the stresses caused by the grounding have distorted the main engine.

Should the ship experience unusual vibrations of hull or unexplained movements, it could be that the ship has touched bottom or a submerged object. Consult navigation charts, Pilot if applicable, and check for visible oil traces in the wake. Plot position of ship and note depth readings. Take the same actions as if ship has run aground.

To assist the Master in the event of a grounding, checklist No.3 contained in appendix V should be completed.

Initial Duties

| Officer of the Watch | Master | Engineer of Watch |
| --- | --- | --- |
| - On feeling the vessel grounding put the engines to stop or, if fitted, use the Emergency Stop Button. - Sound the Emergency Alarm. - Plot the vessel's position. - Advise the Duty Engineer about the situation. - Display the International Signals for a vessel aground. | - On hearing Emergency Alarm go to the Bridge and receive a full briefing from Duty Officer. - Take over command and assume full control of the situation. - Ensure a muster check is made and all persons are accounted for. Advise Emergency Party of any missing crew. | - If safe to do so, Watch keepers will remain on watch until ordered to leave the ER. - Chief Engineer and Electrical Officer should contact Bridge prior to entering the ER. If on duty in ER they should contact the Bridge. - Duty Engineer during UMS operation will contact the Bridge prior to entering the ER. - Check engine suction. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| - Muster - Establish Communication. - Complete Muster Check. - Take soundings of all tanks, holds and bilges. Take soundings of water depth around vessel. Pass the information immediately to Bridge. - Locate source, if any, of oil or cargo spill. At night a white cloth lowered over the side can be used to check for oil leak. - If possible the Chief Officer and Chief Engineer will assess damage and report to the Bridge. - Ensure all plugs, sighting ports etc. are not left open as a loss of buoyancy may result. - Use loadicator to assess: -drafts required to refloat vessel; -ballast, if any, to discharge to refloat vessel; -stability of vessel on refloating; -stresses of vessel on refloating. - On refloating, all compartments sounded and recorded. - A full inspection of the vessel to be carried out. - After each action a report must be made to Bridge. | - Act as back up to Emergency Party. - Stand-by Party will muster outside Hospital. - Prepare lifeboats and liferafts. - Conduct search and rescue as directed by the Chief Officer. - On Master's orders prepare anchors for release. | - Get assessment of hull damage and situation: - Is there any obvious pollution from cargo or fuel? - Are holds or tanks taking in water? - What are the soundings around the vessel? - What are the charted soundings for the position? - What are the prevailing and forecast weather conditions? - What are the tidal predictions? - If vessel is aground, notify the nearest State's authorities as required under MARPOL and National Regulations. - Inform the Company of the situation. - Send emergency message to vessels and Coastal States in vicinity. - Commence record of incident and inform crew of situation. - After assessment of the situation consider: - the use of astern power to refloat vessel; - the condition of the vessel and whether it is likely to sink, break-up or capsize once it is off the bottom; - should salvage assistance be sought; - the likelihood of further damage to the vessel's hull, rudder and machinery are warranted by immediate removal action. - In a tidal area consider waiting for the rising tide before maneuvering. - Transfer oil cargo/bunkers from any tanks that are identified as being holed. - Prepare detailed tank/cargo disposition plan for Damage Stability Organization. - Except for the immediate safety of the vessel, do not discharge oil into the sea to reduce draft; lightering should be carried out to barges or other vessels. - If weather is deteriorating, consider ballasting down. - If the vessel is not aground: - the actual position has been verified; - it has been checked for pollution and water ingress while stopped; - proceed to a port of refuge for hull examination, etc. |

## Fire / Explosion

It is particularly difficult to bring fires under control because of the multiple sources of combustible material available and the complex structures found in Machinery Spaces, Pump Room, etc.

In all cases the ability to control the fire will depend on the size and extent of the fire compared to the nature and condition of the fire-fighting systems available onboard the vessel and the capabilities and training of the crew.

If fires cannot be contained by the ship's equipment it is imperative that the vessel either seeks a port of refuge to obtain assistance or contacts fire-fighting vessels. For United States waters check VRP for contractors.

Fire often breaks down the structural containment systems and inspections of adjoining compartments to a fire must always be carried out, including (if possible) spaces above and below the seat of the fire.

To deal with a fire / explosion effectively, speed is essential and actions taken must be prompt and effective. Fire parties should operate as a minimum in pairs and never exposed to more risk than is absolutely necessary to control the fire.

The initial actions should include:

- Sounding of the alarm

- Muster of crew and determine any personnel are missing or injured

- Request assistance if possible

- Determine the nature, extent and cause of the fire

checklist 4~8 (as applicable)

Is there any pollution or possibility of pollution associated with the fire Sources and locations of fires on board vessels are many and varied.

To assist the Master, event of a fire.

in appendix V has been prepared for use in the

In the event of a fire in port, the Master should alert the authorities as soon as possible and request assistance. When the shore fire teams arrive on board the ship’s staff must co-operate closely with them and ensure that maximum use is made of their expertise and resources. As a minimum, when the fire team arrives on board, the Master must provide the following information to the Senior Fire Officer:

- location and type of emergency;

- information from the vessel's fixed fire detection equipment;

- details of the vessel's stability;

- action already taken;

- vessel's equipment and manpower available;

- tank and hold disposition, cargo contained and associated hazards, if known, including the Material Safety Data Sheet;

- Number of casualties and persons missing, if any.

Post-fire Considerations

When the fire is under control or extinguished it is necessary to take the following actions:

- Test all vessel spaces for hydrocarbon gas.

- Sound all tanks and holds to check for oil or water ingress.

- In gaining access to spaces, care must be taken to ensure that progressive flooding or loss of critical buoyancy is not induced by opening access.

- Use inert gas wherever possible in cargo tanks.

- Before transferring water or oil from tanks, ensure that damage stability is considered regarding transverse and longitudinal stability, bending moments and shear force.

- Ensure that fire watches and special precautions to prevent the introduction of ignition sources are maintained until all risk of fire is exhausted.

Initial Duties

| Person who Discovers the Fire |  | Engineer of Watch |  | Officer of Watch |  |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 | Operate the nearest Fire Alarm. Advise Engineer of Watch. Inform Officer of Watch. Minor Fires Only - use portable fire extinguisher to put out fire. | 1 2 3 4 5 | For ER Watch: Raise alarm and report situation to Bridge. For UMS operation: contact Bridge before entering ER. Stop any machinery on fire. Start additional generator if more electrical power is needed. Wait for Emergency Party in Control Room and, on their arrival, give them assistance. | 1 2 3 4 5 | Sound Fire Alarm, General Alarm and ship's whistle. Shut down accommodation fans. Make preparations for closing electrical power and main propulsion (blackout). Hand over to Master with full information regarding the situation. Hand over to Duty Officer and go to Fire Station. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| After each action, a report must be made to Bridge. - Muster at ER entrance or, if UMS, at Fire Station. - Safe running of machinery must be given a high priority. - Water tight doors must be closed, all vents and fire flaps to be shut down. - Two-man Fire Fighting Team with full fireman's outfit and CABA ready to fight fire. - Determine the extent of the fire. - Fire Fighting Party to fight fire with two hoses, one with foam attachment and one using water wall for protection. - If alternators to shut down ensure that the emergency alternator is started. - Main Engine to be stopped by Bridge order. - When operations fail, preparations to be made for use of fixed system where possible. | - Responsible for search and rescue of personnel. - Rig hoses at nearest entrance to ER with foam equipment. - Prepare lifeboats and liferafts for launching. - Assemble oxygen equipment, rescue harness and line for evacuation. - Assist the ER Emergency Party as required by Master. | - Commence record of incident and inform crew of situation. - Muster list checked and search and rescue organized. - Comply with International Regulations. - Steer vessel away from nearest danger - keep in mind the vessel could lose its electrical power or main propulsion. - Send emergency message to vessels and Coastal States in vicinity. - Inform Company. - Safety Items to consider: - Main Engine to be ordered to stand-by or stopped as required; - Display the necessary lights or shapes, e.g. NUC; - Steering gear to be switched over to manual control; - Emergency Fire pump to be started; - Fixed installation to be operated as necessary; - Abandoning vessel. - Pollution items to consider: - if main engine shut down, isolate all fuel tanks, boundary cool day tanks, etc.; - if blackout was to occur, how close is the nearest danger that the vessel could ground itself upon and cause a pollution hazard; - Make arrangements for towage assistance. |

## Collision with Fixed or Moving Object

If the vessel is involved in a collision with a fixed or moving object , the extent of injuries to personnel and the damage to both parties shall be identified and necessary steps taken to safeguard both vessels and crews and minimize damage to the environment.

Masters must take all steps possible to minimize the impact of the collision and ensure the safety of his crew, ship and the environment. The following points should be considered when taking any actions:

      - Minimize possibility of any further injuries to crew

      - Confirm vessels position and navigational situation

      - Is there any fire or risk of fire

      - It may be safer for the vessels to stay interlocked because of the risk of a spark causing a fire / explosion and the fact that pulling apart may allow a sudden ingress of water to one or both vessels and cause or increase pollution

      - Assess vessels watertight integrity and possible loss of watertight integrity

      - Is there any pollution or risk of pollution and assess best actions to minimize further pollution e.g. transfer of cargo or bunkers from damaged tanks to secure tanks

      - Alerting of coastal states and calling for assistance if appropriate

      - Masters are reminded of the legal duty to assist the other vessel.

To assist the Master in the event of a collision, checklist No. 9 contained in appendix V should be completed.

Initial Duties

| Officer of Watch | Engineer of Watch | Master |
| --- | --- | --- |
| - Evasive action to be used prior to collision. - Sound General Alarm. - Advise ER about the situation, if manned. - Plot vessel's position. - Display appropriate lights or shapes, e.g. NUC. | - Check ER bilge, tanks, cofferdams for leakage. - During UMS operation contact Bridge. - Start additional generator if more electrical power is required. | - On hearing the General Alarm go to Bridge. - After full briefing from Officer of Watch, assume full control of the situation. - Ensure muster check is completed and all persons are accounted for. - Commence record of incident and inform crew of situation. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| - Muster at Fire Station. - Complete Muster Check. - Check all watertight doors are shut. - Take soundings of all tanks, holds and bilges. - Chief Officer and Chief Engineer to assess damage, establish the position and extent of damage. - Extinguish any fires that have resulted from the collision. - Take any necessary action to stop the ingress of water. - Check for signs of pollution. - After each action a report must be made to Bridge. | - Act as back up to Emergency Party. - Prepare lifeboats and liferafts. | - Send a Urgency message. - Comply with International Regulations and MARPOL Provisions. - Inform nearest CoastalState of potential pollution. - Inform Company. - Safety Items to consider - Is there danger of sinking? - Are the ships interlocked? Is it more prudent to the safety of the vessels that they stay interlocked? - Are the vessels of greater danger to traffic in the area if they remain interlocked? - Will separation cause sparking or increase the risk of fire or explosion? - Is there any danger of fire or explosion onboard either vessel, if so, is there risk of it spreading to the other vessel? - Pollution items to consider - Are any tanks penetrated above the waterline? - Is there penetration of the ship's hull below the waterline caused by a bulbous bow? - Is any oil spilling at present and, if so, is the spill large or small? - Are there any tanks that spilling oil can be transferred to? - If separation of vessels is considered - plan to position the vessel upwind of the oil slick; - shut all non-essential air intakes; - isolate the penetrated tank(s) by hermetically sealing them as much as possible. - Consider stopping ER sea suction if oil in water. |

## Hull Failure

Hull failure may occur in certain contingencies or because of a defect weld or design fault that causes the hull to fracture. Excessive steel wastage can also result in failure and leakage from the hull.

The following table gives guidance on actions to be taken in the event of hull leakage and checklist No. 2 contained in appendix V should be used.

Initial Duties

| Person who Discovers an Oil Spill | Duty Officer - Deck/Bridge | Duty Engineer |
| --- | --- | --- |
| - Any one seeing or suspecting an oil spill must immediately notify the Duty Officer. - The speed of notification is essential. - If the vessel is transferring oil, stop all oil transfer operations onboard or ashore. | - On being informed of a spill or suspected spill, all loading, discharge, transfer, bunkering, ballasting, gas freeing and tank cleaning operations must be stopped and valves closed. Openings in Cargo and Consumable Tanks must be closed. - If in port, inform the terminal operators by the agreed communication method. - Inform Master and Duty Engineer of spill or suspected spill. | - On being informed of a spill or suspected spill, all loading, discharge, transfer, bunkering, ballasting, gas freeing and tank cleaning operations must be stopped and valves closed. Openings in Cargo and Consumable Tanks must be closed. - If leakage occurs when bunkering, inform bunker barge, tanker or facility by the agreed communication method. - Inform Bridge, Master and Chief Engineer of spill or suspected spill. |

Emergency Duties and Actions

| Chief Officer - Spill Officer | Master | Emergency Party/Technical Party |
| --- | --- | --- |
| After each action a report must be made to | 1 Comply with International Regulations | Hull Leakage - can occur when loading cargo, bunkering, or internally transferring fuel or cargo and stress has been put on tank welds or pipe joints. Contact with the quay, flotsam or other vessels can also cause hull leakage. Vessels in heavy seas can also experience hull failure. - Stop all oil movement in vessel by stopping shoreside or vessel pumps. - Close the manifold valves after pumps have stopped. - Where difficulty is experienced in identifying source of spill, tank ullages should be checked against loading or estimated used/discharged ullages for unexplained changes. - Commonalty between tanks either through leakage in common bulkheads, pipelines or valves can fail, check tank levels of adjacent tanks. - If leakage is identified or suspected to be coming from the bottom shell or near the turn of the bilge shell plating, further leakage can be prevented after the hydrostatic head is reduced by pumping water into the tank to establish a water bottom buffer cushion under the remaining tank contents. - Should spillage be identified during de- ballast operations, potential leakage of cargo lines passing through ballast tanks should be suspect. - Unexplained continuing oil discharge from the ship could originate from residual oil in the condensers/heat transfer units, taken in through the sea chest during spillage or passage of a slick past the ship. Such oil can be deposited on the condenser/heat transfer unit surfaces and then gradually discharge through the overboard discharge during subsequent routine operation of the cooling system. |
| Bridge. 1 On notification of a spill, mobilize Emergency Party. | and MARPOL Provisions. 2 Inform the relevant persons and organizations regarding oil spill or |  |
| - Ensure that all oil operations are stopped. - Ascertain: | potential oil spill: - nearest Coastal State; - local Port Authorities; |  |
| - the cause and location of the spill; - the quantity of oil spilled; - whether or not the source of the spill has been stopped; - Whether or not any discharge has | - In US - Qualified Individual; - Terminal Operators; Using the Initial Notification Form. - Inform Company. - Commence record of incident and |  |
| gone overboard or is in imminent danger of doing so. 4 Reduce the level in the leaking tank by | inform crew of situation. 5 Continually up-date the relevant authorities as soon as new information |  |
| opening slack tanks and draining oil into them. 5 If necessary, direct the spill mitigation, containment, and removal and clean-up activities of the Emergency Party using onboard spill equipment. | become known. 6 Safety Items to consider: - Is there danger of fire? - Is the crew safe from the toxic or other health hazards of the spill? - Will the stability conditions of the |  |
| 6 If an overboard discharge has occurred, immediately call for shore side assistance. DO NOT PUT CHEMICALS | ship be affected by the spill? 7 Pollution items to consider: - Are there alternative tanks into |  |
| INTO WATER. 7 Take samples of the fuel oil and/or bunkers on board, and, if practicable, samples of the oil slick. | which oil can be transferred? - If oil in water from spill, stop ER sea suction. - Where transfer actions are required to reduce spillage, cargo/fuel quality and segregation concerns should be |  |
|  | secondary. |  |

## Excessive List

In the event of an excessive list being experienced, the following priorities must be observed: safety of life; control of damage to the vessel and cargo; and prevention of environmental pollution. Personnel must not take risks which may endanger their own lives or the lives of any other crew members.

If excessive list occurs rapidly and unexpectedly, it may be due to:

Failure of the hull plating

Failure of an internal bulkhead between compartment shift of cargo Flooding of the engine room, where free surface can cause a list Damage through grounding or collision

Incorrect operational procedures

Depending on what the vessels situation is, i.e. alongside working cargo or at sea, the following steps should be taken immediately:

Stop any cargo, bunkering or ballast operations in progress Sound the General Emergency Alarm and muster the crew If under way, reduce speed or stop

Establish reason for list

Once the situation is stabilised the following should be considered:

Sound all tanks and compare soundings with departure soundings

If oil has spilled, or it is necessary to jettison oil for the safety of the crew and vessel and in order to maintain stability, inform the appropriate parties in accordance with Section 2 of this Plan

If possible, take corrective action to rectify the situation.

To assist the Master in the event of excessive list, checklist No. 10 contained in appendix V should be completed

Initial Duties

| Officer of the Watch | Master | Engineer of Watch |
| --- | --- | --- |
| - In the event of an excessive list occurring, all cargo work, bunkering, transferring of oil or water must stop immediately. - Sound the General Emergency Alarm. - Advise the Duty Engineer about the situation. | - On hearing General Emergency Alarm, go to the Bridge and receive a full briefing from Duty Officer. - Take over command and assume full control of the situation. | - If safe to do so, Watch keepers will remain on watch until ordered to leave the ER. - Chief Engineer and Electrical Officer should contact Bridge prior to entering the ER. If on duty in ER they should contact the Bridge. - Duty Engineer during UMS operation will contact the Bridge prior to entering the ER. - Check that list is not caused by water in ER by immediately taking bilge soundings. - Check sea suction is not drawing in oil from the sea. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| On leaving the Muster Station the Emergency Party must use a portable VHF to communicate with the Bridge. After each action a report must be made to Bridge. - Muster at Fire Station No.2, Chief Officer will lead the Emergency Party. - Complete Muster Check. - Check all watertight doors are shut. - Take soundings of all tanks, holds and bilges and pass the information immediately to the Bridge. - Chief Officer must determine the cause of the list and, if possible, take appropriate action to correct both the list and the cause of the list. - If cause of list not obvious, compare existing ullages and soundings with loading ullages and soundings, make allowance for the for the list. - If the list has been caused by flooding of empty tanks by sea water, ensure that water pumped out is clean and will not pollute sea. | - Act as back up to Emergency Party. - Stand-by Party will muster outside Hospital. - Prepare lifeboats and liferafts. | - Get assessment of any damage to the vessel, its equipment and cargo. - Could there be any pollution from cargo or fuel spilling from tanks? - What are the prevailing and forecast weather conditions? - Notify the nearest State's authorities as required under MARPOL and National Regulations. - Inform the Company of the situation. - Commence record of incident and inform crew of situation. - Prepare detailed tank/cargo disposition plan for Damage Stability Organization. - Safety Considerations: - the list could increase and turn the vessel over; - when trying to put vessel upright, always fill the lower tanks first, this increases list but also increases stability; - If flooding rate is greater than pump capacity, emergency pumps will be required to be brought onboard. |

## Containment System Failure

Should the containment system fail it is essential that all transfer of oil is immediately stopped and the containment system fully examined by a competent person to ascertain the reason for the system failure. If any oil spillage has occurred, as a result of the containment system failure, immediate action should be taken to stop the spillage.

To assist the Master in the event of Containment System Failure, checklist No.11 contained in appendix V should be completed.

Initial Duties

| Person who Discovers an Oil Spill | Duty Officer - Deck/Bridge | Duty Engineer |
| --- | --- | --- |
| - Any one seeing or suspecting an oil spill must immediately notify the Duty Officer. - The speed of notification is essential. - If the vessel is transferring oil, stop all oil transfer operations onboard or ashore. | - On being informed of a spill or suspected spill, all loading, discharge, transfer, bunkering, ballasting, gas freeing and tank cleaning operations must be stopped and valves closed. Openings in Cargo and Consumable Tanks must be closed. - If in port, inform the terminal operators by the agreed communication method. - Inform Master and Duty Engineer of spill or suspected spill. | - On being informed of a spill or suspected spill, all loading, discharge, transfer, bunkering, ballasting, gas freeing and tank cleaning operations must be stopped and valves closed. Openings in Cargo and Consumable Tanks must be closed. - If leakage occurs when bunkering, inform bunker barge, tanker or facility by the agreed communication method. - Inform Bridge, Master and Chief Engineer of spill or suspected spill. |

Emergency Duties and Actions

| Chief Officer - Spill Officer | Master | Emergency Party/Technical Party |
| --- | --- | --- |
| After each action, a report must be made to Bridge. 1 On notification of a containment failure | - Comply with International Regulations and MARPOL Provisions. - Inform the relevant persons and | Containment Failure - can occur when loading cargo, bunkering, or internally transferring fuel or cargo and stress has been put on tank welds or pipe joints. - Stop all oil movement in vessel by stopping shoreside or vessel pumps. - Close the manifold valves after pumps have stopped. - Where difficulty is experienced in identifying source of spill, tank ullages should be checked against loading or estimated used/discharged ullages for unexplained changes. - Commonalty between tanks either through leakage in common bulkheads, pipelines or valves can fail, check tank levels of adjacent tanks. - If leakage is identified or suspected to be coming from the bottom shell or near the turn of the bilge shell plating, further leakage can be prevented after the hydrostatic head is reduced by pumping water into the tank to establish a water bottom buffer cushion under the remaining tank contents. - Should spillage be identified during de- ballast operations, potential leakage of cargo lines passing through ballast tanks should be suspect. - Unexplained continuing oil discharge from the ship could originate from residual oil in the condensers/heat transfer units, taken in through the sea chest during spillage or passage of a slick past the ship. Such oil can be deposited on the condenser/heat transfer unit surfaces and then gradually discharge through the overboard discharge during subsequent routine operation of the cooling system. |
| organize Emergency Party. 2 Ensure that all oil operations are stopped. | organizations regarding oil spill or potential oil spill: - nearest Coastal State; |  |
| 3 Ascertain: - the cause and location of the spill; - the quantity of oil spilled; - whether or not the source of the spill has been stopped; - Whether or not any discharge has gone overboard or is in imminent danger of doing so. | - local Port Authorities; - In US - Qualified Individual; - Terminal Operators; Using the Initial Notification Form. - Inform Company. - Commence record of incident and inform crew of situation. - Continually up-date the relevant |  |
| 4 Reduce the level in the leaking tank by opening slack tanks and draining oil into them. | authorities as soon as new information becomes known. 6 Safety Items to consider: |  |
| 5 Direct the spill mitigation, containment, and removal and clean-up activities of the Emergency Party using onboard | - Is there danger of fire? - Are the crew safe from the toxic or other health hazards of the spill? |  |
| spill equipment. - If an overboard discharge has occurred, immediately call for shore side assistance. DO NOT PUT CHEMICALS INTO WATER. - Take samples of the fuel oil and/or | - Will the stability conditions of the ship be affected? 7 Pollution items to consider: - Are there alternative tanks into which oil can be transferred? - If oil in water from spill, stop ER |  |
| bunkers on board, and, if practicable, samples of the oil slick. | sea suction. - Where transfer actions are required to reduce spillage, cargo/fuel quality |  |
|  | and segregation concerns should be secondary. |  |

## Submerged/Foundered

If the vessel is in serious situation of being partly submerged or foundering as a result of an accident, explosion or other similar occurrence, the Master should consider the prevalent conditions and make preparations to immediately evacuate the vessel. Consideration should be given to the safety of the personnel on board and the prevention of possible pollution.

Weather conditions and the vessel’s situation will dictate the decision to abandon ship or otherwise

– the safety of lives must take precedence over all other considerations.

To assist the Master in the event of a submerged or foundered incident, checklist No.14 contained in appendix V should be completed.

Initial Duties

| Officer of the Watch | Master | Engineer of Watch |
| --- | --- | --- |
| - Immediately inform Master of the seriousness of the situation. - Sound the General Emergency Alarm. - Plot the vessel's position. - Advise the Duty Engineer about the situation. | - On hearing General Emergency Alarm go to the Bridge and receive a full briefing from Duty Officer. - Take over command and assume full control of the situation. - Assess the vessel’s situation - Give instructions to proceed to Boat Stations, if situation warrants. | - If safe to do so, Watch keepers will remain on watch until ordered to leave the ER. - Chief Engineer and Electrical Officer should contact Bridge prior to entering the ER. If on duty in ER, they should contact the Bridge. - Duty Engineer during UMS operation will contact the Bridge prior to entering the ER. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| - Muster - Establish Communication - Complete Muster Check. - Check all watertight doors are closed. - Take soundings of all tanks, holds and bilges and pass the information immediately to Bridge. - Locate source, if any, of ingress of water. - If possible the Chief Officer and Chief Engineer will assess damage and report to the Bridge. | - Act as back up to Emergency Party. - Stand-by Party will muster outside Hospital. - Prepare lifeboats and liferafts. | - Get assessment of hull damage and situation: - Could there be any pollution from cargo or fuel? - Which holds or tanks are taking in water? - What are the prevailing and forecast weather conditions? - Notify the nearest State authorities as required under MARPOL and National Regulations. - Inform the Company of the situation. - Commence record of incident and inform crew of situation. - Transfer oil cargo/bunkers from any tanks that are identified as being holed, if feasible. - Prepare detailed tank/cargo disposition plan for Damage Stability Organization. |

## Wrecked or Stranded

In the event of a vessel resting on the bottom the following priorities must be observed: safety of life; control of damage to the vessel and cargo; and prevention of environmental pollution. Personnel must not take risks, which may endanger their own lives or the lives of any other crew members. Where possible Masters should seek immediate shore based assistance.

To assist the Master in the event of the vessel becoming wrecked or stranded, checklist No. 3 contained in appendix V should be completed.

Initial Duties

| Officer of the Watch | Master | Engineer of Watch |
| --- | --- | --- |
| - When made aware of the vessel being stranded, stop all cargo work, ballasting, etc. - Sound the General Emergency Alarm. - Plot the vessel's position. - Advise the Duty Engineer about the situation. - Display the International Signals for a vessel aground. | - On hearing General Emergency Alarm, go to the Bridge and receive a full briefing from Duty Officer. - Take over command and assume full control of the situation. - Ascertain the condition of the seabed. - Advise shore authorities/Company | - If safe to do so, Watchkeepers will remain on watch until ordered to leave the ER. - Chief Engineer and Electrical Officer should contact Bridge prior to entering the ER. If on duty in ER, they should contact the Bridge. - Duty Engineer during UMS operation will contact the Bridge prior to entering the ER. - Check engine suction. - ER party will start additional alternators, if necessary. |

Emergency Duties and Actions

| Emergency Party /Technical Party | Stand-by Party | Master and Bridge Party |
| --- | --- | --- |
| - Muster - Establish Communication - Complete Muster Check. - Take depth soundings at the fore, aft and midship position (both sides) of the vessel. - Take soundings of all tanks, holds and bilges and pass the information immediately to Bridge. - Locate source, if any, of oil or cargo spill. At night a white cloth lowered over the side can be used to check for oil leak - If possible the Chief Officer and Chief Engineer will assess damage and report to the Bridge. - Use loadicator to assess: - drafts required to refloat vessel - ballast (if any) to discharge to refloat vessel - stability of vessel on refloating - stresses of vessel on refloating. - On refloating all compartments sounded and recorded. A full inspection of the vessel to be carried out. - Ensure all plugs, sighting ports etc. are not left open as a loss of buoyancy may result. | - Act as back up to Emergency Party. - Stand-by Party will muster outside Hospital. - Prepare lifeboats and liferafts. | - Get assessment of hull damage and situation: - Is there any obvious pollution from cargo or fuel? - Are holds or tanks taking in water? - What are the depth soundings around the vessel? - What are the charted soundings for the position? - What are the prevailing and forecast weather conditions? - What are the tidal predictions? - Notify the nearest State's authorities as required under MARPOL and National Regulations. - Inform the Company of the situation. - Commence record of incident and inform crew of situation. - After assessment of the situation consider method to use to refloat vessel. - In a tidal area consider waiting for the rising tide before maneuvering. - Transfer oil cargo/bunkers from any tanks that are identified as being holed. - Prepare detailed tank/cargo disposition plan for Damage Stability Organization. - Except for the immediate safety of the vessel, do not discharge oil into the water to reduce draft, lighter into barges or other vessels. - If oil pollution is evident from a known tank, reduce the Inert Gas pressure to that tank. |

## Hazardous Vapour Release

Whenever there is a real or perceived threat of the release of hazardous or toxic vapours, it is essential to immediately evacuate the area, to avoid the possibility of personnel inhaling toxic vapours. Care should be taken to ensure that vapour does not enter the accommodation or engine room as this may have disastrous consequences resulting in harm to the crew and possible explosions within engine room machinery; it should be understood that there may also be the threat of fire or explosion.

checklist 15

To assist the Master,

hazardous vapour release incident. (Deleted)

Initial Duties

in appendix V has been prepared for use in the event of a

Person who discovers the escape of any hazardous Vapor release should inform the Officer of the watch.

| Officer of the Watch | All personnel | Master |
| --- | --- | --- |
| - All cargo operations stopped. - Raise General Emergency Alarm. - All valves in the liquid line closed both on the ship and ashore as necessary. - All accommodation access doors shut and all ventilation (except closed-circuit systems) shut. - Start Water Spray and Fire pumps for dispersation of Liquid Spill and Vapour Emissions. - If at sea, adjust the ship is heading so that the vessel is free from gas cloud. - The terminal/port authorities informed. - Brief the Master of the situation. | - All bunkering, gas freeing, cleaning and ballast operations stopped, if any, and valves closed. - On hearing General Emergency Alarm, all personnel must quickly go to their Muster Stations. - Officer arriving at the Muster Station must contact the Bridge and determine the nature of emergency. - Smoking and naked lights prohibited everywhere on the ship, and electrical switches used as little as possible. | - Arrive to the Bridge and assume control of the situation. - Ensure that no personnel enter the area of hazardous vapour release until it has been confirmed safe to do so and that all personnel are suitably attired, with breathing apparatus and protective clothing. |

Emergency Duties and Actions

| Bridge Party | Emergency and Stand-by Party | Technical Party |
| --- | --- | --- |
| - Commence incident log and inform crew of the situation. - Muster list checked and search and rescue organised, if required. - A radio warning to all other ships in the vicinity broadcast (VHF CH16). - Co-ordinate actions of Emergency Parties. - Communicate with terminal/port authorities. - Safety Items to Consider: - liquid spillage and/or vapour emission occurred; - wind direction; - main engine to be order to stand-by or stopped as required; - electrical power to be isolated; - display necessary lights/shapes; - steering gear, if applicable, to be switched to manual control; - evacuating the personnel; - abandoning the ship (if this is required refer to sec.5.10.6) | - Complete Muster Check. - Assemble appropriate fire- fighting equipment and breathing apparatus for immediate use. - Emergency parties wear breathing apparatus and protective clothing. - Check Material Safety Data Sheet (MSDS) to ascertain hazards of cargo. - Rectify the leak, if possible. - Dispersation of liquid spill and vapour emissions by water. - Follow instructions from the Bridge. - Prepare lifeboats for launching if instructed by the Bridge. - Evacuate personnel if required. | - Ensure that there is sufficient power to run Water Spray and Fire pumps and start additional alternators if required. - Ensure that there is sufficient water pressure in Water Spray and Fire Systems. - Provide running Engine Room plant. - Leave the Engine Room when instructed. |

  - STABILITY AND STRESS CONSIDERATIONS

When taking actions to mitigate oil spillage or to free the ship from a strand by transfer of cargo or bunkers, great care should be taken to consider stability and stress factors with respect to the ship.

The Master should be fully aware that any substantial damage to the ship's structure may change her strength characteristics markedly. The impact of internal transfers on stress and stability may therefore be impossible for ship personnel to assess without outside assistance.

Prior to considering remedial action, the Master will need to obtain detailed information on the damage sustained by his ship. A visual inspection should be carried out and all cargo tanks, bunker tanks and other compartments should be sounded. Due regard should be paid to the indiscriminate opening of ullage plugs or sighting ports, especially when the ship is aground, as loss of buoyancy could result.

Before making any transfer of cargo or bunkers, the Master will accordingly seek assistance to ensure that the transient and resultant stability and stress changes imposed by such transfer are within the limits dictated by the ship's damage stability and stress plans or calculations as determined by those plans and/or by programmes maintained for that purpose by a technical organization capable of performing the salvage, stability and hull stress assessments details in Appendix III.

Once the Master has contacted the company, the Company's Emergency Response Team will activate the necessary procedures for providing technical assistance to the ship.

Information required for making damage stability and damage longitudinal strength assessment will be required from the vessel and will include:

- Distribution of cargo, fuel ballast, fresh water and other consumable or variable weights;

- Weights, quantity and/or ullages in compartments known or suspected to be open to the sea or known or suspected to be common with compartments open to the sea. The contents of breached compartments should be identified as oil, water or a mixture of the two. If possible in mixtures, the relative proportion of the two should be obtained.

- State whether drafts and angle of heel are observed or estimated and whether they are stable. If measurements are changing, the rate of change should be obtained.

- Any changes in loading carried out since the casualty along with weights changed and detail of the location change.

- If stranded, the extent of grounding as indicated by soundings at the bow, stern and equidistant points, port and starboard, in between. If tidal, the height of the tide when the soundings were taken, and the predicted tidal range for the immediate future.

- Information on the makeup of the sea bed and the firmness of the strand (whether the vessel is working or liable to slip off her strand).

- Weather forecast and outlook for the immediate future.

- As complete a report of damage which may affect the hull's residual strength as is possible. Included should be shell, bulkhead and longitudinal/transverse structural member damage giving extent and location of damage as specifically as is possible. The exact source of the data should be identified along with the method by which observations were made, and the identity and qualifications of those making them (divers, salvage personnel, officers etc.).

- Condition and availability of ship systems.

- Periodic updates containing corrections to initial reports and/or changes in conditions as information is developed.

#### LIGHTERING

Should the ship sustain extensive structural damage, it may be necessary to transfer all or part of the cargo to another ship. If lightering is required for this or any other purpose, the Master should refer to guidance for such transfer in the company's procedures for ship-to-ship transfer and in

checklist No. 16

addition lightering.

has been prepared to assist the Master in the event of an emergency

| The Master should be aware that the Coast state in which the lightering operation will take place is |  |
| --- | --- |
| likely to want to monitor the operation. The Master must ensure that any lightering | operation is |

carried out in close co-operation with the coast state.

#### PLANS, DRAWINGS AND SHIP-SPECIFIC DETAILS

To assist in responding to the above situations, certain plans, drawings and Ship-Specific details may need to be referred to. The following plans and loading arrangements must be available both on board in the ships office and in the PGSHIPS office:

General Arrangement Plan; Midship Section Plan; Tank / Cargo Pipeline Plan; Capacity Plan;

Load Line Assignment (Deadweight Scale); Light Ship Characteristics;

All Tank Disposition (Departure Condition, i.e.)

- Cargo Stowage Plan;

- Bunker Specifications and Stowage Information;

- Ballast Loading and Stowage Information;

Fire Plan

E.R. Piping Diagram

Trim and Stability (and damage) Book Docking Plan

Crew List

Shell Expansion

## Shipboard Spill Response Resources

The equipment to be carried on board the vessel to deal with shipboard spills is itemized in Section 5.

The Chief Officer is responsible for ensuring all equipment is as per the list regarding quantity and that it is at all times ready for immediate use.

The Master is responsible for ensuring that the crew are familiar with the various items on the list and that they are exercised in the use of these items.

#### MITIGATING ACTIVITIES

When the safety of both the ship and personnel has been addressed, the Master can initiate mitigating activities. The following activities should be considered as a minimum however these may vary according to the circumstances of the incident:

Assessment and monitoring requirements

Prior to initiating mitigating activities the Master should make a full assessment of the situation. This assessment should include the following points as a minimum:

Has spillage been stopped? Type of substance spilled Quantity of substance spilled

Where spillage has originated from has spillage been contained on deck or has it entered water

Data Sheets & Cargo Information

The Master should also consult any MSDS or Characteristics of Marine Chemicals Proposed for Marine Transport in Bulk data sheet. This gathering of information identifiesphysical properties of the chemical in question, special protective equipment or unusual response techniques required.

Once the situation has been assessed the Master should initiate the mitigating activities. Once initiated a member of the ships crew should be nominated to monitor the situation. This person should be responsible for monitoring the situation to ensure that themitigating activities are being successful and the situation is under control. Should it become apparent that the mitigating activities are not being successful and the situation is worsening the Master should carry out a reassessment of the situation.

Before any response is considered the nature of the cargo being carried should be considered. In particular the following elements should be assessed

Solubility of cargo Density of cargo

Water reactivity of cargo Solidification of cargo

- Compatibility of cargo with contents of other tanks on board

Personnel Protection

Before embarking on the mitigating activities it is essential that personnel engaged in the clean up are provided with suitable personal protective equipment. The requirements fro PPE will have to be assessed on each occasion and will depend upon the nature of the

substance spilled. As a minimum normal working clothes with full boiler suits and gloves, with slip resistant shoes or boots should be worn.

Threats to Health and Safety

Depending on the nature of the substance spilled there may be a threat to the health and safety of the personnel involved in the clean up. Before engaging on any clean up activities the Master should ascertain any risks associated with it. If it is not clear what therisks are or what actions should be take the master should contact the MSQS for advice.

Clean up activities should not be commenced until a full assessment of the Threats to Health and Safety have been made and risks to crew minimised

Response Techniques

There are many methods of responding to a spill of pollutant e.g. containment, dispersant, absorption, neutralisation etc. The Master must make an assessment of the most appropriate technique to use depending on the circumstances of the incident. It should be remembered that dispersant or any other chemicals should not be put into the water without the express permission of the local authorities.

Before deciding on the best technique to be used the Master should carefully consider the nature of the product in question. This information can be obtained from data sheets and cargo information carried on board.

Isolation Procedures

Depending on the nature of cargo it may be necessary to ensure that the spilled cargo is isolated from other cargo and or personnel. In such cases the crew members that have been exposed to the spilled substance must be isolated until they and their clothing have been decontaminated.

Decontamination of Personnel

Once the mitigating activities have been completed it is important that those personnel involved are decontaminated The nature and extent of the decontamination will depend on the substance involved but Masters should consider the following options as aminimum:

Water showers removal and disposal of all clothing

Before any decontamination is commenced the Master should consult the MSDS for the cargo and any other source available to assess which decontamination procedures are most appropriate.

Disposal of Oil and Clean Up Materials

Prior to completion it is essential that all substances which need to be disposed of are collected together and made ready for disposal. The materials should be collected into sturdy containment e.g. industrial strength plastic bags or empty oil drums. The Master should make whatever arrangements are necessary for the safe disposal of these substances and this may, where appropriate, involve the use of a subcontractor to remove the substance from the vessel.

# PROCEDURES AND POINT OF CONTACT	FOR	CO- ORDINATION

#### NATIONAL AND LOCAL CO-ORDINATION

Quick and efficient co-ordination between the ship and CoastalState(s) or other involved parties is vital in mitigating the effects of a pollution incident. All efforts at mitigation should therefore be undertaken with the foreknowledge of and in co-ordination with the CoastalState(s) or other local authorities who may be affected by the results of such efforts. The Master should contact and keep such CoastalState(s) appraised of plans for and the results of such efforts.

The Master or his recognised delegate is the shipboard point of contact for co-ordination of spill response with national and local authorities. The Master contact information is located in Appendix III

#### IDENTITIES OF NATIONAL AND LOCAL AUTHORITIES

The identities and roles of various national and local authorities involved will vary widely from State to State and even from port to port. Appendices I and II contain the contact numbers for CoastalStates and local authorities.

#### RESPONSIBILITIES

When a report concerning a pollution incident, or emergency, which threatens the environment is submitted to the CoastalState or local authorities, the Master maintains his primary responsibility for the safety of the crew and the vessel.

However, the Master must realize the extreme importance of coordination and cooperation with national and local authorities. The national and local agencies who are charged with protecting the public interests exist for the purpose of ensuring adequate response. They possess valuable resources and information, which can be of great assistance, even if the vessel (owners/operators) is responsible for providing the response contractors as is the case in the United States.

Without interfering with ship owners’ liability, some coastal States consider that it is their responsibility to define techniques and means to be taken against an oil pollution incident and approve such operations which might cause further pollution, i.e. lightering. States are in general entitled to do so under the International Convention relating to Intervention on the High Seas in Cases of Oil Pollution Casualties, 1969 (Intervention Convention).

The Master is responsible for the safety of the vessel and its operation at all times. He must also be aware that not all coastal states address pollution incidents in the same manner.

    - Coastal State Response to Pollution Incident

Responsibilities

Most coastal states have established marine pollution response activities to protect their coastal environment. Therefore, the coastal state contact is absolutely essential in order to mobilize

response resources for pollution containment, removal and other assistance. The Master is the principle liaison with the national and local authorities until the representative of the owner or operator arrives. It is imperative that all factual information required by coastal state authorities is transmitted promptly and accurately.

Response Actions

Since the Master is primarily responsible for the safety of the crew and vessel, his priority is to ensure that the situation on his vessel is under control and that the source of pollution or threat of pollution is correctly identified, that all necessary actions have been initiated to mitigate the actual or probable pollution incident and to minimize danger to the vessel if a casualty has occurred.

The coastal states will advise the Master which resources have been activated and identify what information or assistance is needed from the vessel.

Again, it is repeated - when the coastal states provide the pollution response - the Master is to care for his crew and vessel, stop the source of pollution or initiate corrective action for any casualty, and to cooperate fully with the national and local authorities.

If there are any questions, the Master should contact the operating company for advice.

## The Vessel Response to a Pollution Incident

Responsibilities

In some coastal states, primarily the United States, the vessel (its owner and operator) is responsible for not only notifying the national and local authorities, but is also responsible for the spill responses.

This situation makes the Master, or his agents, responsible for making necessary arrangements to provide the contractors and equipment to contain the spill, remove the oil from the water and clean up damaged shoreline or resources.

The Master will be aware of any contractual arrangements, which are applicable within any State, through the owners/managers of the vessel and will have onboard full details of the contact telephone numbers and advice of communications.

Response Actions

It is the Master's responsibility, in addition to the safety of the vessel, to be in charge of the spill response operations until relieved by a representative of the operating company or P & I Club.

When possible, the operating company will have arranged in advance for a Shoreside Spill Response Coordinator or Qualified Individual who will ensure that proper spill response is carried out, so that the Master and crew are free to handle the problems on the vessel.

Where a Shore based Spill Response Co-ordinator or Qualified Individual is engaged, the Master should initially communicate directly with this individual, should there be a situation where a spill has occurred or there is a possibility of a potential oil or chemical spill.

The response co-ordinator or Qualified Individual will then be in a position to initiate response actions, commence mobilising shore response personnel and equipment consistent with the potential oil or chemical spill.

The Shore based Spill Response Co-ordinator or Qualified Individual will ensure that:

- proper notifications have been made and qualified response contractors are engaged;

- effective liaison with national or local authorities is established;

- public affairs matters are handled in the best interests of the vessel;

- coordination with the P & I Club, operators and owners is maintained; and

- all support necessary for the crew and vessel is arranged.

More detailed descriptions of response requirements from the vessel for the coastal states that require the vessel to initiate spill response resources and management are contained elsewhere in this manual.

  - PLANNING STANDARDS

In general, whenever there is a spill or the potential of a spill the Master and Crew should initially combine their resources to stop the spill and make every effort to prevent pollution to the environment.

If, however, pollution occurs every effort must be made to contain the spill and prevent and reduce the expansion of the area of pollution. Shore based resources are much better equipped and experienced to undertake the task of preventing the spread of oil or chemical pollution and the correct and immediate notifications required to be made, according to this manual, will achieve the best means of preventing the spread of pollution. Much will depend upon the location of the vessel when a spill occurs as to the potential spread of pollution – strong tidal rivers will transfer oil or chemicals spilled from the vessel rapidly to large areas. Whilst at anchor or in a seaway, it is extremely difficult to contain any pollution, if there are strong sea conditions prevailing.

The Master should, however, be aware of the equipment which can be made available through shore based Oil Spill Response Organization. (OSROs)

Booms – are floating barriers, which can be towed to form a barrier around the vessel or in areas where the oil or chemical spill has occurred and thus contain the spill. These floating barriers are the main means used by the shore-based resources of preventing the expansion of the pollution area.

Most oil terminals have boom readily available, which can be quickly placed into position in the event of a spill.

Oil Spill Response vessels – vessels which are based at strategic ports, particularly in the vicinity of the large oil terminals and will respond to a spill or potential spill. They have all the equipment and personnel on board necessary to assist in the event of a spill and the prevention of the spread of pollution. The equipment includes boom, chemical detergents, clothing, pumps and logistical back-up.

Barges – may be required when large quantities of oil or chemicals have been contained in the boom “area” and the oil or chemical residue can be pumped off the surface and into the barge for ultimate disposal.

Skimmers – are specialized boats, which can skim the oil or chemical residue from the surface of the water.

Where vessels are aground or badly damaged, as a result of an incident, salvage and fire fighting services may be required and these are normally already contracted by many owners/operators.

The extent of a spill is generally categorized in the following manner:

Average most probable discharge. (AMPD) 1-50 barrels.

This type of oil spill would normally be handled by local facilities, in conjunction with the ship’s personnel. Typical resources required would be boom, possibly detergent and means of pumping off the residue, if necessary.

Maximum most probable discharge. (MMPD) 50 – 2500 barrels

This category is a major oil spill and will require experienced shore based support assistance, including extensive amounts of boom, Oil Spill response vessels, barges and skimmers.

Worst case discharge (WCD) 2500 barrels and over

This is a catastrophic spill, where the vessel is likely to be aground and breaking up or badly damaged. Only the full resources of a large oil spill response organization and probably a salvage and firefighting vessel can undertake the prevention of serious pollution.

It should be fully understood by the Master that the full potential of possible pollution should be reported initially, in order that ample assistance and equipment is made available at the onset. Invariably, in the past, Masters have always underestimated the spill potential when making the initial report to the authorities, thus slowing the assembly of suitable response equipment.

# ADDITIONAL NON-MANDATORY PROVISIONS

#### SHIP-SPECIFIC INFORMATION

Ship-Specific information including call sign, country of registry, principal characteristics and location is located in Appendix IV.

#### ON BOARD RESPONSE EQUIPMENT

The following equipment and materials must be carried on board to respond to and clean up deck spills:

| ITEM | SIZE | QTY |
| --- | --- | --- |

The resources provided are intended to mitigate, contain and remove internal and deck spills only. The ship should not attempt to put crew over the side to deal with spillage in the water but should rather rely on available shoreside resources for such response. Such resources may be available from the facility if the ship is alongside, from pre-arranged contracted resources, from contractor resources, or if in waters of a State taking direct responsibility for response, from that State.

Detergents, degreasers and the like may be provided for cleanup of the deck, contaminated on board structures and equipment.

“No such chemicals should be applied to oil over the side or on the ship, where the resultant mixture will be discharged over the side, without the express consent and authorisation of the affected Coastal State(s) or local authorities.”

If required by local law, the consent and authorisation of the State or local authorities for on-water use of containment or recovery equipment under the direction of the ship should also be obtained.

#### LEGAL ASSISTANCE

The Master should be aware that, in some States, criminal penalties, prison and fines, can attach to him and other crew members as a result of the incident, i.e. collision, grounding, pollution spill and/or other events. Criminal proceedings can also brought against him and other crew members for actions undertaken in responding to pollutant spills.

Similarly, the Master and crew should be aware that civil proceedings can be brought against them, the ship's owner and the Management Company by organisations involved in the incident.

Owners have arrangements for legal assistance in these cases, usually through P & I Clubs local representatives and retained lawyers.

Before answering questions or making statements, the Master and Crew should be aware of the identity and organisation of the questioner. Identification must be shown.

Masters and crew must always take care in making statements to State, local and port authorities, or answering questions from any person who have not identified themselves or are asking casual questions. Any statements made or answers given may be self incriminating.

When making a statement, be sure to give only the facts of the situation and avoid speculation and guesses.

Many States' laws provide protection from self-incrimination and it is permissible to refuse to answer questions on the grounds that "you wish to preserve your legal rights".

Always ask for legal advice before making a statement or answering questions regarding the causes of an incident.

All vessels must have onboard the latest list of P&I correspondents for the club that they are entered with.

#### PUBLIC AND MEDIA AFFAIRS

The reaction of the media and public to an incident, particularly one which involves pollution, can be as important as any other factor of the response.

The Master and crew are advised that public/media relations must be handled with care and, if available, with the advice of trained professionals in this area. The Management Company have arranged for the services of media consultants who are knowledgeable of shipping incidents and media/public relations matters. This organisation will normally make contact with the vessel and be available to assist with media/public relations.

Prior to the arrival of the media consultants, Masters should:

    - Pass on all media requests for information and interviews to the Management Company.

    - In the initial stages of an incident, explain that he is busy handling the incident and the inquirer should contact the Management Company for information.

    - Tell subsequent inquirers that the authorities have taken charge of investigations and it would be inappropriate to comment, but that such inquirers may wish to contact the Management Company for information.

If it is impossible to avoid making a comment

    - The Master should ensure that the questioners are true representatives of the legitimate media by asking for their credentials before interviews or conferences.

    - The Master should state the known facts relating to the incident and should NEVER speculate or guess about causes, the size or effect of any pollution or the eventual outcome of the response efforts.

    - The Master should be straightforward, co-operative and appear willing to assist the media in gaining the available information.

    - Statements made should be simple, concise recitations of the facts known, without opinions or elaboration. Do not get lured into commenting on facts not known about or on the speculations of other people.

    - Use a straight "I do not know" in all cases where information is unknown.

    - Under no circumstance should false or misleading statements be made nor should an attempt be made to obscure the facts.

#### RECORDKEEPING

For purposes of establishing documentary evidence against subsequent litigation, for formulating summary reports and for providing a learning tool against future casualty responses, the Master should maintain accurate records of events concerning the casualty, the extent and impacts of the spill and the resulting response efforts.

Included in these documents should be:

- Event and communications logs (both individual and collective);

- Contracts, agreements and financial transactions;

- Cargo/bunker samples taken from tanks, the deck or hull and alongside the vessel, if possible. These samples must be retained in clean, sealed containers in the custody of responsible parties accompanied by a "chain of custody" document (signed by each custodian, showing continuous and verifiable custody of the sample);

- Photographs, videos and other visual records of the casualty and response; and

- Newspaper, television, radio and other media clippings or extracts concerning the incident.

#### PLAN REVIEW

Information in this plan is very much subject to change with changing personnel, locations, organisations and laws. Owners have arranged for continual review of the data in the Plan that concerns these factors and for updates of the Plan to reflect changes noted.

- Periodic review; as indicated above, data in the Plan will be under continuous review. The Plan itself will be updated to reflect routine changes on an annual basis with interim updates in the event major revisions are required by change of law or other events; and

- Event review; after use of the Plan in response to an incident, its effectiveness will be evaluated by the owner or operator and the crew. Any necessary modifications will be made accordingly.

#### TRAINING, DRILLS AND EXERCISES

The crew will be given onboard training in the use of all vessels’ safety and shipboard environmental response equipment, including fire fighting, lifesaving and emergency equipment,

e.g. fire pump and emergency generator.

Training will be carried in basic in spill response strategies and tactics to cover areas such as:

  - Reporting requirements

  - Oil spill response team training

  - Vessel casualties threatening the environment

  - Behaviour and fate of oil on water

  - Bunker and cargo system casualties

  - Shipboard environmental response equipment

  - Cargo system casualties and spill mitigation

  - Shipboard spill response management

The Ship’s Crew will be trained by the Senior Officers in the deployment, operation and maintenance of the onboard response equipment and other relevant resources.

The MARPOL Shipboard Oil Pollution Emergency Plan and Emergency Contingency Procedures will be tested on board by monthly drills involving the personnel who will use it in the event of an incident.

The Drills and Exercises will include fire fighting, lifeboat and emergency scenarios as well as pollution incidents.

Vessel's officers and crew will also be expected to take part in Drills and Exercises involving the Operator's staff concerned with managing the ship's movements, as well as any shore-based support provided for response.

Examples of Drills are as follows:

Scenario Subject Matter

Transfer System (pipeline/valve/pump) Leakage Tank Overflow

Tank or Hull Leakage Stranding of the Vessel Grounding (touching bottom) Collision

Fire in various parts of the vessel Explosion

Hull Failure Excessive List

Main Engine Failure Steering Gear Failure

Damage/Stability Information Transfer

The Operator's staff will be given relevant training in Contingency Planning and will be exercised at regular intervals.

#### CREW SAFETY AND WELFARE

At all times the crew's safety and welfare must be given highest priority.

In the event of an incident that causes the crew to be landed ashore, facilities must be provided for their immediate shelter and eventual repatriation.

The Master must liaise with the Management Office, personnel departments and ships Agent to ensure that these facilities are provided

#### GENERAL EMERGENCY ACTIONS

This section provides guidelines for incidents that would not normally result in an oil pollution incident. If the situation requires, the Master may deviate from these guidelines for the safety of life, the ship, cargo or the environment; he has the authority to do so. The incidents covered in this section are:

    - Steering Failure / Electrical Power Failure / Technical Failure

In the event of steering failure it is essential that steering ability is returned as Soon as possible.

If the vessel has traffic in the vicinity they should be warned by the transmission of a “Pan Pan” message and the displaying of ‘Not Under Command’ Signals. If steering cannot be returned promptly the engines should be stopped and the vessel placed in a position of safety, this may include anchoring if possible.

To assist the Master checklist 17 / 18 / 19 has been prepared for use in the event of a Steering failure / Electrical Power Failure / Technical Failure.

## Shifting of Cargo

In the event of excessive list being experienced due to shifting of cargo the following priorities must be observed: safety of life; control of damage to the vessel and cargo; and prevention of environmental pollution. Personnel must not take risks which may endanger their own lives or the lives of any other crew members. In the event of shifting of cargo without a list being experienced this same checklist should be followed where appropriate

In the event of a shift of cargo it is important that actions are taken promptly and safely. To assist the Master checklist 20 has been prepared for use in the event of a shift of cargo situation.

# APPENDIX I - COASTAL STATE CONTACT INFORMATION

Refer to the following pages for Coastal State Contact Information

# for the following information:

- CONTACT ADDRESSES OF RESPONSIBLE NATIONAL AUTHORITIES ( FlagState contact points for port state control)

# LIST OF NATIONAL OPERATIONAL CONATCT POINTS RESPONSIBLE FOR THE RECEIPT , TRANSMISIION AND PROCESSIJNG OF URGENT REPORTS ON INCIDENTS INVOLVING HARMFUAL SUBSTANCES, INCLUDING OIL FROM SHIPS TO COASTAL STATES

NB : Pls note that the above mentioned are subject to changes

# , therefore they shall be updated by the management office .

APPENDIX II - EMERGENCYPORT CONTACT SHEET

Ship’s Name: Date:

Port: Cargo:

Distribution Master

Contingency Manual Cargo Officer

Agent

This Emergency Port Contact Sheet is to be completed on arrival at all ports traded to by vessel.

In the event of an incident the agent is to fax this Emergency Contact Sheet to the Managers.

Coastal State Authorities:	e.g. Coast Guard, Pollution Authority

Name:

Office Tel:

AOH Tel:

Fax:

Telex:

Qualified Individual (if appropriate): Office Tel:

AOH Tel:

Fax:

Telex: Managers:

Office Tel:

AOH Tel:

Fax:

Telex:

Local Authorities: e.g. state officials, port and terminal authorities

Name:

Office Tel:

AOH Tel:

Fax:

Telex: Where more than one authority is known, all organizations are to be identified.

Ship’s Agents: Office Tel:

AOH Tel:

Fax:

Telex:

Local P&I Correspondent: Office Tel:

AOH Tel:

Fax:

Telex:

Spill Contractor:	as per MSC-MEPC.6/Circ.17

Office Tel:	AOH Tel:

Fax:	Telex:

# APPENDIX III SHIP INTEREST CONTACT DETAILS

OWNER:

COMMERCIAL OPERATOR:

FLAG STATE:

###### CLASSIFICATION SOCIETY:

#### DAMAGE STABILITY PROVIDER –

P & I CLUB:

H & M Insurer:

# COMPANY	EMERGENCY	RESPONSE	TEAM	CONTACT DETAILS

COMPANY	:

# Designated Person Ashore (DPA)

Alternative DPA:

# APPENDIX IV - SHIP'S PARTICULARS

1	VESSEL PARTICULARS

Name Flag

Type of Vessel Call Sign

IMO Number GRT

Year Built Classification

BUILDER: DATE OF DELIVERY:

LENGTH OVERALL:

LBP:

BREADTH MOULDED:

DEPTH MOULDED:

SUMMER DRAUGHT:

BUNKER CAPACITY:

GAS OIL:

DETAILS OF ANY MAJOR MODIFICATIONS OR REBUILDING:

DIAGRAM: GENERAL ARRANGEMENT, CAPACITY PLAN, MID SHIP SECTION, SHELL EXPANSION , PIPING ARRANGEMENT.

OTHER RELEVANT INFORMATION SPECIFIC TO THE SHIP:

# APPENDIX V – ADDITIONAL CONTINGENCY

CHECKLISTS/PROCEDURES

###### Contingency Checklist 01	Pipeline Leakage and Tank Overflow

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop Operations & Close all valves |  |
| Sound General Emergency Alarm |  |
| Inform Terminal / Supplier about incident |  |
| Record of incident commenced |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Clean up squad mustered |  |
| Notifications made in accordance with national and international requirements. |  |
| Is there a risk of fire |  |
| Smoking on board banned |  |
| Office Notified |  |
| Air intakes to Accommodation and ER shut down |  |
| If spill in water consider shutting down ER sea water intakes |  |
| Are crew at risk from toxic hazards |  |
| Is external assistance required |  |
| Is an internal transfer of oil necessary |  |
| Will vessels’ stability be compromised by spill and or transfer |  |
| All parties informed of situation |  |
| Emergency Party |  |
| Nature and source of spill confirmed |  |
| Quantity of spill confirmed |  |
| Spill stopped |  |
| Spill contained on board |  |
| Commence clean-up operation |  |
| Drain effected area of oil |  |
| Portable salvage pumps used where possible |  |
| Arrangement made for collection of spill |  |
| Arrangements made for disposal of residues |  |
| Lower level of oil in overflowed tank |  |
| Sample of spilled oil taken |  |
| Engine Room Party |  |
| Engine room party mustered |  |
| Report given to CE about incident |  |
| Instructions issued to ER party |  |
| If leakage in ER stop machinery in vicinity of leakage |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in	blanks

###### Contingency Checklist 02	Hull Leakage / Failure

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop Operations & Close all valves |  |
| Sound General Emergency Alarm |  |
| Inform Terminal / Supplier about incident |  |
| Record of incident commenced |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Clean up squad mustered |  |
| Notifications made in accordance with national and international regulations |  |
| Is there a risk of fire |  |
| Smoking on board banned |  |
| Office Notified |  |
| Air intakes to Accommodation and ER shut down |  |
| Consider shutting down ER sea water intakes |  |
| Are crew at risk from toxic hazards |  |
| Request external assistance |  |
| Is an internal transfer of oil necessary |  |
| Will vessels stability be compromised by spill and or transfer |  |
| Is leakage coming from de-ballasting operation |  |
| Arrange for deployment of booms if possible |  |
| All parties informed of situation |  |
| Emergency Party |  |
| Nature and source of spill confirmed |  |
| Quantity of spill confirmed |  |
| Spill stopped |  |
| Commence cleanup operation |  |
| Drain effected area of oil |  |
| Check levels of tanks and monitor for changes |  |
| Lower level of oil in tank affected |  |
| Sample of spilled oil taken |  |
| Engine Room Party |  |
| Engine room party mustered |  |
| Report given to CE about incident |  |
| Instructions issued to ER party |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 03	Grounding, Wrecked or Stranded

/Major Flooding

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop Main Engines |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed |  |
| Display Appropriate Signals |  |
| Record of incident commenced |  |
| Latest weather forecast obtained |  |
| Advise Engine Room of Situation |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Appropriate message sent e.g. PAN PAN |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors closed where appropriate |  |
| Assess extent of damage – is vessel stable |  |
| Are any crew injured |  |
| Is there a risk of fire/explosion (consider possibility of fumes being drawn into accommodation and ignited by smoking) |  |
| Is there any pollution or risk of pollution? |  |
| Office Notified |  |
| CoastState notified |  |
| Nearest safe port identified |  |
| Consider transfer / discharge of bunkers if necessary |  |
| Consider transfer / discharge of ballast / FW if necessary |  |
| Consider transfer / discharge of cargo if possible |  |
| State of tide determined |  |
| Is astern engine movement appropriate to re-float the vessel? |  |
| Will vessel remain stable when refloated? |  |
| If weather is poor will it be more appropriate to ballast down? |  |
| Emergency Party |  |
| Report on damage received |  |
| Results of internal and external soundings received |  |
| Steps being taken to minimise ingress of water if appropriate |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Results of visual inspection received |  |
| Results of soundings after refloating received |  |
| Engine Room Party |  |
| Report on condition of main engine received |  |
| Report on condition of auxiliary machinery received |  |
| Water provided to fire main |  |
| Suctions to high level if appropriate |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 04	Fire / Explosion

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| General Emergency Alarm Sounded |  |
| Prepare for loss of power |  |
| Vessels’ position plotted and navigational situation assessed if appropriate |  |
| Display Appropriate Signals |  |
| Shut down appropriate ventilation |  |
| Record of incident commenced |  |
| Engage hand steering |  |
| In port - Engine to standby; At Sea – engine stop |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Message sent e.g. Mayday, Pan Pan |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors and fire doors are closed where appropriate |  |
| Are any crew injured |  |
| If possible consult MSDS for substance on fire |  |
| Is there risk of explosion / spread of fire |  |
| Consider use of fixed FFA if available |  |
| Cargo tanks pressurised with IG if appropriate |  |
| If fixed FFA to be used re-muster personnel |  |
| If fixed FFA to be used ensure space ventilation fully secured |  |
| Office Notified |  |
| CoastState notified |  |
| Nearest safe haven identified |  |
| Nearest Hazard identified |  |
| Personnel allocated for medical party e.g. stretcher resuscitator etc |  |
| Is there risk of pollution |  |
| If yes consider shutting down ER suctions and accommodation ventilation |  |
| Is vessels’ course and speed affecting fire e.g. supply of air |  |
| Is vessels’ stability being compromised by supply of water |  |
| Emergency Party |  |
| On UMS ships bridge to be contacted before entering ER |  |
| Confirmation that ventilation shut down received |  |
| Seat of fire located |  |
| Confirm nature/class of fire e.g. oil |  |
| Can fire be fought with first aid measures e.g. extinguisher |  |
| Commence boundary cooling |  |
| Lifeboats ready for embarkation |  |
| Fire fighting party ready; BA Control Established |  |
| Carry out search and rescue if necessary |  |
| Engine Room Party- |  |
| Prior to entering ER confirm it is safe to do so |  |
| Stop any machinery on fire |  |
| Emergency fire pump started |  |
| Emergency generator started |  |
| Water provided to fire main |  |
| Lighting / power confirmed as reliable |  |
| Electrical circuits isolated in vicinity of fire |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 05	Fire in Accommodation

Ships Name:	Date & Time:

| Action: (As Soon As The Fire Has Been Detected) | Initials |
| --- | --- |
| Deck Officer has been informed about a beginning of a fire through the manual fire alarm, phone etc.) |  |
| Sound fire alarm |  |
| Call the Master |  |
| Inform the Engine Room |  |
| An investigation of the area involved has been done |  |
| Evacuate unnecessary personnel and passengers and make sure that all accesses are sealed |  |
| Vessel position available in radio room, satellite terminal and other automatic distress transmitters, updated as necessary |  |
| Inform the Company |  |
| At sea alter course and speed to reduce air draft and minimize fire spread |  |
| If in or near port alert Port Authorities, shore fire service, fire brigades, make sure that the International Shore Connection is properly fitted in place |  |
| Close automatic and manual fire doors |  |
| Close watertight doors |  |
| Shut down cargo and /or bunker operations if any |  |
| Shut down Fire Dampers |  |
| Isolate electrical circuits |  |
| Close doors, skylights, portholes |  |
| Stop engines if necessary |  |
| Check for missing crew |  |
| OPERATE FIRE EXTINGUISHING SYSTEM FOR THE CONCERNED AREA |  |
| At sea exhibit light / shapes |  |
| At sea prepare lifeboats (not affected by fire) |  |
| Navigational warnings broadcasted on all distress frequencies |  |
| Distress message send if fire not under control on all distress frequencies |  |
| IF THE FIRE HAS BEEN PUT OUT |  |
| If fire extinguishing system has been operated, sufficient time has elapsed to allow cooling of the affected area |  |
| Thorough investigation of the area has been carried out |  |
| Has been ascertained that all source of ignition have been removed |  |
| Area has been properly ventilated and all residual fumes blown out |  |
| Electrical power mechanical ventilation has been restored |  |
| Determine cause of fire |  |
| Assess damage to own vessel |  |
| If there is a spill of oil in connection with the fire, inform appropriate parties( refer to SOPEP or VRP plan if within USA territorial waters |  |
| Take photos of area damaged by the fire |  |
| Notify if spare parts required |  |
| Enter all times and facts in log book |  |
| Surveyor attendance/report |  |
| P & I and H&M attendance / report |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

Contingency Checklist 06	Cargo Fire on Deck

Ships Name:	Date & Time:

### If the fire involves stores or machinery, the appropriate extinguishershould be used. The fire may affect the cargo and increase the boil-off: this can be reduced by cooling the area with water spray. However a water jet should never be used on a liquid fire.

If a fire occurs in a cargo equipment space (compressor room), the source of fuel should be cut off and the initial attack should be with dry powder. If necessary all personnel should be evacuated, the compartment closed down and the fixed fire fighting system (foam, CO2) activated. The area should be cooled with water sprays. Once the fire is out, the space should be ventilated carefully to disperse any vapour.

| Action: (As soon as the fire has been detected) | Initials |
| --- | --- |
| Deck Officer has been informed about a beginning of a fire through the manual fire alarm, phone etc.) |  |
| Sound Fire Alarm |  |
| Call the Master |  |
| Inform the Engine Room |  |
| An investigation of the area involved has been done |  |
| Evacuate unnecessary personnel and passengers and make sure that all accesses are sealed |  |
| Vessel position available in radio room, satellite terminal and other automatic distress transmitters, updated as necessary |  |
| Inform the Company |  |
| At sea alter course and speed to reduce air draft and minimize fire spread |  |
| If in or near port alert Port Authorities, shore fire service, fire brigades, make sure that the International Shore Connection is properly fitted in place |  |
| Close automatic and manual fire doors |  |
| Close watertight doors |  |
| Shut down cargo and /or bunker operations if any |  |
| Shut down Fire Dampers, fuel valves, fuel pumps, fuel service tanks, settling tanks etc. |  |
| Isolate electrical circuits |  |
| Close doors, skylights, portholes |  |
| Stop engines if necessary |  |
| Check for missing crew |  |
| OPERATE FIRE EXTINGUISHING SYSTEM FOR THE CONCERNED AREA |  |
| At sea exhibit light / shapes |  |
| At sea prepare lifeboats (not affected by fire) |  |
| Distress message send if fire not under control on all distress frequencies |  |
| IF THE FIRE HAS BEEN PUT OUT |  |
| If fire extinguishing system has been operated, sufficient time has elapsed to allow cooling of the affected area |  |
| Thorough investigation of the area has been carried out |  |
| Has been ascertained that all source of ignition have been removed |  |
| Area has been properly ventilated and all residual fumes blown out |  |
| Electrical power mechanical ventilation has been restored |  |
| Assess damage to shore installation if in port |  |
| Assess damage to the cargo (if any) |  |

| Determine cause of fire |  |
| --- | --- |
| Assess damage to own vessel |  |
| If there is a spill of oil in connection with the fire, inform appropriate parties( refer to SOPEP or VRP plan if within USA territorial waters |  |
| Take photos of area damaged by the fire |  |
| Notify if spare parts required |  |
| Enter all times and facts in log book |  |
| Surveyor attendance/report |  |
| P & I and H&M attendance / report |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

Contingency Checklist 07	Fire on Deck

## Ships Name:	Date & Time:

### If the fire involves stores or machinery, the appropriate extinguishershould be used. The fire may affect the cargo and increase the boil-off: this can be reduced by cooling the area with water spray. However a water jet should never be used on a liquid fire.

If a fire occurs in a cargo equipment space (compressor room), the source of fuel should be cut off and the initial attack should be with dry powder. If necessary all personnel should be evacuated, the compartment closed down and the fixed fire fighting system (foam, CO2) activated. The area should be cooled with water sprays. Once the fire is out, the space should be ventilated carefully to disperse any vapour.

| Action: (As soon as the fire has been detected) | Initials |
| --- | --- |
| Deck Officer has been informed about a beginning of a fire through the manual fire alarm, phone etc.) |  |
| Sound Fire Alarm |  |
| Call the Master |  |
| Inform the Engine Room |  |
| An investigation of the area involved has been done |  |
| Evacuate unnecessary personnel and passengers and make sure that all accesses are sealed |  |
| Vessel position available in radio room, satellite terminal and other automatic distress transmitters, updated as necessary |  |
| Inform the Company |  |
| At sea alter course and speed to reduce draft and minimize fire spread |  |
| If in or near port alert Port Authorities, shore fire service, fire brigades, make sure that the International Shore Connection is properly fitted in place |  |
| Close automatic and manual fire doors |  |
| Close watertight doors |  |
| Shut down cargo and /or bunker operations if any |  |
| Shut down Fire Dampers, fuel valves, fuel pumps, fuel service tanks, settling tanks etc. |  |
| Isolate electrical circuits |  |
| Close doors, skylights, portholes |  |
| Stop engines if necessary |  |
| Check for missing crew |  |
| OPERATE FIRE EXTINGUISHING SYSTEM FOR THE CONCERNED AREA |  |
| At sea exhibit light / shapes |  |
| At sea prepare lifeboats (not affected by fire) |  |
| Distress message send if fire not under control on all distress frequencies |  |
| IF THE FIRE HAS BEEN PUT OUT |  |
| If fire extinguishing system has been operated, sufficient time has elapsed to allow cooling of the affected area |  |
| Thorough investigation of the area has been carried out |  |
| Has been ascertained that all source of ignition have been removed |  |
| Area has been properly ventilated and all residual fumes blown out |  |
| Electrical power mechanical ventilation has been restored |  |
| Assess damage to shore installation if in port |  |
| Assess damage to the cargo (if any) |  |

| Determine cause of fire |  |
| --- | --- |
| Assess damage to own vessel |  |
| If there is a spill of oil in connection with the fire, inform appropriate parties( refer to SOPEP or VRP plan if within USA territorial waters |  |
| If available, take photos of area damaged by the fire |  |
| Notify if spare parts required |  |
| Enter all times and facts in log book |  |
| Surveyor attendance/report |  |
| P & I and H&M attendance / report |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

Contingency Checklist 08	Fire in Pump Room

## Ships Name:	Date & Time:

### If the fire involves stores or machinery, the appropriate extinguishershould be used. The fire may affect the cargo and increase the boil-off: this can be reduced by cooling the area with water spray. However a water jet should never be used on a liquid fire.

If a fire occurs in a cargo equipment space (compressor room), the source of fuel should be cut off and the initial attack should be with dry powder. If necessary all personnel should be evacuated, the compartment closed down and the fixed firefighting system (foam, CO2) activated. The area should be cooled with water sprays. Once the fire is out, the space should be ventilated carefully to disperse any vapour.

| Action: (As soon as the fire has been detected) | Initials |
| --- | --- |
| Deck Officer has been informed about a beginning of a fire through the manual fire alarm, phone etc.) |  |
| Sound Fire Alarm |  |
| Call the Master |  |
| Inform the Engine Room |  |
| An investigation of the area involved has been done |  |
| Evacuate unnecessary personnel and passengers and make sure that all accesses are interdicted |  |
| Vessel position available in radio room, satellite terminal and other automatic distress transmitters, updated as necessary |  |
| Inform the Company |  |
| At sea alter course and speed to reduce draft and minimize fire spread |  |
| If in or near port alert Port Authorities, shore fire service, fire brigades, make sure that the International Shore Connection is properly fitted in place |  |
| Close automatic and manual fire doors |  |
| Close watertight doors |  |
| Shut down cargo and /or bunker operations if any |  |
| Shut down Fire Dampers, fuel valves, fuel pumps, fuel service tanks, settling tanks etc. |  |
| Isolate electrical circuits |  |
| Close doors, skylights, portholes |  |
| Stop engines if necessary |  |
| Check for missing crew |  |
| OPERATE FIRE EXTINGUISHING SYSTEM FOR THE CONCERNED AREA |  |
| At sea exhibit light / shapes |  |
| At sea prepare lifeboats (not affected by fire) |  |
| Distress message send if fire not under control on all distress frequencies |  |
| IF THE FIRE HAS BEEN PUT OUT |  |
| If fire extinguishing system has been operated, sufficient time has elapsed to allow cooling of the affected area |  |
| Thorough investigation of the area has been carried out |  |
| Has been ascertained that all source of ignition have been removed |  |
| Area has been properly ventilated and all residual fumes blown out |  |
| Electrical power mechanical ventilation has been restored |  |

| Assess damage to the cargo (if any) |  |
| --- | --- |
| Determine cause of fire |  |
| Assess damage to own vessel |  |
| If there is a spill of oil in connection with the fire, inform appropriate parties( refer to SOPEP or VRP plan if within USA territorial waters |  |
| Take photos of area damaged by the fire |  |
| Notify if spare parts required |  |
| Enter all times and facts in log book |  |
| Surveyor attendance/report |  |
| P & I and H&M attendance / report |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

Contingency Checklist 09	Collision

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed |  |
| Display Appropriate Signals |  |
| Record of incident commenced |  |
| Latest weather forecast obtained |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Urgency message sent |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors closed where appropriate |  |
| Assess extent of damage – is vessel stable |  |
| Are any crew injured |  |
| Is there any fire |  |
| Is there a risk of fire/explosion (consider possibility of spark if vessels separate or fumes being drawn into accommodation) |  |
| Is there any pollution |  |
| Is there a risk of pollution |  |
| If pollution confirmed have ER sea suctions been closed |  |
| Establish communications with other vessel |  |
| Office Notified |  |
| Coast State notified |  |
| Nearest safe port identified |  |
| Nearest safe beaching option |  |
| Consider transfer of bunkers if necessary |  |
| Consider transfer of ballast / FW if necessary |  |
| Consider Transfer of cargo if possible |  |
| Exchange info with other vessel |  |
| Emergency Party |  |
| Report on damage received |  |
| Results of soundings received |  |
| Steps being taken to minimise ingress of water if appropriate |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Engine Room Party |  |
| Report on condition of main engine received |  |
| Report on condition of auxiliary machinery received |  |
| Water provided to fire main |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 10	Excessive List

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop all operations if appropriate |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed if appropriate |  |
| Display Appropriate Signals |  |
| Record of incident commenced |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Urgency message sent if appropriate |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors closed where appropriate |  |
| Are any crew injured |  |
| Is there any fire |  |
| Is there a risk of fire/explosion |  |
| Is there any pollution |  |
| Is there a risk of pollution |  |
| If pollution confirmed have ER sea suctions been closed |  |
| Office Notified |  |
| CoastState notified |  |
| Nearest safe haven identified |  |
| Consider isolation of damaged area |  |
| Consider transfer of bunkers if necessary |  |
| Consider transfer of ballast / FW if necessary |  |
| Consider Transfer of cargo if possible |  |
| Emergency Party |  |
| Cause of list identified |  |
| Confirm vessel stability calculation |  |
| Results of soundings received |  |
| Steps being taken to minimise ingress of water |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Engine Room Party |  |
| Report on condition of main engine received |  |
| Report on condition of auxiliary machinery received |  |
| Water provided to fire main |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 11	Containment System Failure

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop all operations if appropriate |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed if appropriate |  |
| Display Appropriate Signals |  |
| Record of incident commenced |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Urgency message sent if appropriate |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors closed where appropriate |  |
| Are any crew injured |  |
| Is there any fire |  |
| Is there a risk of fire/explosion |  |
| Is there any pollution |  |
| Is there a risk of pollution |  |
| If pollution confirmed have ER sea suctions been closed |  |
| Office Notified |  |
| CoastState notified |  |
| Nearest safe haven identified |  |
| Consider isolation of damaged area |  |
| Consider transfer of bunkers if necessary |  |
| Consider transfer of ballast / FW if necessary |  |
| Consider Transfer of cargo if possible |  |
| Emergency Party |  |
| Cause of failure identified |  |
| Steps taken to minimise effect of failure |  |
| Results of soundings received |  |
| Steps being taken to minimise ingress of water |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Engine Room Party |  |
| Report on condition of main engine received |  |
| Report on condition of auxiliary machinery received |  |
| Water provided to fire main |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 12	Sea Pollution

Ships Name:	Date & Time:

Include all situations like: loss or contamination of cargo, hull failure, transfer system failure, tank overflow during loading or bunkering, hull leakage, emergency cargo transfer / lightering, pipe leakagefor all aspects like reporting procedures, action to be taken to minimise the oil spill, stability assessment, notification to the company and shore authorities, clean up contractors follow instructions contained in the SOPEP and VRP plan if within USA territorial waters

| Action | Initials |
| --- | --- |
| Call Master |  |
| Inform the Engine Room |  |
| Stop all cargo / bunkering / oil transfer operations |  |
| If at sea advise the Engine Room to slow down and stop the vessel if conditions allow |  |
| If deemed necessary by the Master sound GENERAL EMERGENCY ALARM and muster the crew at their own emergency stations |  |
| Inform the Company |  |
| If at sea warn ships in the vicinity |  |
| Keep continuous watch on channel 16 VHF |  |
| Ensure that fire fighting equipment is available (portable and fixed) |  |
| Eliminate source of ignition and prevent flammable vapours from entering engine room and accommodation |  |
| Ready to start fire pumps |  |
| The ship should keep the wind speed from astern or move astern so that the superstructure and afterdecks remain clear of the cargo vapour |  |
| Check crew for absence / injury |  |
| Fix vessel position and pass it to radio room |  |

Initialing confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 13	Bunker Spills

Ships Name:	Date & Time:

Include all situations like: loss or contamination of cargo, hull failure, transfer system failure, tank overflow during loading or bunkering, hull leakage, emergency cargo transfer / lightering, pipe leakagefor all aspects like reporting procedures, action to be taken to minimise the oil spill, stability assessment, notification to the company and shore authorities, clean up contractors follow instructions contained in the SOPEP and VRP plan if within USA territorial waters

| Action |  |  |  |  |  | Initials |
| --- | --- | --- | --- | --- | --- | --- |
| Raise the Alarm |  |  |  |  |  |  |
| Stop all operations and close all valves |  |  |  |  |  |  |
| Call Master |  |  |  |  |  |  |
| Notifications |  |  |  |  |  |  |
| Make necessary notifications e.g. nearest coastal state; local port authorities; In US Qualified Individual; terminal operators. Using the initial notification form |  |  |  |  |  |  |
| Inform Company |  |  |  |  |  |  |
| Commence record of incident and inform crew of situation |  |  |  |  |  |  |
| Continually up-date the relevant authorities as soon as new information become known. |  |  |  |  |  |  |
| Safety Items to consider: is there danger of fire? - are the crew safe from the toxic or other health hazards of the spill? - will the stability conditions of the ship be affected by the spill? |  |  |  |  |  |  |
| Pollution items to consider: - Are there alternative tanks transferred? - If oil in water from spill, stop ER sea suction. | into | which | oil | can | be |  |
| Mobilize Spill Squad |  |  |  |  |  |  |
| Ensure that all oil operations are stopped |  |  |  |  |  |  |
| Ascertain: the cause and location of the spill; the quantity of oil spilled; whether or not the source of the spill has been stopped; whether or not any discharge has gone overboard or is in imminent danger of doing so. |  |  |  |  |  |  |
| Direct the spill mitigation, containment, and removal and clean-up activities of the Emergency Party using onboard spill equipment. |  |  |  |  |  |  |
| If an overboard discharge has occurred, immediately call for shore side assistance. DO NOT PUT CHEMICALS INTO WATER. |  |  |  |  |  |  |
| If an overboard discharge has not occurred, evaluate the likelihood of such a discharge and the necessity of mobilizing shoreside assistance as a precaution |  |  |  |  |  |  |
| Reduce the level in the overflowing tank by opening slack tanks and draining oil into them |  |  |  |  |  |  |
| Take samples of the fuel oil and/or bunkers on board, and, if practicable, samples of any oil spills |  |  |  |  |  |  |
| Prepare ships pumps for possible discharge ashore |  |  |  |  |  |  |
| Check all scuppers to ensure that they are plugged in accordance with pre-transfer procedures, taking into account increases in contained volume from changes in trim or collection of rain or sea water. |  |  |  |  |  |  |
| Check that deck containment systems are adequate to prevent leakage from going overboard. |  |  |  |  |  |  |
| Establish portable pumps to put excess spillage into slack or slop tanks or other suitable containment equipment |  |  |  |  |  |  |
| Un-pumpable spillage must be removed in liquid form, so far as practicable, by sweeping, scooping and/or shoveling it into containers for removal to slop tanks or containers. Clingage remaining after these actions will be mopped up using sorbents, rags and/or |  |  |  |  |  |  |

| cleaning agents as necessary |  |
| --- | --- |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 14	Submerged / Foundered

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed |  |
| Display Appropriate Signals |  |
| Record of incident commenced |  |
| Latest weather forecast obtained |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure Distress message sent |  |
| Ensure Crew Mustered |  |
| Ensure Watertight doors closed where appropriate |  |
| Assess extent of damage – is vessel stable |  |
| Are any crew injured |  |
| Is there any fire |  |
| Is there a risk of fire/explosion |  |
| Is there any pollution |  |
| Is there a risk of pollution |  |
| If pollution confirmed have ER sea suctions been closed |  |
| Office Notified |  |
| CoastState notified |  |
| Nearest safe haven identified |  |
| Consider isolation of damaged area |  |
| Consider transfer of bunkers if necessary |  |
| Consider transfer of ballast / FW if necessary |  |
| Consider Transfer of cargo if possible |  |
| Emergency Party |  |
| Report on damage received |  |
| Results of soundings received |  |
| Steps being taken to minimise ingress of water |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Engine Room Party |  |
| Report on condition of main engine received |  |
| Report on condition of auxiliary machinery received |  |
| Water provided to fire main |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 15	Hazardous Vapour Release

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Stop all operations and close all valves |  |
| General Emergency Alarm Sounded |  |
| Vessels position plotted and navigational situation assessed if appropriate |  |
| Display Appropriate Signals if appropriate |  |
| Record of incident commenced |  |
| Stop all air intakes to engine room and accommodation |  |
| Start water spray system if appropriate |  |
| Master |  |
| After briefing from OOW assume control of vessel |  |
| Ensure appropriate message sent |  |
| Ensure Crew Mustered |  |
| Ensure all external doors closed |  |
| Are any crew injured |  |
| Smoking banned on board |  |
| Port / Terminal advised |  |
| Is there any fire |  |
| Is there a risk of fire/explosion consider possibility of fumes being drawn into accommodation / ER) |  |
| Is there any pollution |  |
| Is there a risk of pollution |  |
| If pollution confirmed have ER sea suctions been closed |  |
| Office Notified |  |
| CoastState notified |  |
| If at sea can vessel be manoeuvred up wind of vapour |  |
| Emergency Party |  |
| Cause of release identified |  |
| Confirm vessel stability calculation |  |
| Consult MSDS of substance |  |
| Prepare parties in protective clothing to address the problem |  |
| Steps being taken to minimise pollution if appropriate |  |
| Lifeboats ready for embarkation |  |
| Engine Room Party |  |
| Start emergency fire pump |  |
| Ensure power maintained |  |
| Shut down air intakes where appropriate |  |
| Lighting / power confirmed as reliable |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 16	Emergency Lightering

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| Ensure that bitts of sufficient strength are suitably located inboard of enclosed fairleads to receive eyes of mooring lines |  |
| Establish communications with lightering vessel and discuss ship interface requirements, method of approach, and mooring procedures |  |
| Obtain weather forecast and consider effects of weather on operation |  |
| Test engines, steering gear, controls, and navigation equipment |  |
| Check fenders and handling equipment (if applicable) |  |
| Review contingency plan |  |
| Plug and seal scuppers |  |
| Determine effects of ship to ship electric currents and necessary precautions |  |
| Establish efficient deck watch paying particular attention to moorings, fenders, hoses, and manifold observation |  |
| Eliminate sources of ignition and prevent flammable vapours from entering engine room and accommodation |  |
| Tightly close and seal area and overboard discharge valves of cargo system |  |
| Set air conditioning system to recirculation mode and close all access doors to accommodation and engine room spaces |  |
| Check and prepare fire fighting and anti-pollution equipment |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 17	Steering Failure

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Alternative mode of steering engaged |  |
| Call Master |  |
| Advise engine room / duty engineer of situation |  |
| Engine to standby |  |
| Engine stopped |  |
| Pan Pan Message transmitted |  |
| NUC Signals displayed |  |
| Check proximity of navigational hazards |  |
| Watertight doors closed if appropriate |  |
| Commence record of incident |  |
| Master |  |
| After briefing from OOW assume control of the ship |  |
| Give order to lower boats |  |
| Report made to coast state authority |  |
| Managing office informed |  |
| Is there a suitable area where the vessel can be anchored |  |
| Engine Room Party |  |
| Main engine ready for manoeuvring |  |
| Steering flat manned |  |
| Engage emergency steering |  |
| Commence investigation into reason for failure |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 18	Electrical Power Failure

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| Both at sea and port following an electrical power failure Engine Department will first check that emergency generator has started regularly and after, on board IAQ1 vessels, that main electrical supply has been restored and all the services on board are fed again. |  |
| In case vessel should experience electrical power failure during transfer operations: |  |
| The terminal, or alongside ship, must be immediately informed by the Deck Department to stop any transfer operations |  |
| All the cargo valves, on the manifolds and on the tanks, must be closed |  |
| To resume the operations only when all the services and cargo plant have been checked and found fully working again |  |
| When electrical power failure has concluded and all the services are running regularly the Engine Department Will check the conditions of emergency generator (battery, gas oil…) in order it is ready to start in case of another electrical power failure. |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 19	Technical Breakdown

Ships Name:	Date & Time:

| Initial Duties | Initials |
| --- | --- |
| Engineer of the watch |  |
| When a technical incident occurs which means disabling the vessels motive, directive or electrical power, ensure that it is operationally safe to do so |  |
| Check with the bridge that the navigational situation allows stopping of the vessel |  |
| Take all precautions to stop engine, etc safely |  |
| Inform the Chief Engineer that the vessel has a serious technical breakdown |  |
| Prepare any engine room back up facilities – emergency steering etc |  |
| Officer of the watch |  |
| In navigational hazardous areas the vessel should be on standby with anchors cleared and watches readily available |  |
| On being informed of a technical breakdown, ascertain if it safe to allow the vessels to stop, If not safe to stop then tell the engine room |  |
| If the vessels stops without warning, identify any possible navigational hazards, such as proximity to land, vessels in vicinity etc |  |
| While the vessel has steerage try to get away from the nearest navigational hazard |  |
| If steering failure take all way of the vessel using the engine and if practical the anchors. In restricted waters using anchors could swing the vessel into additional danger |  |
| Immediately display the international signal for the vessel not under command |  |
| Call master and in emergency situation sound the General Emergency Alarm |  |
| Master |  |
| On being called to the bridge, check if the vessel is in immediate danger |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 20	Cargo Shift

Ships Name:	Date & Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| All operations on board stopped |  |
| General Alarm Sounded |  |
| Advise Duty Engineer |  |
| Close Watertight doors |  |
| Commence record of incident |  |
| Master |  |
| After briefing from OOW assume command of the vessel |  |
| Send Pan Pan Message if appropriate |  |
| Notify nearest coast state |  |
| Managing office informed |  |
| Has vessels stability been compromised |  |
| Is there a risk of capsize |  |
| Obtain latest weather forecast |  |
| Identify closest place of refuge |  |
| Emergency Party |  |
| Crew Mustered |  |
| Soundings being taken |  |
| Is there damage to cargo |  |
| If yes assess extent |  |
| Consider possibility to re stow cargo |  |
| Can list be reduced by shifting of cargo |  |
| Engine Room Party |  |
| Remain in ER until instructed to leave |  |
| Check that bilges are as dry as possible |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in blanks

###### Contingency Checklist 21	Gyro Compass Malfunction Ships Name:		Date/Time:

| Action | Initials |
| --- | --- |
| OOW |  |
| Inform Master |  |
| Follow Magnetic Compass heading |  |
| Change over to Hand steering |  |
| Helmsman Assigned |  |
| Check vessel’s position and Proximity of Navigational hazard |  |
| Inform Engine Room and Call Electrician |  |
| Ensure Gyro fully settled and gyro error determined before switching Auto Pilot |  |
| Master |  |
| After briefing from OOW assume control of the ship |  |
| Consider Autopilot steering with Transmitting Magnetic Compass (If available) |  |
| Switch over to other Gyro Compass (If available) |  |
| Follow Instructions/Trouble Shooting from Manufacturers Manual |  |
| Managing Office Informed |  |
| Inform Port authority if unable to repair before arrival |  |

Initialling confirms compliance. If not applicable enter N/A. Any additional items to be entered in Blank areas.

|  | - | SOPEP | DOCUMENT NUMBER: SECTION REVISION NUMBER: | IMS/SOPEP 00 |
| --- | --- | --- | --- | --- |
| SECTION – APPENDIX VIII GENERAL |  |  | PAGE NUMBER: | 1 of 1 |

  - CAUTION NOTES:

    - This Manual is valid from the issue date.

    - This Manual is to be used only on LIGHTSHIP managed vessels.

    - This Manual is not to be copied or marked up.

    - Review of this Manual or any suggested alteration is to be referred to the PILOTSHIP MANAGEMENT Shipping office.

    - Further revisions of this Manual will be recorded on the Revision Sheet (Page 2).

The procedures contained within this manual are to ensure the safe operation of the vessel, the safety of the crew and the protection of the environment. The contents also ensure compliance with legislation and industry best practice. Failure by any member of staff, ashore or afloat, to comply with procedures will not be tolerated and shall be treated as a breach of contract.

Copyright 2024 PILOTSHIP MANAGEMENT

All rights strictly reserved. No part of this publication may be reproduced, stored in a retrieval system or transmitted in any form or by any means electronic, mechanical, photocopying, recording or otherwise without the prior written permission of PILOT SHIP.

Any person infringing the copyright will be liable to criminal prosecution and a civil claim for damages.

# APPENDIX VIII- GENERAL

- Location of Oil Spill Equipment:

Main deck

- Plan for deployment (To include Oil Spill Emergency Team layout diagram specifying duties including specific crew member duties for handling small spills)

Following diagrams are to be attached to this appendix:

  - General arrangement

  - piping arrangement

  - Capacity Plan

  - Mid ship Section

  - Shell expansion