Ship To Ship Operations Plan

# (STS Plan)

For compliance with Resolution MEPC.186(59), Annex I, Chapter 8 of MARPOL and

CDI/ICS/OCIMF/SIGTTO publication “Ship-to-Ship Transfer Guide for Petroleum, Chemicals, and Liquefied Gases,” Second edition 2020

For

| Ship Name |  |
| --- | --- |
| IMO Number |  |
| Call Sign |  |
| MMSI No. |  |
| Ship Type |  |
| Port of Registry |  |
| Flag |  |
| Deadweight |  |
| Moulded Depth |  |

# Contents

# REVISION SHEET	6

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 01 | 10-sept-2025 | MD |

# REFERENCES	7

# GLOSSARY	8

# SECTION I: RESPONSIBLE PERSONS FOR STS OPERATIONS	12

# SECTION II : MARPOL ANNEX I AMENDMENTS	13

# SECTION III.  SHIP’S PARTICULARS	14

# INTRODUCTION	16

# APPROVAL	16

  - SCOPE	17

# OBJECTIVES	17

# STS PROCESS	18

# PRE-ARRIVAL PLANNING	19

# CONDITIONS AND REQUIREMENTS	19

# SHIP COMPATIBILITY	19

# PERSON IN OVERALL ADVISORY CONTROL (POAC)	19

    - MASTER	20

# STS SUPERINTENDENT / LIGHTERING COORDINATOR / OR MOORING MASTER	21

# RECORDING AND CHECKLISTS	21

# APPROVAL FROM AUTHORITIES	21

# TRANSFER AREA	22

# WEATHER CONDITIONS	23

# NIGHT TIME STS MANOEUVRING	23

# QUALITY ASSURANCE OF SHIP TO SHIP SERVICE PROVIDERS	24

# COMMUNICATIONS	24

# LANGUAGE	24

# STS INSTRUCTIONS	25

# NAVIGATIONAL WARNINGS	26

# COMMUNICATIONS DURING OPERATIONS	26

# COMMUNICATIONS DURING APPROACH, MOORING AND UNMOORING	26

# PROCEDURES FOR COMMUNICATION FAILURE	27

# EQUIPMENT	27

# FENDERS	27

    - HOSES	30

# HOSE HANDLING	30

# HOSE CONNECTION	31

# PIPE REDUCERS	31

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

# MOORING EQUIPMENT	31

# PERSONNEL TRANSFERS	31

# LIGHTING	32

# ANCILLARY EQUIPMENT	32

# EQUIPMENT NOISE LEVELS	32

# EMERGENCY	32

# EMERGENCY SIGNAL	32

# EMERGENCY SITUATIONS	33

# EMERGENCIES DURING MANOEUVRING	34

# PROCEDURES IN THE EVENT OF GAS ACCUMULATION ON DECK	34

# ACCIDENTAL CARGO RELEASE	34

# SOPEP or VRP	34

# STATE OF READINESS FOR AN EMERGENCY	34

# RISK ASSESSMENT	35

# CONTINGENCY PLAN	36

  - SAFETY	37

# GENERAL SAFETY	37

# PREVENTION OF FATIGUE	37

# SAFE WATCH KEEPING	38

# HELICOPTER OPERATIONS	38

# DRILLS AND TRAINING	38

# PERSONAL PROTECTIVE EQUIPMENT AND LIFE SAVING APPLIANCES	38

# ACTION IN CASE OF INFRINGEMENT OF SAFETY	39

# ACTION IN CASE OF CARGO LEAKAGE	39

# SECURITY	39

# ARRIVAL	40

# OPERATIONAL PROCEDURES BEFORE MANOEUVRING	40

# PREPARATION OF SHIPS	40

# JOINT PLAN OF OPERATION	40

# NAVIGATIONAL SIGNALS	41

# LIGHTERING SUPPORT VESSELS	42

# BERTHING	43

# MANOEUVRING	43

# BASIC BERTHING PRINCIPLES	43

# MANOEUVRING ALONGSIDE WITH TWO SHIPS UNDER POWER	43

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

# GENERAL ADVICE FOR CONTROLLING THE TWO SHIPS	43

# ADVICE FOR MANOEUVRING ALONGSIDE	44

# MANOEUVRING A COMBINED TWO-SHIP SYSTEM TO ANCHOR	45

# UNDERWAY TRANSFER	45

# MANOEUVRES WITH ONE SHIP AT ANCHOR	46

# MANOEUVRING FOR IN PORT OPERATIONS	47

# MANOEUVRING WITH ONE SHIP ALONGSIDE A TERMINAL	47

# MOORING	48

# MOORING PREPARATIONS	48

# TENDING MOORING LINES	50

# TENSION IN MOORING LINES	51

# MOORING LINE LEAD ANGLES	51

# WEATHER THRESHOLDS	51

# LONG PERIOD WAVES	51

# DIRECTION OF WAVE ENCOUNTER	51

# ELASTICITY OF MOORING ARRANGEMENT	51

# CHAFING PROTECTION	52

# IN PORT MOORING OPERATIONS	52

# CARGO TRANSFER	53

# PRE-TRANSFER PROCEDURES	53

# RESPONSIBILITY FOR CARGO OPERATIONS	53

# PLANNING FOR CARGO TRANSFER	54

# CARGO TRANSFER	55

# GENERAL	55

# BALLAST OPERATIONS	55

# SUSPENSION OF CARGO TRANSFER	55

# VAPOUR BALANCING CONSIDERATIONS	56

# VAPOUR BALANCING CONSIDERATIONS BEFORE COMMENCING CARGO TRANSFER	56

# VAPOUR BALANCING CONSIDERATIONS DURING CARGO TRANSFER:	57

# VAPOUR HOSE CONSIDERATIONS	57

# SAFETY DURING CARGO TRANSFER	57

# SMOKING AND NAKED LIGHTS	57

# EARTHS ON ELECTRICAL SWITCHBOARDS	57

# MACHINERY OPERATION	58

# SHIP-TO-SHIP ELECTRIC CURRENTS	58

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - THE USE OF RADIO AND SATELLITE COMMUNICATIONS EQUIPMENT	59

    - RADAR USE	60

    - GAS ACCUMULATION	60

    - ELECTRICAL STORMS	60

    - GALLEY STOVES	61

    - READINESS FOR FIRE-FIGHTING EQUIPMENT	61

    - ACCOMMODATION OPENINGS	61

    - UNAUTHORISED CRAFT	61

  - OPERATIONS AFTER COMPLETION OF CARGO TRANSFER	61

  - BUNKERING AND STORING	62

  - DEPARTURE	63

  - UNMOORING	63

    - UNMOORING PROCEDURE WHILE ONE SHIP IS AT ANCHOR	63

    - UNMOORING PROCEDURE AFTER UNDERWAY TRANSFER	63

    - UNMOORING CHECKS	63

    - UNMOORING FROM A SHIP ALONGSIDE A TERMINAL	64

  - PROCEDURE FOR UNBERTHING	64

    - UNMOORING USING QUICK RELEASE APPARATUS OR TOGGLE PINS	64

APPENDIX I – CHECKLIST	66

APPENDIX II - PLAN	70

- GENERAL ARRANGEMENT PLAN	70

- MOORING ARRANGEMENT	71

3.0 TANK CAPACITY PLAN	72

APPENDIX III - RECORDS	73

RECORD OF STS OPERATIONS	73

APPENDIX IV - CONTACTS	74

APPENDIX V - EXAMPLE HOSE CONNECTION GUIDELINES	75

E.1 EXAMPLE HOSE CONNECTION GUIDELINES	76

APPENDIX VI - EXAMPLES OF MOORING EQUIPMENT MAP	77

VI. I EXAMPLE STS MOORING EQUIPMENT MAP	77

APPENDIX VII - MARPOL ANNEX II CARGOES (CHEMICAL)	78

G 1.0 CONTROL OF OPERATIONS	78

G 2.0 NOTIFICATION TO AND APPROVAL FROM AUTHORITIES	78

G 3.0 PRE-ARRIVAL COMMUNICATIONS	78

G 4.0 PLANNING FOR CARGO TRANSFER	79

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

# G 4.1 SOLIDIFYING CARGOES/INSULATION/HEAT DANGERS	79

# G 4.2 TANK CLEANING	79

# G 5.0 CARGO TRANSFER - GENERAL GUIDANCE	80

# G 5.1 VAPOUR CONTROL	80

# G 6.0 OPERATIONS AFTER COMPLETION OF CARGO TRANSFER	80

# G 6.1 CLEARING LINES AND DISCONNECTING	80

# G 6.2 PRECAUTIONS	81

## REVISION SHEET

All sections have been issued at Revision Number 0. Where individual sections have been subsequently revised, these are listed below. The changes within these sections are clearly highlighted in yellow.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| Section Amended | Revision Number | Date Amended |
| --- | --- | --- |
| CONTENTS & REVISION |  |  |
| RESPONSIBLE PERSONS |  |  |
| MARPOL ANNEX I AMENDMENTS |  |  |
| VESSEL PARTICULARS |  |  |
| SECTION 1 – INTRODUCTION |  |  |
| SECTION 2 – PRE-ARRIVAL PLANNING | 01 | 10-sept-2025 |
| SECTION 3 – ARRIVAL |  |  |
| SECTION 4 – BERTHING |  |  |
| SECITON 5 – CARGO TRANSFER |  |  |
| SECTION 6 – DEPARTURE |  |  |
| APPENDIX I – CHECKLISTS |  |  |
| APPENDIX II – PLANS |  |  |
| APPENDIX III – RECORD OF STS OPERATIONS |  |  |
| APPENDIX IV - CONTACTS |  |  |
| APPENDIX V - EXAMPLE HOSE CONNECTION GUIDELINES |  |  |
| APPENDIX VI - EXAMPLE STS MOORING EQUIPMENT MAP |  |  |
| APPENDIX VII – MARPOL ANNEX II CARGOES (CHEMICAL) |  |  |

## REFERENCES

- The CDI, ICS, OCIMF & SIGTTO “Ship to ship transfer guide for petroleum, chemicals and liquefied gases, Second edition 2020”

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

- International Maritime Organization, “Manual on Oil Pollution, Section I, Prevention”, Revised 1983

- International Chamber of Shipping, Oil Companies International Marine Forum & The International Association of Ports and Harbours, “International Safety Guide for	Oil Tankers and Terminals”, Witherbys Publishing & Seamanship International, Sixth Edition 2020.

- Oil Companies International Marine Forum, “Mooring Equipment Guideline,” Witherbys Publishing, February 2007.

- Oil Companies International Marine Forum, “Recommendations for Oil Tanker Manifolds and Associated Equipment,” Witherbys Publishing, February 2007.(NB - new document in progress by OCIMF)

- Oil Companies International Marine Forum, “Guidelines for the Handling, Storage, Inspection and Testing of Hoses in the field,” Witherbys Publishing, February 1995.

- Standard Marine Communication Phrases (IMO)

- International Regulations for Preventing Collisions at Sea (COLREGS) (IMO)

- Ship and Port Facility Security (ISPS) Code (IMO)

- International Convention on Standards and Training, Certification and Watch keeping and STCW Code (IMO)

- Guide to Helicopter / Ship Operations (ICS)

- Guidelines on the Use of High-Modulus Synthetic Fibre Ropes as Mooring Lines on Large Tankers (OCIMF)

- Recommendations for Ships’ Fittings for Use with Tugs with Particular Reference to Escorting and Other High Load Operations (OCIMF)

## GLOSSARY

Within this plan, the terms below have the following meaning.

### AT SEA

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

The term "at sea" is used throughout this guide. It is intended to indicate offshore waters or partially sheltered waters. It may be, however, that an STS transfer operation at sea is to be conducted within the jurisdiction of a local (port) authority or national government. In such cases reference has to be made to local regulations and it may also be necessary to obtain local approval.

### CLOSED OPERATIONS

Ballasting, loading or discharging operations carried out without recourse to opening ullage and sighting ports. In these cases ships will require the means to enable closed monitoring of tank contents, either by a fixed gauging system or by using portable equipment passed through a vapour lock.

### CONSTANT HEADING SHIP

During manoeuvring and mooring, the ship that maintains course and speed to allow the manoeuvring ship to approach and moor is referred to as the constant heading ship.

### DEDICATED LIGHTERING SHIP

A dedicated lightering ship is a ship designed to perform multiple STS operations. These ships are usually fitted with adequate primary and secondary (baby) fenders, which upon completion of an STS transfer are capable of being lifted and stowed in onboard cradles. They are usually fitted with their own hoses and are generally capable of performing STS operations without external assistance such as support craft. They may also be fitted with bow and stern thrusters, or large angle rudders to assist with manoeuvrability.

### DISCHARGING SHIP

The ship containing cargo for transfer to the receiving ship, and which may also be known as the Ship To Be Lightened (STBL).

### JOINT PLAN OF OPERATION {JPO)

An operation-specific plan that includes, as appropriate, reference to ship compatibility, manoeuvring, approach, mooring and transfer and, if applicable, references the ship-specific STS operations plans.

### MANOEUVRING SHIP

During manoeuvring and mooring, the ship that approaches the Constant Heading Ship for mooring operations is referred to as the manoeuvring ship.

### ORGANISERS

Organisers are shore-based operators responsible for arranging an STS transfer operation. The Organiser may be an STS Service provider.

### PERSON IN OVERALL ADVISORY CONTROL

For transfers at sea involving MARPOL Annex I cargoes, the person agreed to be in overall advisory control of an STS operation. It may be one of the Masters (generally the Master of the Manoeuvring ship) or it may be an STS Superintendent.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

### PRIMARY FENDERS

Primary fenders are large fenders capable of absorbing the impact energy of berthing and wide enough to prevent contact between the ships should they roll while alongside one another.

### RECEIVING SHIP

The ship to which cargo is transferred from the Discharging Ship. The Receiving Ship may also be known as the lightering ship, or Service Ship (SS).

### SECONDARY FENDERS

Secondary (baby) fenders are fenders used to prevent contact between the two ships, should they be rolling or not parallel to each other. They are especially effective when rigged towards the ends of a ship and are of most benefit during mooring and unmooring operations.

### SHIP-TO-SHIP (STS) TRANSFER OPERATION

An STS transfer operation is an operation where crude oil or petroleum products are transferred between seagoing ships moored alongside each other. Such operations may take place when one ship is at anchor or when both are underway. In general, the expression includes the approach manoeuvre, berthing, mooring, hose connecting, safe procedures for cargo transfer, hose disconnecting, unmooring and departure manoeuvre.

### STS SERVICE PROVIDER

Companies sometimes employed to organise and assist with STS transfers .The services offered by these companies vary, but often include the provision of personnel and equipment to facilitate the STS transfer.

The STS Service Provider may also supply the essential personnel and equipment needed, such as hoses, fenders and support craft.

The STS Service Provider may also be referred to as an STS Contractor or STS Resource Provider.

### STS SUPERINTENDENT

A person who may be designated to assist a ship's Master in the co-ordination and

supervision of the STS operation .This may include mooring and unmooring of the ships and/or the cargo transfer operation.

For transfers at sea involving MARPOL Annex I cargoes, the STS Superintendent may also fulfil the role of POAC. The STS Superintendent may also be known as Lightering Master, Mooring Master or Transfer Supervisor.

### SWL

SWL or Safe Working Load is the operating limit to which equipment is tested for day-to-day use. Equipment should never be used beyond its SWL.

### TRANSFER AREA

A transfer area is an area within which an STS transfer operation takes place. Transfer areas should be selected in safe sea areas (see Section 2.3). In coastal areas they will be agreed with nearby coastal authorities and, as appropriate, in accordance with specific port or national regulations. The expression "transfer at anchor" describes an operation where a cargo transfer is carried out

### TRANSFER AT ANCHOR

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

Transfer between ships when they are moored alongside each other and one of the ships is at anchor. The operation is an alternative to underway transfer.

### UNDERWAY

By definition under the International Regulations for Preventing Collisions at Sea (COLREGS) a ship when underway is not at anchor, made fast to the shore or aground. However, she may be either steaming or drifting freely with current and weather.

### VAPOUR BALANCING

May be used to avoid the release of cargo vapours to the atmosphere by interconnecting the vapour systems of the receiving and discharging ships.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

## SECTION I: RESPONSIBLE PERSONS FOR STS OPERATIONS

The document is to be circulated to all ship’s staff that will be responsible for ship to ship transfer operations. After reading the STS Plan below is to be signed.

| Name | Rank | Date Joined | Signature and date |
| --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

## SECTION II : MARPOL ANNEX I AMENDMENTS

### MARPOL Annex I Amendments

Resolution MEPC.186(59) was adopted at MEPC 59 and contains a new Chapter 8 to MARPOL Annex I on the prevention of pollution during the transfer of oil cargo between oil tankers at sea. Find below Regulation 40, 41 & 42 from the new Chapter 8 to MARPOL Annex I.

### Regulation 40 Scope of application

  - The regulations contained in this chapter apply to oil tankers of 150 gross tonnage and above engaged in the transfer of oil cargo between oil tankers at sea (STS operations) and their STS operations conducted on or after 1 April 2012. However, STS operations conducted before that date but after the approval of the Administration of STS operations Plan required under regulation 41.1 shall be in accordance with the STS operations Plan as far as possible.

  - The regulations contained in this chapter shall not apply to oil transfer operations associated with fixed or floating platforms including drilling rigs; floating production, storage and offloading facilities (FPSOs) used for the offshore production and storage of oil; and floating storage units (FSUs) used for the offshore storage of produced oil1.

  - The regulations contained in this chapter shall not apply to bunkering operations.

  - The regulations contained in this chapter shall not apply to STS operations necessary for the purpose of securing the safety of a ship or saving life at sea, or for combating specific pollution incidents in order to minimize the damage from pollution.

  - The regulations contained in this chapter shall not apply to STS operations where either of the ships involved is a warship, naval auxiliary or other ship owned or operated by a State and used, for the time being, only on government non-commercial service. However, each State shall ensure, by the adoption of appropriate measures not impairing operations or operational capabilities of such ships that the STS operations are conducted in a manner consistent, so far as is reasonable and practicable, with this chapter.

### Regulation 41 General Rules on safety and environmental protection

- Any oil tanker involved in STS operations shall carry on board a Plan prescribing how to conduct STS operations (STS operations Plan) not later than the date of the first annual, intermediate or renewal survey of the ship to be carried out on or after 1 January 2011. Each oil tanker’s STS operations Plan shall be approved by the Administration. The STS operations Plan shall be written in the working language of the ship.

- The STS operations Plan shall be developed taking into account the information contained in the best practice guidelines for STS operations identified by the Organization2. The STS operations Plan may be incorporated into an existing Safety Management System required by chapter IX of the

- Revised Annex I of MARPOL, chapter 7 (resolution MEPC.117(52)) and UNCLOS article 56 are applicable and address these operations.

- IMO’s “Manual on Oil Pollution, Section I, Prevention” as amended, and the ICS and OCIMF “Ship to Ship Transfer Guide, for Petroleum, Chemicals and Liquefied Gases”, 1st edition, 2013

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

International Convention for the Safety of Life at Sea, 1974, as amended, if that requirement is applicable to the oil tanker in question.

- Any oil tanker subject to this chapter and engaged in STS operations shall comply with its STS operations Plan.

- The person in overall advisory control (POAC) of STS operations shall be qualified to perform all relevant duties, taking into account the qualifications contained in the best practice guidelines for STS operations identified by the Organization3.

- Records4 of STS operations shall be retained on board for three years and be readily available for inspection by a Party to the present Convention.

### Regulation 42 Notification

- Each oil tanker subject to this chapter that plans STS operations within the territorial sea, or the exclusive economic zone of a Party to the present Convention shall notify that Party not less than 48 hours in advance of the scheduled STS operations. Where, in an exceptional case, all of the information specified in paragraph 2 is not available not less than 48 hours in advance, the oil tanker discharging the oil cargo shall notify the Party to the present Convention, not less than 48 hours in advance that an STS operation will occur and the information specified in paragraph 2 shall be provided to the Party at the earliest opportunity.

- The notification specified in paragraph 1 of this regulation5 shall include at least the following:

  - name, flag, call sign, IMO Number and estimated time of arrival of the oil tankers involved in the STS operations;

  - date, time and geographical location at the commencement of the planned STS operations;

  - whether STS operations are to be conducted at anchor or underway;

  - oil type and quantity;

  - planned duration of the STS operations;

  - identification of STS operations service provider or person in overall advisory control and contact information; and

  - confirmation that the oil tanker has on board an STS operations Plan meeting the requirements of regulation 41.

- If the estimated time of arrival of an oil tanker at the location or area for the STS operations changes by more than six hours, the master, owner or agent of that oil tanker shall provide a revised estimated time of arrival to the Party to the present Convention specified in paragraph 1 of this regulation.”

Note :        PANAMA FLAG Vessels - additional requirements;

a)     To ensure that ships flying the flag of the Republic of Panama adhere to measures which lawfully

prohibit or regulate STS transfers operations, and further adhere to the spirit of the safety and pollution

prevention requirements in IMO conventions, especially chapter 8 of Annex I of the International

Convention for the Prevention of Pollution from Ships (MARPOL), 1973, to minimize the risk of oil pollution; the Republic of Panama requires according to Resolution No.106-035-DGMM from April 11, 2025, that ships update their STS Operations Plan to include notifying this Administration when and where they are engaged in such operations, especially when they are engaged in a mid-ocean STS transfer

b)  All Ship Operator Companies of all Panamanian flagged oil tankers of 150 GT and above shall update their STS Operations Plan to include the Notification to the Flag State regarding when and where this STS operations will take place

c)  All Ship Operator Companies of all Panamanian flagged oil tankers of 150 GT and above should contact the Recognized Organizations to obtain approval for updated STS Operations Plan

d) Records of oil cargo transfer operations required to be recorded in the Oil Record Book and Emissions Part II – Cargo and Ballast operation Book, and any records required by the STS Operations Plan shall be retained on board for three years.

# STS transfer Notification in a mid-ocean operation

- STS transfer notification is crucial to minimize the risk of oil pollution and ensuring compliance with safe shipping standards, especially when transferring cargo between ships in a mid-ocean

- Starting from August 6th, 2025, all Panamanian flagged oil tankers, acting as discharging or/and receiving vessel must notify their STS transfer operations to the Flag State through the E-Segumar Platform at the following link: https://certificates.amp.gob.pa/certificates

- In the event that the STS operation is carried out between two Panamanian flagged oil tankers, both vessels must notify the STS transfer operation to Panama Flag State through the E-Segumar Platform

- All the Panamanian flagged oil tankers must maintain valid and updated Continuous Synopsis Record (CSR) and Conformance Test Report (CTR) on board at the time of notifying the STS operations. Failure to comply with the aforementioned will result denying the authorization for STS operations until compliance is demonstrated.

- The STS transfer Notification must be provided well in advance and at least forty-eight (48) hours prior to the planned STS transfer operations. An acknowledgement of receipt will be issued by the Flag State through the E-Segumar Platform which shall be kept on board in electronic format for a minimum period of three (3) years.

  - The STS transfer notification shall include the following information:

    - Name, flag, call sign, IMO Number and estimated time of arrival of the oil tankers involved in the STS transfer operation.

    - Date, time and geographical position of the commencement of the planned STS transfer operation.

    - Mode in which the STS transfer operation will be carried out: at anchor or underway.

    - Type of oil and its quantity.

    - Expected duration of the STS transfer operation.

    - Identification and contact information (email, telephone number) of the Designated Person Ashore (DPA) of each oil tanker involved in the STS operation.

    - Confirmation that the oil tanker has on board an STS operations Plan complying the requirements of regulation 41, Annex I, MARPOL Convention

- If the estimated time of arrival of an oil tanker at the location or area for the STS transfer operation changes by more than six hours or in the event that the STS operation is cancelled due to any issue, the Master, Owner, or Designated Person Ashore (DPA) must immediately inform the Administration via the following email address : stspma@amp.gob.pa

- Failure to provide timely notification may lead to administrative measures, in accordance with the applicable maritime regulations and oversight procedures established by the Administration

# Enforcement

- All Panama flagged Oil Tankers of 150GT and above, shall comply with the provisions of the Resolution No. 106-048-DGMM of November 19, 2019, on LRIT and AIS provisions (especially regulations 19 and 19-1 of chapter V of SOLAS Convention), and the National legislation of the Republic of Panama; meaning that when ships are engaged in STS transfer operations the AIS and LRIT must be working properly

- It is prohibited that Panamanian flagged Oil Tankers of 150GT and above engage in STS transfer operations with sanctioned vessels included in the international sanction lists recognized by the Panamanian State.

## SECTION III. SHIP’S PARTICULARS

- IMO’s “Manual on Oil Pollution, Section I, Prevention” as amended, and the ICS and OCIMF “Ship to Ship Transfer Guide, for Petroleum, Chemicals and Liquefied Gases”, 1st edition, 2013

- Revised Annex I of MARPOL chapters 3 and 4 (resolution MEPC.117(52)); requirements for recording bunkering and oil cargo transfer operations in the Oil Record Book, and any records required by the STS operations Plan.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

# Ship Data

| Deadweight (t) |  |
| --- | --- |
| Length Overall (m) |  |
| Breadth Extreme (m) |  |
| Depth Moulded (m) |  |
| Max. Manifold height above waterline (m) |  |
| Liquid Manifold (number/size) |  |
| Vapour Manifold (number/size) |  |
| Manifold dimensions |  |
| Bow to Centre Manifold |  |

### Tank Capacities:

Capacity plan included Appendix ii section plans

Manifolds port and starboard have drip trays with capacity

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

  - INTRODUCTION

This STS Transfer Operations Plan (STS Plan) has been developed in accordance with the standards describe in MARPOL Annex I, as amended by Resolution MEPC.186(59), Chapter 8: Prevention of Pollution during Transfer of Oil Cargo between Oil Tankers at Sea, Regulations 40, 41, 42.

This plan is a ship specific Ship to Ship (STS) operations plan for the transfer of crude oil and petroleum products between two ships moored together. The plan was developed using ICS/OCIMF Ship to Ship Transfer Guide [2], IMO Manual on Oil Pollution [3] and other publications as referenced. It is recommended that publications referred to in this STS plan should be kept onboard.

When transfers are performed within certain Exclusive Economic Zones, Territorial Waters or Port Limits it may be found that national or local regulations apply. National or local regulations should be followed and, where appropriate, the recommendations made in this plan can be used as additional advice.

The Person in Overall Advisory Control (POAC) of STS operations shall be qualified to perform all relevant duties, taking into account the qualifications contained in the best practice guidelines for STS operations identified by the Organization, as required by MARPOL Annex I, Chapter 8, Regulation 41.4

Records of STS operations shall be retained on board for three years and be readily available for inspection by a party to the MARPOL Convention, as required by MARPOL Annex I, Chapter 8, Regulation 41.5.

The Plan contains guidance on general conditions and operational requirements for vessel involved in Ship to Ship Transfer Operations. The Appendices contain list of contact points for the Notification of STS operations, records of STS operations, hose connection guide, mooring arrangements and other reference material.

In ship to ship transfers, both tankers should comply fully with the safety precautions required for normal cargo operations.

Routine drills conducted on board will not only ensure that the ship’s staff are familiar with their duties but will assist in forming a proficient team to combat all pollution incidents in an efficient manner

The Plan has been approved by …on behalf of The Flag Administration and, except as provided below, no alteration or revision shall be made to any part of it without the prior approval of the …Changes to the Appendices will not be required to be approved by the … The Appendices should be maintained up to date by the vessel's managers, owners and operators.

APPROVAL

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

  - SCOPE

The Manual should be written in the working language of the master and officers and, if the working language of the master and officers is not English, French, or Spanish, include a translation into one of these languages.

As per Management System (SMS) Fleet Operations 4.5 the command language to be used in port is English. This applies to STS Operations

A copy of the STS Plan is to be available at the following locations on each oil tanker.

    - Bridge

    - Cargo transfer control station

    - Engine-room.

  - OBJECTIVES

The STS Plan should contain the following information:

    - a step-by-step description of the entire STS operation;

    - a description of the mooring and unmooring procedures and arrangements, including diagrams where necessary, and procedures for tending the oil tankers’ moorings during the transfer of cargo;

    - a description of the cargo and ballast transfer procedures, including those used while underway or anchored and procedures for connecting cargo hoses, topping off cargo tanks and disconnecting cargo hoses;

    - the titles, locations and duties of all persons involved in the STS operation;

    - procedures for executing the emergency shut-down and communication systems and for rapid breakaway;

    - a description of the drip trays and procedures for emptying them;

    - procedures for reporting spillages of oil into the water;

    - an approved contingency plan. (See section 3)

    - a cargo and ballast plan;

    - a description of deployment and un-deployment of fenders; and

    - a description of hose connection and disconnection procedures.

Note: It is understood that some local authorities will required that an STS Operator Company intended to provide STS transfer services within their territorial waters must be licensed by the appropriate Authorities. The STS Operator Company in some cases should also have in place a Site Specific Manual for Ship to Ship transfer. This Site Specific Manual will need to be approved by the appropriate local Authority, to be able to operate in its Territorial Waters.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

  - STS PROCESS

The purpose of this STS operations plan is to provide guidance to the master and or STS operator on board the ship in conducting the transfer in a safe and efficient manner. The STS operation are divided into five phases; Pre-Arrival Planning, Arrival, Berthing, Cargo Transfer and Departure. During each phase of the operation there are different procedures to follow and check-lists to complete. A risk assessment should be carried out before operations commence and a contingency plan should be put in place to deal with emergencies.

### Phase 1 – Pre-Arrival Planning:

The Pre-Arrival Planning should be carried out before any STS operations commence. The planning of the STS operation could be performed by the ships operators and managers together.

### Phase 2 – Arrival:

During the Arrival stage of the operation the two ships conducting the STS operation should establish radio communication, and concur that all operational procedures has been assessed and agreed upon.

### Phase 3 – Berthing:

The Berthing phase of the STS operation consists of the manoeuvring and mooring of the two ships. For the manoeuvring operation a contingency plan should be in place to deal with any emergency situation.

### Phase 4 – Cargo Transfer:

The Cargo transfer phase should only commence once the two ships are securely moored. The Master should ensure that all the recommended safety procedures in this plan are followed. A contingency plan should be in place to deal with any emergency situations during cargo transfer.

### Phase 5 – Departure:

Departure is the last phase of the STS operation.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

## PRE-ARRIVAL PLANNING

  - CONDITIONS AND REQUIREMENTS

    - SHIP COMPATIBILITY

It should be ensured that the ships are compatible in design and equipment; that they comply with the various recommendations included in this plan; and that mooring operations, hose handling and communications can be conducted safely and efficiently. It is essential that information relating to the overall dimensions, freeboard, position of manifolds, mooring points and fenders should be passed to the Masters of the other ship(s).

It is recommended that ships with bridge wings that extend beyond the ship’s maximum breadth are not used for STS transfer operations. Consideration should be given to any precautions and mitigating measures necessary regarding bridge wings that do not extend to ship side.

The following should be determined prior to berthing:

| CARGO HANDLING COMPATIBILITY |  |
| --- | --- |
| 1. | The size and number of manifolds to be used. |
| 2. | The minimum and maximum expected height of the manifold above the waterline during the transfer operation, and the freeboard differences during the cargo transfer (see 5.2). |
| 3. | Whether the cargo cranes or derricks are in satisfactory condition and of suitable safe working load (SWL). |
| 4. | The hose supports at the ship’s side are the adequate to prevent damage to the hose through chafing. |
| 5. | That both ships have manifolds that comply with OCIMF Recommendations for Oil Tanker Manifolds and Associated Equipment. |

### Table 1: Cargo handling compatibility

    - PERSON IN OVERALL ADVISORY CONTROL (POAC)

A ship-to-ship transfer operation should be under the advisory control of a designated Person in Overall Advisory Control (POAC). The POAC will either be one of the Masters of the vessels concerned or an STS Superintendent, Lightering Coordinator or Mooring Master employed by an STS Resource Provider. It is not intended that the POAC in any way relieves the ships’ Masters of any of their duties, requirements or responsibilities.

The Administration, cargo owners or oil tanker's operators should agree and designate the POAC who should have at least the following qualifications:

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

      - An appropriate management level deck license or certificate meeting international certification standards, with all STCW and dangerous cargo endorsements up to date and appropriate for the ships engaged in the STS operation;

      - Attendance at suitable ship-handling course;

      - Conduct of a suitable number of mooring/unmooring operations in similar circumstances and with similar vessels;

      - Experience in oil tanker cargo loading and unloading;

      - A thorough knowledge of the geographic transfer area and surrounding areas;

      - Knowledge of spill clean-up techniques, including familiarity with the equipment and resources available in the STS contingency plan;

      - Thorough knowledge of the STS Plan.

The POAC should:

      - Ensure that the cargo transfer, mooring and unmooring operations are conducted in accordance with the required STS plan, the contents of this chapter of the Manual and take into account the recommendations contained in the industry publication "CDI, ICS, OCIMF & SIGTTO -Ship to ship transfer guide for petroleum, chemicals and liquefied gases, first edition 2013";

      - Advise the Master(s) of the critical phases of the cargo transfer, mooring and unmooring operation;

      - Ensure the provisions of the contingency plan are carried out in the event of a spill;

      - Ensure that all required reports are made to the appropriate authorities;

      - Ensure that crewmembers involved in each aspect of the operation are properly briefed and understand their responsibilities;

      - Ensure that fenders appropriate for the size of ships involved in the STS operation are deployed so as to provide most effective standoff between the structure of both ships;

      - Ensure that approach and mooring operations are not attempted until proper effective communication has been confirmed between the two oil tankers and appropriate checks have been completed;

      - Ensure that a pre-transfer STS safety check is undertaken in accordance with accepted industry guidance; and

      - Closely monitor prevailing environmental conditions and weather forecasts relevant to the location at which the STS is being conducted;

      - Ensure connection and disconnection of cargo hoses and fenders is conducted in a manner which will prevent the potential for loss of containment.

As well as the Masters, the POAC has the authority to:

      - Suspend or terminate the transfer operation;

      - Amend the transfer plan for the particular operation.

    - MASTER

It is the responsibility of the Master to ensure that the Plan is current and that the STS operations are conducted according to the requirements described on this Plan, ICS/OCIMF Ship to Ship transfer Guide –

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 1 | 10-sept-2025 | MD |

Petroleum and ISGOTT, as well as to maintain relevant records (see Appendix III). Both ship’s Masters are responsible for respective operations, in spite that one of them may be agreed to be in overall advisory control of the operation (POAC). Each must satisfy himself that procedures reflect safe practice. Even in case an STS Superintendent is employed, the Master retains his overall responsibility for the safety of the ship and its crew.

    - STS SUPERINTENDENT / LIGHTERING COORDINATOR / OR MOORING MASTER

If the Master is unfamiliar with, or inexperienced in STS transfer operations, an STS Superintendent (employed by specialized STS Resource provider) may be employed to advise them. In that case, the quality of services and equipment it can provide are paramount if operations are to be carried out safely, reliably and efficiently. Before employing such assistance the Master, shipping company, or STS organizer should ensure that STS Resource provider will provide trained and experienced STS Superintendents. The STS Resource provider should provide evidence of the quality and competence of the intended STS Superintendent.

    - RECORDING AND CHECKLISTS

The STS operation should be recorded in the Oil Record Book Part II, as required for all oil cargo transfer and ballast operations. The checklists contained in Appendix I, should be followed and filled out concurrently with the STS operation.

All records should be retained onboard for at least three years.

    - APPROVAL FROM AUTHORITIES

When the STS transfer is performed within the territorial waters or Exclusive Economical Zone of a country, local and national regulations should be checked and appropriate approvals obtained. The responsible person should inform the appropriate authorities of STS transfer operations to be conducted in the lightering area.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| Minimum 48 hours in advance NOTICE TO AUTHORITIES |  |
| --- | --- |
| 1. | Name, flag, call sign, IMO Number and estimated time of arrival of the oil tankers involved in the STS operations. |
| 2. | Date, time and geographical location at the commencement of the planned STS operations. |
| 3. | Whether STS operations are to be conducted at anchor or underway. |
| 4. | Oil type and quantity. |
| 5. | Planned duration of the STS operations. |
| 6. | Identification of STS operations service provider or person in overall advisory control and contact information. |
| 7. | Confirmation that the oil tanker has onboard an STS operations plan meeting the requirements of MARPOL Annex I, Chapter 8, Regulation 40, 41 & 42. |

### Table 2: Advance Notice to Authorities

Where, in an exceptional case, all the information specified above is not available more than 48 hours before the STS operations are to take place, the oil tanker discharging the oil cargo shall notify the authorities not less than 48 hours in advance that an STS operation will occur. The information specified above shall be provided to the authorities at the earliest opportunity.

Once the initial report for any ETA has been made, it should, if possible, be updated when a variance of more than two hours is expected from the time given in the latest report.

    - TRANSFER AREA

The STS transfer area should be specially selected for safe operations, in co-ordination

| TRANSFER AREA |  |
| --- | --- |
| 1. | Notify and obtain agreement from the applicable coastal authority. |
| 2. | The shelter provided from the weather, particularly from sea and swell. |
| 3. | Present and forecast weather conditions. |
| 4. | Tidal current conditions. |
| 5. | Safe distances from offshore installations. |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| 6. | The availability of a designated lightering area. |
| --- | --- |
| 7. | Sufficient sea-room to be available to allow for normal drift or streaming patterns when the cargo transfer operations are conducted underway. |
| 8. | Sufficient sea-room and water depth required for manoeuvring during berthing and unberthing. |
| 9. | Locations of underwater pipelines, cables, artificial reefs or historic sites. |
| 10 | Selection of a safe anchorage with sufficiently good holding ground. |
| 11. | Traffic density. |
| 12. | Availability of emergency and oil spill response capability. |
| 13. | Distance from shore logistical support. |
| 14. | Security threat. |

### Table 3: Transfer area

    - WEATHER CONDITIONS

The weather condition limit will mostly depend on the effect of the sea and swell on the fenders or mooring lines and the rolling movements induced by the participating ships, taking into account the relative freeboard and displacement. as well as the local weather knowledge from the PAOC as well as previous experience of all those involved. The cargo hose disconnect and abort STS weather criteria are to be agreed in writing between all involved – vessels and STS service provider –prior the STS operation commences.

The following items should be taken into account regarding weather condition:

      - If the transfer takes place with one ship at anchor, special care should be given to the ultimate strain placed on the anchor cable due to yawing movements aggravated by the current and weather condition.

      - Weather forecasts should be obtained before and during the transfer. The weather should be constantly conducive for STS operations given the size of the vessels involved

      - Throughout the berthing operation the visibility should be good enough for safe manoeuvring, taking into account the safe navigation and collision avoidance requirements.

      - Special attention should be given to electrical storms.

See OCIMF publication “Mooring Load Analysis during Ship to Ship Transfer Operations” in main STS menu under “Related Publications”.

    - NIGHT TIME STS MANOEUVRING

Should the vessels be required to carry out night time STS manoeuvring for coming alongside then :

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

- A ship specific risk assessment for same should be drawn up ( with the assistance also from the other STS vessel as well as the STS service provider as applicable ) which takes into account the following –

  - Master and Bridge Team experience with night time STS operations;

  - POAC experience with night time STS operations;

  - Plan of approach and contingencies to abort the manoeuvre;

  - Port and Local approvals sought for night-time STS;

  - Weather conditions obtained;

  - Ships mooring equipment, power and propulsion engines in satisfactory operational condition;

  - Fendering equipment details obtained and confirmed in good order;

  - Vessels to display the correct navigational lights;

  - Vessels deck and bridge wing lighting in satisfactory condition.

- Night time STS to take place only once expressly agreed by the master.

Note : If, at any stage before or during such night time STS operations, the Master considers such operations to be unsafe for any reason, including the adequacy of fendering, the weather and sea conditions, the condition of any other vessel involved and the competence of its crew, he may refuse or order immediate cessation of same.

Night time unberthing and cargo operations are allowed.

    - QUALITY ASSURANCE OF SHIP TO SHIP SERVICE PROVIDERS

If an STS service provider is employed, the quality of the services and equipment provided are paramount if operations are to be carried out safely, reliably and efficiently. There are currently no international standards for STS service providers. While some form of International Organization for Standardisation (ISO) accreditation, if available, will provide an assurance that an STS service provider has the necessary resources, afloat and ashore, to provide a quality service, this should not be the only determinant of quality. Performance records and previous industry experience may be important when assessing an STS service provider's ability to meet customer and regulatory requirements.

STS service providers should be aware that they may be subject to assessment by users of their services.

Self-assessment can be used by service providers to verify that their safety management systems (SMS) are comprehensive and sufficiently robust to minimise all potential safety and environmental risks in the execution of their operations and to measure and continuously prove their management systems. Guidance on this process is contained in Ship to Ship Service Provider Management.

  - COMMUNICATIONS

    - LANGUAGE

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

A common language for communication should be agreed before operations commence. This should normally be English.

    - STS INSTRUCTIONS

The organisers generally provide STS instructions. This may be the operator of the ships if carrying out "in- house" operations or it may be an STS Service provider. Normally such providers send advance STS instructions to the ships concerned.

The following information should be received from involved ships.

| INFORMATION REQUIRED FROM THE SHIPS |  |
| --- | --- |
| 1. | Confirmation of the integrity of systems e.g. navigational, machinery, steering gear, cargo system, crude oil washing (COW), inert gas system (IGS), fire-fighting, mooring equipment, derrick or cranes, etc. |
| 2. | Confirmation of estimated time of arrival (ETA) at agreed intervals. |
| 3. | Confirmation that copies of the Ship to Ship Transfer Guide and appropriate accepted industry guides are on board and that ship's personnel are conversant with the procedures within them. |
| 4. | Cargo details, including copies of the material safety data sheets (MSDS). |
| 5. | Confirmation of arrival draught, freeboard and height of manifold above water line, including anticipated changes during the transfer. |
| 6. | Confirmation that the ship comp lies with applicable local, national and international requirements, including those relating to hours of work/rest. |

### Table 4: Information required from the ships

| ADVICE TO BE GIVEN TO SHIPS BY THE SHIP TO SHIP ORGANISERS |  |
| --- | --- |
| 1. | The STS organiser’s full title and contact numbers. |
| 2. | A description of the planned STS operation, including the location of the transfer area. |
| 3. | Details of equipment (including confirmation of integrity of hoses, fenders, etc), logistical support and personnel provided. |
| 4. | Requirements for the preparation of moorings, manifold and lifting gear |
| 5. | Local and national STS regulations where applicable. |
| 6. | Identity of the STS service provider and/or STS Superintendent where applicable |
| 7. | Copy of risk assessment identifying appropriate measures to ensure the safe conduct of STS at scheduled transfer area. |
| 8. | Copy of Joint plan of operation (JPO) |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Table 5 : Advice to be given to the ships by the ship to ship organisers

    - NAVIGATIONAL WARNINGS

The following navigational warnings should be broadcasted, before and during the transfer operation, to all ships advising:

| NAVIGATIONAL WARNINGS |  |
| --- | --- |
| 1. | The name and flag of the ships involved. |
| 2. | Geographical position of operations and general headings. |
| 3. | Nature of operations. |
| 4. | Time of starting operations and expected duration. |
| 5. | Request for wide berth and the need to exercise caution when navigating in the transfer area. |

### Table 6: Navigational warnings during transfer

Note: The Automatic Identification System (AIS) must be operational on at least one ship.

On completion of the STS operation, the person having overall advisory control or his designee should cancel the navigational warning

    - COMMUNICATIONS DURING OPERATIONS

Contact should be established on the appropriate VHF channel at the earliest opportunity. Approach, mooring and unmooring should not be attempted until proper effective communication has been confirmed between the two ships.

It should be confirmed that the portable radios on each ship are capable of working on the same frequencies, alternatively that one of the vessels have enough portable devices to adequately supply both vessels. Ship's officers responsible for mooring stations should be provided with portable radios. Ship's emergency portable VHF radios should not be used for routine operations. AIS must be operational on at least one ship.

    - COMMUNICATIONS DURING APPROACH, MOORING AND UNMOORING

As the ships come into the transfer area, contact should be established on the appropriate VHF channel at the earliest opportunity, thereafter switching to a mutually agreed working channel. Approach, mooring and unmooring should not be attempted until proper effective communication has been confirmed between the two ships. Ship’s officer responsible for mooring stations should be provided with portable radios.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Confirmations should be sought that the portable radios on each ship are capable of working on the same frequency. In the event that common frequency is not available, provision should be made to exchange compatible equipment between the ships.

It is recommended that internal communications on each ship are maintained on separate frequencies to avoid misunderstanding when operations are being conducted on multiple vessels. In addition all radio transmissions should be prefixed with the vessel’s name.

When conducting operations in port, there may be a need to communicate with additional parties such as the port authority, a terminal, pilots, tugs and line handlers.

    - PROCEDURES FOR COMMUNICATION FAILURE

If communication breakdown occurs during the approach manoeuvre, the manoeuvre should be aborted if appropriate and safe to do so and the subsequent actions taken by each ship should be indicated by the appropriate sound signals as prescribed in the International Regulations for Preventing Collisions at Sea (COLREGS) - [8].

If communication is lost during cargo transfer the emergency signal should be sounded and all operations in progress should be suspended immediately if it is safe to do so, operations should not be resumed until satisfactory communications have been re-established.

  - EQUIPMENT

    - FENDERS

Fenders may be secured in place on either ship, but it is recommended that they are placed on the manoeuvring ship. It should be noted that where fenders are to be rigged on the manoeuvring ship there may be more stresses on the head fender wire, and that the fender wire utilizes one head wire per winch.

### Figure 1: Typical arrangement of fenders rigged in a continuous string

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Figure 2: Fenders rigged in pairs

When fenders are fitted to the manoeuvring ship, primary fenders should be positioned one at each end of the parallel body, with similar additional units fitted in between. The fender string may be made up to a pre- arranged length. Alternatively in some operations where four fenders are used, it has been found beneficial to position them in two groups of two. In this way, and with each group positioned well forward or well aft on the parallel body, better protection can be provided. Secondary fenders may be positioned fore and aft of the parallel body. Fender moorings should be monitored frequently and tended as necessary to ensure that they do not become too slack or too taut and that the fenders remain in position. The length of the fender string should be such that the fenders will be able to distribute the maximum anticipated impact load within the parallel body of both ships.

If fenders are provided by an STS Service provider, the Master, responsible person or organiser should ascertain the age of the fenders to be used and should be satisfied that reasonable measures have been taken to ensure that they are fit for the intended service. The fender certificates should be made available to assist with this.

FENDER SELECTION ASSISTANCE REQUEST FORM For Ship-to-Ship Use (To be filled out prior to contacting fender providers)

Location of Site Potential Sea State Potential Beaufort Scale Ship A Ship B

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| Type of ship Displacement Tonnage (start of STS ops.) Gross Tonnage Deadweight Tonnage Length Overall Length Between Perpendiculars Loaded Draft Beam Freeboard when coming into contact Relative approach velocity of ships |  |  |
| --- | --- | --- |
| Other relevant information |  |  |

### Table 8 : Fender Selection Assistance Request Form

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - HOSES

The diameter for a chosen cargo transfer hose is governed mainly by the required flow rate and the manifold dimensions.

Hose lengths should be considered on a case-by-case bases and but as a reference, hose lengths equal to twice the maximum difference in manifold height between the two ships are usually sufficient to allow for variables during transfer.

A rule of thumb for calculating the minimum bending radius (MBR) of a rubber hose is given by the formula:

### MBR = Nominal Bore of Hose x 6

As the tanker rises or falls as a result of cargo transfer, the hose should be adjusted so as to avoid undue strain on the hoses, connections and ship’s manifold and to ensure that the radius of the curvature of the hose remains within the limits recommended by the manufacturer.

If hoses are provided by an STS Service provider, the Master, shipping company or organiser should ascertain the age of the hoses to be used and should be satisfied that reasonable measures have been taken to ensure that they are fit for the intended service.

The hoses used for the STS transfer of crude oils or petroleum products should be specially designed and constructed for the product being handled and the purpose for which they are being used. Hoses used should comply with EN1765 (or latest equivalent) with regard to specification for the assemblies and with BS1435 (or latest equivalent) and OCIMF guidelines with regard to their handling, inspection and testing. A visual inspection of each of the hose assemblies should be carried out before they are connected to the manifolds to determine if any damage has been caused.

Hoses should be marked with the following:

- the manufacturer’s name or trademark;

- identification of the standard specification for manufacture;

- factory test pressure (Note: equal to rated working pressure, maximum working pressure, maximum allowable working pressure);

- month and year of manufacture and manufacturer’s serial number;

- indication that the hose is electrically continuous or electrically discontinuous, semi-continuous or anti-static; and

- the type of service for which it is intended, e.g., oil or chemical.

The hose certificates should be made available by the STS Service provider and test data with respect to each hose should be available and should be sighted prior to the hose being used for transfer.

If there is any doubt regarding the condition of the hose(s), they should not be used.

    - HOSE HANDLING

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Although oil hoses are robustly constructed for a marine environment, they can be damaged from improper handling. In general, while handling hoses adequate support is the key to the prevention of over-bending (kinking), which can lead to premature hose retirement.

When transferring the one end of the hose to the other ship, lifting straps should be used that are preferably flat nylon or equivalent reinforced cloth bands, and at least 150 mm wide to prevent any chafing of the hose cover. If nylon or equivalent straps are not available, the best substitute is a sling of large circumference nylon or polypropylene rope. Wire should not be used.

    - HOSE CONNECTION

Flanges and quick connect/disconnect (QC/DC) couplings should be in good condition and properly secured and properly secured to ensure leak tight connections. The gaskets used at the ship’s manifolds and between each hose should be made from a material suitable for use with the cargo to be transferred. The vessel will be expected to provide all necessary personnel to connect the hoses. The connection operation should be properly supervised and integrity of the connection confirmed. Prior commencing transfer, the integrity of the hose assemblies should be verified by a pressure test.

    - PIPE REDUCERS

It should be ascertained that adequately sized pipe reducers with packings are available onboard, to accommodate actual manifold and hose dimensions.

    - MOORING EQUIPMENT

Typically a mooring pattern for exposed locations for lightering vessels not fitted with special mooring arrangements would consist of at least six head lines, two forward and two back springs, and four stern lines. Where specialised mooring equipment is fitted (e.g. on dedicated STS transfer ships) the number of headlines could be reduced to four where this has proven to be reliable for the local operating environment.

    - PERSONNEL TRANSFERS

      - PERSONNEL TRANSFERS AT SEA OPERATIONS

In general, it is recommended that the transfer of personnel between ships be kept to an absolute minimum. If the transfer of personnel is unavoidable, the safest means of transfer should be verified by risk assessment.

This assessment should consider the prevailing circumstances and conditions at the time and place in question and should compare the risks associated with any alternative means of transfer that may be available.

The lifting equipment shall be inspected, maintained, load tested and certified as required. Checklist for “transfer of personnel” shall be included as appendix.

      - PERSONNEL TRANSFERS - IN PORT OPERATIONS

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

For STS operations conducted at anchor in sheltered waters or alongside in port, there will be varying needs to establish a safe means of access between involved vessels. For example, during double banked transfers, personnel on the outer vessel(s) may need to traverse the vessel alongside to access the terminal.

In all cases, a formal risk assessment should be utilised to identify the safest means of personnel transfer between the vessels. If safe access can be established by means of a gangway, a safety net should be rigged. Otherwise, access may be via a pilot ladder rigged from the vessel with the higher freeboard directly to the deck of the vessel with the lower freeboard. Where there is a disparity in deck levels, or the distance between the vessels is too great to maintain safe access between them, it may be necessary to employ a launch to ferry personnel between two safe access points.

Only personnel wearing a persona l flotation device should be permitted to move between the two vessels and their transit should be supervised by a responsible officer. A lifebuoy should be ready for use at the transfer point. Wharf ladders should be used with caution.

    - LIGHTING

During STS transfers at night, normal in-port deck lighting should be adequate. The minimum recommended lighting is five foot-candles (lumens) at transfer connection points and one foot-candle in oil transfer operation work areas (measured one metre above the deck). Portable spotlights, which should be flameproof, and bridge wing spotlights are useful for night mooring and unmooring operations.

    - ANCILLARY EQUIPMENT

All ancillary equipment - wires, messengers, stoppers, strops and shackles etc. should be inspected for condition prior to commencing the STS operation.

    - EQUIPMENT NOISE LEVELS

Excess noise levels in the vicinity of equipment can influence operational communication safety, as well as affecting off duty personnel during rest periods and contributing to fatigue.

On deck and in cargo control areas on most tankers this is not usually an issue. High noise areas are to be identified e.g. adjacent cargo pumps or deck fans are marked accordingly. Personnel responsible for communications must not linger in these areas.

  - EMERGENCY

    - EMERGENCY SIGNAL

The agreed signal to be used in the event of an emergency on either ship should be clearly understood by the personnel on both ships. An emergency on either ship should be indicated immediately by sounding the ship's internal alarm signal and by sounding seven or more short blasts on the whistle to warn the other ship. All personnel should then proceed as indicated by the contingency plan.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - EMERGENCY SITUATIONS

In an emergency, the Masters involved should assess the situation and act accordingly. The following actions should be taken, or considered, in the event of any emergency arising:

| ACTIONS TO BE TAKEN DURING EMERGENCY |  |
| --- | --- |
| 1. | Stop the transfer. |
| 2. | Sound the emergency signal. |
| 3. | Inform both crews on the ships of the nature of the emergency. |
| 4. | Man emergency stations. |
| 5. | Implement emergency procedures. |
| 6. | Drain and disconnect cargo hoses. |
| 7. | Send mooring gangs to stations. |
| 8. | Confirm the ships main engine is ready for immediate use. |
| 9. | Advise standby boat of the situation and any requirements. |

### Table 8 : Emergency Situations

In addition, Masters should decide jointly, particularly in the case of fire, whether it is to their mutual advantage for the ships to remain alongside each other.

The basic actions, as listed above, should be included in individual STS contingency plan and be consistent with the ship's safety management system.

The following table, containing emergency duties assigned to designated members of the crew, may be used as general guidance:

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

### Table 9 : Example of emergency duties allocation of Appendix A of CDI, ICS, OCIMF & SIGTTO “Ship to ship transfer guide for petroleum, chemicals and liquefied gases, first edition 2013” [1]

    - EMERGENCIES DURING MANOEUVRING

The Masters of both ships and the POAC should always be prepared to abort a berthing operation if necessary. The decision should be taken in ample time while the situation is still under control. The Masters of both ships should be immediately informed of each other's actions. The International Regulations for Preventing Collisions at Sea (COLREGS) must be complied with.

    - PROCEDURES IN THE EVENT OF GAS ACCUMULATION ON DECK

The transfer operation should be suspended if excessive cargo vapours are detected around the decks or manifolds of either ship, and should not be resumed until the risk to both ships and their personnel is considered to have been averted.

    - ACCIDENTAL CARGO RELEASE

Any leakage or spillage should be reported immediately to the officers on cargo watch who should stop the cargo transfer and advise the person in overall advisory control. The transfer must remain suspended until it is agreed between the relevant persons/authorities that it is safe to resume.

    - SOPEP or VRP

Risk of oil pollution during STS transfer operations need not be greater than during in-port cargo transfers. However, as a transfer area may be out of range of port services, a contingency plan within the SOPEP or VRP to cover such risk should be available and should be activated in the case of an oil spill. SOPEP, SMPEP and VRP are kept on bridge.

    - STATE OF READINESS FOR AN EMERGENCY

The following arrangements should be made on both ships:

| PREPARED FOR EMERGENCY |  |
| --- | --- |
| 1. | Main engines and steering gear ready for immediate use. |
| 2. | Cargo pump and all other equipment trips relevant to the transfer to be tested prior to the operation. |
| 3. | Crew available and systems prepared to drain and disconnect hoses at short notice. |
| 4. | Oil spill containment equipment prepared and ready for use. |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| 5. | Mooring equipment ready for immediate use and extra mooring lines ready at mooring stations as replacements in case of breakage. |
| --- | --- |
| 6. | Fire fighting equipment ready for immediate use. |

### Table 10 : State of readiness for emergency

  - RISK ASSESSMENT

A risk assessment should be undertaken before committing to an STS transfer operation. It should include sufficient information to ensure a good understanding and effective control of the operation. The risk assessment should cover both the physical and operational hazards and the means by which they are managed, and should also address the suitability of equipment.

Risk assessments are an important part of the pre-STS planning process and, as a minimum, should consider the following:

- Ship compatibility, including mooring arrangements.

- Suitability of the location for the particular operation.

- Properties of the cargo to be transferred.

- Training, experience and qualifications of personnel.

- Suitable preparation of ships for the proposed operations and sufficient control during operations.

- Adequacy of navigational processes.

- Adequate number of personnel assigned to control and perform the transfer operation.

- Adequacy of communications between ships and/or responsible persons.

- Implications of differences in freeboard or the listing of ships when transferring cargo.

- Equipment, including fenders and transfer hoses.

- Anticipated environmental conditions.

- Emergency planning and procedures.

The scope of the Risk Assessment should include confirmation of the following:

| RISK ASSESSMENT SCOPE |  |
| --- | --- |
| 1. | Adequate training, preparation or qualification of oil tanker's personnel. |
| 2. | Suitable preparation of oil tankers for operations and sufficient control over the oil tankers during operations. |
| 3. | Proper understanding of signals or commands. |
| 4. | Adequate number of crew assigned to controlling and performing oil transfer operations. |
| 5. | Suitability of the agreed STS plan. |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| 6. | Adequate communications between oil tankers or responsible person(s). |
| --- | --- |
| 7. | Proper attention given to the differences in freeboard or the listing of the oil tankers when transferring cargo. |
| 8. | The condition of transfer hoses. |
| 9. | Methods of securely connecting hose(s) to the oil tanker(s) manifold(s). |
| 10. | Recognition of the need to discontinue oil transfer when sea and weather conditions deteriorate. |
| 11. | Adequacy of navigational processes. |

### Table 51: Risk Assessment Scope

The level of complexity required will depend on the type of operations. For particular transfer area utilising standard approved STS equipment and ships that are fully operational, a generic risk assessment might be appropriate. For STS operations being undertaken in a new area, or in the event of a deviation from a routine STS transfer, e.g. VLCC to VLCC operation, a risk assessment should be carried out for each ‘non standard’ activity and the Master shall advise the Company. Operations shall not proceed until a full risk assessment has been carried out and agreed by the Master and the Company.

Risk Assessments are to be carried out using ShipSure following procedures in Safety and Environmental manual section 3 – Risk Assessments.

  - CONTINGENCY PLAN

A contingency plan should be put in place covering all possible emergencies, especially during the manoeuvring and cargo transfer phase, and provide for a comprehensive response. In addition, contingency plans should have relevance to the location of the operation and take into account the resources available both at the transfer area and with regard to nearby back-up support. Where appropriate, the contingency plan should be integrated with similar plans prepared by the responsible local authority.

The contingency plan should be agreed between both ships, the STS organiser and the local or national authorities (as appropriate) before STS operations commence.

The lightering/receiving ship will generally be playing the lead role in an STS transfer operation. Accordingly, where organisers have delegated the preparation of a contingency plan, it will normally be incumbent on the Master of such a ship to establish the overall plan that will be reviewed and agreed.

The SOPEP or VRP identifies measures to deal with operational oil spills and also spills resulting form casualties. Please see the appropriate section of the SOPEP/SMPEP or VRP for measures to be implemented during such emergencies.

The V.Ships combined SOPEP and Emergency Contingency Plan (ECP) includes the checklists and procedures relevant to STS operations, including:

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Contingency Checklist G1	Pipeline Leakage and Tank Overflow Contingency Checklist G4	Fire / Explosion

Contingency Checklist G14	Sea Pollution

Contingency Checklist G15	Cargo Spills

Contingency Checklist TNK1	Hose burst, pipe work fracture or Cargo Contingency Checklist TNK2	Break Away During Cargo Transfer Overflow

A blank format is included for additional requirements.

The following Emergency Procedures should be taken into account when preparing the contingency plan:

- Each vessel must have emergency duties assigned to designated members of the crew in case of accidents that may arise during the transfer of oil, particularly in the case of spillages of oil. Refer to the SOPEP or VRP for list of responsible persons and actions regarding oil spills.

- Having discovered a spillage, the operation should be stopped and the immediate measures set forth in the contingency plan should be implemented. The appropriate authorities should be informed of any oil spillage together with size, nature and cause. Each case of oil spillage must be entered in the Oil Record Book.

- Article 8 and Protocol I of MARPOL require that the nearest coastal state should be notified of actual or probable discharges of harmful substances to the sea. The intent of the requirement is to ensure that coastal states are informed without delay of any incident giving rise to pollution or threat of pollution of the marine environment, as well as the need for assistance and salvage measures, so that appropriate action may be taken. The report should be in accordance with the IMO Interim Guidelines for Reporting Incidents Involving Harmful Substances.

  - SAFETY

    - GENERAL SAFETY

For all STS transfer operations each Master remains at all times responsible for the safety of his own ship, its crew, cargo and equipment and should not permit safety to be prejudiced by the actions of others. Each Master should ensure that the procedures recommended by this STS transfer operations plan are followed and, in addition, that internationally accepted safety standards are maintained. In this regard, the most prominent international safety manual in use for cargo handling advice is the latest edition of the International Safety Guide for Oil Tankers and Terminals (ISGOTT).

    - PREVENTION OF FATIGUE

Provisions may need to be made to ensure that the hours of work and hours of rest for the persons involved in STS Operation included the 'Person in Overall Advisory Control' are in accordance with requirements of relevant ILO (Regulation 2.3 Standard A2.3 of the 'Maritime Labour Convention 2006' (or equivalent)), IMO (STCW 95) and national regulations. Records of rest and work hour compliance should be retained.

For STS operations of prolonged duration such compliance may require that the duties and accountability for the role is shared between two persons under an agreed rota system.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - SAFE WATCH KEEPING

The Master should take into consideration the estimated duration of operations to ensure that safe and fatigue-free watch keeping can be maintained throughout.

For reasons of crew workload, transfer operations taking place simultaneously from either side of the STBL are generally not recommended unless fully reviewed by risk assessment.

    - HELICOPTER OPERATIONS

While ships are moored together, helicopter operations should not be permitted without the prior approval of the organisers, both ships' Masters, and responsible person. If approved, the responsible person will co- ordinate the operations locally.

No helicopter operations are to be carried out during transfer of cargo and/or bunkers and/or ballasting into cargo tanks.

    - DRILLS AND TRAINING

The drill schedule for all vessels is included in V.Ships Form SAF 024. Included are drills specific to STS operations such as ‘Emergency Letting Go’. Many standard tanker drills are common to STS operations (See 2.4 and 2.6).

Prior to any STS operation a specific STS training drill should be carried out. Vessels regularly carrying out STS operations are to incorporate into their drill schedule.

    - PERSONAL PROTECTIVE EQUIPMENT AND LIFE SAVING APPLIANCES

The appropriate personal protective equipment (PPE) for the products being handled should be worn by crew members engaged in operational tasks on discharging and receiving vessels. Self-contained breathing apparatus (SCBA) sets should be maintained ready for use in their designated locations. When handling certain hazardous products, appropriate PPE may have to be placed at operational locations, particularly around the manifold area .Transfer should not commence until it is ensured that a ll parties on both vessels are wearing appropriate PPE and, w here required, have the appropriate neutralisers/antidotes readily available.

When undertaking multiple transfers simultaneously, either involving several vessels and/or different products, the STS Superintendent is responsible for ensuring that all vessels are aware of the nature of the products being transferred to other vessels and the actions to be taken in the event of an emergency.

All involved vessels should give due consideration to emergency evacuation arrangements, particularly where the launching of lifeboats and life rafts may be impeded by vessels alongside. Mooring plans should consider the need to launch free-fall lifeboats and, if launching arrangements are compromised, alternative means of emergency evacuation should be identified.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

The STS Superintendent should be aware of any additional PPE and life saving appliances (LSA) requirements that may be imposed by local harbour regulations or by the terminal and these should be conveyed to the Masters of all vessels.

    - ACTION IN CASE OF INFRINGEMENT OF SAFETY

If either ship fails to observe any of the safety requirements during the STS transfer operation, this should be brought to the attention of the Master of the ship concerned and the STS Superintendent. Operations should be suspended until the situation is rectified.

    - ACTION IN CASE OF CARGO LEAKAGE

Cargo transfer should be stopped in the event of a cargo leak on either vessel and should not be resumed until the source of the leak is identified, repaired and/or the cause is isolated. Where appropriate, vessel operators and/or local authorities should be notified.

Cargo operations should only resume, with the agreement of both Masters, once the spilled product has been cleaned up or contained, any vapour associated with the leak has dispersed and safe atmospheric conditions have been verified. Due account should be given to any hazardous properties of the cargo or vapour released and the potential dangers to responding personnel.

  - SECURITY

The requirements of ISPS code with any local requirements should be adhered to throughout the operation, with each vessel responsible for its own security at all times.

Prior to commencing the STS transfer operation, the following security issues should be discussed and any associated decisions recorded.

- Exchange of sufficient information to determine if a Declaration of Security is required.

- Agreement on how and between whom, communications regarding security are made.

- Actions to be taken in the event of breach of security, such as suspending operations and separating vessels.

Local regulations may impose exclusion zones around the STS transfer operation. Due consideration should be given to establishing safe distances to be maintained from other vessels and actions to be taken if such distances are compromised.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## ARRIVAL

  - OPERATIONAL PROCEDURES BEFORE MANOEUVRING

    - PREPARATION OF SHIPS

| PREPARATION BEFORE MANOEUVRES BEGIN |  |
| --- | --- |
| 1. | Review the operation-specific risk assessments and, where applicable, the related JPO and ensure all identified preventive and mitigation measures have been implemented. |
| 2. | Ensure familiarity with the procedures contained in this Guide, supplemented by any instructions issued by ship operators or organisers. |
| 3. | Confirm that ships will be able to comply with the requirements of the relevant check lists. |
| 4. | Train and brief crews on procedures and hazards, with particular reference to mooring and unmooring. |
| 5. | Confirm that steering gear and all navigation and communications equipment is in working order. |
| 6. | Test engine and thruster controls and main propulsion plant ahead and astern. |
| 7. | Test essential cargo and safety equipment. |
| 8. | Confirm that each ship is upright, at a suitable trim with the propeller fully immersed. If one vessel has a list, the possibility of safe stand-off distances being compromised should be considered. |
| 9. | Prepare mooring equipment, including messengers, in accordance with the agreed mooring plan. |
| 10. | Ensure that fenders and transfer hoses are correctly positioned, connected and secured as appropriate and as detailed in the JPO. |
| 11. | For specialised lightering vessels, confirm that fender davits are in the stowed position. |
| 12. | Prepare cargo manifolds and hose handling gear. |
| 13. | Obtain area weather forecasts for the transfer period. |
| 14. | Confirm actions to be taken if the agreed emergency signal is sounded. |
| 15. | Confirm the security level at which the ship is operating in accordance with the provisions of the International Ship and Port Facility Security (ISPS) Code [9], and the requirements being undertaken on board to ensure compliance . |

    - JOINT PLAN OF OPERATION

A joint plan of operation in alignment with the STS plan established for each ship should be developed on the basis of information exchanged between the two oil tankers in an STS operation.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

The Joint Plan of Operation should cover the following and should be exchanged between the two ships:

| INFORMATION TO BE EXCHANGED BETWEEN SHIPS |  |
| --- | --- |
| 1. | Mooring Arrangements. |
| 2. | Quantities and characteristics of the cargo(es) to be loaded (discharged) and identification of any toxic components. |
| 3. | Sequence of loading (discharging) of tanks. |
| 4. | Details of cargo transfer system, number of pumps and maximum permissible pressure. |
| 5. | Rate of oil transfer during operations (initial, maximum and topping-up). |
| 6. | The time required by the discharging oil tanker for starting, stopping and changing rate of delivery during topping-off of tanks. |
| 7. | Normal stopping and emergency shutdown procedures. |
| 8. | Maximum draught and freeboard anticipated during operations. |
| 9. | Disposition and quantity of ballast and slops and disposal if applicable. |
| 10 | Details of proposed method of venting or inerting cargo tanks. |
| 11. | Details of crude oil washing, if applicable. |
| 12. | Emergency and oil spill containment procedures. |
| 13. | Sequence of actions in case of spillage of oil. |
| 14. | Identified critical stages of the operation. |
| 15. | Watch or shift arrangements. |
| 16. | Environmental and operational limits that would trigger suspension of the transfer operation and disconnection and unmooring of the tankers. |
| 17. | Local or government rules that apply to the transfer. |
| 18. | Co-ordination of plans for cargo hose connection, monitoring, draining and disconnection. |
| 19. | Unmooring plan. |

### Table 6: Information to be exchanged between ships

    - NAVIGATIONAL SIGNALS

The lights and shapes to be shown, and the sound signals made are those required by the International Regulations for Preventing Collisions at Sea (COLREGS), and local regulations. These lights and shapes should be checked and rigged ready for display prior to the STS operation.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - LIGHTERING SUPPORT VESSELS

Lightering support vessels are often employed to transport equipment and personnel to the STS location and to assist in preparing vessels for operation. If they remain on station once the ships are alongside, the support vessels may assist in ensuring the security and safety of operation.

In addition to providing operational support for activities such as rising of fenders, hoses and specialized mooring equipment, personnel on the support vessel will have roles that may include the following:

      - Maintenance of radio watch on the agreed marined very high frequency (VHF) channels and/or internal communication frequencies.

      - Communicate and agree with the tankers involved in STS operations on approach manoeuvres alongside.

      - An agreed stand by position, in the vicinity, while the vessels are manoeuvring and while STS operations are underway.

      - Maintenance of a good lookout, both visually and by radar, for approaching vessels.

      - Making contact with vessels likely to encroach upon established safe areas.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## BERTHING

  - MANOEUVRING

    - BASIC BERTHING PRINCIPLES

Conduct berthing and unberthing operations during daylight only. Operations required to be conducted during hours of darkness must:

      - Have a risk assessment issued and approved by the management cell;

      - Be conducted by personnel experienced in night-time STS manoeuvring operations.

For some inshore areas the port authority may require a pilot to be taken. In such circumstances the pilot should advise on all aspects of navigation and piloting, but the Master remains in overall control and in command of his ship.

Where the ship to ship (STS) transfer operation is to be undertaken at sea, passage plans should include reference to relevant navigational charts and publications. The passage plan should detail the projected track lines for the approach phase to the mooring operation, the required sea room, the planned anchor postionm anticipated track lines if drifting, potential navigational hazards and awareness of territorial boundaries (if applicable) and prohibited zones.

    - MANOEUVRING ALONGSIDE WITH TWO SHIPS UNDER POWER

The larger of the two ships should maintain steerage way at slow speed (preferably about 5 knots) keeping a steady course heading. Local conditions and knowledge will dictate the appropriate heading with due regard to transfer area and weather conditions. The manoeuvring ship then manoeuvres alongside.

It is recommended that the manoeuvring ship approaches and berths with the port side to the starboard side of the constant heading ship.

Be aware that some local jurisdictions may have regulations specifying some aspects of manoeuvring between the ships.

    - GENERAL ADVICE FOR CONTROLLING THE TWO SHIPS

Each ship should take the following into account:

      - Courses requested by the manoeuvring ship must be followed by the constant heading ship.

      - Ship’s speed should be controlled by adjusting engine revolutions (or propeller pitch). Any adjustment should be limited; for example, to plus or minus 5 rpm rather than using the relatively coarse engine room telegraph system. However normal full operating range must remain readily available.

      - For diesel engines, ascertain number of air starts available.

      - At night the deck should be adequately lit and, if possible, the ship's side and fenders should be lit by spot lights

      - The side for mooring should be clear of all over side obstructions; permanent and otherwise.

      - The navigation lights and shapes appropriate to STS transfers should be displayed.

      - There should be effective radio communications between the bridge and mooring personnel.

      - There should be effective communications between the Masters of each ship.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

POSSIBLE FINAL APPROACH MANOEUVRE

Vessel A: Constant Heading Ship – Constant Speed (About 5 knots) Vessel B: Manoeuvring Ship

### Figure 3: A possible final approach manoeuvre

    - ADVICE FOR MANOEUVRING ALONGSIDE

If either of the Masters of the ships or the STS Superintendent has the slightest doubt about the safety of the manoeuvre, the berthing operation should be aborted.

At all times each ship is responsible for maintaining a proper lookout.

Generally, during manoeuvring, the wind and sea are kept on the port bow of the usually larger constant heading ship; however local conditions or knowledge may indicate an alternative approach.

The angle of approach adopted by the manoeuvring ship should not be excessive.

A common method of berthing is for the manoeuvring ship to approach the constant heading ship from the quarter on the side of berthing. On closer approach the manoeuvring ship should parallel the course of the constant heading ship at a safe distance that is appropriate to the conditions, before then positioning itself relative to the constant heading ship. Contact is made by manoeuvring ship reducing the distance by appropriate rudder and engine movements until the fenders touch.

The two ships should preferably make parallel contact at the same speed with no astern engine movements being necessary.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

No engine movement on the constant heading ship should be made without advising the STS Superintendent or Master of the manoeuvring ship.

The effects of ship interaction should be anticipated when manoeuvring at close quarters.

### Fig.2 The ships should make parallel contact at the same speed

    - MANOEUVRING A COMBINED TWO-SHIP SYSTEM TO ANCHOR

On completion of mooring, the constant heading ship will usually power all future manoeuvres and, if a transfer at anchor is planned, will proceed to the agreed anchoring position. During this time, the (former) manoeuvring ship will have its engines stopped and rudder amidships. It should be emphasised that, for this period, in order to avoid problems for the manoeuvring ship the constant heading ship should not use strong astern engine movements. Speeds through the water should be minimal.

The constant heading ship should use the anchor on the side opposite to that on which the other ship is moored. Once at anchor, each ship is responsible for watch keeping arrangements.

    - UNDERWAY TRANSFER

As long as adequate sea-room is available and traffic conditions, weather, sea conditions and forecasts are suitable, then transfers of this type can be carried out, but it should be noted that speeds through the water should be minimal.

The constant heading ship maintains steerage way at slow speed on a steady course and the (former) manoeuvring ship keeps its rudder amidships and remains (with engines stopped) as a towed ship. In order to minimise towing loads on the moorings, the constant heading ship should alter her engine revolutions

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

sparingly, adjusting speed very gradually. The chosen course and speed should be agreed by the two Masters and the STS Superintendent and should result in minimum relative movement between the two ships and minimum turbulence in the gap between the hulls.

Under such circumstances, while the ships are moored together as a unit, safe navigation and collision avoidance is usually the responsibility of the constant heading ship but may be under the direction of the person in overall advisory control aboard the lightering ship. The two ships can drift freely provided conditions are suitable and a transfer area of suitable size is available.

## The use of the underway transfer system requires a full navigational watch to be kept on the bridge of each ship.

    - MANOEUVRES WITH ONE SHIP AT ANCHOR

One ship anchors in a pre-determined position using the anchor on the side opposite to that on which the other ship will moor. The berthing operation should only commence after the anchoring ship is brought up to her anchor and is lying on a steady heading with reference to prevailing current and wind conditions.

The Master of the ship which is to anchor should allow for the fact that the single anchor will be required to hold both ships. When anchoring in deep water, and using an extended scope of cable, the Master of the ship that is to anchor should also ensure that the windlass is capable of recovering the cable and anchor once the operation is completed.

The type of berthing operation then undertaken by the manoeuvring ship is similar to a normal approach to a jetty. A risk assessment should be undertaken by the organisers to evaluate the necessity of tug assistance for the manoeuvring ship.

A careful watch should be kept on the heading of the anchored ship and the anchored ship should advise the manoeuvring ship immediately if she has any tendency to yaw. Where there is a tendency to yaw excessively, a tug should be employed to hold the anchored ship on a steady heading. If no tug is available, postponement of the operation should be considered.

This manoeuvre can be preferred for more constrained transfer areas, especially when tug assistance is available, or if the manoeuvring ship is fitted with a bow thruster. Where current and wind are not from the same direction or the wind varies in speed or direction the anchored ship can yaw (or lie cross-current), making it difficult for the manoeuvring ship to berth alongside. Also both ships could experience different effects due to their different freeboards and draughts. In these circumstances tug assistance may be advisable to hold the anchored ship on a steady heading during berthing.

It is recommended that the services of an experienced STS Superintendent be utilised for this type of operation. However, berthing should not be attempted when the tidal stream is due to change.

When approaching a ship at anchor some Masters recommend a wider angle of approach than that adopted for manoeuvres underway. A wider angle of approach, especially when tugs are not available, helps to avoid early ship-to-ship contact in cases where the anchored ship might yaw unexpectedly. It is recommended that the manoeuvring ship approaches and berths with her port side to the starboard side of the other ship. When

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

mooring to an anchored ship, care should be taken not to pull the anchored ship quickly towards the manoeuvring ship.

    - MANOEUVRING FOR IN PORT OPERATIONS

STS operations, may be undertaken with in port limits. These operations may involve berthing alongside the discharging ship, which may be at anchor in sheltered waters or alongside a terminal. Permission of the harbour authorities and, where required, the terminal should be obtained prior to commencing manoeuvres to berth alongside another vessel. The STS Superintendent should be aware of the estimated time of arrival (ETA) of the vessels and should be notified when manoeuvres are about to commence.

The manoeuvring vessel should advise the Master of the moored ship of the intended approach and the approach should not be commenced until the procedure has been jointly agreed.

Within port limits, it will in most cases be a requirement to take a berthing pilot on board and, depending on the size of the vessel regulations may require that tugs are used.

    - MANOEUVRING WITH ONE SHIP ALONGSIDE A TERMINAL

This operation, also known as double banking, involves one or more vessels mooring alongside a ship that is a l ready moored to a jetty and is a relatively common practice in certain trades.

For berths that have not traditionally been used for double banking operations, it is recommended that a forma l engineering study and risk assessment is undertaken and a forma l procedure and safety plan is produced prior to conducting such operations.

Before double banking operations are agreed, consideration and agreement should be reached by all parties concerned on issues that may include the following:

      - Safe arrival and departure procedures.

      - Integrity of the berth, including fender panels and mooring fittings, with regard to the	potential loads involved.

      - Personnel access, including emergency escape provisions for personnel on all involved ships.

      - Management of operational safety

      - Roles and responsibilities of involved parties.

      - Contingency planning, fire-fighting and emergency unberthing.

The Master of the ship moored to the terminal should be aware of the total displacement of his ship and vessels berthing alongside. The mooring arrangements should be adequate for the anticipated loads.

Consideration should be given to the availability and need for line handling support. The ship alongside the terminal may provide crew to receive and secure the line but this should not detract from the safety of any ongoing cargo operations. Local port regulations may require that licensed line handlers are used and these should be arranged prior to commencement of manoeuvring.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

  - MOORING

    - MOORING PREPARATIONS

Moorings should be arranged and rigged to allow safe, effective line tending when the ships are secured together. This is very important on board the manoeuvring ship whose mooring lines will normally be used, but this should also be addressed on the constant heading ship where rope messengers have to be made ready between fairleads and deck winches.

When preparing the mooring plan the following factors should be taken into account:

| MOORING PLAN PREPARATION |  |
| --- | --- |
| 1. | Size of each ship and the difference in their sizes. |
| 2. | The expected difference in freeboards and displacements. |
| 3. | The anticipated weather and sea conditions. |
| 4. | The degree of shelter offered by the location. |
| 5. | The efficiency of the mooring line leads available. |

### Table 7: Mooring plan preparation

Most STS Service providers have a standard mooring plan, suitable for the particular location. It is important to ensure moorings allow for ship movement and freeboard changes to avoid over stressing the lines throughout the operation, but that they are not so long that they allow unacceptable movement between the ships. Mooring lines leading in the same direction should be of similar material.

The order of passing mooring lines during mooring, and of releasing lines during unmooring, should be agreed. Where the STS Service providers utilise quick-release mooring hooks, their role and use should be discussed to ensure proper understanding.

The picture illustrates a typical and proven mooring arrangement for an STS transfer operation in offshore waters. Scope for additional head and stern lines is preferred, and at any time spare lines should be readily available to supplement moorings if necessary or in the event of a line failure.

The mooring lines are normally deployed from the manoeuvring ship. However, when prevailing weather conditions or weather forecasts require it, sending lines from both ships can increase the number of mooring lines. Loads should not be concentrated by passing most of the mooring ropes through the same fairlead or onto the same mooring bitts. Use should be made of all available fairleads and bitts.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

### Fig.3 Typical STS mooring arrangement

Fig.4 Example of a ship to ship mooring plan, including contingency unmooring arrangements

During the operation, freeboard differences should be kept to a minimum, with consideration given to ballasting the higher ship and de-ballasting the lower one where this is possible. The steeper the orientation of the mooring lines, the less effective they will be in resisting horizontal loads.

The maximum anticipated freeboard difference should therefore be taken into account when planning the mooring plan in order to ensure that the vertical angle of each mooring line stays as small as practicable throughout the operation. See below figure 5 for maximum and minimum freeboard heights.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

### Figure 5:Maximum and minimum freeboard heights (Ships specific data to be included)

A ship's standard complement of mooring lines is generally suitable for STS transfer operations but ships equipped with steel wire or high modulus synthetic fibre mooring lines should fit soft rope tails to them. The connection between the primary line and the soft rope tail should be made with an approved fitting e.g. Mandel or Tonsberg Shackle.

Rope tails should be at least 11 metres long and have a dry breaking strength at least 25% greater than that of the wires to which they are attached in accordance with OCIMF Mooring Equipment Guidelines . Soft rope tails fitted to wire moorings also introduce the benefit of making the cutting of mooring lines easier in an emergency and, for this purpose, long-handled firemen's axes or other suitable cutting equipment should be available at all mooring stations.

Strong rope messengers should be readied on both ships and in addition rope stoppers should be rigged in way of relevant mooring bitts. Where possible, heaving lines and rope messengers should be made of buoyant materials. A minimum of four messengers should be provided and ready for immediate use.

Non-pyrotechnic line-throwing equipment may be used to make the first connection.

    - TENDING MOORING LINES

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Regular rounds are to be carried out and moorings tended as required. Moorings are to be tightened or slackened in such a way that ensures the ship does not move position. Mooring equipment must be operated by competent persons and care must be taken to ensure that excessive loads do not arise on moorings.

    - TENSION IN MOORING LINES

Excessive or uneven tension in mooring lines should be avoided because it can significantly reduce the weather threshold at which the forces in mooring lines will exceed their SWL. Attention should be given to this throughout the STS operation in order to ensure changes to the relative freeboards do not create excessive strain in the moorings.

    - MOORING LINE LEAD ANGLES

Peak loads on individual head and stern mooring lines can be minimised if the lead angles are similar and thus more effectively share the mooring loads.

    - WEATHER THRESHOLDS

Higher weather thresholds for mooring loads can be tolerated when the STBL is at or close to fully loaded displacement. Masters and persons in overall advisory control should be aware that weather thresholds may change significantly during the course of an STS operation as the STBL is lightened.

    - LONG PERIOD WAVES

STS operations in locations subject to long period waves should be undertaken with caution. The load on mooring lines at any specific significant wave height greatly increases as the wave period, or period of encounter, increases.

    - DIRECTION OF WAVE ENCOUNTER

Wave encounter from a beam direction during STS operations should be avoided. This will need particular attention when using exposed anchorage STS locations subject to strong currents, where the ships can lie at a large angle to the wind and waves.

When conducting underway STS operations, the optimum wave encounter direction to control mooring loads is normally considered to be on the port bow, with the STBL to windward. But, depending on the relative size and displacements of the two ships, this may not always be the case. For instance, when both ships are of similar size and as the lightering ship's displacement increases relative to the STBL, the optimum mooring load wave encounter direction may change to the starboard bow, with the lightering ship to windward. In such a case, it may be advantageous to have sea-room available for course alterations across the wind.

    - ELASTICITY OF MOORING ARRANGEMENT

Nylon tails of 11 meters in length are recommended to improve the elasticity of mooring arrangement. Under certain circumstances in exposed locations, longer tail lengths may improve the

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

overall integrity of the mooring system.

    - CHAFING PROTECTION

Synthetic moorings passed through the shipside fairleads may be subjected to chafing from cyclical loading due to the vessel’s motion. Lines can be protected with suitable chafing covers. The covers may be lubricated to minimise the potential for them being damaged.

    - IN PORT MOORING OPERATIONS

The mooring configuration should be discussed and agreed in advance between the STS vessels expected alongside so that crews have adequate notice to prepare the moorings and to ensure that there is no confusion as to when the mooring ropes will be sent out and from which location.

All lines are led through closed fairleads to avoid the possibility of lines jumping out of the lead.

All mooring lines should be passed through fairleads and secured to bitts, bollards or cleats with an adequate safe working load (SWL).

Off shore anchors may be used in controlling the approach to a ship that is moored alongside. A sufficient length of cable should be used to provide necessary drag force.

Bow and stern thrusters, when fitted, should be used to full advantage,. Consideration should be given to the effects that the use of thrusters may have on the other vessel alongside.

The first line sent to the mother ship will be usually spring lines, followed by heat, breast and stern lines, as per mooring arrangement. Should the spring lines be used to assist in bringing the vessel alongside, due consideration must be given to the impact on the other vessel ad the added force being placed on her moorings.

Every effort should be made to prevent chafing of the mooring lines at chocks or fairleads of both vessels. The moorings should be checked and tended regularly to ensure proper tension is maintained throughout the transfer operation.

Mooring plans should anticipate and allow for the effects of port traffic passing in close proximity to the transfer location.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## CARGO TRANSFER

  - PRE-TRANSFER PROCEDURES

When the two ships are securely moored and before cargo transfer commences, good communications should be established between the personnel responsible for cargo operations on each ship and the pre-transfer checks should be satisfactorily completed.

Before commencing the transfer operation the responsible person(s) on the ships should ensure:

| PRE-TRANSFER REQUIREMENTS |  |
| --- | --- |
| 1. | Proper mooring of the ships. |
| 2. | Availability of reliable communication between the two oil tankers. |
| 3. | Emergency signals and shutdown signals are agreed. |
| 4. | Proper connection and securing of hoses to the oil tanker's manifolds. |
| 5. | Proper condition and position of hoses, hose saddles and supports. |
| 6. | Flanged joints, where used, are fully bolted and sealed and ensured oil tight. |
| 7. | Proper blanking of unused cargo and bunker connections. |
| 8. | Tools required for the rapid disconnection of hoses are located at the Manifold. |
| 9. | Any valve through which oil could be discharged to the sea is closed and inspected and, if not used in the operation, is sealed to ensure that it is not inadvertently opened. |
| 10. | Deck scuppers are properly plugged. |
| 11. | Availability of empty drip trays on both oil tankers under couplings of hoses, and means for drip tray drainage. |
| 12. | Availability of materials on the oil tankers for on-deck clean-up in case of Spillage. |
| 13. | Fire axes or suitable cutting equipment is in position at fore and aft mooring Stations. |
| 14. | An engine-room watch will be maintained throughout the transfer and the main engine will be ready for immediate use. |
| 15. | A bridge watch and/or an anchor watch will be established. |
| 16. | Officers in charge of the cargo transfer are identified and details are posted. |
| 17. | A deck watch is established to pay particular attention to moorings, fenders, hoses and manifold integrity. |
| 18. | Correct understanding of commands and signals by the responsible person(s) on the oil tankers during operations. |
| 19. | Confirm completion of STS checklists 1 & 4. |

### Table 8: Pre-Transfer Requirements

    - RESPONSIBILITY FOR CARGO OPERATIONS

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Cargo transfer operations should be carried out in accordance with the requirements of the receiving ship. The person in charge of the cargo operations for each ship should be positively identified on a list posted on the bridge and cargo control room of both ships, together with the names of other persons supervising the cargo transfer.

The transfer operation may be started only after the responsible person(s) on both oil tankers and the POAC have agreed to do so, either verbally or in writing.

    - PLANNING FOR CARGO TRANSFER

When preparing cargo loading and discharging plans, due regard should be given to ensuring that adequate stability is maintained, hull stresses remain within design limits and free surface effects are kept to a minimum throughout.

The cargo transfer operation should be planned and agreed in writing between the two ships and, where applicable, should include information on the following:

| CARGO TRANSFER OPERATION INFORMATION |  |
| --- | --- |
| 1. | Quantity of each grade of cargo to be transferred. |
| 2. | Sequence of grades, cargo density, temperature and specific precautions such as those that might be necessary for static accumulator products. |
| 3. | Details of cargo transfer system, number of pumps, maximum pressure. |
| 4. | Crude oil washing procedures. |
| 5. | Initial and maximum topping off rates and notice period of rate change. |
| 6. | Normal stopping and emergency shutdown procedures. |
| 7. | Emergency and spill containment procedures. |
| 8. | Watch or shift arrangements. |
| 9. | Critical stages of the operation. |
| 10 | Local or government rules that apply to the transfer. |
| 11. | Material Safety Data Sheet (MSDS) in respect of cargo to be transferred to ensure that the receiving ship is aware of particular properties of the cargo e.g. high Hydrogen Sulphide (H 2S) content, special fire-fighting requirements etc. |
| 12. | Receiving ship to provide details of previous cargo. |
| 13. | Co-ordination of plans for cargo hose connection, monitoring, draining and disconnection. |

### Table 9: Cargo transfer operation information

Before starting cargo transfer, the discharging ship must be informed by the receiving ship of the flow rates required for the different phases of the cargo operation. If variations in transfer rate become necessary, the receiving ship should advise the discharging ship of its requirements. The discharging ship should similarly inform the receiving ship of any variations in flow rates due to its operations.

The agreed transfer rate should not exceed the manufacturer's recommended flow rates for the cargo hoses.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

  - CARGO TRANSFER

    - GENERAL

Throughout cargo transfer operations, the discharging ship and the receiving ship should station a responsible person at the cargo manifold area to observe the hoses and to check for leaks. In addition, throughout the cargo transfer, a responsible person equipped with a portable radio should be stationed at or near the cargo pump controls or in the cargo control room on the discharging ship, to take action as required.

Cargo transfer should begin at the agreed slow rate to enable the receiving ship to check that the cargo pipeline system is correctly set.

Only after being satisfied there is no leakage, that the oil is being transferred into the intended pipelines and tanks and that there is no excessive pressure, may the rate of transfer be increased up to the maximum indicated in the plan of operation.

The transfer rate should also be reduced to an agreed topping off rate when the receiving ship's tanks are reaching their filling limits. Throughout the transfer, as a minimum hourly transfer rate checks and comparisons should be made between the two ships, and the results logged. Any differences or anomalies should be carefully checked and if necessary cargo operations should be suspended until the differences are resolved.

It should be ensured that adequate ullage space is left in each tank being filled. When it is required to stop cargo transfer operations, the responsible person should advise the pumping oil tanker in ample time.

Cargo operations should be conducted under closed conditions i.e. with ullage, sounding/sampling ports securely closed. In some parts of the world vapour balancing procedures are enforced and due regard should therefore be given to local regulations.

Consideration should be given to the prevention of pressure surges by careful planning and control of pump speeds and the operation of valves.

Static accumulator cargoes will require extra precautions and ISGOTT should be referred to when handling this type of cargo.

    - BALLAST OPERATIONS

During cargo transfer, appropriate ballast operations should be performed in order to minimise the differences in freeboard between the two ships, and to avoid excessive trim by the stern. Listing of either ship should be avoided, except as required for cargo tank draining on the discharging ship.

Any national or local regulations controlling discharge of ships' ballast water should be complied with. Constant attention should be paid to mooring lines and fenders to avoid chafing and undue stress, particularly that caused by changes in relative freeboard. If at any time mooring lines need to be re-positioned or adjusted this should only be done under strictly controlled conditions.

Most ships engaged in STS operations are fitted with segregated ballast tanks. However, cases may arise where ships are employed which may require the transfer of ballast to the discharging ship. During deballasting from cargo tanks the inert gas system, where fitted should be operated.

    - SUSPENSION OF CARGO TRANSFER

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Both ships should be prepared to immediately discontinue the lightering operation, and to unmoor if necessary. The operation should be suspended when:

| SUSPENSION OF CARGO TRANSFER |  |
| --- | --- |
| 1. | Movement of the ships alongside reaches the maximum permissible and may cause loss of strength or excessive strain on hoses. |
| 2. | Under adverse weather and/or sea conditions. |
| 3. | Either vessel experiences a power failure. |
| 4. | There is a failure of the main communication system between the ships and there are no proper standby communications. |
| 5. | Escape of oil through sea valve or ship plating is discovered. |
| 6. | There is an unexplained pressure drop in the cargo system. |
| 7. | Fire danger is discovered. |
| 8. | Any oil leakage is discovered from hoses, couplings, or the ship’s deck piping. |
| 9. | Any oil overflow on the ship(s)’ deck caused by over-filling of tank occurs. |
| 10 | Faults or damage threatening the escape of oil are discovered. |
| 11. | There is a significant, unexplained difference between the quantities of cargo delivered and received. |
| 12. | There is any failure of the fendering system. |
| 13. | When hours of work are exceeded on either ship. |

### Table 10: Suspension of Cargo Transfer

  - VAPOUR BALANCING CONSIDERATIONS

Cargo operations should be conducted under closed conditions with ullage, sounding and sampling ports securely closed. Due regard must be given to any local regulations that may require the adoption of vapour balancing procedures.

The operation is to be conducted in fully inert conditions in accordance to ISGOTT and Volatile oranic compound (VOC) management plan.

To ensure cargo vapour emissions to the environment are reduced as much as possible during the vapour balancing operation, the following measures to minimise carg vapour evolution during the operation should be considered:

- Careful operation of cargo handling equipment to minimise cavitation of transfer pumps.

- Minimising use of cargo edictors for stripping and draining.

- Wherever applicable, keeping COW operations to a minimum.

    - VAPOUR BALANCING CONSIDERATIONS BEFORE COMMENCING CARGO TRANSFER:

Equipment should be provided on at least one of the vessels to enable the oxygen content of the vapour stream to be monitored. This should draw samples continuously from a location close to the vapour manifold

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

connection and should include the facility for audible and visual alarms in the event that the oxygen content of the vapour stream exceeds 8% by volume. The oxygen analyser and associated alarms should be tested for proper function prior to each cargo transfer operation.

The oxygen content of the vapour space of each tank connected to the IG main in both ships should be checked and confirmed to be less than 8% by volume.

The vapour transfer hose should be purged of air and inerted prior to commencing transfer of vapours.

The vapour manifold valves should not be opened until the pressure in the cargo system of the receiving vessel exceeds that of the ship discharging cargo.

    - VAPOUR BALANCING CONSIDERATIONS DURING CARGO TRANSFER:

The inert gas system on the discharging ship should be kept operational and on standby, with the inert gas main deck isolating valve closed. The inert gas system should be used if the inert gas pressure in the discharging vessel falls to a low level (300 mm WG).

The inert gas pressure on both ships should be monitored and each ship advised of the other’s pressure on a regular basis.

No air should be allowed to enter the cargo tanks of the discharging ship

Transfer operations should be suspended if the oxygen content of the vapour stream exceeds 8 % by volume and should only be resumed once the oxygen content has been reduced to 8% or less by volume.

The cargo transfer rate must not exceed the design rate for the vapour balancing hose.

    - VAPOUR HOSE CONSIDERATIONS

When connected, care must be taken to ensure that the vapour hose is not kinked anywhere along its length as this may cause significant flow restriction.

Care should be taken to avoid liquid build-up in the vapour hose due to liquid carry over from one of the ship’s vapour mains or from condensation.

  - SAFETY DURING CARGO TRANSFER

The basic safety requirements for a transfer operation are similar to those for a normal port cargo operation as contained in the latest edition of ISGOTT. The following points are emphasised for an STS transfer operation:

    - SMOKING AND NAKED LIGHTS

Regulations regarding smoking and the use of naked lights should be strictly enforced. Warning notices should be displayed and smoking rooms should be designated and clearly marked.

    - EARTHS ON ELECTRICAL SWITCHBOARDS

Earth indicator lights showing on the main switchboard indicate a faulty circuit and such faults should be immediately traced and isolated. This is to avoid the risk of arcing, especially in deck areas where hazardous accumulations of gas may be present.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - MACHINERY OPERATION

Issues that should be considered with regard to the management of main engines and machinery systems include:

      - Sufficient reserve power should be available on the switchboard to overcome sudden loss of generating equipment or increased demand. Standby generators and the emergency generator should be tested and confirmed to be ready prior to STS operations.

      - The main propulsion unit should either be in standby mode or on short notice of readiness. This ensures that lubrication, fuel, jacket water and other services to the propulsion unit will be maintained. The state of readiness of the main propulsion machinery of both vessels should be agreed between the Masters and the STS Superintendent.

      - Steering gear system operational tests should be carried out prior to commencing the STS operation.

      - Any changeover of fuel, for example to low sulphur fuel oil, should be undertaken in good time prior to the start of STS operations. Any automated changeover systems should be stabilised before operations commence.

      - On vessels fitted with controllable pitch propellers the main propulsion unit could be operating on low load at zero pitch for prolonged periods. Manufacturer's recommendations should be followed when preparing the plant for such operation.

      - On ships fitted with boilers, operations such as soot blowing should be carried out prior to commencing the approach manoeuvre. Incinerators should not be used during transfer operations.

      - Exhaust uptakes should be regularly monitored. In the event of sparks being observed, transfer operations should be stopped immediately.

    - SHIP-TO-SHIP ELECTRIC CURRENTS

The Elimination of Electrical Current and Electrostatic Charge in Cargo Hoses

In order to eliminate the potential for incentive arcing between the two ships when presenting the hose string for connection:

      - a single insulating flange should be fitted within each hose string (or at one ship's manifold); or

      - one length of electrically discontinuous hose should be fitted in each hose string; or

      - hoses that are specially constructed to prevent static build-up or electrical currents transferring between ships should be used.

The electrical potential between the two ships should be reduced to the minimum. Switching off cathodic protection systems of the impressed current type is not, in general, considered to be a feasible method of minimising ship-to-ship currents in the absence of an insulating flange or hose. If both ships have properly functioning impressed current cathodic protection systems, this is probably best achieved by leaving them running. Likewise, if one has an impressed system and the other a sacrificial system, the former should remain in operation. However if either ship is without cathodic protection, or its impressed system has broken down, consideration should be given to switching off the impressed system on the other ship well before the two ships come together.

### Other Places Where Electrical Arcing May Occur

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

All ship-to-ship mooring lines should be insulated either by using the natural properties of soft mooring lines or by attaching a soft rope tail to the eye of each steel wire mooring line. If using soft rope tails, they should be of suitable length so that they extend to the outboard side of the ship receiving the mooring. Care should be taken to avoid low resistance ship-to-ship electrical contact in the following areas:

      - non-insulated metallic ladders or gangways between the ships - by the fitting of rubber ends.

      - derrick or crane wire runners and hooks - by careful operation.

      - unprotected bare wires and chains within fender support nets or cages - by good quality maintenance.

    - THE USE OF RADIO AND SATELLITE COMMUNICATIONS EQUIPMENT

### Main Radio Equipment

Transmissions from the ship's main radio station can cause electrical resonance in insulated parts of some ship fittings, such as mast stays, and this can cause arcing across deck fittings. Similarly, arcing can occur on a ship's wireless aerials, especially over the surface of insulators when they have a coating of salt, dirt or water.

The use of the ship's main radio equipment during cargo operations can be dangerous. Radio transmissions should not be permitted during periods when there is the possibility of flammable gas in the region of the antennae or where there is doubt about the effective earthing of stays, derrick equipment and other such fittings.

The main radio transmitting aerials on both ships should be earthed (grounded) and neither ship should use this equipment while alongside one another. Satellite communications equipment can be used for communications; however, the risks described below should be taken into account.

### Satellite Communications

Satellite communications equipment normally operates at 1.6 GHz and the power levels generated are considered to present few ignition hazards. However, this equipment should not be used when flammable gas is in the vicinity of antennae.

### VHF and UHF Radios

Any hand-held VHF and UHF radios, as used for mooring and cargo operations, should be of intrinsically safe manufacture.

### Automatic Identification Systems (AIS)

Where either or both ships involved in STS operations are required to have an AIS operating while underway or at anchor, the AIS equipment should remain in use at all times, including during STS operations.

The VHF equipment used for the AIS broadcasts need not be set to low power output during STS operations. However, during STS operations consideration should be given to using the optimal text entry area in the AIS message to include a phrase to indicate that the ship is restricted in her ability to manoeuvre, underway or at anchor, as a result of conducting STS transfer operations. It may be necessary to abbreviate the optional message to include this information. AIS broadcasts should not supplant the recommendation to broadcast navigational warnings by other means.

### Portable Electronic Devices

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

It should be noted that portable cellular (mobile) telephones, pagers, cameras using batteries, Portable Data Assistants (PDAs), calculators etc, could constitute a risk to the ships if used in a hazardous area. Precautions should be taken to ensure that all personnel involved in the transfer, especially those who may be visiting the ships on other business (technicians, surveyors etc) are made fully aware of the dangers and any restrictions on the use of such items.

    - RADAR USE

### General

The use of radar involves the operation of electrical equipment which is not intrinsically safe. Depending on the relative size of the two ships, during cargo transfer operations the radar beam of one ship may at times sweep the cargo deck of the other and be close enough to create potentially hazardous power densities in areas where flammable gas mixtures could be present. Consultation between Masters is advisable before radar is used during cargo transfer operations. The following section gives further advice.

### The Use of 3cm and 10cm Radar

Radiation from radar operating at frequencies above 9000 MHz (3cm) may be considered safe at distances of over 10 metres. The radiated power from such radar should not present an ignition hazard provided scanners are correctly sited above the superstructure. Radar operating in the 3cm waveband will normally be safe but should only be used with discretion.

At the lower frequencies, as used by 10cm radar, the possibility of induced arcing in parts of a ship's structure is present at ranges of up to 10 metres.

Marine radars normally operate with a pulsed signal and a rotating scanner, so people are not continuously exposed to radiation. Therefore the power-scanner interlocks should never be overridden without an appropriate risk assessment.

    - GAS ACCUMULATION

The STS transfer operation should be suspended if cargo vapour accumulation around the decks or manifolds of either ship constitutes a risk to the ship or personnel, and should not be resumed until it is considered safe to do so.

The receiving ship should, prior to cargo transfer, provide details of the ship's previous cargo to the discharging ship (STBL). This will enable the discharging ship's personnel to take suitable precautions in the event that the previous cargo contained toxic vapours that could be displaced onto the deck of the discharging ship. Particular attention should be given to the potential of high H2S levels in the cargo vapours and all necessary personal safety precautions should be taken.

    - ELECTRICAL STORMS

When an electrical storm is present or imminent in the transfer area, the cargo transfer operation should be suspended and all vent risers, cargo systems and IGS systems secured until such time as it is considered safe to resume operations.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - GALLEY STOVES

Before permitting the use of galley stoves and other cooking appliances while a ship is engaged in STS operations, the ship's Master and the STS Superintendent (if applicable) must, after taking into consideration the location, construction and ventilation of the galley, jointly agree that no associated danger exists. Oil or gas fired stoves or electrical appliances using exposed elements should not be used.

    - READINESS FOR FIRE-FIGHTING EQUIPMENT

Fire-fighting equipment should be ready for immediate use on both ships. Foam monitors on each ship should be pointed towards the cargo manifold in use and left in a suitable condition for hands-off operation. Additional foam fire-fighting equipment should be immediately available for use on deck.

    - ACCOMMODATION OPENINGS

All access doors to the accommodation should be kept closed during cargo transfer operations. The Master of each ship should designate those access doors that are to be used for personnel transit. Where possible, only doors remote from the main deck cargo area should be used. All doors opened for personnel transit should be closed immediately after use. The air conditioning intakes must be set to ensure the atmospheric pressure inside the accommodation is always greater than that of the external atmosphere. Air conditioning systems should not be set to 100% recirculation, as this will cause the pressure of the internal atmosphere to fall less than that of the external atmosphere, due to extraction fans operating in sanitary spaces and galleys.

    - UNAUTHORISED CRAFT

No unauthorised craft should be allowed alongside either ship throughout the transfer.

  - OPERATIONS AFTER COMPLETION OF CARGO TRANSFER

It should be ensured that adequate ullage space is left in each tank being filled. When it is required to stop cargo transfer operations, the responsible person should advise the pumping oil tanker in ample time.

Upon completion of the oil transfer, the oil tanker with the greatest freeboard should close the valve at the manifold and drain the oil contained in the hoses into the tank of the other oil tanker. Any remaining oil in the hoses should be drained, after which the hoses should be disconnected and securely blanked. The cargo manifolds should also be securely blanked.

In accordance with previously agreed procedures, after completion of cargo transfer the following operations should be carried out:

| OPERATIONS AFTER CARGO TRANSFER |  |
| --- | --- |
| 1. | All hoses should be drained into one ship prior to disconnecting. |
| 2. | Hoses should be disconnected and securely blanked. |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

| 3. | Cargo manifolds should be shut and securely blanked. |
| --- | --- |
| 4. | Any spillage from hose disconnection in manifold drip trays / save-alls drained. Drip tray cleaned. |
| 5.. | Authorities should be informed of completion of cargo transfer and the anticipated time of unmooring. |

### Table 11: Operations after completion of cargo transfer

When the loading or discharging of cargo is completed, it must be recorded in the Oil Record Book. Following completion of hose disconnection and prior to unmooring, the POAC and masters on both the vessels should liaise to prepare a coordinated unmooring plan taking into consideration prevailing sea and weather conditions. Ship's masters should be mindful of the fact that historically a significant proportion of

STS incidents have occurred during unmooring operations and should ensure that all critical manoeuvring equipment has been tested and proven as being fully operational prior to commencement of such operations. Once the unmooring plan has been agreed by all elements of Checklist No 5 in the ICS/OCIMF STS Transfer Guide - Petroleum (Appendix E) should be verified and completed by both vessels.

As soon as practicable after the transfer operation has been completed, and before unmooring, the responsible person on each oil tanker should ensure that all valves in their system are closed and cargo tank openings are closed and secured for sea.

The oil transfer documents should be completed, communications checked and the readiness of both oil tankers established, whereupon the ships should unmoor in accordance with the plan

  - BUNKERING AND STORING

It is not recommended that bunkering takes place at the same time as STS cargo operations. A separate risk assessment should be produced if concurrent bunkering is required.

Bunkering and storing activities should only be undertaken at the same time as the cargo transfer if there are sufficient personnel available to effectively monitor both operations.

When transfer operations take place ar anchor, storing may be undertaken from a barge or work boat. It should be noted that personnel on the storing vessel are unlikely to be aware of the hazards associated with products being transferred and may require specific briefing on safety and emergency procedures, including use of personal protective equipment (PPE).

With the discharging ship moored alongside a terminal, any storing would have to be across the discharging ship. Usually this will involve use of the discharging ship’s lifting equipment and should require consent of the STS Superintendent.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## DEPARTURE

  - UNMOORING

    - UNMOORING PROCEDURE WHILE ONE SHIP IS AT ANCHOR

It is recommended that unmooring at anchor be carried out only by persons with considerable experience in STS operations and use of tugs should be considered where available, especially if yawing of the anchored ship is anticipated. It is also recommended not to unmoor during a change of tide.

If, in the judgement of the POAC, the weather and current conditions so require, the constant heading ship should weigh her anchor and unmooring should be carried out while making way.

    - UNMOORING PROCEDURE AFTER UNDERWAY TRANSFER

If the STS transfer operation have taken place while underway, it is normal to unmoor with the wind and sea on the port side and then bring the combined two-ship system head to the wind to spread apart the ships, unless local conditions dictate otherwise.

    - UNMOORING CHECKS

Sufficient crew should be allocated to unmooring stations and consideration should be given to the following points:

| UNMOORING CHECKS |  |
| --- | --- |
| 1. | The cargo transfer side of the ship should be cleared of obstructions including derricks or cranes. |
| 2. | The method of disengagement and letting go of mooring lines should be agreed. |
| 3. | Fenders, including their towing and securing lines, should be checked to be in good order. |
| 4. | Winches and winch lasses should be ready for immediate use. |
| 5. | Rope messengers and rope stoppers should be ready at all mooring stations. |
| 6. | Sharp fire axes or other suitable cutting equipment should be available at each mooring station. |
| 7. | Communications should be confirmed between ships. |
| 8. | Communications should be established with mooring personnel. |
| 9. | Mooring personnel should be instructed to let go mooring lines only when directed. |
| 10. | Shipping traffic in the vicinity should be checked. |
| 11. | Check-List 5 should be completed. |

### Table 12: Unmooring checks and considerations

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

    - UNMOORING FROM A SHIP ALONGSIDE A TERMINAL

Where necessary, the permission of the harbour authorities and the terminal should be obtained prior to commencing unmooring operations. Unmooring operations should be conducted in accordance with procedures agreed by both Masters and particular attention should be given to prevailing weather and tidal conditions.

Where multiple vessels are alongside a mother ship the STS Superintendent should give consent for the unmooring operation and all other vessels should be made aware of the planned departure. This requirement applies equally to the unmooring of unmanned barges.

If unmooring takes place during hours of darkness, consideration should be given to reducing background lighting so as to be fully aware of other vessels and the proximity of navigational hazards, while ensuring that all working areas are adequately and safely lit.

The mother ship may provide crew to release the lines. However this should not detract from the safety of any ongoing cargo operations. Port regulations may require that licensed line handlers are used and these must be arranged in advance to board with the pilot.

  - PROCEDURE FOR UNBERTHING

Special care needs to be taken when unmooring to avoid the two ships coming into contact. While there are other methods, a common method of unmooring is achieved by: The forward gang let go the forward Springs and then the Headlines. The after gang lets go the Stern lines and then the After Springs and the manoeuvring ship moves clear. Special care should be adopted during letting go of the last lines. This needs to be performed both safely and effectively. A method that can facilitate this is the “toggle pin technique” (see section 6.2.1). The mooring lines sequence may be changed at times according to weather conditions and/or at the discretion of the Mooring Master. The two vessels will be informed prior to the unmooring of any changes. The constant heading ship should not independently manoeuvre until advised that the manoeuvring ship is clear.

It should be noted that local conditions or ship configurations may cause difficulties in separating the two ships and alternative plans should be considered.

    - UNMOORING USING QUICK RELEASE APPARATUS OR TOGGLE PINS

Special care should be taken in regard to letting go the last lines in an expeditious and safe manner. This operation should be planned in advance, be undertaken by experienced crew and requires good communications and supervision.

Different methodologies can be applied by STS Superintendents and ships' crews to carry out this task safely and effectively. One such method involves the use of quick release hooks secured around the mooring bitt or a "toggle" pin that is used in conjunction with a messenger to take the load of the mooring line while it is removed from the mooring bitt.

The deployment of the toggle pin technique is shown below.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

Fig.1 Use of Release or Toggle Pins

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX I – CHECKLIST

LIST of STS Checklist

- STS01-

- STS01A-

- STS01B

- STS01C

- STS01D

- STS01E

- STS01F

- STS01G

- STS01H

- STS01I

- STS01J

- STS01K

LIST OF RESPONSIBILITIES AND DUTIES (Routine & Emergency)

| Rank | Responsibility | Duties | Emergency |
| --- | --- | --- | --- |
| Master |  |  |  |
| Person in Overall Advisory Control (POAC) |  |  |  |
| Chief Officer |  |  |  |
| Second Officer |  |  |  |
| Third Officer |  |  |  |
| Chief Engineer |  |  |  |
| Engineering Officers |  |  |  |
| Deck Ratings |  |  |  |
| ER Ratings |  |  |  |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX II - PLAN

- GENERAL ARRANGEMENT PLAN

- MOORING ARRANGEMENT

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

3.0 TANK CAPACITY PLAN

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX III - RECORDS

RECORD OF STS OPERATIONS

To be retained onboard for at least three years along with Checklists and other relevant records for each STS Operation including abstract from Loading/Discharge computer from each STS Operation. Other relevant records include.

  - Oil Record Book

  - Safety checklists

  - Audit checklist (if available)

  - Notification forms

  - Risk assessment form (if available)

  - Records of rest and work hour compliance

These can be filed in usual location. An additional file is not required.

| Voy No: | Dates | Location | Other vessel | Oil Type | Remarks |
| --- | --- | --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX IV - CONTACTS

### SHIP INTEREST CONTACTS

    - Refer to SOPEP for the full lists of Ship interest Contacts.

    - This includes full up to date contact details for Owners, Managers, DPA, P&I, Response Organisations etc.

### COASTAL STATES CONTACTS

Refer to SOPEP for latest version of MSC- MEPC 6/circ.xx List of national operational contact points responsible for the receipt, transmission and processing of urgent reports on incidents involving harmful substances, including oil from ships to coastal states

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX V - EXAMPLE HOSE CONNECTION GUIDELINES

E.1 EXAMPLE HOSE CONNECTION GUIDELINES

The following instructions shall be followed by ship’s personnel to connect the STS Hoses supplied by the STS Service Provider. Ideally this operation should be supervised throughout by the attending STS Superintendent, and should only be performed by the ship’s staff when requested in writing to do so by the STS Service Provider.

If it is possible to do so, ship’s staff should endeavour to complete stages 1 thru 5 of the tightening process, leaving the final stage to complete under the supervision of the attending STS Superintendent following his arrival on site.

Prior to connection of the two hoses, the flange faces should be inspected and cleaned to ensure all residues and debris from previous gaskets or fixatives are removed completely. For best results, use a metal flange scraper and an aerosol gasket remover and inspect the flange for damage.Be sure surface finish and flatness are satisfactory. Avoid using a wire brush for cleaning flange surfaces as this could result in surface scoring.

Only the joints, nuts, bolts and torque wrench supplied with these hose sections are to be used for the purpose of hose connection. Lubricate bolt and nut threads and nut bearing face (where it contacts the flange).

Centre the gasket on the flange. Note: standard ANSI ring gaskets, when cut properly, should centre themselves with the bolts in place.

| After flange assembly and all nuts have been run down by hand with joint in place, start wrench tightening following the sequence of the numbers indicated on the flange diagram above (marking the number on the flange with a crayon aids in keeping track of the tightening process). During all of the following steps, keep any gap between flanges even all around the circumference, and nuts made up approximately the same amount on each end of the bolt. - First time around just snug the nuts with a hand wrench. - Second time around tighten the nuts firmly with the same wrench. Use a torque wrench for the following steps: - Third time around apply approximately 25% recommended torque**. - Fourth time apply approximately 75% of recommended torque**. - Fifth time around, apply 100% of recommended torque**. | Flange Diagram |
| --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

6) Continue tightening nuts all around until nuts do not move under 100% recommended torque**.

If possible, re-torque after completion of each transfer operation as most of the short term bolt preload loss occurs within 24 hours after initial tightening.

** The recommended torque value is dependent upon the gasket manufacturer's recommendation.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX VI - EXAMPLES OF MOORING EQUIPMENT MAP

- I EXAMPLE STS MOORING EQUIPMENT MAP

This is attached as two separate documents.

  - STS Mooring Map Guidelines

  - Mooring Equipment Map

If not already onboard, they need to be completed. A completed example is provided

Note 1: The Example is applicable to ships constructed on or after January 1, 2007, as described in SOLAS II-1

/ 3-8. Owners/operators/managers should be guided by the approved mooring arrangement plan onboard (also to be included in this Manual).

Note 2: For ships constructed before January 1, 2007, having Mooring Plans onboard which may not have been approved by a classification society; it is the owner’s/operator’s/manager’s responsibility to provide correct data when using the STS Compatibility Data Sheet. Verification of strength and loading of shipboard mooring fittings and supports would be the Owners/Master’s responsibility.

Note 3: Class Societies will not be able to verify the accuracy of the information provided for vessels constructed before January 1, 2007

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

## APPENDIX VII - MARPOL ANNEX II CARGOES (CHEMICAL)

G 1.0 CONTROL OF OPERATIONS

Should an STS Superintendent not be appointed for an STS transfer of chemical products, the advisory control of the Superintendent should be undertaken by one of the Masters, typically the Master of the mother ship.

G 2.0 NOTIFICATION TO AND APPROVAL FROM AUTHORITIES

STS transfers involving chemical cargoes will normally be conducted within port areas and, depending on the products to be transferred, transfer organisers may need to notify and supply an operations plan to the harbour authorities, possibly national government agencies and, if alongside, to the facility owner or terminal operator. The cargo product name as per the certificate of fitness, not the commercial name, should be used when notifying authorities. The transfer of some types of cargo may only be permitted in specially designate areas. Transfer organisers should identify any special restrictions and regulations that need to be complied with, along with any requirements of the particular terminal.

The local authority's permission will usually include explicit requirements for:

  - Designation of the STS Superintendent.

  - Day or night cargo operations restrictions.

  - Day or night vessel movement restrictions.

  - Tank cleaning/ventilating restrictions during cargo transfer.

  - Ballast water management.

  - Emergency response and notification in the event of fire, spill or medical incidents.

  - An approved fire and safety patrol vessel supplied and active during all STS operations.

  - Approved MARPOL Annex II surveyors, for any prewash requirements on discharge and the approval of barges, trucks or terminal slop tank reception facilities.

G 3.0 PRE-ARRIVAL COMMUNICATIONS

### STS instructions

The STS organiser should prepare a general set of instructions for the STS operations. A copy of these general instructions should be transmitted in advance to the STS Superintendent and Masters of all vessels to berth alongside the discharging ship. The instructions should detail information that may include:

  - Name and call signs of the discharging ship and all vessels to berth alongside.

  - Location/position of the transfer operation and water depth.

  - Identification and contact details of the STS Superintendent.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

  - Details of the planned transfer operations and expected sequence of vessels alongside the discharging ship.

  - Cargo details for each vessel.

  - Details of equipment required, including fendering, lifting equipment, hoses and connections.

  - Applicable local regulations.

G 4.0 PLANNING FOR CARGO TRANSFER

In addition to the items listed in section 7.3, the following should also be agreed for bulk liquid chemical transfers:

- Applicable port regulations and terminal requirements.

- Personnel transfer.

- Inerting and final cargo padding.

- Sampling procedures.

- Pre-wash requirements.

- Tank washing.

- Any relevant simultaneous operations taking place on either vessel.

G 4.1 SOLIDIFYING CARGOES/INSULATION/HEAT DANGERS

Cargoes with a freezing/solidifying point higher than the ambient temperature must be heated to remain liquid. The vessels involved in the STS transfer operation should exchange information on maximum/minimum temperatures expected during cargo transfer and any limitations imposed due to cargo tank, pipeline and/or hose design.

Where effective insulation is not provided for pipelines and flanges, the crews on the vessels involved should be made aware of the associated safety hazard. In addition to the likelihood of the cargo solidifying in the cargo line, the pressure in the vapour space of the cargo tank should be closely monitored to determine the possibility of vapour solidifying in the vent line and/or pressure/vacuum (P/V) valves. Thermal stress to the vessel's tanks and pipelines should be minimised by commencing operations slowly and gradually increasing to the agreed transfer rate and temperature.

In the event that the cargo flow is unexpectedly stopped, the vessels involved should immediately take steps to clear the cargo lines to avoid any freezing/solidifying and blocking of the cargo system.

G 4.2 TANK CLEANING

For the mother ship, an STS transfer operation will often involve both loading and discharging and this may necessitate tank cleaning. For vessels alongside there may be occasions where tank cleaning is necessary, particularly for cargoes requiring pre-wash as per MARPOL Annex II. The consent of the STS Superintendent, the terminal and the harbour authority should be obtained prior to commencing tank cleaning operations. The

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

other vessels should be informed of the nature of the product being washed, (anticipated) time of commencement and completion and any other relevant safety information.

On completion of washing, and prior to ventilating, consideration should be given to the climatic conditions and wind direction. Particular attention should be paid to the differing cargo deck levels of the mother ship and vessels alongside to avoid creating any vapour clouds over cargo deck areas and accommodation blocks. Notification should be given to all vessels alongside prior to the ventilation of any cargo tank and the atmosphere should be monitored for toxicity and flammable vapour. In the event of toxicity rising above the permitted threshold limit value (TLV), or flammable vapour being detected, all ventilation should be stopped immediately.

For cargoes requiring pre-wash procedures as per MARPOL Annex II and where the pre-wash is to take place while alongside, the authorities should be notified in advance so as to have the authorised surveyor in attendance on completion of discharge. Where the vessel is to discharge the pre-wash to a slop barge for transportation to a reception facility, the barge should moor and conduct operations in accordance with the recommendations contained in this Guide.

G 5.0 CARGO TRANSFER - GENERAL GUIDANCE G 5.1 VAPOUR CONTROL

In addition to the requirements of the IBC Code chapter 15 (reference 11), local regulations may require that the transfer of certain toxic or otherwise hazardous products utilises a return vapour line. Return vapour hoses should be of a construction and material compatible with the product to be handled. The handling, connection and support of return vapour hoses should follow the same guidance as that for cargo hoses.

The pre-transfer meeting should consider the additional precautions to be taken for return vapour, i.e. pressure control of the tank atmospheres on both vessels. The transfer rate should be adjusted to ensure that pressure and vacuum limits for the tanks are not exceeded and excessive pressure or vacuum does not result in a release of toxic vapour to the atmosphere or ingress of air into the tank.

G 6.0 OPERATIONS AFTER COMPLETION OF CARGO TRANSFER G 6.1 CLEARING LINES AND DISCONNECTING

In accordance with MARPOL Annex II, the discharging vessel should strip the tanks of all product and clear the lines and hoses to the receiving vessel. This process should be carried out in accordance with the vessel's procedures and arrangements manual. To avoid any introduction of air, nitrogen should be used for clearing the lines and hoses. The pressure hose supplying the nitrogen should be fitted with a non-return valve.

The process of clearing lines should be carried out with the responsible persons of both vessels in attendance. Line clearing should not be commenced until the responsible person on the receiving vessel is present and in charge of the operation. The receiving vessel should ensure that there is sufficient ullage space to accept the

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |

remaining volume and should constantly monitor the tank ullage and pressure. If a return vapour line is in use, the discharging vessel should also monitor tank pressures carefully.

During clearing of cargo lines, transfer hoses should be adjusted to facilitate effective draining and avoid the possibility of them becoming trapped between the vessels' hulls.

Once lines and hoses are cleared of product, the receiving vessel should close the manifold valves on both cargo and vapour systems. All pressure from the hoses should be released back to the discharging vessel and all valves closed. Simultaneously, and in the same sequence, manifold valves on the vapour system should be closed. Hoses should then be disconnected and recovered by the vessel supplying them. Cargo and vapour manifold flanges should be blanked off. Hoses that are not to be immediately re-used for tank or line cleaning should also be securely blanked.

G 6.2 PRECAUTIONS

  - Communication between vessels should be established and maintained throughout the line clearing operation. A crew member should be assigned to the valve controlling the pressure and be ready to shut off the supply at any time.

  - Safety of personnel should be ensured at all time by using the appropriate personal protective equipment (PPE) for the cargo being handled. Personnel not involved in the line clearing operation should be restricted from entering the area.

  - Potential pollution hazards should be identified to all personnel engaged in the line clearing operation.

  - Pressure should be controlled at all times to prevent cargo overflowing and to prevent over- pressurisation of cargo tanks.

OCIMF – Mooring Load Analysis during Ship to Ship Transfer Operations - 2022

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
| HSSEQ-STS | 1 | 10-sept-2025 | MD |