# Volatile Organic Compound (VOC) Management Plan

REVISED MARPOL ANNEX VI REGULATION 15.6

| Vessel |  |
| --- | --- |
| Call Sign |  |
| Flag |  |
| Port of Registry |  |
| IMO Number |  |
| Class |  |
| GRT |  |
| Deadweight |  |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## INTRODUCTION AND OBJECTIVES

CONTENTS

# INSTRUCTIONS FOR COMPLETION OF VOC MANAGEMENT PLANS	Error! Bookmark not defined.

INTRODUCTION AND OBJECTIVES	2

RECORDS OF CHANGES	4

MAIN FEATURES OF MARPOL 73/78, REVISED ANNEX VI, REGULATION 15	5

- INTRODUCTION	5

- OBJECTIVES	7

- ADDITIONAL CONSIDERATIONS	8

SECTION 1 - THE HULL AND ITS PRESSURE LIMITATIONS	9

  - ALLOWABLE CARGO TANK ULLAGE PRESSURE – GENERAL	9

  - TYPICAL CARGO TANK VENTING SYSTEMS - GENERAL	9

  - TYPICAL SETTINGS OF PRESSURE/VACUUM RELIEF DEVICES	10

  - ARRANGEMENT OF THE SHIP AND DESCRIPTION OF ITS CARGO TANKS	10

  - DESCRIPTION OF CARGO TANKS AND VENTING ARRANGEMENTS	11

SCHEMATIC DRAWING OF THE INERT GAS SYSTEM	11

  - DESCRIPTION OF PRESSURE CONTROL/RELEASE SYSTEMS AND OVER/UNDER PRESSURE ALARMS	12

  - VESSEL LOADING RATES	13

  - COMPANY POLICY ON THE USE OF INERT GAS AND GAS FREEING TANKS	16

SECTION 2 – CRUDE OIL TANKER PRESSURE CONTROL/RELEASE SYSTEMS - GENERAL	17

  - INTRODUCTION	17

  - LOAD PORT DISPLACEMENT OF VOC	17

  - VOC RELEASE DURING THE VOYAGE	19

  - A CRUDE OIL TANKER’S VAPOUR PRESSURE CONTROL MECHANISMS - GENERAL	20

SECTION 3 – VOC GENERATION IN CRUDE OIL	22

NMVOCS	22

  - THE VOLATILITY OR VAPOUR PRESSURE OF THE CRUDE OIL	22

  - THE TEMPERATURE OF THE CRUDE OIL IN A CARGO TANK	23

  - THE PRESSURE SETTING OR CONTROL OF THE VAPOUR PHASE WITHIN THE CARGO TANK .24

  - THE SIZE OR VOLUME OF THE VAPOUR PHASE WITHIN THE CARGO TANK SYSTEM	24

SECTION 4 – THE METHODS AND SYSTEMS FOR THE CONTROL OF VOC	25

  - METHODS AND SYSTEMS FOR THE CONTROL OF VOC DURING LOADING	25

    - BEST PRACTICES AND DESIGN	25

    - VAPOUR EMISSION CONTROL SYSTEMS (VECS)	31

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

  - METHODS AND SYSTEMS FOR THE CONTROL OF VOC DURING TRANSIT	34

    - BEST PRACTICES/DESIGN	34

    - INCREASED PRESSURE RELIEF SETTINGS	37

    - TARGET PRESSURE FOR CARGO TANKS	37

4.2.7 CARGO TEMPERATURE	37

  - Methods and systems for the control of VOC during Discharging/Ballasting	37

  - Control of VOC during cargo tank purging to reduce toxic gas or gas freeing for entry	37

SECTION 5.0 - THE MONITORING AND CONTROL OF NMVOC RELEASES	37

SECTION 6.0 - TRAINING PROGRAMME	38

SECTION 7.0 - DESIGNATED PERSON	39

SECTION 8.0 - LIST OF DRAWINGS/ADDITIONAL INFORMATION	40

ATTACHMENTS ....................................................................................... .

APPENDIX I ...............................................................................................

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## RECORDS OF CHANGES

| Rev. No. | Section | Page | Date | Description | Init. |
| --- | --- | --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## MAIN FEATURES OF MARPOL 73/78, REVISED ANNEX VI, REGULATION 15

The purpose of the VOC Management Plan is to ensure that the operation of a tanker, to which regulation

15.6 of the revised Annex VI applies, prevents or minimizes VOC emissions to the extent possible.

The requirements of regulation 15.1 to 15.5 are applicable only to tankers operating in designated ports or terminals as specified by a Party to the Annex.

Designated ports and terminals will be required to have a Vapour Emissions Control System (VECS) in accordance with MSC/Circ. 585.

A tanker calling at a designated port or terminal regulating VOC shall be provided with a Vapour Collection System approved by the Administration taking into account the safety standards as laid out in MSC/Circ. 585, and shall use this system during the loading of relevant cargoes. A designated port or terminal which has installed a VECS in accordance with regulation 15 may accept tankers which are not fitted with vapour collection systems for a period of 3 years after the effective date of the VOC control being applied (MARPOL Reg. VI/15.5).

Meanwhile, regulation 15.6 refers to a mandatory VOC Management Plan which a tanker carrying crude oil shall implement and have onboard. Ships specific VOC Management Plan shall provide written procedures for minimizing VOC emissions during the loading, sea passage and discharge of cargo.

Emissions of VOC's can be prevented or minimized by:

- Optimizing operational procedures to minimize the release of VOC emissions; and/or

- Using devices, equipment, or design changes to prevent or minimize VOC emissions.

## INTRODUCTION

Regulation 15 of Annex VI of MARPOL 73/78, as revised by IMO Resolution MEPC.176 (58) (hereinafter referred to as “revised Annex VI”) regulate the VOC emissions from a tanker in designated port(s) or terminal(s) of a Party regulating such emissions. Regulation 15.6 requires that a tanker carrying crude oil shall have on board and implement a VOC Management Plan (VOC Plan) approved by the Administration in accordance with IMO resolution MEPC.185 (59). This Plan shall be specific to each ship.

The aim of the VOC Plan is to identify the arrangements and equipment required to enable compliance with regulation 15.6 of the revised Annex VI and to identify for the ship’s officers all operational procedures for VOC emission control.

This VOC Plan has been written pursuant to the requirements in the revised Annex VI regulation 15.6, and it has been developed in accordance with the Resolution MEPC. 185(59) “Guidelines for the Development of a

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

VOC Management Plan”, having taken into account the provisions of MEPC.1/Circ.680 “Technical Information on systems and operation to assist development of VOC Management Plans”.

The VOC Plan describes the specific arrangement, operations and conditions onboard a crude oil tanker with respect to the emission and ability to control VOC emissions. This VOC Plan is not a safety guide and reference shall be made to other publications to evaluate safety hazards.

.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## OBJECTIVES

The IMO Guidelines for the development of a Volatile Organic Compound (VOC)

management plan (Resolution MEPC.185(59):)

### 1 Objectives

- The purpose of the VOC management plan is to ensure that the operation of a tanker, to which regulation 15 of MARPOL Annex VI applies, prevents or minimizes VOC emissions to the extent possible.

- Emissions of VOCs can be prevented or minimized by:

  - Optimizing operational procures to minimize the release of VOC emissions; and/or

  - Using devices, equipment, or design changes to prevent or minimize VOC emissions

- To comply with this plan, the loading and carriage of cargoes which generate VOC emissions should be evaluated and procedures written to ensure that the operations of a ship follow best management practices for preventing or minimizing VOC emissions to the extent possible. If devices, equipment, or design changes are implemented to prevent or minimize VOC emissions, they shall also be incorporated and described in the VOC management plan as appropriate.

- While maintaining the safety of the ship, the VOC management plan should encourage and, as appropriate, set forth the following best management practices:

  - The loading procedures should take into account potential gas releases due to low pressure and, where possible, the routing of oil from crude oil manifolds into the tanks should be done so as to avoid or minimize excessive throttling and high flow velocity in pipes;

  - The ship should define a target operating pressure for the cargo tanks. This pressure should be as high as safely possible and the ship should aim to maintain tanks at this level during the loading and carriage of relevant cargo;

  - When venting to reduce tank pressure is required, the decrease in the pressure in the tanks should be as small as possible to maintain the tank pressure as high as possible;

  - The amount of inert gas added should be minimized. Increasing tank pressure by adding inert gas does not prevent VOC release but it may increase venting and therefore increased VOC emissions; and

  - When crude oil washing is considered, its effect on VOC emissions should be taken into account. VOC emissions can be reduced by shortening the duration of the washing or by using a closed cycle crude oil washing programme.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## ADDITIONAL CONSIDERATIONS

- Additional considerations

- A person in charge of carrying out the plan

  - A person shall be designated in the VOC management plan to be responsible for implementing the plan and that person may assign appropriate personnel to carry out the relevant tasks;

  - Procedures for preventing or minimizing VOC emissions

    - Ship-specific procedures should be written or modified to address relevant VOC emissions such as the following operations:

      - Loading;

      - Carriage of relevant cargo; and

      - Crude oil washing.

    - If the ship is equipped with VOC reduction devices or equipment, the use of these devices or equipment should be incorporated intot he above procedures as appropriate

    - Training

.1 The plan should describe the training programmes ot facilitate best management practices for the ship to prevent or minimize VOC emission.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## SECTION 1 - THE HULL AND ITS PRESSURE LIMITATIONS

#### ALLOWABLE CARGO TANK ULLAGE PRESSURE – GENERAL

The cargo tank structure is designed to withstand a range of design loads and parts of the tank structure will also contribute to the global longitudinal strength of the ship. The Classification Societies’ specified load conditions and loads are applied in verification of the structural design. One such load is the combined pressure from the liquid cargo and the tank ullage pressure. The tank ullage pressure is to be minimum 25 kN/m2 or the opening pressure of the pressure relief device (P/V-valve), whichever is greater. Accordingly, the maximum allowable ullage pressure in a standard tanker is typically interpreted as 25 kN/m2 (i.e. approximately 2550 mmWG). It should however be noted that global strength considerations and the impact of other design loads may imply that actual allowable pressure could be higher.

In terms of under pressure, SOLAS II-2 Reg.11.6 indicates an allowable under pressure of -700 mmWG. From a structural point of view, the maximum allowable tank under pressure is clearly lower.

### Exceeding the maximum allowable pressures could lead to structural failures. If such a structural failure results in opening of the tank structure to atmosphere, uncontrolled VOC emissions will occur together with the possibility of oil pollution to the seas. Further, it could result in loss of inert gas protection with subsequent hazards related to fire and explosion.

#### TYPICAL CARGO TANK VENTING SYSTEMS - GENERAL

The design of cargo tank venting and inert gas systems is governed by SOLAS II-2 Reg. 11.6 and Reg. 4, 5.3. Most crude tanker have a common cargo tank venting and inert gas main pipeline which is also used for vapour emission control (ref. section 4). Branches to each cargo tank are provided with isolation valves and blanking arrangements. The isolation valves and blanks are typically only used in connection with tank entry. SOLAS II- 2 Reg 4 5.3 requires that the isolation valves are to be provided with locking arrangements to prevent inadvertent closing/opening of said tanks.

The cargo tank venting/inert gas main is connected to a mast riser. The mast riser has a min height of 6 m with an IMO approved flame arrestor at its outlet. An isolation valve is provided between the cargo tank venting/inert gas main and the mast riser. Some designs have a small capacity pressure/vacuum valve fitted in a bypass across the isolation valve. This latter enables thermal breathing from cargo tanks when the isolation valve is closed.

A liquid filled P/V-breaker is typically connected to the cargo tank venting/inert gas main. The P/V-breaker has a capacity to accommodate the gas flow from cargo tanks during loading (125% the loading rate and discharge rate). The cargo tank venting/inert gas main is typically used during loading and discharging operations. During loading the mast riser valve is open (unless vapour emission control is performed) and VOC is expelled to air. During discharge the same valve is closed and inert gas used to replace the tank atmosphere. The cargo tank venting/inert gas main is also used during voyage but the mast riser valve will be operated only in the event of increasing ullage pressure.

In addition to the common cargo tank venting/inert gas main, each cargo tank is required to have a pressure/vacuum relief device for thermal breathing in the event the cargo tank is isolated from the common cargo tank venting/inert gas main. Although Classification Societies accept that these devices have capacity to

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

accommodate gas volumes resulting from variations in cargo temperature only (i.e. thermal breathing), latest industry practices have led to the installation of devices with capacity to accommodate the full gas flow from loading of cargo tanks.

#### TYPICAL SETTINGS OF PRESSURE/VACUUM RELIEF DEVICES

Although the design pressure of cargo tanks is typically …… mmWG and …. mmWG, the typical setting of pressure/vacuum valves on crude tankers is …… mmWG and…..mmWG.

The typical settings of the P/V-breakers are ….. mmWG and …. mmWG. It should be noted that for liquid filled P/V-breakers, the settings have to take into account ship movements as specified by the Classification Societies.

#### ARRANGEMENT OF THE SHIP AND DESCRIPTION OF ITS CARGO TANKS

| Item | Description |
| --- | --- |
| Cargo Tank Layout |  |
| Cargo Capacity (CBM) |  |

Attach Capacity plan for indicating position and numbering of cargo tanks.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### DESCRIPTION OF CARGO TANKS AND VENTING ARRANGEMENTS

| Item | Description (This must be ship specific – example entries are provided which must be amended accordingly) |
| --- | --- |
| Cargo tank venting and inert gas main pipeline |  |
| Isolation valves, locking and blanking arrangements |  |
| Connections between cargo tank venting/inert gas main to mast riser. |  |
| Isolation valve by pass line fitted with a pressure/vacuum valve? |  |
| Vapour Manifold |  |
| Cargo Heating |  |

### Attach Schematic drawing of the Cargo tank venting system

#### SCHEMATIC DRAWING OF THE INERT GAS SYSTEM

### Attach Schematic drawing of the inert gas system

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### DESCRIPTION OF PRESSURE CONTROL/RELEASE SYSTEMS AND OVER/UNDER PRESSURE ALARMS

| Item | Description (This must be ship specific – example entries are provided which must be amended accordingly) |
| --- | --- |
| Cargo Tank Design Pressure | The design pressure of cargo tanks is Maximum: …… WG Minimum: ….. mmWG |
| Primary Tank Pressure/Vacuum Valve (P/V) . | Each tank is fitted with an individual P/V valve. This valve is the primary mechanism for the protection from cargo tank over pressure is a P/V valve on each tank. The settings on the P/V valves are: Pressure …… mm WG Vacuum …… mm WG |
| P/V Breaker | Supporting the over pressure safety system of the P/V valve is the secondary safety mechanism of the P/V breaker fitted on the main venting line The settings of the P/V Breaker are: Pressure …… mm WG Vacuum ….. mm WG |
| Mast Riser |  |

Attach Schematic drawings and the pressure protection devices details.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### VESSEL LOADING RATES

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 01 | JULY 2024 | CR |

#### COMPANY POLICY ON THE USE OF INERT GAS AND GAS FREEING TANKS

The Company considers that the continuous availability of inert gas and its correct use is of paramount importance. The cargo spaces and slop tanks are to be kept fully inerted at all times unless it is necessary to enter these spaces. There may be circumstances in a particular trade where Inert Gas is not used for high specification, high flash point cargoes.

If entry is required the space will be thoroughly cleaned whilst using inert gas. After cleaning the tank atmosphere must be purged with inert gas to reduce the hydrocarbon levels to less than 1.4% by volume to prevent the atmosphere from entering the flammable range during venting. The Tankscope is used for this purpose. The space must then be vented with fresh air ensuring that the space has not less than 20.8% of oxygen, not more than 1% LEL Explosimeter reading and that concentration of toxic vapours less than TLV before entry is made.

Clearly this process will result in quantities of VOC’s being released and therefore this should only proceed if tank entry is absolutely necessary. Records are to be retained as per Section 5.

The Master must ensure that the oxygen content of the inert gas supply in the main line to the tanks is not more than 5% by volume, at any required rate of flow.

Local regulations may require the vessel to operate to a lower limit.

If for any reason, the inert gas system cannot be operated, or the oxygen content of the inert gas

supply cannot be reduced to below 5% by volume, or a level required by the local port regulation, the Master must stop operations (ref 9.8 of these operating instructions).

In addition to the shipboard PMP an Inert Gas Operations and Maintenance Log is to be maintained for recording of all required checks and tests and operational parameters of the IG plant and related equipment.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## SECTION 2 – CRUDE OIL TANKER PRESSURE CONTROL/RELEASE SYSTEMS - GENERAL

#### INTRODUCTION

Vapour release from crude oil tankers generally occurs on four occasions; during loading, during the loaded voyage, during gas freeing of tanks (see 1.8) and less frequently during the ballasting of cargo tanks at the discharge port.

Since the introduction MARPOL, tankers built after 1st June 1982 (Regulation 18), are all designed with totally segregated (designated) ballast tanks. With these Regulations in force, the use of cargo tanks for the loading of ballast is only on very rare occasions for bad weather purposes where one of the Crude Oil Washed cargo tanks is dedicated to take in ballast water. Therefore, the displacement of vapour from the relevant crude oil cargo’s tank at the discharge port has ceased to occur for the MARPOL compliant type tankers.

#### LOAD PORT DISPLACEMENT OF VOC

Displacement of crude oil cargo vapours at the loading port continues to occur. The reasons for the existence of these volumes of displaced vapour is attributed to two reasons; namely existing vapour in the cargo tank system before loading and, the evolved vapour created during the loading programme.

The first portion of the vapour displaced from the cargo tanks is that from the evolved vapour generated during the previous discharge programme and in particular that vapour generated as a result of the Crude Oil Washing of the cargo tanks. The second portion of vapour displaced is that that develops or evolves during the loading programme itself. This vapour evolves as a result of, both, the turbulence generated in the cargo tanks due to the rate of loading and the pressure differentials within the loading pipeline system creating a degree of "flashing" of the vapour from the incoming crude oil.

To illustrate the extent of these gases within a cargo tank system on a tanker during a loading process the figure on the right shows the measurements of hydrocarbon vapour concentrations as taken from a tanker during its loading programme. The “X” axis of the graph records the percentage of hydrocarbon vapour (VOC) concentration whereas the “Y” axis records the percent status of loading of the tanker. The graph primarily records the total hydrocarbon gas concentration at the differing percentages of loading of the cargo tanks. The total figure is proportioned and sub-divided, taking into consideration the diminishing size of the vapour volume in the cargo tanks, into the two concentrations of vapours, namely, those present at the commencement of loading and the concentration of vapours that evolve as a result of the loading process.

These vapours are displaced by the incoming cargo volumes, throughout the loading period, and released through the ship's vapour pipeline system (Inert Gas pipeline) to atmosphere via the ship's Mast Riser. In

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 01 | JULY 2024 | CR |

order to prevent excess pressures within the cargo tank system the isolation/control valve to the Mast Riser is fully opened at the commencement of loading and remains opened until completion of loading. Once the Mast Riser valve is shut and loading is completed, the necessary "in tank" positive pressure is achieved to prevent any form of air/oxygen entry into the cargo tank vapour system as is required by the SOLAS Regulations.

Mast

P/V

P/V Valves

In figure 2.2 below a photograph shows the deck of a tanker and highlights the relevant pressure control and release mechanisms, namely, the Vessel’s Mast Riser, the individual tank Pressure/Vacuum (P/V) valves and the secondary safety mechanism of the P/V Breaker. These mechanisms will be explained further below in this section

### Figure 2.2 – Main Cargo Deck of a Crude Oil Tanker

Typically a normal loading programme will take about 24 hours for a VLCC with a volumetric rate of loading of up to 20,000 m3/hour. The Mast Riser is normally used during loading for tank vapour pressure control. Its exit location, being at least 6 meters above the deck, allows for the free flow of the vapours displaced from the cargo tanks by the incoming liquid crude oil at the rate of loading of the cargo. The rate of displacement of NMVOC vapours from the cargo tank system will be the same as the loading rate but the concentration of NMVOC vapours in the displaced stream will be greater dependent upon the extent and rate of evolution of NMVOC vapours (vapour growth) from the incoming cargo that would add to the volume of gas/vapour mixture already existent in the cargo tank prior to loading as shown in figure 2.1 above

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### VOC RELEASE DURING THE VOYAGE

During the voyage, the temperature of the gases/vapours in the ullage space of the cargo tanks and the liquid cargo varies. The gas phase consists of a mixture of unsaturated gases (Inert Gas - for tank safety and protection) and saturated vapours (evolved hydrocarbon vapours from the cargo). The temperature of the gas phase of the tank varies diurnally with its maximum temperature being achieved by mid afternoon and its coolest temperature in the early hours of the morning. The liquid phase temperature varies very much slower and is dependent upon both the hull design and the temperature of the surrounding seawater.

Figure 2.3 below, records, as an example, the vapour pressure and cargo temperature data of a reported voyage for a single hulled (but segregated ballast) tanker. The graph records on the “X” axis the days of the voyage whereas the “Y” axis records both the cargo temperature (oC) and the pressure (mmWG) within the vapour phase of the cargo tank system. Superimposed upon the graph is both the normal operational release pressure as well as the P/V valve opening pressure levels. The vapour pressure readings were recorded every four hours whereas the cargo liquid temperature readings (blue) were recorded daily.

P/V Valve Opening Pressure

Maximium Normal Control Operating Pressure before

30	1600

1400

25

1200

20

Temperature Deg C

1000

Tank temp

Pressure mmWG

15	800

600

10

400

5

200

0	0

0	5	10	15	20	25	30	35	40	45	50

Day Number

### Figure 2.3 – Temperature and Pressure profile for a crude oil voyage

In double hulled vessels the temperature of the liquid cargo remains closer to the temperature of the cargo upon loading for a longer period due to the so called “Thermos Effect" or heat loss insulation created by the void or empty ballast space. The cargo temperature profile, as shown in Figure 2.3, reflects the expected changes to temperature for a cargo carried onboard a single hulled vessel where the impact of the seawater temperature upon the cargo is more apparent. This aspect can be more clearly seen in Figure 2.3 for the early/interim days of the 47 day voyage from North Sea to the Far East.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### A CRUDE OIL TANKER’S VAPOUR PRESSURE CONTROL MECHANISMS - GENERAL

A crude oil tanker is designed and constructed to withstand high vapour pressures up to a certain value. In order to protect the vessel's structure against excessive pressures two differing levels of safety mechanisms are installed to control and limit the pressures exerted in vapour phase of the cargo system. The installation of both of these systems is a requirement within SOLAS. These mechanisms are:

    - The individual tank Pressure/Vacuum (P/V) valve

    - The common Pressure/Vacuum (P/V) breaker.

The P/V valve is the primary mechanism for the protection from cargo tank over pressure. The design and operational requirements of the P/V valves is set out in the ISO 5364:2000 standard but the opening and closing pressure setting of the individual valves is set in accordance with the designed tolerance of the relevant structure having applied the necessary safety margins.

A design of a P/V valve can be seen in figure 2.4 next to this. The valve is fitted to a vertical pipeline connected directly to the vapour space of a cargo tank. The valve consists of two sections, namely, the vacuum protection section on the left hand side of the valve as shown and the pressure control mechanism of the right hand side. Both mechanisms rely upon a weighted diaphragm that will be lifted when the pre-designed pressures are met. On the pressure side of the valve the exit nozzle is designed such that the exit velocity of the vapours reach the required velocity so as to maintain the deck working area clear of hydrocarbon vapours.

Figure 2.4 – A Design and Construction of a P/V valve

Each cargo tank is normally equipped with its valve so that full protection is available should the individual cargo tank be isolated from the main common vapour system onboard the tanker. The typical pressure setting for a P/V valve is traditionally measured in millimetres of water gauge and would be in the range from 1400 to 1800 mmWG. These valves are supported on a connecting pipeline to the tank's atmosphere by a 100 to 150 mm diameter pipeline and located at least 2 meters above the deck. Due to the requirements to prevent mechanical damage to these valves the closing pressure is controlled by a damping mechanism (to prevent hammering of the valve). As a result of the damping mechanism the closing pressure of the valve will vary but in the range of 400 - 800 mmWG.

Supporting the over pressure safety system of the P/V valve is the secondary safety mechanism of the P/V breaker. In the event of a rapid pressure fluctuation within the common vapour system the P/V breaker is

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

available to relieve such an over pressure. The single P/V breaker is located on the common vapour pipeline, serving all the cargo tank branch pipelines, which ends at the vessel's Mast Riser (see Figure 2.2)

### Figure 2.5 - The Design and operation of a P/V Breaker

The construction and operation of the P/V breaker can be seen in Figure 2.5 above. The pressure setting in the P/V breaker is achieved by way of the internal water column with an equivalent pressure setting of approximately 2000 mmWG. The water column also isolates the vapour phase from external air ingress into the system. In the event of an excessive pressure surge within the tank vapour system the water column would either be displaced out of the breaker onto the deck, in the event of excessive pressure, or drawn into the cargo tanks in the event of an under pressure. This will, therefore, open the total vapour system to the external environment and atmospheric pressure and, due to the equipment’s dimensions, will relieve the pressure in the system very quickly. Thus, this safety mechanism, due to its pressure setting will only operate if the vessel's tanks P/V valves fail to operate or are not of sufficient capacity to relieve the pressure surge adequately.

It should, however, be noted that once the P/V Breaker operates then, as stated above, it will reduce the pressure within the tank vapour system to atmospheric pressure thereby exposing the tank system to ingress of oxygen. It is therefore that this system is a "last resort" system to preserve the structure of the tanker from damage.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## SECTION 3 – VOC GENERATION IN CRUDE OIL

NMVOCS are a pollutant to the air and act as a precursor to the formation of Tropospheric Ozone – commonly termed Smog. Tropospheric Ozone is identified as a Greenhouse Gas with a greater contribution per unit volume or tonnage to Climate Change than the base gas, namely Carbon Dioxide (CO2).

Thus, to control this emission there are four criteria that impact the extent and rate of evolution of gaseous non methane VOC from crude oils and its subsequent release to atmosphere. These are:

    - The volatility or vapour pressure of the crude oil

    - The temperature of the liquid and gas phases phase of the crude oil tank

    - The pressure setting or control of the vapour phase within the cargo tank.

    - The size or volume of the vapour phase within the cargo tank.

Each of these criteria are defined and briefly explained below together with any interaction between the criteria for general operational circumstances.

#### THE VOLATILITY OR VAPOUR PRESSURE OF THE CRUDE OIL

Reid Vapour Pressure (RVP) – This is an industrially developed standard test method to determine the Air Saturated absolute Vapour Pressure of a volatile, non-viscous hydrocarbon liquid in compliance with the requirements specified in the Institute of Petroleum test procedure IP 69.

The RVP is the vapour pressure obtained within a standardised piece of test equipment for the evolved hydrocarbon vapour at a temperature of 100 0F or 37.8 0C. The standard test parameters for the determination of this pressure are important to identify and relate to the ratio of a fixed liquid volume to a fixed vapour volume. This ratio is one part of liquid to four parts of vapour. Thus, the pressure reported for this parameter reflects, in principle, the pressure that would be registered when the cargo tanks are about 20% loaded.

Data for this pressure may be recorded in the relevant Materials Safety Data Sheet (MSDS) for the relevant crude oil carried onboard. In the event that there more than one parcel of crude oil onboard then a mathematical average can be calculated for the differing types.

There are two other parameters, namely, the Saturated Vapour Pressure and Unsaturated Vapour Pressure. These two parameters, and the physics behind them, give more clear indications and guidance with respect to a crude oil’s volatility with respect to vessel operations and NMVOC control.

Saturated Vapour Pressure (SVP) - is the equilibrium pressure generated by the liquid phase for the vapour volume within a defined system. The Saturated Vapour pressure is developed only by the evolved hydrocarbon vapours from the crude oil liquid phase. For a Saturated Vapour to be present it must have contact with its own liquid phase. If the liquid phase temperature increases or decreases so will the Saturated Vapour pressure vary accordingly – an increase the liquid temperature will cause an increase in the Saturated Vapour Pressure.

However, if the vapour volume increases or decreases for a known liquid temperature, the pressure should, in theory, remain constant. These circumstances respectively will only cause the vapour to condensate and fall back to the liquid phase or more vapour to evolve from the liquid phase to maintain the Saturated Vapour Pressure. This physical characteristic is indicative of equilibrium pressure – between the liquid and vapour phases within the defined system.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

From the foregoing it can be readily recognised that Saturated Vapour pressure should not vary with the size of the vapour volume and will only vary with the temperature of the liquid phase – not the vapour phase temperature.

Unsaturated Vapour Pressure (UVP) - Contrary to the concept of Saturated Vapour Pressure, an Unsaturated Vapour is not in contact with its liquid phase. In this case the vapour is obtained from other sources such as air or more likely Inert Gas. Thus, by reference to the standard laws of physics and what is termed the Ideal Gas Law, both variations in volume and/or temperature (this time of the gas or vapour phase) will vary the pressure within a closed system.

From an operational perspective this type of behaviour is the primary cause of the variation of pressures within a cargo tank system over a 24 hour period and is to be associated with the Inert Gas phase within a cargo tank. However, the pressure generated from this type of gas/vapour is not the total vapour pressure in the cargo system.

Behind the pressure generated from the Unsaturated Vapours (Inert Gas) lays the pressure generated by the Saturated Vapours (the hydrocarbon vapours evolving from the crude oil cargo). As stated above, this pressure will remain as a constant for a given cargo/liquid temperature and, as is well recognised, a cargo temperature will not vary to the same extent as the vapour temperature due to heating or cooling from external sources (sunlight, sea temperature, air temperature etc). Thus, the variation for the Tank Observed Total Vapour Pressure is due to the presence of Inert Gas in the cargo tank.

Total/True Vapour Pressure (TVP) - this pressure is the total pressure to be achieved within a defined closed system given the variable parameters of vapour volume and the differing control temperatures. In fact it is the combination or addition of the Saturated and Unsaturated Vapour Pressures (Dalton’s Law of Partial Pressure1) within a closed and defined system.

Thus, onboard a tanker, the pressure measured within Vapour System is the Total Vapour Pressure of the system which is the sum of the two individual pressures generated by the differing types of gases present in the system.

#### THE TEMPERATURE OF THE CRUDE OIL IN A CARGO TANK

It is necessary to consider the two phases (liquid and vapour) separately with regard to the impact of temperature.

The temperature of the liquid in a crude oil cargo tank in a crude oil cargo tank will vary little over the period of a voyage unless cargo heating is being undertaken. It is this temperature that determines the Saturated Vapour pressure that will be exerted by the evolving NMVOCs from the cargo volume and contribute to the Total Vapour Pressure in the cargo tank at any one time. The cooler the liquid phase temperature the lower will be the Saturated Vapour pressure of the crude oil but care should be taken not to allowing cooling of waxy cargoes too much such that it promotes wax precipitation.

The temperature of the vapour or gas in a crude oil cargo tank will change more rapidly and vary during the day/night cycle. As this phase in the cargo tank contains a mixture of Saturated (evolved hydrocarbon gases)

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

and Unsaturated (Inert gas) gas specie the pressure in this space will vary with temperature due to the reaction of the Unsaturated Gas component to temperature. Thus, during the day when the gas phase warms the pressure in the tank will increase so long as there is an Inert Gas component in the gas phase. The obverse will occur at night as the gas phase cools.

#### THE PRESSURE SETTING OR CONTROL OF THE VAPOUR PHASE WITHIN THE CARGO TANK

The technologies available onboard crude oil tankers for the control of pressure within the cargo tank vapour system. It is important to identify the significance of pressure with respect to the evolution of hydrocarbon vapours from a crude oil liquid phase.

Control of the extent of the pressure within a crude oil cargo tank vapour system will determine the extent of further vapour evolution from a crude oil cargo. If the pressure within the system is controlled at the Saturated Vapour pressure then equilibrium pressure between the liquid and vapour phase is obtained and no further NMVOC will evolve from the cargo. However, if the vapour pressure in the crude oil tank vapour system is reduced to a pressure below the Saturated Vapour pressure of the cargo then NMVOC will evolve to restore the equilibrium balance in the system.

#### THE SIZE OR VOLUME OF THE VAPOUR PHASE WITHIN THE CARGO TANK SYSTEM

The size or volume of the gas or vapour phase in the cargo tank system (usually a common system on a crude oil tanker due to the interconnection through the Inert Gas pipeline system) is an important criterion to establish the pressure within the system. Again separate consideration should be given to the two differing types of gases to be found in the vapour phase and how volume may impact these component gases.

Saturated vapours from the crude oil liquid phase – as described above in paragraph 3.2.2. under theoretical conditions the pressure generated by saturated vapours will not be affected by a change in the volume space occupied by the vapours. However, due to the numerous specie of hydrocarbon types to be found in evolved vapour from a crude oil it has been found that a volumetric change of the vapour phase from a 2% volume to a 20% volume (V:L ratio 0.2) will impact the saturated vapour pressure of a crude oil at a constant temperature. For vapour volumes greater than 20% of the total volume the pressure behaves similar to that expected of a Saturated Vapour; namely nearly isobaric. These circumstances can be seen in Figure 3.1 below for a selection of crude oil types.

21

19	 Champion

17	 Forcados

Palanca

Pressure (psia)

15

Brent

13	 Maya

11	 Oseberg

Oman

9

Iranian Light

7

5

3

0	0.1	0.2	0.3	0.4	0.5

Vapor to Liquid Ratio

### Figure 3.1

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 01 | JULY 2024 | CR |

The change in pressure with respect to volume, for a vapour percent volume from 2% to 20%, for complexed vapour phases evolved from crude oils is due to the influence of the individual volatile hydrocarbon types and their varying proportions in both the liquid and vapour phase that separately contribute to the final saturated vapour pressure under equilibrium conditions. The ratio of concentration of the individual hydrocarbon compounds in the vapour phase is due to the Partition Coefficients for each hydrocarbon type in relation to another type. This will cause a differing distribution of hydrocarbon specie to that in the liquid phase when the vapour phase volume is smaller.

Unsaturated gases (Inert Gas) in the Vapour phase system – this type of gas behaves in a manner simulated by the Ideal Gas Law equation. Therefore any reduction in the volume occupied by this gas will cause an increase in the pressure exerted by the gas at a known temperature.

## SECTION 4 – THE METHODS AND SYSTEMS FOR THE CONTROL OF VOC

#### METHODS AND SYSTEMS FOR THE CONTROL OF VOC DURING LOADING

## BEST PRACTICES AND DESIGN

      - Manual pressure relief procedures (tank pressure control) IG risers to be left open throughout the loading operation.

      - P/V valve condition and maintenance

Type/Model

Maker:. Type:

Size:

Operating Pressure: Pressure:

Vacuum:

Maintenance Procedure

VMS

5- Months Maintenance

Before doing any maintenance, the tank must be gas-free. Use non-Spark tools only if maintenance is done while valve mounted on tank.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 01 | JULY 2024 | CR |

# 10- Months Maintenance:

# 30-Months Maintenance:

# Gaskets for hatches:

.

# Piping:

- Pressure test before discharge to check pipes and flanges condition.

      - Inert gas topping up procedures

        - When at a terminal without use of vapour return:

Gas riser mast valve should be full open. This will result in a minimum positive pressure in the cargo tanks of 100 mm WG. Maximum pressure in the cargo tanks is not applicable on condition that the loading rate does not exceed the maximum allowable rate of 11,000 m3/hr and that the flame screen in the gas riser mast is clean and well maintained.

        - When at a terminal which does use vapour return:

Minimum cargo tank pressure 100 mm WG; maximum pressure 500 mm WG. The terminal should reduce the loading rate if their vapour return rate cannot be increased, and the cargo tank pressure reaches 500 mm WG.

Vessel to use the vapour return system onboard when shore, facilities are available. Refer to VRS manual.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

Vessel equipped with …. vapours return at forward manifolds, both sides (Port & Starboard)

- Size

- Number of reducers

- Presentation flanges

- Reserve reducer stocks

#### MANUAL PRESSURE RELIEF PROCEDURES (TANK PRESSURE CONTROL)

When loading crude oils a large build up of “gas” pressure can be registered on the Inert Gas pressure gauge in cargo control room. This increase in pressure is to be lowered by release of gas either through the vessel’s Mast Riser, or through the hi-jets

As part of the pre-loading checks, the Chief Officer is to ensure that all valves on the tank’s vent system are correctly set. Tank hatches must not be opened, or ullage plugs left open during loading.

The atmospheric conditions at the time of loading are the main factors in the successful dissipation of gas at a safe distance from the gas line outlet. Hydrocarbon gases are, on average, 1.5 times heavier than air, and in still air or near calm conditions they will fall to deck level and may accumulate there.

In these conditions the Master may consider stopping loading operations, if the gas concentrations are approaching hazardous levels, until conditions improve. Similarly, if the existing atmospheric conditions cause the gas to flow towards the accommodation, and should gas be detected inside the accommodation, loading must be stopped immediately, and the Emergency Alarms sounded. The Master must ensure all precautions are taken to prevent vapours entering the accommodation.

It is essential that the accommodation is kept under positive pressure to prevent the entry of Hydrocarbon Vapours – the operation of sanitary and galley extraction fans will cause a vacuum and therefore the air conditioning system intakes must not be kept fully closed. The accommodation air conditioning system should be maintained on partial re-circulation during cargo operation.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### P/V VALVE CONDITION AND MAINTENANCE

Pressure/Vacuum valves are designed to provide protection of all cargo tanks and provide for the flow of small volumes of tank atmosphere resulting from temperature variations in the cargo tank(s) and should operate in advance of the pressure/vacuum breaker.

P/V valves are to be clearly marked with their high pressure and vacuum opening pressures. The correct maintenance of these valves is essential to the safe operation of the vessel. To ensure this, these valves are to be inspected regularly and thoroughly overhauled and decarbonised at least every six months, or more frequently if specified by the manufacturers’ instructions. In addition annual testing of each PV valve is to be carried out in the workshop every 2.5 years / dry-dock to ensure that both sides of the valve open at the specified pressure and vacuum. All such maintenance is to be recorded in the ships PMP.

Pressure/vacuum valves should be kept in good working order by regular inspection and cleaning. It is essential that venting systems are thoroughly checked to ensure that they are correctly set for the intended operation, if required, the outlets should be protected by a device to prevent the passage of flame. High velocity vents should be set in the operational position to ensure the high exit velocity of vented gas. Once operations have started, further checks should be made for any abnormalities, such as unusual noises of vapour escaping under pressure or pressure/vacuum valves lifting.

The operation of the P/V valves and/or high velocity vents should be checked using the testing facility provided by the manufacturer. Furthermore, it is imperative that an adequate check is made, visually or otherwise, to ensure that the checklift is actually operating the valve. On occasion, a seized or stiff vent has caused the checklift drive pin to shear and the ship's personnel to assume, with disastrous consequences, that the vent was operational.

Flame gauzes/screens on P/V valves, Hi-Jet type valves, vapour lines, mast risers, purge pipes, p/v breakers and on ullage ports are to be inspected every three months and replaced as necessary.

Flame screens on ballast tank and bunker tank vents must be inspected every six months and replaced, as necessary.

When volatile cargo is being loaded into tanks connected to a venting system which also serves tanks into which non-volatile cargo is to be loaded, particular attention should be paid to the setting of pressure/vacuum valves and the associated venting system, including any inert gas system, in order to prevent flammable gas entering the tanks to be loaded with non-volatile cargo.

Whenever tanks are isolated to prevent cross contamination, the likelihood of oxygen entering the tank due to it ‘breathing’ on passage should be taken into consideration and measures may need to be taken to restore the inert condition prior to discharge.

#### CONDITION OF GASKETS FOR HATCHES AND PIPING

Tanks must be gas tight at all pressures. Gaskets on access hatches and all deck openings must be in good condition to achieve this and should be examined on each occasion a lid or hatch is opened. Replace if in doubt.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

All cargo, COW, ballast and bunker pipework must be tight and not leak under pressure. To ensure this, all cargo, ballast and bunker pipework is to be pressure tested every 2.5 years / dry-dock, and must not leak under static liquid pressure at 1.5 times the maximum allowable working pressure.

On satisfactory completion of the test a suitable Deck Log entry and remark on the pressure test in the Oil Record Book Part II under letter “O” is to be made as recommended by Intertanko Guide for correct entries in the ORB. The entry required is as follows: “Deck cargo lines tested to … (insert test pressure)”. The pressure used and the date is to be stencilled on each line adjacent to the manifold on both sides of the vessel.

The following items are to be contained in the maintenance records:

        - Pressure testing of cargo, COW, ballast and bunker piping (1.5 x WP). (30 months)

        - Pressure testing of cargo and COW lines to working pressure(prior to each discharge)

        - Function testing and pressure testing of cargo valves (6 Months)

#### INERT GAS TOPPING UP PROCEDURES

The vessel should arrive at the loading berth with tanks properly inerted and at least 100mm WG positive pressure. During loading, gas from tanks will be expelled either through main mast riser or through the tank hi-jets. HI-jets are designed to open at a pressure high enough to expel vapour away from the cargo tank. This is not achieved at lower pressures and for this reason hi-jets must not be jacked open.

Prior loading it is important to ensure that all tank gas lines are open.

If for any reason, the inert gas system cannot be operated, or the oxygen content of the inert gas supply cannot be reduced to below 5% by volume, or a level required by the local port regulation, the Master must stop operations.

#### PARTIALLY FILLED TANKS

Partially filled tanks should be avoided whenever possible as sloshing will increase VOC generation.

#### LOADING SEQUENCE AND RATE

Prior to arrival at the loading port all tanks are to be fully inerted and the oxygen content of the atmosphere in the tanks reduced to below 8% by volume or the limit specified by port regulations if lower. Ample time is to be allowed for this operation to ensure that the cargo tanks are properly inerted. Immediately prior to arrival at the loading port, a check is to be made that the atmosphere in all cargo tanks contains 8% oxygen by volume or less. The inert gas plant must be operated when ballast is discharged from any cargo tanks/holds prior to loading to ensure that the space is kept fully inerted.

Once the flow has been established and into the correct tanks, the loading rate may be increased to the agreed figure.

High loading rate per tank means shorter filling time and less vapour release from the crude oil. If practicable, considering trim and bending moment/shear force, loading segregation by segregation at max rate is favourable.

High vapour space pressure reduces vapour release from the cargo during loading. A procedure where the initial slow loading takes place with the mast riser valve open followed by closing the riser valve and let the

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

P/V valves on the tanks take care of the release of excess vapour/inert gas during the subsequent loading will be beneficial.

In order to guard against the possibility of tank over pressurisation, it is essential that loading rates are kept in accordance with the tank venting arrangements. Accordingly there must be knowledge of the maximum filling rates of individual tanks, and for the whole vessel available and posted in the cargo control room, as governed by P/V safety device limitations. The closed loading system, whereby vapours are exhausted through either vent risers or high velocity vents, is always to be used where possible. (see 1.7)

The maximum loading rate described above will only be valid when all the vessels main pipelines are used for loading. The rate of loading must be adjusted to take into consideration the number of tanks open at any one time. Where all main lines are not being used for loading or the number of tanks open on each line is not sufficient the maximum rate will be reduced proportionally.

It must be noted that the maximum permissible loading rate will also be governed by the nature of the cargo to be loaded.

Another consideration for restricting the maximum rate at which a vessel may load is to prevent the excessive build up of pressure within the cargo system. This is governed by a number of factors including the diameter of the smallest section of pipe in the system, the material from which the pipe is constructed, the angle and frequency of bends etc. It is important that the maximum loading rate is not exceeded in order to avoid placing undue stress on the pipeline system, and to allow all the gas being given off by the cargo, to dissipate to below the lower explosive limit at a safe distance from the gas line outlet.

It is important that the cargo being loaded is correctly distributed. It is not permissible to load at the maximum loading rate into one single tank, or into tanks served by only one section of the gas line so that all the gas being given off by the cargo is expelled through a single outlet

The maximum permissible system loading rate for either oil or water ballast on any particular vessel will depend on the design of the cargo system and its associated parts, including the gas venting system. Where a maximum rate has been stated by a shipbuilder, this value must not be exceeded. In the absence of a stated maximum, this figure will not exceed 8.5% of the ship’s summer deadweight per hour.

#### USE OF VAPOUR RETURN MANIFOLDS WHEN SHORE FACILITIES AVAILABLE

This is the optimum way to minimise VOC release during loading as all vapour is returned to shore tank or other facility (e.g. a flare).Refer 4.1.2.

#### SHIP SPECIFIC LOADING PROCEDURES

The ship should define a target operating pressure for the cargo tanks. This pressure should be as high as safely possible and the ship should aim to maintain tanks at this level during the loading and carriage of relevant cargo.

Vessels fitted with VECS should use in all ports and terminals where available.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

Vessels not fitted with any automated equipment for VOC control and dependent on a manual valve only for pressure control must set a limit well below the P/V valve release pressure. THIS IS IN ORDER TO PREVENT A TANK OVER PRESSURISATION SITUATION OCURRING. The requirement is that cargo tanks pressure during loading should be as high as safely possible. On a manual system with standard gas release via mast riser(s) this should never be allowed to exceed 600 mmwg. If any pressure surge or unexpected increase of flow rate, the P/V valves will release the pressure if it should reach 1360 mmwg.

A Critical Operation Checklist is to be prepared and used during all loading operations, and included in this section of the VOC Manual. If the Master should experience any concerns in controlling the vapour pressure

e.g. due to a frequently changing loading rate, he should stop manual pressure control and fully open the vent valve fully.

#### VAPOUR EMISSION CONTROL SYSTEMS (VECS)

The principle behind VECS is that VOC generated in cargo tanks during loading is returned to shore terminal for processing, as opposed to being emitted to atmosphere through the mast riser.

Vapour Emission Control Systems (VECS) were introduced in 1990 as a requirement for tankers loading oil and noxious liquid substances at US terminals (USCG 46 CFR Part 39). IMO followed up with the introduction of IMO MSC.Circ.585 “STANDARDS FOR VAPOUR EMISSION CONTROL SYSTEMS” in 1992. International

regulations requiring vapour emission control was introduced through Reg.15 of Annex VI to MARPOL 73/78 as adopted in 1997, although it is only required for ships loading cargo at terminals where IMO have been informed that VECS is mandatory.

Since 1990, most crude tankers have installed a VECS system in compliance with USCG regulations. The regulations cover the technical installation (vapour recovery piping & manifold, vapour pressure sensors and alarms, level gauging, high level and independent overflow alarms) as well as operational restrictions and training. The operational restrictions are found in a mandatory VECS manual which also includes maximum allowable loading rates. The maximum allowable loading rate is limited by one of the following:

- The pressure drop in the VECS system from cargo tank to vapour manifold (not to exceed 80% of the P/V-valve setting).

- The maximum pressure relief flow capacity of the P/V-valve for each cargo tank.

- The maximum vacuum relief flow capacity of the P/V-valve for each cargo tank (assuming loading stopped while terminal vacuum fans are still running).

- The time between activation of overfill alarm to relevant cargo tank is full (min. 1 minute).

The calculations are to be based on maximum cargo vapour/air densities as well as maximum cargo vapour growth rates, which again may limit the cargoes that can be loaded with VECS.

Further, the calculations are to be carried out both for single tank and multiple tank loading scenarios.

The USCG regulations also contain additional requirements to vapour balancing, i.e. for tankers involved in lightering operations. These include operational requirements as well as technical requirements for an in-line detonation arrestor, oxygen sensors with alarms and possibly means to prevent hazards from electrostatic charges.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

For ships provided with a VECS system as per IMO or USCG regulations, the control of NMVOC emissions will be through returning NMVOC to shore terminal in accordance with the procedures found in the onboard VECS manual.

The maximum allowable loading rates and corresponding maximum vapour/air densities and vapour growth rates should be specified in this manual.

#### DETAILS OF ONBOARD VAPOUR EMISSION CONTROL SYSTEM (VECS)

Enter details of VECS system fitted for reducing VOC emissions. Include a description of the system, method of operation and equipment parameters applying to control VOC emissions during cargo loading.

The control of VOC emissions when using VECS is as per approved VECS Manual. The main details of the system are as follows

| Description of the system |  |
| --- | --- |
| Method of operation |  |
| Equipment parameters applying to control VOC emissions during cargo loading. |  |
| Maximum allowable loading rates |  |
| Maximum vapour growth / density |  |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### 4.1.5 INCREASED PRESSURE RELIEF SETTINGS (APPLICABLE ALSO FOR SEA PASSAGE)

Note: before increasing pressure relief set points of the P/V breaker and the pressure/vacuum valves, extensive analysis of strength need to be considered. The pressure set points required by SOLAS are not to be exceeded in any case.

As described in sections 2 and 3, as long as the tank pressure is maintained above the Saturated Vapour Pressure of the cargo, then equilibrium is obtained between the liquid and vapour phase of the cargo and no further VOC will evolve from the cargo. This means that if the pressure/vacuum relief settings are increased to, e.g., 2,100 mmWG, VOC will not evolve from a cargo as long as the Saturated Vapour Pressure of said cargo is below the pressure relief setting.

As indicated earlier, the maximum design pressure of a cargo tank is at least 2,500 mmWG and, as such, increasing the settings of the pressure/vacuum devices up to, e.g., 2,100 mmWG, should not require additional strengthening. It will however require adjustment/replacement of P/V valves. Note that for some P/V valves designs, the pressure after initial opening increases, and this has to be taken into account if an owner intends to increase the setting of P/V valves.

Needless to say it will also require replacement/modifications to the P/V breaker, as well as water loops serving the inert gas deck water seal, as well as settings of pressure sensors and alarms in the inert gas and VECS system. It is of course also essential that onboard operational procedures in terms of manual pressure release have to be adjusted.

One additional benefit is that increasing the pressure/vacuum relief settings will increase the acceptable loading rate during VECS.

Although the primary benefit of increasing set pressure will occur during voyage. It will also have an effect related to loading, as the increased set pressure will limit the existing vapour in the cargo tanks, i.e. the vapour generated during the previous discharge and Crude Oil Washing.

For ships that have been provided with increased pressure relief settings, the VOC emissions will be controlled when the saturated vapour pressure of the crude oil is below that of the pressure relief valve settings.

It is important that terminals and cargo surveyors acknowledge that if ships with higher pressure settings are required to de-pressurize prior to cargo handling operations, this will limit the ships’ ability to control VOC emissions.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### METHODS AND SYSTEMS FOR THE CONTROL OF VOC DURING TRANSIT

#### BEST PRACTICES/DESIGN

      - P/V VALVE CONDITION AND MAINTENANCE

Pressure/Vacuum valves are designed to provide protection of all cargo tanks and provide for the flow of small volumes of tank atmosphere resulting from temperature variations in the cargo tank(s) and should operate in advance of the pressure/vacuum breaker.

P/V valves are to be clearly marked with their high pressure and vacuum opening pressures. The correct maintenance of these valves is essential to the safe operation of the vessel. To ensure this, these valves are to be inspected regularly and thoroughly overhauled and decarbonised at least every six months, or more frequently if specified by the manufacturers’ instructions. In addition, testing of each PV valve is to be carried out every 2.5 years / dry-dock in the workshop to ensure that both sides of the valve open at the specified pressure and vacuum. All such maintenance is to be recorded in the ships PMP.

Pressure/vacuum valves should be kept in good working order by regular inspection and cleaning. It is essential that venting systems are thoroughly checked to ensure that they are correctly set for the intended operation, if required, the outlets should be protected by a device to prevent the passage of flame. High velocity vents should be set in the operational position to ensure the high exit velocity of vented gas. Once operations have started, further checks should be made for any abnormalities, such as unusual noises of vapour escaping under pressure or pressure/vacuum valves lifting.

The operation of the P/V valves and/or high velocity vents should be checked using the testing facility provided by the manufacturer. Furthermore, it is imperative that an adequate check is made, visually or otherwise, to ensure that the checklift is actually operating the valve. On occasion, a seized or stiff vent has caused the checklift drive pin to shear and the ship's personnel to assume, with disastrous consequences, that the vent was operational.

Flame gauzes/screens on P/V valves, Hi-Jet type valves, vapour lines, mast risers, purge pipes, p/v breakers and on ullage ports are to be inspected every three months and replaced as necessary.

Flame screens on ballast tank and bunker tank vents must be inspected every six months and replaced, as necessary.

When volatile cargo is being loaded into tanks connected to a venting system which also serves tanks into which non-volatile cargo is to be loaded, particular attention should be paid to the setting of pressure/vacuum valves and the associated venting system, including any inert gas system, in order to prevent flammable gas entering the tanks to be loaded with non-volatile cargo.

Whenever tanks are isolated to prevent cross contamination, the likelihood of oxygen entering the tank due to it ‘breathing’ on passage should be taken into consideration and measures may need to be taken to restore the inert condition prior to discharge.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

#### CONDITION OF GASKETS FOR HATCHES AND PIPING

It is essential that all cargo, COW, ballast and bunker pipework remains in good condition. To ensure this, all cargo, ballast and bunker pipework is to be pressure tested every 2.5 years / dry-dock, and must not leak under static liquid pressure at 1.5 times the maximum allowable working pressure.

On satisfactory completion of the test a suitable Deck Log entry is to be made and remark on the pressure test in the Oil Record Book Part II under letter “O” as recommended by Intertanko Guide for correct entries in the ORB. The entry required is as follows: “Deck cargo lines tested to … (insert test pressure)”. The pressure used and the date is to be stencilled on each line adjacent to the manifold on both sides of the vessel.

The following items are to be contained in the maintenance records:

        - Pressure testing of cargo, COW, ballast and bunker piping (1.5 x WP). (30 months)

        - Pressure testing of cargo and COW lines to working pressure (prior to each discharge)

        - Function testing and pressure testing of cargo valves (6 Months)

#### INERT GAS TOPPING UP PROCEDURES

A positive pressure of inert gas should be maintained in the ullage space at all times during the loaded passage in order to prevent the possible ingress of air. If the pressure falls below the low-pressure alarm level, it will be necessary to start the inert gas plant to restore an adequate pressure in the system.

The most important issue is to avoid overcapacity when inerting (i.e. use IG generator if available and not flue gas system) and maintain inert gas pressure below P/V-valve setting

Loss of pressure is normally associated with leakages from tank openings and falling air and sea temperatures. In these cases, it is all the more important to ensure that the tanks are gas tight. Gas leaks are usually easily detected by their noise and every effort must be made to eliminate leaks at tank hatches, ullage lids, tank cleaning machine openings, valves, etc. Leaks which cannot be eliminated should be marked and recorded for sealing during the next ballast passage or at another suitable opportunity.

Certain oil products, principally aviation turbine kerosenes and diesel oil, can absorb oxygen during the refining and storage process. This oxygen can later be liberated into an oxygen deficient atmosphere such as the ullage space of an inerted cargo tank. Although the recorded incidence of oxygen liberation is low, cargo tank oxygen levels should be monitored so that any necessary precautionary measures can be taken prior to the commencement of discharge

#### PRECAUTIONS FOR HIGH H2S CARGOES

Whilst the vessel is on passage, a careful watch on tank pressure must be maintained throughout. A log is to be maintained of tank pressure(s), air temperature, atmospheric pressure and weather. This will assist in identifying pressure characteristics of cargo.

If cargo or vapour leakage is identified or suspected, the cargo area is to be evacuated immediately and the Officer of the Watch (OOW) informed. Deck apertures, where leakage may occur (tank lids, purge pipes, valve glands etc) should be monitored on a regular basis.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

During the heat of the day, pressure will rise as the tank’s atmosphere heats up. Pressure may be allowed to rise to 90% of the relief valve setting before venting takes place. There may be a reluctance to hold onto the gas and a tendency to relieve the pressure at too low a level. However, holding on to gas pressure will, on most occasions, ensure that the introduction of inert gas later in the day, as the atmosphere cools, will not be required. If venting is necessary then it is preferable that it is carried out in daylight, during scheduled breaks in the normal working day. Cooling of deck by water to control pressure should be considered.

Should the pressure rise to greater than the 90% value, and therefore require venting, the procedures as per the Tanker manual must be adopted.

#### COW PROCEDURES DURING DISCHARGE (CLOSED CYCLE)

Any vessel equipped with an approved C.O.W. system is required to have on board an approved “Crude Oil Washing Operations and Equipment Manual”. This manual must always be referred to

prior to C.O.W. operations. The procedures and checklists contained in the manual must be strictly adhered to.

Only fixed tank washing, pipework and machines may be used for crude oil washing. The oxygen content of the tank atmosphere must not exceed 8% by volume. Oxygen content is to be checked at points 1 metre below the deck and at the middle level of the ullage space in accordance with ISGOTT. (ISGOTT Ref. 7.1.6.9). A record of oxygen content reading for the various levels MUST be maintained.

Under no circumstances may Crude Oil Washing operations be commenced without operational inert gas equipment.

A ‘closed cycle’ is essential. All openings in the cargo tanks must be closed.

Prior to crude oil washing the blanking of the tank washing heater line is to be checked. These blanks may consist of spectacle blanks or a removable ‘spool’ piece

The pressure/vacuum valve is to be set with the by-pass valve closed to assist in keeping the positive pressure inside the tank being cleaned.

- When the rate of pressure drop becomes constant (after the initial rapid pressure drop) then the gas release should be stopped and the valve closed.

- Monitor the Tank Gas Pressure after completion of the controlled release in order to check the final pressure obtained within the Vapour/Inert Gas system.

### Advice Notes

- A review of figure 4.1 show a clear change in the rate of pressure drop during the release period. If the gas release continues after this point then the pressure in the Inert Gas system will be quickly restored to the pressure associated with the point where the rate of pressure drop changes.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

### If there is a Straight line drop of pressure observed and no inflection observed by 800 mm WG, then close the release valve anyway.

- By reference to the ISGOTT Publication, all safety measures should be taken to minimise the hazards associated with vented gases from the vessel’s cargo tank system.

#### REDUCTION OF EXCESS NMVOC PRESSURE AND TANK ABSORPTION (VENTURI SYSTEM)

    - INCREASED PRESSURE RELIEF SETTINGS

See 4.1.5

#### VAPOUR RECOVERY SYSTEMS

See 4.1.6

#### TARGET PRESSURE FOR CARGO TANKS

The ship should define a target operating pressure for the cargo tanks. This pressure should be as high as safely possible and the ship should aim to maintain tanks at this level during the loading and carriage of relevant cargo.

#### CARGO TEMPERATURE

A description of cargo heating requirements to maintain the vapour/liquid equilibrium should be included.

The ship should define a target operating pressure for the cargo tanks. This pressure should be as high as safely possible and the ship should aim to maintain tanks at this level during the loading and carriage of relevant cargo.

| Target Pressure Range |  |
| --- | --- |
| Cargo Temperature |  |

  - Methods and systems for the control of VOC during Discharging/Ballasting

Emissions of VOC during ballasting had relevance when tankers took ballast into cargo tanks for stability and longitudinal strength reasons and thus displaced VOC from cargo tanks being ballasted. After the implementation of requirements to segregated ballast tanks and, of course, double hull, VOC releases during discharge and ballasting are no longer an issue. During discharging of cargo tanks, it is important that pressure monitoring is exercised in order to avoid excessive supply of inert gas to cargo tanks.

  - Control of VOC during cargo tank purging to reduce toxic gas or gas freeing for entry

See 1.8

## SECTION 5.0 - THE MONITORING AND CONTROL OF NMVOC RELEASES

Record keeping is necessary in order to document compliance with the requirements of the management plan and, potentially, the extent of release of gases from the crude oil cargo tanks.

Recording is only required if manual pressure release is carried out.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

The form of record keeping is dependent upon the specific form of method used to minimise the emission of NMVOC from the crude oil cargo. It will also be dependent upon the operation being performed by the ship necessitating the release of NMVOC; namely loading, during the carriage or as a result of a crude oil washing (COW) operation.

As a general example of the type and scope of record keeping to be undertaken on board the crude oil tanker, the methodology of the manual VOCON procedure is used. The appropriate record keeping is as follows::

- The target or minimum pressure within the tank gas/vapour system for the specific voyage

- A record of the time and pressure within the tank gas/vapour system before the release takes place.

- A record of the time and pressure within the gas/vapour system after the release has been completed.

- For those ships operating with manual NMVOC control by a vapour release procedure as per the requirements in this manual, the released volume of gas/vapour can be estimated by use of the pressure change (opening to closing pressures) relationship to the total gas/vapour volume in the cargo tank vapour system, times for the start and stop of the release.

- A form (spreadsheet) for calculating emissions is included with this Plan.

Enter form for VOC Calculations

## SECTION 6.0 - TRAINING PROGRAMME

A training programme is to be developed for the persons intended to assume overall charge of the VOC management onboard each ship. The programme is to include the following:

- An introduction to the purpose of VOC emission control:

  - Volatile organic compounds (VOCs) may be toxic, and when they evaporate into the air where they can react with Nitrogen Oxides (NOx) in sunlight and split apart oxygen molecules in air and thereby form ground-level ozone, commonly referred to as smog. The layer of brown haze it produces is not just an eyesore, but also is a source of serious illnesses. Ozone is extremely irritating to the airways and the lungs, causing serious damage to the delicate cells lining the airways. It contributes to decreased lung function, increased respiratory symptoms and illnesses. Tropospheric Ozone is also deemed to be one of the identified Greenhouse gases.

  - Regulation 15 of Annex VI to MARPOL 73/78

An introduction to the principles of VOC emission control:

- VOC generation systems in crude oil (ref. section 3)

- Crude oil tanker pressure control/release systems (ref. section 2) General VOC emission control options

- Methods and system for the control of VOC emissions (ref. section 4) Ship specific VOC emission control options

- Ship specific methods and system for the control of VOC emissions (ref. section 4) Monitoring and recording of VOC release

- Methods for monitoring and recording of VOC emissions (ref. section 5) Hazards and Safety related to VOC emission control

- The hull and its pressure limitations (ref. section 1).

- Personnel safety hazards related to exposure to crude oil vapour.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

### V.Ships Training Programme

- All Chief Officers on crude oil tankers will undergo VOC management training in training centres prior joining vessel. A certificate will be issued.

- Senior Second Officers will also undergo training as above.

- Follow up training will be carried out by Onboard Training Superintendents when identified as a training need.

- A copy of the training presentation will be carried onboard all crude tankers.

- All crude tankers will carry title No. 0196 - Volatile organic compound in OTG library.

## SECTION 7.0 - DESIGNATED PERSON

A person shall be designated to assume overall charge of the VOC management onboard the ship. The designated person should preferably have:

- At least one year’s experience on crude oil tankers where his duties have included all cargo handling operations relevant to VOC management. In the absence of experience with VOC management, he or she shall have completed a training programme in VOC management in as specified in this manual.

- Have participated at least twice in cargo loading operations, Crude Oil Washing Operations and transit where VOC management procedures have been applied, one of which shall be on the particular ship or a similar ship in all relevant aspects, for which he or she is to undertake the responsibility of VOC management.

- Be fully knowledgeable of the contents of this manual.

THE CHIEF OFFICER IS THE DESIGNATED PERSON ON THIS VESSEL.

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

## SECTION 8.0 - LIST OF DRAWINGS/ADDITIONAL INFORMATION

  - The following drawings are included as appendices to the management plan: General Arrangement drawing (All Ships)

    - Tank plan	(All Ships)

    - Schematic drawing(s) of the Cargo tank venting system (All Ships)

    - Schematic drawing of the inert gas system	(All Ships)

    - Schematic drawing of the vapour emission control systems (only if applicable)

    - Schematic drawing(s) Vapour Recovery System or other VOC control systems. (only if applicable)

    - Details of pressure vacuum relief devices including settings and capacities.

Cross References to ship specific manuals or documents such as:

    - Vapour Emission Control System manual (only if applicable)

    - Vapour Recovery System manual (only if applicable)

    - Other VOC control system manual (only if applicable)

    - Inert Gas manual (All Ships)

    - COW manual  (All Ships)

    - The SMS (All Ships)

    - The Tanker Manual  (All Ships)

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

### Appendix 3 - Voyage NMVOC Emission Calculation Spreadsheet

| Column ID- A | B | C | D | E | F | G | H | I | J | K | L | M | N | O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Voyage | NMVOC | Emission | Calculation | Spreadsheet |  |  |  |  |  |  |
| Date | Time at start of release | Time at stop of release | Duration of release | Tank pressure at start of release (mm wg) | Tank pressure at stop of release (mm wg) | Mean pressure during release (Meters Wg) | Diameter of release pipeline from tank (meters) | Length of release pipeline from tank (meters) | Mean velocity of release (m3) | Total vapour volume of release (m3) | % of NMVOC | NMVOC vapour release | NMVOC Equiv liquid vapour release | NMVOC equiv. weight release (Tonnes) |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

### Voyage NMVOC Emission Calculation Spreadsheet

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |

| Document Number | Version Number | Date | Approved By |
| --- | --- | --- | --- |
|  | 01 | JULY 2024 | CR |