Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  |  | • |  |
| Advise terminal - order emergency release of cargo gear | • |  |  | • |  |
| Stop operations - activate emergency shut down |  |  | • | • |  |
| Standby at fore and aft stations to tend moorings | • | • | • |  |  |
| Initiate steps to re-establish mooring lines to secure the vessel | • |  | • | • |  |
| Call for tugs/pilot to assist | • |  |  |  |  |
| If cargo has spilled initiate containment / clean up measures |  |  | • | • |  |
| Re-access the weather conditions and mooring requirements | • |  |  | • |  |
| Monitor other vessel movements in the vicinity | • |  |  | • |  |
| Start fire pumps |  | • |  |  |  |
| Isolate non-essential equipment |  | • | • |  |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Advise engine room - order emergency start of main engine | • | • |  |  |  |
| If dangerous cargo, review MSDS |  |  |  |  |  |
| Assure accommodation ventilation is in full recirculation |  | • |  |  |  |
| Muster crew - conduct tally of personnel. Ensure staff provide emergency response equipment | • |  | • | • |  |
| Prohibit all smoking | • | • | • | • |  |
| Advise company by emergency phone followed by message and advise charterers | • |  |  |  |  |
| Advise shore emergency services of situation, dangers from cargo, if toxic fumes expected | • |  |  |  | • |
| Consider to Alert vessels in vicinity | • |  |  |  |  |
| Maintain accommodation and engine room gas free |  | • | • |  |  |
| Use thruster(s) to assist steering - if fitted | • | • |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Display N.U.C signals | • |  |  | • |  |
| Observe movement of gas cloud | • |  | • |  |  |
| Do not direct water jet on any liquid pool |  |  | • | • |  |
| If required, protect deck steel by water spray from possible brittle fracture (as applicable) | • |  | • | • |  |
| Use hose water spray to dilute gas cloud and direct it away from ignition sources and personnel (as applicable) | • |  | • | • |  |
| Use portable gas detection equipment to monitor gas accumulation on deck |  |  | • | • |  |
| Do not switch on radars if gases present on deck | • |  | • | • |  |
| Advise Company and update situation | • |  |  |  |  |
| Preserve VDR / SVDR records if not automatically protected | • |  |  |  |  |