Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master | • |  |  |  |  |
| Report Casualty | • |  |  | • |  |
| Alert vessel’s crew – account for and ensure their safety |  |  |  | • |  |
| If vessel is unable to maneuver due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Implement VRP / SMPEP / SOPEP as applicable | • |  | • |  |  |
| Broadcast distress message if vessel in imminent danger. | • |  |  |  |  |
| Determine location and extend of casualty |  | • | • |  |  |
| Initiate damage control and fire-fighting measures |  | • | • |  |  |
| Isolate cargo and fuel pipeline |  | • | • |  |  |

| Determine vessel’s structural integrity (damage stability and hull stress) | • | • | • |  | • |
| --- | --- | --- | --- | --- | --- |
| Report casualty to P&I, owners, etc. | • |  |  |  | • |
| If fire on deck, consider transferring cargo internally | • |  | • |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Determine need for commercial salvage assistance or lightering assistance | • |  |  |  | • |
| Ascertain cause of casualty | • | • | • |  | • |
| If fire in accommodation, engage with fire team |  |  | • |  |  |
| If uncontrolled fire in Engine Room, make head counts of all crew / passenger and activate Foam / CO2 with master’s order. | • | • | • |  |  |
| If uncontrolled fire in Pump Room, make head counts of all crew / passenger and activate Foam / CO2 with master’s order. | • | • | • |  |  |
| Contact Class ERS for damage stability assessment and advise on Cargo / Ballast transfer. | • |  | • |  |  |
| Transfer cargo and / or ballast as required. | • |  | • |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |