Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound alarm - General | • |  | • | • |  |
| If in port - advise terminal | • |  | • | • |  |
| Stop cargo / bunker operations - activate emergency shut down |  |  | • | • |  |
| Start fire pumps |  | • |  |  |  |
| Assure accommodation ventilation is in full recirculation |  | • |  |  |  |
| Shut all cargo system valves |  |  | • | • |  |
| Operate deck water spray system |  |  | • |  |  |
| Muster crew - conduct tally of personnel. (Ensure that all staff wear the emergency escape sets) | • |  | • | • |  |
| If at sea - consider alteration of course or speed to carry vapour cloud away from ignition sources or accommodation / personnel, is gas toxic | • |  |  |  |  |
| Advise Company by emergency phone followed by message and advise charterers | • |  |  |  |  |
| Respond as per VRP SOPEP/SMPEP as applicable | • |  |  |  | • |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| If at sea - have ready at radio details of ship's position and any other information for a distress / urgency safety message | • |  |  |  |  |
| Prohibit all smoking | • | • | • | • |  |
| Advise engine room and emergency parties of situation | • | • | • | • |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |