Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • | • |  |
| Muster crew and initiate damage control measures | • | • | • | • |  |
| Advise engine room of situation, order a slowdown/stop main engines | • | • |  | • |  |
| Determine if any causalities suffered and render first aid, if necessary | • | • | • | • |  |
| Assess the immediate danger of sinking and capsizing | • | • | • | • |  |
| Check for oil / hazardous substances pollution and possibility of one | • | • | • | • |  |
| Monitor latest weather forecast and assess impact on present situation | • |  |  | • |  |
| Consider changes in course and speed to reduce level of water ingress | • | • | • |  |  |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |

| Attempt to remove flooding effect by pumping out affected space if feasible | • | • | • |  |  |
| --- | --- | --- | --- | --- | --- |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Have ready at position of radio, details of ship's position and any other relevant information | • |  | • | • |  |
| Determine extent of flooding, exercising caution as some spaces my be under pressure due to flooding | • | • | • |  |  |
| Advise nearest coast state using "standard reporting format" | • |  |  | • |  |
| Advise Company by emergency phone followed by message and advise charterers | • |  |  |  |  |
| Determine possible final trim/list and stability due to flooding | • |  | • |  | • |
| Consider ballasting / de- ballasting to reduce level of flooding | • |  | • |  |  |
| Advise company using "standard format" (updated situation | • |  |  |  |  |
| Notify owners, H&M, etc. |  |  |  |  | • |
| Call Master | • | • |  |  |  |
| Close all water tight doors | • | • |  |  |  |
| Maintain log/records of events and decisions | • | • |  |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| In case of flood in |  |  |  |  |  |
| Engine Room – Checklist Main Engine Failure, complete as appropriate | • | • |  |  |  |
| Steering Gear Room – Checklist Steering Failure, complete as appropriate | • | • |  |  |  |
| Generator Compartment – Checklist Total Electrical Power Failure, complete as appropriate | • | • |  |  |  |