Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • |  |  |
| If in port - advise terminal | • |  | • |  |  |
| Stop air conditioning / ventilation system |  | • |  |  |  |
| If at sea - consider alteration of course or speed to facilitate firefighting and dispersion of any un-ignited vapor | • |  |  |  |  |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Muster crew - conduct tally of personnel. Ensure staff provide emergency equipment | • |  | • | • |  |
| Advise fire parties of situation / location | • | • | • |  |  |
| If at sea - advise nearest coast state and alert vessels in vicinity | • |  |  |  |  |
| Advise Company by emergency phone followed by message and advise charterers | • |  |  |  |  |
| Coordinate Fire Fighting, Lightering, & Salvage Services |  |  |  |  | • |
| Notify owners, P&I Club, etc. as per contingency plan |  |  |  |  | • |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master | • |  |  |  |  |
| Assess proximity of navigational hazards, including traffic, and manoeuvre the ship as appropriate | • |  |  |  |  |
| Broadcast Urgency or Distress message, as appropriate | • |  |  |  |  |
| Maintain log/records of events and decisions | • |  |  |  |  |
| Preserve VDR / SVDR records if not automatically protected | • |  |  |  |  |
| In case of fire - |  |  |  |  |  |
| Engine Room – Checklist Main Engine Failure, complete as appropriate | • | • |  |  |  |
| Steering Gear Room – Checklist Steering Failure, complete as appropriate | • | • |  |  |  |
| Generator Compartment – Checklist Total Electrical Power Failure, complete as appropriate | • | • |  |  |  |

Accommodation Spaces / Galley

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Refer to Accommodation Plans for understanding of the layout | • |  |  |  |  |
| Isolate Electrical Circuits | • |  |  |  |  |
| Conduct Boundary Cooling from all Six Sides |  | • | • |  |  |

| In Galley Fires, preferred means if extinguishing would be Fire Blanket /CO2 / Dry powder to avoid Boil over |  |  | • |  |  |
| --- | --- | --- | --- | --- | --- |

Engine Room

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Activate Emergency Fuel trips etc. |  | • |  |  |  |
| Confirm Emergency Power & Fire pump have been started |  | • |  |  |  |
| Conduct Boundary Cooling from all Six Sides |  |  | • |  |  |
| If the Fire is too large, Evacuate Engine room and use fixed CO2 / Foam |  | • | • | • |  |

Cargo fire on deck / cargo tank

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Stop any Cargo Operations. Activate Emergency Shut down, shut all Cargo valves |  |  | • | • |  |
| Refer to MSDS Sheets | • |  |  | • |  |
| Activate deck and bridge and front water sprays, foam systems, Inert Gas system, Steam etc. as applicable |  |  | • | • |  |

Pump Room

| Activate Emergency trips for all cargo pumps |  |  |  | • |  |
| --- | --- | --- | --- | --- | --- |
| Confirm all Cargo System Valves shut to isolate the pump room |  |  | • | • |  |
| Conduct Boundary Cooling |  |  | • |  |  |
| If the Fire is too large, evacuate pump room and use fixed CO2 / Foam |  | • | • | • |  |