Note:

Hardcopy to be maintained on bridge.

# 1	Main engine failure

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Notify Office Notify interested parties if delay anticipated. | • |  |  | • | • |
| Master Informed | • |  |  |  |  |
| Check for other vessels in the vicinity. Check for navigational hazards in vicinity. | • |  |  |  |  |
| Change over from bridge control to Engine Control Room (ECR) and check for ME response. | • | • |  |  |  |
| Change over from ECR to local control station if main engine is not responding from ECR |  | • |  |  |  |
| Fix position of vessel. | • |  |  |  |  |
| Broadcast SAFETY or URGENCY message, if appropriate. | • |  |  |  |  |

| Rudder & Bow Thruster used to best navigational advantage | • |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prepare for anchoring if in shallow water | • |  | • |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| If vessel is unable to maneuver due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Exhibit NUC lights/signals/shapes. Change the navigation status on the AIS. Inform VTS or port authority, as appropriate. | • |  |  |  |  |
| Assess weather, sea conditions, currents and drift. Forecasted weather/sea conditions. (Refer to publication “Peril at sea and salvage” for guidance on set and drift calculation) | • |  |  |  | • |
| Ascertain cause of casualty | • | • |  |  |  |
| Report casualty | • |  |  |  | • |
| Provide support as needed |  |  |  |  | • |
| Maintain log/record of events and decision | • | • |  |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |