Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Master and Chief Engineer Informed | • |  |  |  |  |
| Alternative Steering Engaged and Disengage auto pilot | • | • |  | • |  |
| Sound signal made when appropriate | • |  |  | • |  |
| Reduce Speed and Manoeuvre as appropriate / stop engines(s) | • |  |  | • |  |
| If vessel is unable to manoeuver due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Warning broadcast to other ships and broadcast Safety or Urgency message as appropriate | • |  |  | • |  |
| Notify Office | • |  |  |  |  |
| Ascertain cause of casualty | • | • |  |  |  |

| Report casualty | • |  |  |  | • |
| --- | --- | --- | --- | --- | --- |
| Provide support as needed |  |  |  |  | • |
| Prepare for anchoring if water depth and conditions are appropriate | • |  | • |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Check for navigational hazards | • |  |  |  |  |
| Check positions of vessel in the vicinity | • |  |  |  |  |
| Maintain log/record of events and decision | • |  |  |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |