Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Master Informed |  |  |  | • |  |
| Change over to manual steering and steer by magnetic compass. Magnetic compass or other alternative means used | • |  |  | • |  |
| Switch radars to ‘head up’ display and commence manual radar plotting. | • |  |  | • |  |
| If determining positions by radar, use cross-point of 3 or 4 ranges rather than bearings. | • |  |  | • |  |
| Make appropriate entries in the log book recording actions taken. | • |  |  | • |  |
| Electrical Officer Notified | • | • |  | • |  |
| Effect on navigational instruments considered | • |  |  | • |  |
| Preserve VDR or S-VDR records if not automatically protected | • |  |  |  |  |

| Notify Office | • |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ascertain cause of casualty | • | • |  |  |  |
| Report casualty | • |  |  |  | • |
| Provide support as needed |  |  |  |  | • |