Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Position & bearing of distress noted |  |  |  | • |  |
| Master Advised |  |  |  | • |  |
| Distress Message Re- transmitted | • |  |  | • |  |
| Volume III, Mobile Facilities, (IAMSAR) consulted | • |  |  | • |  |
| Communication established between surface unities and SAR aircraft on 2182 KHZ and / or VHF Channel 16 and maintain radio watch | • |  |  |  |  |
| Position, course and speeds of other assisting units plotted | • |  |  |  |  |
| Look-Outs Posted |  |  | • |  |  |
| Radar made available for locating survival craft transponder signal and AIS | • |  |  |  |  |
| Rescue boat/Fast rescue boat deployed |  |  | • |  |  |

| Report casualty | • |  |  |  | • |
| --- | --- | --- | --- | --- | --- |
| Preserve VDR or S-VDR records if not automatically protected | • |  |  |  |  |
| Provide support as needed |  |  |  |  | • |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Records contents of distress alert and/or message | • |  |  |  |  |
| Monitor the distress situation | • |  |  |  |  |
| Prepare for recovery of persons from the water Action required will be in accordance with the ship specific plan for recovery of persons from water | • |  |  |  |  |
| Maintain log/records of events and decisions | • |  |  |  |  |