Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master | • |  |  |  |  |
| Report casualty | • |  |  | • |  |
| Alert vessel’s crew and other vessels in vicinity; and Broadcast Urgency or Distress message, as appropriate, Inform VTS or port authority, as appropriate | • |  |  | • |  |
| Ascertain vessel’s position | • |  |  | • |  |
| Display ‘Aground’ signal. Change the navigation status on the AIS | • |  |  | • |  |
| Implement VRP / SMPEP / SOPEP as applicable and conduct damage control procedure | • |  |  |  | • |
| Account for and ensure the safety of the vessel’s crew | • |  | • |  |  |
| Initiate oil spill removal and verify containment, if applicable | • |  | • |  |  |

| Isolate pipeline valves |  |  | • |  |  |
| --- | --- | --- | --- | --- | --- |
| Sound internal spaces and initiate structural damage survey |  |  | • | • |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Take soundings around the vessel and asses the nature of sea bed |  | • | • |  |  |
| Report casualty to Co. Office | • |  |  |  |  |
| Determine location of “grounding point” | • | • | • |  |  |
| Assess vessel stability condition and calculate for potential risk eg. loss of buoyancy, overstress, excessive list, capsized, cargo shifting. |  |  | • |  |  |
| Contact Class ERS for damage stability assessment and advise on Cargo / Ballast transfer. | • |  | • |  |  |
| Transfer cargo internally and/or consider lightering | • |  | • |  |  |
| Monitor weather, sea conditions, tidal direction and height effects on vessel | • |  | • | • |  |
| Determine need for commercial salvage assistance | • |  |  |  | • |
| Coordinate re-floating with authorities | • |  |  |  | • |
| Ascertain cause of casualty | • | • | • |  |  |
| Preserve VDR or S-VDR records if not automatically protected and preserve ECDIC records | • |  |  |  |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general emergency alarm | • |  |  |  |  |
| Switch to high cooling water |  | • |  |  |  |
| Manoeuvre as appropriate/stop engine | • |  |  |  |  |
| Consider use of anchor | • |  |  |  |  |
| Muster crew for damage control | • |  |  |  |  |
| Determine location of deep water on relation to the ship | • |  |  |  |  |
| Consider taking additional ballast to prevent unwanted movement and damage | • |  | • |  |  |
| Maintain log/records of event and decision | • |  |  |  |  |