Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Report Casualty | • |  |  | • |  |
| Alert vessel’s crew |  |  |  | • |  |
| Implement VRP / SOPEP / SMPEP as applicable | • |  |  |  | • |
| Verify vessel’s position |  |  |  | • |  |
| Alert other vessel in the area |  |  |  | • |  |
| Establish and maintain communications with appropriate maritime government agency | • |  |  |  |  |
| Determine if ship’s crew is capable to repair source of casualty |  | • |  | • |  |
| Determine the estimated time of repair completion |  | • |  | • |  |
| While running the emergency generator a continuous watch should be maintained, for closely monitoring the performance parameters and the general condition. | • | • |  |  |  |
| Determine need for commercial salvage assistance | • | • |  |  |  |
| Prepare ship’s personnel to initiate oil spill mitigation and removal action as warranted |  |  | • |  |  |

| Determine cause of casualty |  | • |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Preserve VDR or S-VDR records if not automatically protected | • |  |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| DOTCOM laptop on bridge | • |  |  | • |  |
| Call master | • |  |  |  |  |
| Take immediate action to keep ship away from danger | • |  |  |  |  |
| Contact E/R, duty Engineer | • |  |  |  |  |
| Select emergency power supplies for bridge and navigational equipment | • |  |  |  |  |
| Check for navigational hazards | • |  |  |  |  |
| Not Under Command (NUC) lights, shapes and sound signals as appropriate, modify AIS status | • |  |  |  |  |
| Prepare for anchoring if water depth and condition appropriate | • |  | • |  |  |
| Maintain log of events and decision | • |  |  |  |  |