Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • |  |  |
| If in port – advise terminal and notify local and national authorities | • |  | • |  |  |
| Initiate medical assistance onboard and from shore if required | • | • |  | • |  |
| Stop any cargo operations – and shut tank valves | • |  | • |  |  |
| Fire pumps in readiness |  | • |  |  |  |
| Stop air conditioning / ventilation |  | • |  |  |  |
| At night – switch on all deck lighting |  |  |  | • |  |
| Muster crew – conduct tally of personnel (Ensure that all staff wear the emergency escape sets) | • |  | • | • |  |
| Identify the chemicals involved and determine the degree of risk. | • |  | • | • |  |
| Consider the need for all personnel to evacuate the vessel | • |  | • | • |  |
| Activate water sprays |  | • | • | • |  |
| Shut all cargo system valves |  | • | • |  |  |

| Operate deck water spray system |  | • | • |  |  |
| --- | --- | --- | --- | --- | --- |
| Prohibit all smoking |  | • | • |  |  |
| Advise engine room and emergency parties of situation | • |  |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| If at sea – advise nearest coast state and alert vessels in vicinity | • |  |  |  |  |
| Commence notifications | • |  |  |  | • |
| Refer to MSDS emergency procedures | • |  | • |  |  |
| If at sea – consider alteration of course of speed to carry vapour cloud away from ignition sources of accommodation | • |  |  | • |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |