Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • |  |  |
| If in port – advise terminal | • |  | • |  |  |
| Activate ESD. Stop any cargo operations – and shut tank valves | • |  | • |  |  |
| Try to stop the leak if that is possible (wrapping with wet rugs, tightening bolts) |  | • | • |  |  |
| Fire pumps in readiness and make the dry powder system ready for possible outbreak of fire |  | • | • |  |  |
| Stop air conditioning / ventilation and close all watertight doors |  | • |  |  |  |
| At night – switch on all deck lighting |  |  |  | • |  |
| Muster crew – conduct tally of personnel (Ensure that all staff wear the emergency escape sets) | • |  | • | • |  |
| Activate water sprays and flush water in leak area |  | • | • | • |  |

| Shut all cargo system valves |  | • | • |  |  |
| --- | --- | --- | --- | --- | --- |
| Operate deck water spray system |  | • | • |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Prohibit all smoking and avoid any kind of external ignition source |  | • | • |  |  |
| Advise engine room and emergency parties of situation | • |  |  |  |  |
| If at sea – advise nearest coast state and alert vessels in vicinity | • |  |  |  |  |
| Commence notifications | • |  |  |  | • |
| Refer to MSDS emergency procedures | • |  | • |  |  |
| If at sea – consider alteration of course of speed to carry vapour cloud away from ignition sources of accommodation | • |  |  | • |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |