Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • |  |  |
| If in port – advise terminal | • |  | • |  |  |
| Activate ESD. Stop any cargo operations – and shut tank valves | • |  | • |  |  |
| Stop dual firing. FO only, isolate engine room from cargo side |  | • |  |  |  |
| Fire pumps in readiness |  | • |  |  |  |
| Shut down air conditioning / ventilation and close all watertight doors |  | • |  |  |  |
| At night – switch on all deck lighting |  |  |  | • |  |
| Muster crew – conduct tally of personnel (Ensure that all staff wear the emergency escape sets) | • |  | • | • |  |
| Attend any injured people | • |  | • |  |  |

| Activate water sprays |  | • | • | • |  |
| --- | --- | --- | --- | --- | --- |
| Close the relevant isolation valves to prevent the spread of fire to the other cargo tanks and cargo pipes |  | • | • |  |  |
| Operate deck water spray system |  | • | • |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Use water fire hoses for radiation protection in the approach to shut off valves or to leaking jet |  | • | • |  |  |
| Prohibit all smoking |  | • | • |  |  |
| If fire cargo is involved in the fire, do not apply water directly to the fire since it will provide a heat source for more rapid vaporisation of the liquid and therefore increase the fire |  | • | • |  |  |
| Advise engine room and emergency parties of situation | • |  |  |  |  |
| If at sea – advise nearest coast state and alert vessels in vicinity | • |  |  |  |  |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Commence notifications | • |  |  |  | • |
| Refer to MSDS emergency procedures | • |  | • |  |  |

| If at sea – consider alteration of course of speed to carry vapour cloud away from ignition sources of accommodation | • |  |  | • |  |
| --- | --- | --- | --- | --- | --- |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Consider running inert gas into void spaces |  |  | • |  |  |
| Consider abandon vessel | • |  |  |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |