Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Inform Master call extra lookout. |  |  |  | • |  |
| Increase bridge watch level and man engine room. | • |  |  | • |  |
| Verify status of secondary ECDIS, passage plan and commence monitoring using secondary ECIDS is functional. | • |  |  |  |  |
| In case both the ECDIS fail, use back up arrangements of loading ENC on backup hardware as provided by the chart outfit vendor e.g. PC or tablet. | • |  |  |  |  |
| Notify office – MS or VM | • |  |  |  |  |
| Refer to TGM 2.3.7 A5 flow chart on ECDIS failure | • | • | • | • | • |
| Fix ship’s position by all available means. | • |  |  |  |  |
| Switch on echo sounder and monitor UKC. | • |  |  |  |  |
| Reduce to safe speed and M/E to be on standby for stopping. | • | • |  |  |  |
| Consider anchoring if in anchoring depth during poor visibility or if entering heavy traffic zone | • | • | • |  |  |
| Inform VTIS or relevant traffic services regarding the deficiency | • | • | • |  |  |
| Compile communications report and inform VM & MS on situation | • | • |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Initiate trouble shooting in communication with the Makers | • | • |  |  | • |
| In consultation with MS &VM Plan for a safe area to stop or to proceed to a safe port where emergency services can be arranged. | • | • |  |  | • |
| Notify flag state / classification society and coastal state as applicable. |  |  |  |  | • |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |