Note:

Hardcopy to be maintained on bridge.

| Actions to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master | • |  |  |  |  |
| Report casualty | • |  |  | • |  |
| Alert vessel's crew - account for and ensure their safety |  |  |  | • |  |
| Establish communications with the other vessel | • |  |  | • |  |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Implement VRP / SOPEP as applicable | • |  |  |  | • |
| Initiate damage control measures | • | • | • |  |  |
| Isolates cargo and fuel pipeline sources to affected areas of vessel |  | • | • |  |  |
| Determine vessel's structural integrity (damaged stability and hull stress) | • | • | • |  | • |

| Sound internal spaces and initiate structural damage survey |  |  | • | • |  |
| --- | --- | --- | --- | --- | --- |
| Report casualty to Co. Office | • |  |  |  |  |
| Actions to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Contact Class ERS for damage stability assessment and advise on Cargo / Ballast transfer. | • |  | • |  |  |
| Transfer cargo internally and/or consider lightering | • |  | • |  |  |
| Determine need for commercial salvage assistance | • |  |  |  | • |
| Ascertain cause of casualty | • | • | • |  |  |
| Preserve VDR or S-VDR records if not automatically protected | • |  |  |  |  |
| Sound general emergency alarm | • |  |  |  |  |
| Manoeuvre as appropriate/stop engine | • |  |  |  |  |
| Close watertight doors and automatic fire doors |  |  | • |  |  |
| Muster any passenger | • |  |  |  |  |
| Prepare survival crafts if there is need to abandon ship. |  | • | • |  |  |
| Broadcast Urgency or Distress message, if appropriate | • |  |  |  |  |
| Offer assistance to other vessel | • |  |  |  |  |
| Switch on deck light | • |  |  |  |  |
| Preserve ECDIS records | • |  |  |  |  |
| Maintain log/records of events and decisions | • |  |  |  |  |

| Actions to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Inform VTS or port authority, as appropriate | • |  |  |  |  |