Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master |  |  |  | • |  |
| Master to Inform Thome Officer. | • |  |  |  |  |
| Start Navigation with 2nd GPS (if available) | • |  |  | • |  |
| Check positions of vessels in the vicinity. | • |  |  | • |  |
| Fix ship’s position by all available means (celestial & terrestrial navigation) - proceed with DR and EP. | • |  |  | • |  |
| Sound emergency alarm if running into danger. | • |  |  |  |  |
| Reduce to safe speed and M/E to be on standby. | • | • |  |  |  |
| Proceed cautiously if in shallow depth or during poor visibility or if entering heavy traffic zone. Post additional lookout, if necessary. | • | • | • |  |  |
| Inform VTIS or relevant traffic services regarding the deficiency. | • |  |  |  |  |

| Broadcast SAFETY or URGENCY message, if appropriate. | • |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Compile communications report and inform VM & MS on situation. Maintain log / record of events and decisions. | • |  |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |
| Initiate trouble shooting in communication with the Makers. | • | • |  |  | • |
| In consultation with MS &VM Plan for a safe area to stop or to proceed to a safe port where emergency services can be arranged. | • | • |  |  | • |