Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm |  | • |  |  |  |
| Operate inert system to purge the fuel system lines of vapour build-up |  | • | • |  |  |
| Donning of Chemical Suits (with BA Sets) for Emergency Squad |  | • | • |  |  |
| If methanol leak is detected and without internal sensors alerting same, then emergency stop of Methanol supply. ME switches to pure DO mode. |  | • |  |  |  |
| Purge system, reset alarm system, monitor situation and identify fault/s. |  | • |  |  |  |
| Fire pumps in readiness and portable dry powder, CO2 extinguisher system ready for possible outbreak of fire |  | • | • |  |  |
| Muster crew – conduct tally of personnel | • |  | • | • |  |
| Advise Bridge and emergency parties of situation |  |  | • |  |  |
| If at sea – advise nearest coast state and alert vessels in vicinity as authorized by | • |  |  |  |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Master |  |  |  |  |  |
| Commence notifications to Company as per Incident reporting procedures | • |  |  |  | • |
| Refer to MSDS emergency procedures | • |  | • |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |