Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Notify Master | • |  |  |  |  |
| Report equipment failure | • |  |  | • |  |
| Verify vessel's position (Time, location and details of event) | • |  |  | • |  |
| If main engine failure – comply as per TGM 2.13.1 A24 MATRIX – Main engine failure | • | • | • |  |  |
| Assess if M/E need to be stopped or consider for controlled stop if required | • | • |  |  |  |
| Determine cause of failure and estimated completion time of repair. Refer to TGM 2.10.11 A16 – Scrubber Operation / Malfunctioning |  | • |  |  |  |
| Determine if ship's crew capable to repair the equipment and put back into a compliant condition within one hour |  | • |  |  | • |
| If equipment cannot be rectified within 1 hour, changeover to compliant fuel / LSMGO. |  | • |  |  |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Monitor parameters of other machineries (Fuel temperature, viscosity, A/E, Boiler, etc). Log down entry in Annex VI record book |  |  |  |  |  |
| Evaluate ROB of compliant fuel / LSMGO on board |  | • |  |  | • |
| If insufficient compliant fuel on board, propose an action plan for bunkering compliant fuel. |  | • |  |  | • |
| Plan for equipment repair |  | • |  |  | • |
| Notify flag state and destination port authority on current status, bunkering and repair plan. Seek approval before consuming the non-compliant fuel oil. |  | • |  |  | • |
| Log entry made (Deck logbook and Engine logbook, EGCS record book, etc.) | • | • |  |  |  |
| Preserve VDR / SVDR records if not automatically protected |  |  |  |  |  |