Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Notify Master | • |  |  |  | • |
| Report the details of the container to charterers and planner and Thome (container no./cell posn/port of load/port of discharge/attached photos of the situation/cargo /container contaminated) | • |  |  |  | • |
| Verify no leak is gone overboard. (refer to the EMS/MFAG/IMDG and MSDS as required) | • |  | • |  |  |
| Verify vessel’s position (Time, location and details of event, next port eta etc. if in port then state details to all) | • |  |  | • |  |
| Check for other vessels in the vicinity at sea. Check for navigational hazards in vicinity. | • |  |  | • |  |
| Broadcast SAFETY or URGENCY message, if appropriate if any DG cargo overboard. | • |  |  | • |  |
| Assess weather, sea conditions, currents and drift. | • |  |  | • |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Forecasted weather/sea conditions. |  |  |  |  |  |
| Determine if ship’s crew capable to contain the leak |  | • | • |  |  |
| Verify containment Plan. Activate the SOPEP/EMS procedures. | • | • | • |  |  |
| Log entry made (Annex VI logbook, Deck logbook and Engine logbook, EGCS record book, etc.) | • |  |  | • |  |
| Notify flag state (if any incident / deviation from regulation / violation) occurred. | • |  |  |  | • |
| Prohibit all smoking. | • | • | • | • | • |
| Implement oil spill removal and verify containment. |  | • | • | • |  |
| Determine rate flow loss. | • |  | • | • |  |
| Coordinate shore-side response. | • |  |  | • | • |
| Document spill by photos or video. |  | • | • |  |  |
| At night – switch on deck lighting as required. | • |  |  | • |  |
| Identify the chemicals involved and determine the degree of risk. (do not handle the with hands and to use chemical suit as required- refer EMS/MSDS/IMDG). | • |  | • | • |  |
| Check if reefer container and malfunction & malref report raised. | • |  | • | • |  |

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Have the areas cleaned and keep the liquid and the cleaning material separate and ask the agents for disposal.( refer to the EMS/MFAG/IMDG and MSDS as required ) | • | • | • |  |  |
| Implement oil spill removal and verify containment |  | • | • | • |  |
| Determine rate flow loss | • |  | • | • |  |
| Coordinate shore-side response | • |  |  | • | • |
| Document spill by photos or video |  | • | • |  |  |
| At night – switch on deck lighting as required | • |  |  | • |  |
| Identify the chemicals involved and determine the degree of risk. ( do not handle the with hands and to use chemical suit as required- refer EMS/MSDS/IMDG) | • |  | • | • |  |
| Check if reefer container and malfunction & malref report raised. | • |  | • | • |  |
| Have the areas cleaned and keep the liquid and the cleaning material separate and ask the agents for disposal.( refer to the EMS/MFAG/IMDG and MSDS as required ) | • | • | • |  |  |