Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Sound general alarm | • |  | • |  |  |
| If in port - advise terminal | • |  | • |  |  |
| Stop cargo operations - activate emergency shut down | • |  | • | • |  |
| Isolate all cargo tanks / lines |  | • | • | • |  |
| Isolate non-essential equipment |  | • | • |  |  |
| Assure accommodation ventilation is in full recirculation |  | • | • |  |  |
| Muster crew - conduct tally of personnel. Ensure staff provide emergency gear | • |  | • | • |  |
| Prohibit all smoking | • | • | • | • |  |
| Eliminate all sources of ignition in area |  | • | • | • |  |
| Maintain accommodation and engine room gas free | • | • | • |  |  |
| Assess and monitor extent of leakage | • |  |  |  |  |
| Advise management and charterers | • |  |  |  |  |

| If at sea - advise nearest port state and alert vessel in vicinity | • |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Purge space continuously with inert gas (if fitted) or allow void space to vent | • |  | • |  |  |
| Disperse gas cloud with water spray | • |  | • | • |  |
| Consider deballasting double bottom or circulating ballast water, if liable to freezing | • |  | • |  |  |
| Consider transfer of cargo | • |  | • |  |  |
| Implement VRP/SOPEP/SMPEP | • |  | • |  | • |
| Preserve VDR / SVDR records if not automatically protected | • |  |  |  |  |