Note:

Hardcopy to be maintained on bridge.

| Actions to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Call Master | • |  |  |  |  |
| Alert vessel’s crew by emergency alarm and announcement on PA system | • |  |  | • |  |
| Alert other vessels in vicinity; Broadcast Urgency or Distress message, as appropriate. Inform VTS or port authority, as appropriate | • |  |  | • |  |
| Account for and ensure the safety of the vessel’s crew. Report casualty if any | • |  | • |  |  |
| Ascertain vessel’s position | • |  |  | • |  |
| Post extra lookouts | • |  |  |  |  |
| Sound internal spaces and initiate structural damage survey if required | • |  | • |  |  |
| Assess vessel stability condition and calculate for potential risk eg. loss of buoyancy, overstress, excessive list, capsizing, cargo shifting etc. |  |  | • |  |  |
| Contact Class ERS for damage stability assessment and advise on Cargo / Ballast transfer | • |  | • |  |  |

| Monitor weather, sea conditions | • |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Seek assistance to call ice breaker if the vessel is stuck in ice | • |  |  |  | • |
| Actions to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Consider ballasting/deballasting to counter excessive trim or list |  |  | • |  |  |
| Manoeuvre as appropriate/stop engine | • | • |  |  |  |
| Refer to the relevant publications like Polar code etc. | • |  |  |  |  |
| Maintain log/records of event and decision | • |  |  |  |  |