Note:

Hardcopy to be maintained on bridge.

| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| --- | --- | --- | --- | --- | --- |
| Notify Master | • |  |  |  |  |
| Report casualty | • |  |  | • |  |
| Alert vessel's crew |  |  |  | • |  |
| Implement VRP / SOPEP / SMPEP as applicable |  |  |  |  | • |
| Verify vessel's position |  |  |  | • |  |
| Alert other vessel in the area |  |  |  | • |  |
| If vessel is unable to manoeuvre due to exceptional circumstances, display ‘Not under command’ signal (as per COLREGS Rule 27). Change the navigation status on the AIS | • |  |  | • |  |
| Establish and maintain communications with appropriate maritime government agency | • |  |  |  |  |
| Determine if ship's crew is capable to repair source of casualty |  | • |  | • |  |

| Determine the estimated time of repair completion |  | • |  | • |  |
| --- | --- | --- | --- | --- | --- |
| Determine need for commercial salvage assistance | • | • |  |  |  |
| Action to be taken | Bridge Team | Technical Team | Emergency Squad | Watch Officer | Office |
| Prepare ship's personnel to initiate oil spill mitigation and removal action as warranted |  |  | • |  |  |
| Determine cause of casualty |  | • |  |  |  |
| Preserve VDR / SVDR records if not automatically protected | • |  |  |  |  |