| S.No. | Item Description | Y | N |
| --- | --- | --- | --- |
| 1 | Docking condition to be planned and office, local agent and dockyard advised of same. |  |  |
| 2 | For gas carriers, cargo tanks are to be inerted and then gas freed. Cargo holds are to be gas freed only after cargo tanks are gas freed. Cargo tanks and hold space inspection to be carried out during docking. For LNG carrier Inter barriers space must be depressurised and made gas free. Cargo holds / tanks, slop tanks are to be cleaned & de-mucked (Tankers / Cargo). |  |  |
| 3 | All ballast tanks, especially for tanker in which internal repairs are planned, are to be demucked to possible extent. |  |  |
| 4 | All manholes of each cargo / ballast tanks and void spaces are to be unbolted and left open with protectors, if possible. For gas carrier cargo tanks, only manhole that are required for removal of cargo pumps and ventilation to be opened. Suitable protection to be erected around cargo dome to prevent ingress of dirt and water. Entry in cargo tanks to be limited only to personnel for inspection, overhaul of cargo pumps and level gauge checking. |  |  |
| 5 | All cargo, COW & MARPOL lines to be cleaned and gas-freed and all valves /drains left open. (Tankers). For gas carriers, all cargo lines including compressor vapour and condensate lines must be inerted and made gas-free. Only manifold valves and blanks must be kept open. For LNG carriers, the Fuel Gas line to ER is to be inerted and gas freed additionally. |  |  |
| 6 | Frame nos. are to be marked with paint on deck from aft to forward on each side of main deck if any repairs are planned on the hull. |  |  |
| 7 | All work spots to be undertaken by dockyard to be marked by Job No. as per DD specification for quick identification with yard people. |  |  |
| 8 | Spaces in which yard people are not to enter, to be posted with notices and crew / officers should be made aware of same. |  |  |
| 9 | One suitable office to be made ready as DD office. Ship's General Arrangement & Capacity Plan to be displayed in this office for easy reference. |  |  |
| 10 | A chart to be prepared for jobs and planned no. of days in the dockyard and to be displayed in Gymnasium or ships office. Same to be filled in every day (in evening meeting of all officers) as the work progresses. |  |  |
| 11 | All necessary plans & manuals are to be kept ready beforehand for quick reference and issuance to dockyard official on receipt signature only. It is important that the inventory and locations of all plans/ manuals are updated well in advance. |  |  |
| 12 | Test of steering gear as per "Sea Trial Booklet" procedures (as far as practical) to be carried out and office Superintendent informed of the results. |  |  |
| 13 | List of CMS and other Class surveys to be ready and discussed with technical Superintendent beforehand. An updated latest survey report is to be procured from Class through the office prior to DD. For gas carriers list of all cargo safety valves & relief valves that are to be overhauled is to be prepared as per class requirements. |  |  |
| 14 | Cargo derrick winches, L/B winches, all mooring wires & ropes and all L/rafts to be covered with polythene / canvas (if not to be attended in DD) in order to protect them from sand / grit from own ship and /or from adjoining ships. Last brake test report to be kept ready as can be handy to judge the condition of brakes, etc., on opening of winches. |  |  |
| 15 | Ship Management to prepare a list of vital jobs, which is required to be completed well in advance of sailing date in order to carry out proper inspection & test of these jobs. Same to be brought to the notice of office Superintendent and to dockyard on first meeting. |  |  |
| 16 | In case vessel is to be laid-up after DD a list of priority jobs to be agreed upon as above well in advance with office Superintendent. |  |  |
| 17 | All required spares, including bearings and consumable, to be procured before arrival DD. A DD Requisition Status to be ready for pending stores or spares which may be required during stay in dockyard. |  |  |
| 18 | Spares are to be assembled, marked and kept ready as per Job Nos., and all crew / officers made aware of the same. |  |  |
| 19 | A printout of spares status according to important jobs should be in hard copy and ready for reference. All such lists should be filed together with Job Nos., as extra spares above anticipated could be required. |  |  |
| 20 | A list of parts to be landed for reconditioning / repairs to be ready with Landing Form and office Superintendent informed of same beforehand. |  |  |
| 21 | All officers and crew should be briefed about all work planned well advance. Officers & crew should be allotted some specific jobs (like keeping account of Scaffoldings at various locations) as well, to monitor & recording during stay in dockyard. Such briefing shall take place at frequent intervals prior arrival DD. For gas carriers' cargo tank preparation plan for gas freeing, aeration, regassification and coolant loading after the DD to be prepared prior arrival to last discharge port. |  |  |
| 22 | Fixed Fire Fighting systems, like CO2 system for Paint Locker & E/R (where applicable) shall be disconnected and locked after entry in yard as per yard regulations. Fire lines to be provided by yard as per yard regulations for the duration of the dock. Additionally for gas carriers CO2 for compressor room, motor room & E/R (as applicable) shall be disconnected and locked. |  |  |
| 23 | All equipment from both lifeboats is to be stored inside a locker. All other loose equipment on deck is also to be kept in deck stores. |  |  |
| 24 | Annual testing / routine maintenance of equipment like Radars, Gyro & other Bridge Equipment, ME Manoeuvring System, Fuel Pumps, Turbocharger, ODM, Oily Water Separator, Echo Sounder, Gas Meters, UTI/MMC, etc., to be planned and office to be informed well in advance. Dockyard to advise if maker's authorised representative available. In the absence of availability of maker's authorised representative, such equipment should be attended before or after DD. Additionally, for gas carriers' cargo pump, cargo safety valves and cargo compressors attendance of makers technician to be arranged as required. For LNG carrier Global test (Vacuum test) to be planned. The test for checking the integrity of the cargo tanks is to be decided well in advance (TAMY test or SBTT or equivalent.) |  |  |
| 25 | Insulation of all machinery and pipelines to be checked. |  |  |
| 26 | Shore Power Supply Breaker to be inspected and tested if required, for proper working. |  |  |
| 27 | A list of any additional jobs generated at last moment to be prepared for discussion with the technical Superintendent on the first day of docking. |  |  |
| 28 | Information for the following HSQE issues shall be procured from DD in advance and if same is not available to be discussed upon arrival: - Safe access to ship and Muster point in case of emergency - Fire protection & Firefighting arrangements - Medical and General Emergency help numbers and contacts - Garbage disposal and segregation arrangement - Hot Work Policy and related requirements - Local Security regulations |  |  |

_________________________                                   _____________________

Chief Engineer					                                                Chief Officer

_________________________

Master

Note: 1. Completed form should be retained with DD reports on board.