Note:	This checklist is to be used to supplement a critical operations checklist for during drydock and departure. Reference is to be made to fleet operations technical procedures.                                       Note: Checklist compliant with 2017 Intertanko Drydock Checklist (V1)

Vessel:						Location / Yard:

- Checklist – All Vessels

### Certification and Documentation

Certificates, test records, calibration reports and receipts 	Onboard

Environmental requirements & documentation	In compliance / Onboard

Navigational, Security Plans and Ships Plans	Approved & Onboard

Contingency plan	Verified

### Crew Management

Ships personnel work & rest hours	Compliant

All crew onboard prior sailing	Verified

### Navigation

Navigation equipment / instruments	Tested

Dead man alarm, navigation bridge alarm system (Inc. public areas & cabins)	Tested

Echo sounder & Doppler transducers (transducers uncovered)	Functioning

SSAS	Tested

Passage plan (Inc. Navtex, weather info etc.)	Prepared

Charts, publications and NTM	Latest & Available

Suez canal lights	Tested

Heavy objects	Secured for sea

### Safety Management

Security inspection & logbook entry (ensure shipyard personnel disembarked)	Completed

Accommodation ladders	Tested

Water tight & fire proof doors	Operational & Closed

Firefighting equipment–accessories (Inc. updated fire plans)	Stowed

Lifesaving equipment (L/rings, EEBDs, ELSA etc.)	Ready

Emergency generator (Inc. auto start and operation of steering gear from emergency Gen. board)	Tested

Emergency power supply (Batteries, UPS & chargers)	Tested

Fire, Gas, Smoke, Thermal detection systems (Inc. extension, repeaters & internal fault alarm)	Tested

Operational test of fire pump, fire main & fire hoses	Completed

Fire dampers & remote operating cables	Satisfactory

CO2 system connections	Satisfactory

Permanent fire extinguishing systems (Engine, Deck, Galley & Accommodation)	Ready

- (Air tested to confirm clear with no blanks)

PPE 	Available

Fan stops (lifeboat stations, fire station, CO2 station)	Tested

Lifeboat engine operation & emergency air supply	Tested

Lifeboat davits swung out & limit switch test	Complete

### Pollution Prevention

Environmental seals	In place

Garbage, oily liquids and rag delivery receipts with Master	 Confirmed

Hydraulic lines, bunker and discharge manifolds valve/joint tightness	Confirmed

Main Cool S.W, Fire Pumps, Emergency Bilge Suction	Closed

Tank overfill alarms (engine room, pump room, steering gear room)	Set / Tested

Bilge alarms & remote indication (Inc. those out with machinery space)	Tested

OWS (Inc. three-way valve and internal examination of overboard pipe)	Tested

OWS overboard valves (seal no. recorded in record book)	Closed

Manifolds, valves and drains	Bolted / Closed / Blanked

### Structural Condition

Tank plugs	In place

Upper deck, accommodation and out-fittings	Inspected

Inspected tanks, void spaces, chain locker and access hatches	Closed/Secured

Tank soundings (ready for departure)	Satisfactory

Stability & trim during dock flooding / undocking 	Satisfactory

### Cargo Systems

Cargo hold bilge alarm / Water Ingress Alarm System	Tested

Hydraulic System 	Tested

Cargo, ballast and void spaces	Clean / Watertight

Ballast pumps (Inc. lines, valves and joint pressure checks & auto and emergency trip)	Tested

WBTs level gauges & indicators 	Tested / Accurate

CCR draft gauge indicators	Accurate

### Mooring & Deck Machinery

Mooring machinery and windlass	Tested

Crane, limit switch, slack wire and high hook trip	Tested

### Communications

Communication/Radio (Inc. Bridge wings, Upper deck fwd. & aft., Steering gear, ECR and CCR)	Tested

### Engine and Steering Compartments

Quick closing valves	Tested

UMS alarms and repeaters	Tested

Aux. Engine (Inc. load sharing stability, reverse power trip and preferential trips)	Tested

Stand-by Generators auto start and auto connection	Tested

Main & Aux. Engine – Oil leakage detection alarm	Tested

M/E alarms and trips (Inc. oil mist detector)	Tested

M/E turning gear interlock	Tested

Engine side & control room operation of M/E	Tested

Bridge control of M/E	Tested

Low L/O pressure, high F/W temperature alarms and trips on M/E & Aux. engines	Tested

Main & emergency switchboard automation test (Inc. cleaning and bus bar securing bolt test)	Complete

Double skin fuel pipes (M/E & Aux. Engines)	Satisfactory

Shielding on fuel oil pipe flanges	Satisfactory

Insulation and cladding of hot surfaces	Satisfactory

Aux. boiler(s), alarms and trips (Inc. flame failure alarm / trip)	Tested

Rudder / steering (Inc. emergency steering system)	Tested

Stern tube system (Static head test prior to undocking) 	Confirmed

Change-over of stand-by pumps and motors	Tested

Overhauled sea chests, valves, suction and overboard	Satisfactory

Air condition units tested & fresh air intake suction filters renewed	Confirmed

Refrigerator chamber alarm and hospital calling system	Tested

Emergency lights 	Tested

M/E L.O sump tank manhole  gaskets inside cofferdam	Inspected

Thrusters, if available	Tested

Special tools &  equipment provided to yard / contractors	Onboard

# Ship Specific Checks

- All Tankers

### Safety Management

- Pump room - rescue winch, stretcher, FFA and safety equipment	In place

- Pump room - ventilation and dampers	Satisfactory

### Mooring & Deck Machinery

Emergency towing systems	Inspected

- Oil Tankers

### Pollution Prevention

- O.D.M spectacle flange closed & testing ODME via self-test	Confirmed

### Cargo Systems

C.O.Ps (Inc. lines, valves and joint pressure checks & auto and emergency trip)	Tested

Tank ullage vapour locks condition / gas tightness	Satisfactory

COW machines manual operation test and checks for damage	Satisfactory

Heating coils 	Tested / Inspected

Hydraulic system	Tested

P/V valves operation / leak test	Satisfactory

IG / N2 system (Inc. alarm & trips)	Tested

IG / N2 system operation test for leaks (Inc. cargo spaces, accesses, hatches and piping) 	Satisfactory

COTs level gauges (Inc. indicators in CCR)	Satisfactory

IG inlet branch, mast riser and main valves (Inc. security pins and sockets)	Tested

IG / N2 remote tank s pressure indicators	Satisfactory

- LPG Tankers

### Safety Management

- Educator system valves for cargo hold / interbarrier spaces secured (blanks in place)	Confirmed

- Crew briefed on post drydock duties	Confirmed

### Cargo Systems

Independent high level alarms (95% and 98%)	Tested

Emergency Shut Down (ESDS)	Tested

Tanks, pipes, valves and equipment	Inert

Water hydrate formation / correct dew point	Confirmed

- Chief Engineer:	 		Date:

- Chief Officer:			Date:

- Master:			Date:

Superintendent:						Date: