To be completed by the Chief Officer

Ship:					                         Voyage: 	________

Cargo No: 		  Grade: 		                   Date:

Operation to be carried out: Purging  / Padding (delete as applicable)

| AAA Operational Information (To be filled after carrying out Ship-Shore Safety Meeting) 1. Tanks to be Purged or Padded (In operational order) 1; 2; 3; 4. 5; 6; 7; 8. 9; 10; 11; 12. 13; 14; 15; 16. 2. Requirement on Completion of Operation Oxygen Content; ppm or %; Tank Pressure; mbar. 3. Nitrogen Supply Source of Nitrogen Supply; Ship Supply / Terminal Supply / Barge / Truck (delete as applicable). Maximum Flow Rate; m3/h. Size of Connection; Inches / mm. Shore Source; Buffer tank / Evaporation through Vaporizer (delete as applicable). 4. Nitrogen Receiving Arrangements Shore Source connected to; Vent Stack / Manifold Line / Vapour Return Line (delete as applicable). Method of Controlling Pressure; Small diameter hose and controlling valve on vessel.; Yes / No. Vessel MUST control nitrogen flow to the tank(s) by way of Ball Valve fitted to Small Diameter Hose at inlet on ship's side. If this arrangement is not met then an Operation Specific Risk Assessment is to be completed and submitted to the office for approval prior to commencing the operation.. Tank Atmosphere vented through P/V valves / Gas freeing cover and maximum flow rate; Yes / No m3/h; Maximum acceptable flow rate of Nitrogen should not exceed the tanks / vessel's maximum loading rate. The tank should never be pressurized above the setting of the P/V Valve.. Basis Above – Maximum allowable pressure/flow in supply line; m3/h / psi / bar. |
| --- |
| 5. Communications Have the Communication Method and Shut Down Procedures been agreed with the Terminal; Yes / No. Communication Method. Emergency Shut Down Procedure. |
| Guidance on Cubic meters of gas at various pressures received in 1 minute through various size hoses. |  |  |  |  |  |  |  |  |  |
| Hose Diameter / Pressure |  | 200mm(8") |  | 150mm(6") |  | 100mm(4") | 50mm(2") | 25mm(1") |  |
| 5.2 bar (75 PSI) |  | 1771 m3/min |  | 914 m3/min |  | 343 m3/min | 67m3/min | 12 m3/min |  |
| 3.4 Bar (50 PSI) |  | 1286 m3/min |  | 662 m3/min |  | 243 m3/min | 48 m3/min | 9 m3/min |  |
| 2.1 Bar (30 PSI) |  | 886 m3/min |  | 457 m3/min |  | 171 m3/min | 33 m3/min | 6 m3/min |  |
| 0.7 Bar (10 pSI) |  | 471 m3/min |  | 214 m3/min |  | 80 m3/min | 16 m3/min | 3 m3/min |  |

BBB    Checks to be Made Prior Commencing Nitrogen Operations

CAUTION - Nitrogen Purging / Padding may result in tank over pressurisation. The operation is to be controlled by the Chief Officer and is not to be done concurrent with other cargo / tank cleaning operations. Flow rate of Nitrogen must be controlled by vessel.

| Item to be checked |  | Yes / No |
| --- | --- | --- |
| Have voyage orders been checked for 'Oxygen content requirement' and 'Inhibitor Oxygen dependency' for this cargo ? | Oxygen dependency of Inhibitor % |  |
| Has a manifold gauge (0 to 15 bar) been fitted at manifold to monitor pressure? Has a tank pressure gauge been fitted (- 0.5 to 1.0 bar)? |  |  |
| Are sufficient number of Nitrogen bottles (If Applicable) of required purity available onboard for the voyage? | Nitrogen purity required _______ Nitrogen bottles available _______ |  |
| Has Oxygen meter been zeroed and tested prior to commencing operation ? |  |  |
| Is the cargo tank pressure alarm system switched ON and found satisfactory? Note: Tank never to pressurised above the PV Valve setting, ensure tank pressure alarm system is set 10% below the PV valve opening pressure. | PV Valve Pressure Side Setting __________________________ Tank Pressure Alarm Setting __________________________ |  |
| Has the line up been cross checked by the Chief Officer and another Responsible officer? Name ________________ Rank ______________ Initials_____________ |  |  |
| Have relevant crew members been briefed of their duties? |  |  |
| Are all personnel involved in the operation aware of the dangers and health hazards of Nitrogen? Nitrogen tags applied and areas fenced off as required to control access? |  |  |
| Has a person been designated to monitor tank pressure regularly during the operation? Name ________________ Rank ______________ Initials_____________ |  |  |
| In case of venting through PV or Vapour Return Line, Has PV valve 'Operation Test' been carried out and found satisfactory? |  |  |
| Has possibility of cargo contamination due to introduction of Nitrogen gas during padding operation been considered and found acceptable ? |  |  |

CCC    Checks to be Made After Completion of Nitrogen Operations

| Item to be checked | Yes / No |
| --- | --- |
| Have all valves been closed and manifold blanks put in place ? |  |
| Have all dead ends in the line system been flushed adequately with Nitrogen ? |  |
| Have all tank openings been secured and tank pressure monitored to ensure tank pressure is steady ? |  |
| PV valves and pressure alarms returned to 'Normal Operational' condition. |  |

Any negative entries above require an explanation / comment to be entered against the appropriate item

and compensatory measures put in place as appropriate.

| Signed: | Date: |
| --- | --- |
| Name (Print) |  |
| Master: | Date: |
| Name (Print) |  |