| Name of Ship: |  |
| --- | --- |
| Hose Identification: |  |
| Hose Manufacturer: |  |
| Hose Size: |  |
| Maximum working pressure / Test Pressure: (This should be marked on the hose) |  |
| Maximum & minimum working temperatures: (These should be marked on the hose) |  |
| Expected Retirement Date (as per manufacturer’s recommendations) |  |

| Date | Before Pressure Test |  |  | At Rated Working Pressure (Duration > 5 min) |  |  |  | At Test Pressure (Duration > 5 min) |  |  |  | After Pressure Test |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Length (M) | Visual Condition | Electrical Continuity | Length (M) | Visual Condition | Electrical Continuity | Length (M) |  | Visual Condition | Electrical Continuity | Length (M) |  | Visual Condition | Electrical Continuity |

Remarks:

- The cargo hoses are to be pressure tested to 1.5 times the Rated working pressure every 12 months.

- Those found defective such as leakage, kink, bulges, soft spot, etc. must be reported to the office and marked as “DEFECTIVE” and must not be used for any cargo transfer.

- Hoses with satisfactory result shall be marked with date of test, maximum working pressure and working temp range.

- The temporary elongation should not exceed 1.5 times the temporary elongation when the hose assembly was new otherwise the hose should be removed from service.

- Electrically continuity of cargo hoses should be in compliance with latest ISGOTT requirements.

- Cargo hoses shall be inspected internally as far as practical for inner core (innermost layer) delamination and / or blistering.  If such condition in found, this must be reported ot the office and hose marked as “DEFECTIVE” and must not be used for any cargo transfer.

| CHIEF OFFICER Name: | MASTER Name: |
| --- | --- |