Instructions: Ensure below tasks are performed to facilitate safe cargo hold ballast operations.

| Vessel: |  | Port: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

| Responsible Officer: Ch. Officer |  | Yes | No | N/A | Notes / Remarks |
| --- | --- | --- | --- | --- | --- |
| In Port |  |  |  |  |  |
| 1 | Does current condition of unloaders require hold to be ballasted to allow sufficient air draft? If yes proceed to ballasting procedures. |  |  |  |  |
| 2 | Does intended voyage requires hold ballasting for stability purposes? |  |  |  |  |
| 3 | Does the current port allow hold ballasting? |  |  |  |  |
| 4 | Is the water depth at berth sufficient to avoid sucking sediments (mud)? If yes to 3 & 4, then proceed to ballasting procedure. |  |  |  |  |
| 5 | Is the weather / sea condition outside while underway after departure safe to effect hold ballasting? If yes to this point and no to 3 & 4, then proceed to ballasting after sailing. |  |  |  |  |
| Ballasting |  |  |  |  |  |
| 1 | Has vessel’s stability and stress condition ascertained to be safe? |  |  |  |  |
| 2 | Sequence of alternate filling / discharging between hold ballast and DBT/TST been established and acceptable? |  |  |  |  |
| 3 | Hold properly swept /cleaned? |  |  |  |  |
| 4 | Tank top ballast line cover removed, and perforated suction screen installed? |  |  |  |  |
| 5 | Bilges covers perforation clear to allow stripping? |  |  |  |  |
| 6 | Firefighting extinguishing line isolated if any? Fixed CO2 system valves at the hatch coaming side are closed / Shut. |  |  |  |  |

| Responsible Officer: Ch. Officer |  | Yes | No | N/A | Notes / Remarks |
| --- | --- | --- | --- | --- | --- |
| 7 | Access manhole to hold bilges from lower stool compartment tight and secured if any? |  |  |  |  |
| 8 | Bilges line from lower stool isolated if any? |  |  |  |  |
| 9 | All electrical connections inside ballast hold isolated? |  |  |  |  |
| 10 | All ventilations i.e. hatch cover vents (4 nos.), air vents, etc opened up? |  |  |  |  |
| 11 | Ballast hold filled up close to 98%? (Avoid sloshing damage). |  |  |  |  |
| 12 | Heavy ballast hatch cover securing devices installed and secured? |  |  |  |  |
| 13 | Ballast hold valves tested for leak |  |  |  |  |
| 14 | The manhole lid rubber packing / nuts & bolts conditions checked |  |  |  |  |
| De-ballasting |  |  |  |  |  |
| 1 | Is weather/sea condition safe to perform de-ballasting? If not do it in confined water or in port. |  |  |  |  |
| 2 | Ensure sufficient ventilation is opened before starting pump. |  |  |  |  |
| 3 | Has vessel’s stability and stress condition ascertained to be safe? |  |  |  |  |
| 4 | Sequence of alternate discharging / filling between hold ballast and DBT/TST been established and acceptable? |  |  |  |  |
| 5 | Bilges line from lower stool that has been isolated been opened up if any? |  |  |  |  |
| 6 | Initial stage of de-ballasting should be by gravity. |  |  |  |  |
| 7 | Once water cleared, dry up bilges. |  |  |  |  |
| 8 | Put back ballast line tank top cover. Ensure no protrusions. |  |  |  |  |

| Responsible Officer: Ch. Officer |  | Yes | No | N/A | Notes / Remarks |
| --- | --- | --- | --- | --- | --- |
| 9 | Fixed CO2 system valves at hatch coaming side are opened. |  |  |  |  |

Note:

Check	at	Six	months	interval	-	The	indication	of	ballast	hold	valve	on	the	panel ( CCR / Ships office) and carry out check on the physical valve actuator for alignment.

| Date verified: |  |
| --- | --- |
| Officer’s name, rank & Signature: |  |