Bunker Plan	                         Vessel:

The Chief Engineer is to prepare a bunkering plan prior to the pre-transfer conference. The plan is to be retained onboard along with the bunker delivery note/Sludge or Bilge water receipt in the MARPOL Annex VI file and retained on board for a period of at least 3 years.

The Bunker Plan is to be developed in accordance with the requirements of the SMS Fleet Operations Procedures.

Port________________________________      Date___________________________________

The term “bunkering” refers to the bulk transfer from shore facility, barge etc. to vessel of fuel and lubricating oils and the transfer from ship to shore or barge of sludge and other residual oils.

The Company defines the following bunker tank filling limits (i.e. excluding Lub oil tanks which must not be filled in excess of 95%):-

Tanks up to 700m³ total capacity 				90%

Tanks over 700m³ total capacity					95%

Tanks with known air-locking difficulties experience		90%

The practice of simultaneous receipt of different grades of bunkers is prohibited.  Only one grade at a time is to be taken.  This restriction does not apply to lubricating oils.

Where available, Bunker Tank adjustable high-level alarms shall be set to the SMS allowed filling limit. Where the high level alarms are fixed, the tanks shall normally be filled to the set high level alarm capacity.

If filling limits need to be increased up to 95%, it shall be subject to Office FSI authorisation, specific Risk Assessment and additional control measures:

-              Reduced bunkering rate when topping up.

-              Continuous manual soundings.

-              Good communication lines (both internal and with bunker barge).

-              If this tank is last in sequence, it shall not be filled over 90%.

| Obtain pre-delivery typical fuel parameters in order to provide some warning of any specific differences between current bunkers on board and that to be supplied. This can be in the form of a Certificate of Quality giving details such as the fuel being residual or distillate fuel, viscosity, density, sulphur content and cold flow properties. The full ISO 8217 suite of tests with a stability TSP result is preferable. |  |
| --- | --- |
| Details of bunkers to be loaded as per the Bunker Stem received / oil residues or sludge to be transferred ashore. |  |
| Caution : The density of VLSFO can vary significantly according to method of production. The density of the product to be bunkered shall be determined prior to commencing bunkering and where required the bunkering plan / checklist revised regarding tank filling data. |  |

Bunker / Oily Mixture or Sludge Distribution

| Tank Characteristic |  |  | Grade / Oily Mix or Sludge | Volume to bunker (m3) |  |  |  | Product density/ temp. used during planning | Transfer Rates (m3) |  |  | Max Line Pressure (Kg/Psi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID / No | Vol at max. filling |  |  | OBQ ** | Available Volume | Planned Filling |  |  | Initial | Max. | Topping Off |  |
|  | m3 | %* |  |  |  | m3 | %* |  |  |  |  |  |

*   - for maximum filling % see above /  Element 6C.2.1.4

** - volume in tank prior bunkering

| Describe the bunker line up requirements and detail staff responsible for verifying this to be correct. |  |
| --- | --- |

| Describe the sequence of loading bunkers. |  |
| --- | --- |

| Detail any special requirements of the vessel for segregation and required stoppages. |  |
| --- | --- |

| Detail the procedure to be followed for changing over tanks whilst loading bunkers or transferring oily mixtures or sludge to shore reception facility. This should include details of additional valve line-up checks, level monitoring in closed tanks, and monitoring of manifold back pressures. |  |
| --- | --- |

| Describe the procedures to be followed for topping off / discharge completion. This must include details of the vessel’s required topping off / discharge completion rate, the notice period to be given to suppliers, and increased levels of supervision for this critical stage of the operation. |  |
| --- | --- |

| Detail the persons responsible for making anti-pollution preparations such as fitting of scupper plugs, checking save-all plugs are in place, ensuring all bunker vents are clear and ready for the operation, and preparing the oil spill response kit. |  |
| --- | --- |

| Detail the Oil Spill Equipment Available and its location during the bunkering operation. |  |
| --- | --- |

| Describe the level measurement system for the bunker / sludge tanks. Describe the procedure to be followed for monitoring the level in the bunker / sludge or Bilge tanks during bunkering and the frequency of measurements. This should include a method for checking temperature if so required. Comparison records should be maintained between manual and automated soundings to verify the accuracy of the automated system ( if applicable ). Any discrepancies must be reported to the management office. Details of the trim correction factor to be included. REMEMBER to manually check all bunker tanks on the system in case of incorrect valve line-up. |  |
| --- | --- |

| Describe details of the High Level Alarm Systems onboard the vessel, including alarm set points. Describe how tested and date of last test. Describe the actions to be taken onboard upon activation of a high level alarm. Emergency stop tested ? |  |
| --- | --- |

| Describe the Tank overflow arrangements and actions to be taken in the event of overflow. |  |
| --- | --- |

| Communications | Primary means of communication with suppliers /shore reception facility Secondary means of communication with suppliers /shore reception facility Emergency Stop Signal to be used Internal Communications – VHF Channel to be used |
| --- | --- |

| Sampling Arrangements As required by SMS FOP |  |
| --- | --- |

| Safety State PPE to be worn during bunkering Arrangements for testing the presence of toxic and flammable gas. Ship’s personnel should always be alert to the possible presence of H2S and Benzene in bunker fuel. Procedures to be followed onboard in the event of high H2S and/or Benzene levels. |  |
| --- | --- |

| Manning requirement to execute the operation safely. Personnel involved in the bunkering operation onboard should have no other tasks and should remain at their workstations during topping off / start of transfer of Sludge or Bilge water ashore. This is particularly important when bunkers are being loaded concurrent with cargo operations, so that conflicts of interest for operational personnel are avoided. |  |
| --- | --- |

| Additional Notes |  |
| --- | --- |

Signed:									Chief Engineer

Signed:									Designated person in charge

(State name / rank)

Signed:									Chief Officer

Signed:									Others (State name / rank)

Reviewed and Signed :                                                                               Master

To be attached to this plan:-

- Material Safety Data Sheet

- Bunker Stem / Sludge or Bilge water transfer agreement

- Bunker system diagram showing distribution of tanks and valves.