| Vessel Name: |  | Date: |  |
| --- | --- | --- | --- |

This checklist is to be filled up only when OWS is operated for discharging bilges in open sea and not required to be filled up for monthly test in recirculation Mode. The OWS-OCM operation should be carried out as far as practicable during daylight hours and with ER in Manned condition.

CE Is the environmental officer and overall responsible for training of engineers to correctly operated OWS as per Makers guidelines. CE responsible and overall, in charge for each OWS operation.

Note: Operation of OWS to discharge overboard is prohibited in following areas:

- Arctic & Antarctic area.

- When less than 12 nm from any base line (territorial waters).

- In Baltic Sea including Kattegat (South of Skagen).

- In Canadian Territorial waters, this includes entire Saint Lawrence seaway enclosed by Cabot Straits and Belle Isle Straits.

- Great Barrier Reef and part of the Torres Straits (Particularly Sensitive Sea Area (PSSA)

- When navigating in Canals, Straits, Rivers and inland waterways. (Including Singapore and Malacca Straits)

- Other MARPOL Annex-1 Special Areas (Master to communicate with ECD for one-time waiver)

| ##### Items Prior and During Discharge |  |
| --- | --- |
| Confirm with bridge that the ship is proceeding En-route (more than 6 Knot speed and not circling), greater than 12N-Miles from base line and not in special areas. (For Annex-I special areas- ECD for one-time waiver received). | ☐ |
| Test OCM 15ppm alarm system & 3-way valve changeover/ Bilge Pump stops (as the case may be) and sounds in ER Alarm panel. The Oil probe confirmed working satisfactory. | ☐ |
| OCM timings to be in UTC. Time difference between UTC & OCM to be adjusted if greater than 15 Minutes & I entry made in ORB after adjustment. | ☐ |
| During OWS operation ensure that there is uninterrupted and true representative sample of bilge water through 15 ppm OCM AND the FW line valve is strictly closed (Unless systems designed for Auto FW Flushing) | ☐ |
| Contents of Bilge holding tank(s) in M3 Start Time: Start Qty: Stop Time: Stop Quantity: Total Quantity discharged in M3: | ☐ |
| The Time delay for OCM 15ppm alarm and 3-way Valve changeover should be preferably set between 8-10 Secs, but not exceeding 10 secs (where possible as per OCM design). | ☐ |
| During operation, ensure that the oil sensing probe is working satisfactorily to auto discharge accumulated Oil in OWS into Bilge Oil Tank. | ☐ |

| Item After OWS discharge completion |  |  |
| --- | --- | --- |
| To ensure that The OWS system has been secured, overboard valve shut and sealed, and box enclosure locked, keys handed over to Master/CE and Bridge informed. |  | ☐ |
| Ensure that appropriate Oil record book entry for the above operation and that the timings/ Position/ Qty recorded consistent with OWS/ OCM Data. |  | ☐ |
| During and after every OWS operation, CE to check OCM data for discharged bilge water and confirm all in order. The PPM to record as below. Start ppm: Total Average ppm for discharge operation Only (Eg: Every 30 Minutes check): |  | ☐ |
| This is to state that the Oily water separator and 15ppm Oil Content Monitor was operated as per makers instructions. No tampering of the equipment was noted during entire OWS operation and uninterrupted true representative sample of bilge water was continuously fed to the 15ppm OCM throughout the discharge operation. |  | ☐ |
| Engineer Officer Assisting CE with OWS discharge Ops (Name & Signature) | Chief Engineer (Name & Signature) |  |

References:

- EMS 04-06-02

- MARPOL Annex-1/ Ch3/ Reg 15