| Vessel Name: |  |  |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

This checklist is to be filled up only when ODME is intended to be operated for decanting in open sea (Ref Annex I-Ch 4/ Reg 31) & not required for Monthly tests. The ODME operation must be carried out during daylight hours only and when vessel is En-Route, in ballast Condition and not in special area.

Note: Operation of ODME to discharge overboard is prohibited in following scenarios but not limited to:

- When less than 50 nm from any coast line (territorial waters).

- Vessel In Special Area

- Vessel is not proceeding en-route.

- When navigating in Canals, Straits, Rivers and inland waterways.

- Any other Restricted Areas defined by Local Port authorities.

| ##### Item |  |  |
| --- | --- | --- |
| Before Decanting Operations: - Has Master approved the Decanting operations. Ensure key with the master If the ODME is fitted with such a key for Manual/Auto changeover. |  | ☐ |
| - ODME Self-test & Self calibration done as per makers instructions. |  | ☐ |
| - ODME is in AUTO mode and NOT Manual. Master to confirm status prior commencement of operation. (For operation in Manual mode in Ballast- One Time Flag & HOC / ECD approval Mandatory) |  | ☐ |
| - Set the total oil quantity/ last Cargo qty Input in ODME for Computation (Master should check this input value). Ensure sufficient settling time (At least 24-36 hrs) |  | ☐ |
| - Vessel Not in Special Area & Decanting not being done in Loaded Condition. Inform Bridge and ensure duty officer verifies the vessel position |  | ☐ |
| - Operations done between Sunrise to Sunset. Continuous monitoring of Oily-Water Interface to be done during operation. (Decanting to stop when Oily-Water interface >/= 0.5Mtr). |  | ☐ |
| - ODME Discharge Overboard is above the water line for Visual checks of discharges and CO to ensure visual checks throughout ODME operation. |  | ☐ |
| - Operations carried out in compliance with Makers Instructions & MARPOL Annex I-Ch 4/ Reg 31. |  | ☐ |
| After Decanting Operations: - Upon completion of De-canting, Master to review with CO the data from ODME so as to verify all operations done with ODME in “AUTO”. (Pls Note: Any concerns sighted with data to be raised to HOC / ECD immediately) |  | ☐ |
| - ODME discharge Overboard valve unsealed prior operation and Re-sealed after operations to be recorded under “O” in ORB-II. (Seal log E72) |  | ☐ |
| Chief Officer (Name & Signature) | Master (Name & Signature) |  |