THIS CHECKLIST IS TO BE COMPLETED BY THE WATCH OFFICER AT

LEAST ONCE DURING HIS WATCH AND A LOG BOOK ENTRY MADE

| HAVE THE FOLLOWING BEEN UNDERTAKEN: | ## Initials |
| --- | --- |
| The Master's and Chief Officer’s standing and special orders regarding vessel safety, security and cargo operations read and understood? |  |
| Is the cargo plan being adhered to? |  |
| Is the temperature of the incoming gas within 80C of the receiving tank? |  |
| Is the maximum transport pressure of the incoming gas below the relief valve setting by least 0.1 bar (10.2v)? |  |
| Is the reliquefaction plant (if in use) running properly? |  |
| Is the ship stability and stresses within acceptable limits? |  |
| Is a deck watch being maintained and are they aware of the cargo being loaded? |  |
| Are cargo tank pressures, temperatures, liquid levels and manifold connections being monitored? |  |
| Are soundings checked, and recorded with loading rates being calculated every hour? |  |
| Are ship figures being compared with shore figures at least once every watch and recorded? (Any discrepancy noted has to be immediately investigated). |  |
| Is the vessel securely moored, aligned correctly with the berth and is the deck watch maintained properly? |  |
| Are fire wires ready and rigged in accordance with terminal requirements? |  |
| Are all deck scuppers plugged and is the deck clear of any oil or cargo leakages? |  |
| Has the vessel’s draft been visually checked and remote read-out compared against it? |  |
| ### Have tidal ranges, tidal or river currents and vessel traffic movements which may affect mooring safety been continuously monitored for unpredicted changes? |  |
| ### Have weather forecasts been obtained at least once each watch from relevant sources, including NAVTEX, the information recorded and relevant changes and trends passed between watch keepers? |  |
| Has examination of the cargo pipeline been made in order to detect any leaks? |  |
| Has the water in segregated ballast tanks been sampled/visually checked prior to discharge? |  |
| Have arrangements been made to monitor the offshore manifolds during starting of cargo operations and while increasing the loading/discharging pressure? |  |
| Are all cargo manifolds not in use being monitored for pressure build up? |  |
| Is the chicksan operating envelope being maintained (watch for surging)? |  |
| Are all cargo lines and valves properly set and cross-checked by a further member of staff? |  |
| Are all save alls clear, dry and plugged? |  |
| Is the safety equipment rigged and is protective equipment ready for use? |  |
| Is fixed gas monitoring equipment operational? |  |
| Are cargo tank high level and overfill alarm systems operational and their set points known to the officer on watch? |  |
| Shore / Terminal Emergency Shutdown (USC) at manifold is attended by deck watch? |  |
| Are the requirements and agreements made within the Ship Shore Safety Check List (SSSCL) being complied with, including communication protocols and repetitive checks? |  |
| The number of crew members on board and the presence of any other persons such as contractors performing work on board |  |

| COMPLETION OF LOADING: | ## Initials |
| --- | --- |
| Are cargo pipes drained and depressurized before disconnecting of loading arm? |  |
| Are all manifolds blanked? |  |

| Signed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Watch Officer |  |  |  | Date |

A ship’s watch conducted visual monitoring of the water around and behind the vessel for visible sheens, dust, chemicals, abnormal discoloration or foaming, and other indicators of pollutants or constituents of concern originating from the vessel. Attention paid to deck runoff and ballast water. Ref VGP 2013 Routine Visual Inspections (4.1.1)