THIS CHECKLIST IS TO BE COMPLETED BY THE CHIEF OFFICER PRIOR TO

ARRIVAL IN PORT AND COMMENCEMENT OF CARGO AND A LOG BOOK ENTRY MADE

|  | ## Initials* |
| --- | --- |
| Is the cargo change grade log book started? |  |
| Is remaining liquid from the cargo pipe system drained? |  |
| Have the maximum liquid residues been removed from tank before heating or boiling off? |  |
| Has all liquid in sumps been boiled off and temperature in sumps started to increase? |  |
| Has a positive temperature reading in the sump been obtained before shutting off the flow of hot gas? |  |
| Has the wind conditions been taken into account before venting to atmosphere? |  |
| Are inert gas/or nitrogen/or air being introduced and are cargo vapours discharging properly (depending on density)? |  |
| Is the pressure in the tanks lower than 0.3 barge during inerting or purging with dry air (when using onboard inert gas/dry air generator)? |  |
| Is the dew point of inert gas (or dry air) sufficient to prevent formation of water and ice during subsequent cool down? |  |
| Is the contamination of vapour in tanks less than LEL before opening of tank lids? |  |
| Are the tanks gas-free before washing? |  |

| Signed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Chief Officer |  |  |  | Date |