# ENCLOSED SPACE REGISTER

In accordance with IMO Resolution MSC.581(110) – Revised Recommendations for Entering Enclosed Spaces (2025)

| VESSEL NAME : |  | SHIP TYPE : |  |
| --- | --- | --- | --- |
| IMO NUMBER : |  | DATE OF ISSUE : |  |
| FLAG : |  | NEXT REVIEW DATE : |  |

Approved and Verified by Master

Name :

Signature :

# PURPOSE

This Enclosed Space Register is prepared in accordance with:

  - IMO Resolution MSC.581(110) – Revised Recommendations for Entering Enclosed Spaces (2025)

  - SOLAS

  - ISGOTT

All enclosed spaces on board the ship to be identified and recorded in Enclosed Space Register, which should be maintained on board the ship as well as ashore.

Hazards associated with the individual spaces identified and appropriate risk mitigation measures to be taken.

# PERIODIC REVIEW AND CONTROL

Vessel to ensure Enclosed space register is periodically updated and reviewed.

The Enclosed Space Register shall be:

Reviewed annually

Reviewed after modification of tanks/spaces

Reviewed after any enclosed space incident

Verified during internal audits

Shall be sent for review to office after any changes

# MANDATORY CONTROLS REQUIRED DURING ENCLOSED SPACE ENTRY

  - Enclosed Space Entry Permit HSSEQ-11 to be filled correctly

  - Ensure PTW, Lock out tag out and "Safe to enter"/"Unsafe to enter" signage arrangements are in place

  - Continuous atmosphere monitoring

  - Attendant stationed at entrance

  - Rescue team on standby

  - No entry during cargo, inerting, tank washing

  - SCBA available and ready for immediate use

  - Precautions shall be taken to ensure no personnel gets trapped in enclosed spaces

# ATMOSPHERE ACCEPTANCE CRITERIA

| Gas | Acceptable Limit |
| --- | --- |
| Oxygen (O₂) | 20.8 – 21.0 % |
| Flammable Gas | 0 % LEL |
| Carbon Monoxide (CO) | ≤ 30 ppm |
| Hydrogen Sulphide (H₂S) | ≤ 10 ppm |
| Carbon Dioxide (CO₂) | ≤ 0.5 % (5,000 ppm) |

# EMERGENCY RESCUE EQUIPMENTS

Below table to be updated ship specific as per correct storage locations

(items not limited to the following)

| Equipment | Location |
| --- | --- |
| SCBA Sets | Fire Station |
| Lifeline & Harness | Bosun Store |
| Rescue Tripod (if applicable) | Deck Store |
| Stretcher | Hospital |
| Portable VHF | Bridge Locker |
| Portable Oxygen Resuscitator | Hospital |
| Intrinsically safe Lamps | Deck Store |

LIST OF ENCLOSED SPACES (TO BE ENSURED SHIP SPECIFIC BY MASTER)

NAME OF VESSEL :	Port of Registry:	IMO number:

| SL. NO./ REF. NO. | SPACE NAME/ LOCATION | TYPE OF SPACE | REMARK |
| --- | --- | --- | --- |
| 1 | CARGO TANK NO.1P | CARGO TANK |  |
| 2 | CARGO TANK NO.1S | CARGO TANK |  |
| 3 | CARGO TANK NO.2P | CARGO TANK |  |
| 4 | CARGO TANK NO.2S | CARGO TANK |  |
| 5 | CARGO TANK NO.3P | CARGO TANK |  |
| 6 | CARGO TANK NO.3S | CARGO TANK |  |
| 7 | CARGO TANK NO.4P | CARGO TANK |  |
| 8 | CARGO TANK NO.4S | CARGO TANK |  |
| 9 | CARGO TANK NO.5P | CARGO TANK |  |
| 10 | CARGO TANK NO.5S | CARGO TANK |  |
| 11 | CARGO TANK NO.6P | CARGO TANK |  |
| 12 | CARGO TANK NO.6S | CARGO TANK |  |
| 13 | CARGO TANK NO.7P | CARGO TANK |  |
| 14 | CARGO TANK NO.7S | CARGO TANK |  |
| 15 | SLOP TANK P | CARGO TANK |  |
| 16 | SLOP TANK S | CARGO TANK |  |

Signature of Master ………………………………

# LIST OF ENCLOSED SPACES (TO BE ENSURED SHIP SPECIFIC BY MASTER)

NAME OF VESSEL :	Port of Registry:	IMO number:

| SL. NO./ REF. NO. | SPACE NAME/ LOCATION | TYPE OF SPACE | REMARK |
| --- | --- | --- | --- |
| 17 | BALLAST TANK FWD | BALLAST TANK |  |
| 18 | BALLAST TANK AFT | BALLAST TANK |  |
| 19 | BALLAST TANK NO. 1P | BALLAST TANK |  |
| 20 | BALLAST TANK NO. 1S | BALLAST TANK |  |
| 23 | BALLAST TANK NO. 2S | BALLAST TANK |  |
| 24 | BALLAST TANK NO. 2P | BALLAST TANK |  |
| 23 | BALLAST TANK NO. 3S | BALLAST TANK |  |
| 24 | BALLAST TANK NO. 3P | BALLAST TANK |  |
| 25 | BALLAST TANK NO. 4S | BALLAST TANK |  |
| 26 | BALLAST TANK NO. 5P | BALLAST TANK |  |
| 27 | BALLAST TANK NO. 5S | BALLAST TANK |  |
| 28 | BALLAST TANK NO. 6P | BALLAST TANK |  |
| 29 | BALLAST TANK NO. 6S | BALLAST TANK |  |
| 30 | BALLAST TANK NO. 7P | BALLAST TANK |  |
| 31 | BALLAST TANK NO. 7S | BALLAST TANK |  |

Signature of Master ………………

# LIST OF ENCLOSED SPACES (TO BE ENSURED SHIP SPECIFIC BY MASTER)

NAME OF VESSEL :	Port of Registry:	IMO number:

| SL. NO./ REF. NO. | SPACE NAME/ LOCATION | TYPE OF SPACE | REMARK |
| --- | --- | --- | --- |
| 32 | DOUBLE BOTTOM TANK | DB TANK |  |
| 33 | PUMP ROOM | MACHINERY SPACE |  |
| 34 | COFFERDAM P | COFFERDAM |  |
| 35 | COFFERDAM S | COFFERDAM |  |
| 36 | FO TANK | FUEL TANK |  |
| 37 | LO TANK | OIL TANK |  |
| 38 | VOID SPACE | VOID |  |
| 39 | CHAIN LOCKER P | EQUIPMENT SPACE |  |
| 40 | CHAIN LOCKER S | EQUIPMENT SPACE |  |
| 41 | SEWAGE TANK | WASTE TANK |  |
| 42 | PIPE TUNNEL | TRUNK SPACE |  |

Signature of Master ………………………………

( FOLLOWING SECTIONS SHALL BE RECORDED SEPARATELY FOR EACH ENCLOSED SPACE ENTERED IN THE REGISTER)

| NAME OF SPACE : |  |
| --- | --- |
| REF NO : |  |
| TYPE OF SPACE : |  |

PHYSICAL LAYOUT

(DIAGRAM TO BE ATTACHED)

| ACCESS POINTS : |  |  |
| --- | --- | --- |
| EXIT POINTS : |  |  |
| ADJACENT / CONNECTED SPACES : (LIST ALL ADJACENT /CONNECTED SPACES) |  |  |
| LIGHTING REQUIRED : | FIXED LIGHTING : | ☐ Yes ☐ No |
|  | TEMPORARY LIGHTING : | ☐ Yes ☐ No |
|  | INTRINSICALLY SAFE REQUIRED : | ☐ Yes ☐ No |

| VENTILATION ARRANGEMENTS |  |  |
| --- | --- | --- |
| NATURAL VENTILATION : (Please specify existing Natural Ventilation arrangements) |  |  |
| FORCED VENTILATION : | IS FIXED VENTILATION ARRANGEMENTS IN PLACE ? (If yes please specify) |  |
|  |  | ☐ Yes ☐ No |
|  | PORTABLE BLOWER STORAGE LOCATION : |  |
|  | VENT DUCTING LENGTH REQUIRED : |  |
| ESTIMATED TIME REQUIRED TO ACHIEVE AIR CHANGES FOR SAFE ENTRY USING FORCED OR NATURAL VENTILATION : |  |  |
| TANK CONDITION MONITORING : | Any remote advanced monitoring systems including fixed gas detectors installed in the required location ? | ☐ Yes ☐ No |
|  | If yes provide more details |  |
| TYPE AND DETAILS OF PORTABLE GAS DETECTOR TO BE USED : |  |  |
| PHYSICAL HAZARDS : (LIST ALL POSSIBLE PHYSICAL HAZARDS) |  |  |
| SPECIFIC HAZARDS : (THESE ARE SPECIFIC HAZARDS WITH RESPECT TO THE SPACE) |  |  |
| SPECIFIC OPERATIONAL RESTRICTIONS IF ANY : (IF NOT APPLICABLE PLEASE WRITE NA) | ( For example , Entry prohibited when heating coils in use) |  |
| PPE / SAFETY EQUIPMENTS REQUIRED : |  |  |

# MASTERS DECLARATION

I confirm that the enclosed space register onboard this vessel has been reviewed and verified in accordance with Company SMS and IMO requirements.

Master Name __________________

Date: ___________________                                                       Signature: ___________________