Note: The 2nd Engineer or Chief Officer is responsible for issuing this permit

Location of work:

Reason for proposed work:

Description of Work:

Tools to be used for work:

- Work permitted as per SMS instruction: _____________________________ Yes   No

Special Conditions:

Tool operator(s):

- Permit Validity

This permit is valid for maximum of 12 hours

Permit valid:

From (Date / Time): 				  To (Date/Time):

- Preparation

Task and location-specific JSA (JSA Ref # :           )	Completed

Tool operator(s)	Trained

Fatigue management (rest hours)	Checked

Tool integrity	Inspected

Portable power cutting tools

Check atmosphere for explosive gases, if tools are used outside the ER workshop

Ensure correct cutting disc is used for intended material

Check RPM rating is correct

Check condition of cutting disc

Check securing of cutting disc to the machine

Confirm guards in place

Confirm handles in place

Check condition of electrical cable or air hose and fittings

Test operating lever and/or emergency stop function

- Disconnect power & secure equipment during breaks

Hydro blaster

Check safety devices, including emergency stop and relief/ by-pass valves are working

Check condition of electrical fittings – electrically continuously conductive and grounded

Work area verified gas free < 1% LFL

Check condition of hose and fittings

Check pressure setting for intended use/purpose

Select correct nozzles, fittings and handles

Ensure adequate supply of fresh water and fittings/arrangement

Post ‘WARNING’ signs around pressure hoses

Display ‘NO GO AREA’ around the work site

Supervisor	Identified

Other permits to work	Issued

Tankers – no cargo operations, tank cleaning, gas freeing, purging or inerting in progress. All vessels–no bunkering in progress	Confirmed

Note:	Other permits include Hot Work, Enclosed Space Entry, Isolation Control, and any other required by HSE Permit to Work System.

Tool	Issued

- Safety

- Correct Personal Protective Equipment	Donned

Tool Box Talk	Held

- Personal Statement and Authorization

This permit has been completed and found in order.

| Name: | Responsible Officer | Signature |
| --- | --- | --- |
|  | Rank: |  |
| Date: | Time: |  |

I will follow instruction and training given by Responsible officer.

| Name: | Rank: | Signature |
| --- | --- | --- |

- I authorize the use of tools to perform the work listed above:

- Name:   ______________________ (Master)            Signature: _______________________

- Date:      ______________________

- Time:      ______________________

Permit Closure

- Work	Completed

Tool(s)	Secured

Remove tool(s) from work area and return to storage area

Inspect tools for damage

Permit	Closed for filing

| Name: | Responsible Officer | Signature |
| --- | --- | --- |
|  | Rank: |  |
| Date: | Time: |  |

Completed