| SYSTEM - EQUIPMENT TEST DESCRIPTION | DATE & TIME |
| --- | --- |
| A compatibility study is available onboard and has been taken into consideration, no any issues regarding forthcoming operation are pending (e.g manifold connections, mooring procedure, gangway & ESD compatibility etc) | ☐ OK |
| A detailed discharging and ballasting plan to be prepared, approved by Master and signed by all participant Officers | ☐ OK |
| Check that cargo tanks pressure & temperature are within receiving terminal’s acceptable limits. MAX PRESSURE: MAX TEMP: | ☐ OK |
| V-PIN verification to be carried out on CTS as per maker’s Instructions (As applicable) | ☐ OK |
| Check loading computer (Cargo max) for proper function (test results have been verified within last 3 months) | ☐ OK |
| Check Fire pumps and spray pump proper function. Test manifolds water curtains. Compare local and remote pressure and valve position indications. | ☐ OK |
| Dry powder system visual inspection, check that monitors are aiming at Manifolds. | ☐ OK |
| Check hydraulic units for Cargo and Ballast valves remote control and change from Top-up pump, to one Main pump per unit (1 for Cargo and 1 for Ballast) | ☐ OK |
| Check and test all manually and remotely operated valves related to the next cargo operation. Compare local and remote valve position indications. | ☐ OK |
| Check ESD, telephone, earthing and connections on the manifold box; clean if necessary. (ESD to be tested from various points including melting fuses in a rotating basis. Records to be kept as per form LNG 02 | ☐ OK |
| Test Ship-Shore Link ESD activation, by simulation, utilizing each system test devices as applicable for the following: - Optical link (including communications) - Electrical ESD (Miyaki/Pyle) as applicable - Pneumatic ESD Test both Ship->Shore and Shore->Ship activation of ESD | ☐ OK |
| Performed GCU testing of Start/Stop sequence with minimum Gas Flow. (Applicable to 155 K vessels only) | ☐ OK |
| Check ESD system closing time of manifold valves (within 30 sec) | ☐ OK |
| Check CTS & IAS high/low cargo tank level alarms. | ☐ OK |
| Check very high level (CTS & IAS), alarms and close function of filling valve. | ☐ OK |
| Check Independent (Omicron) High-High and Extremely High level alarms and ESD function. - Verify with alarm list in IAS and after complete the check list. - Allow 5 sec delay between testing of HHL & EHL alarms. | ☐ OK |

| VESSEL: | PORT/TERMINAL: |
| --- | --- |

| The ESD system’s electric circuit, and in particular the control relays, to be inspected. | ☐ OK |
| --- | --- |
| Compare local and remote vapor pressure indications for: Cargo tanks & vapor header, Cargo Machinery room and Cargo Manifolds gauges. | ☐ OK |
| Lower the floats and check the secondary sounding system’s (Whessoe) proper function. (Retract the floats and secure in stowing position after checking). (As applicable) | ☐ OK |
| Check that sufficient and suitable (as per with terminal’s agreement if any) manifold gaskets are available. | ☐ OK |
| Check bolts and nuts torque on any blanks at both port & starboard manifolds. Repack/re-tight any leak flanges during previous operation. | ☐ OK |
| Check liquid lines for cracked nuts. | ☐ OK |
| SYSTEM - EQUIPMENT TEST DESCRIPTION | DATE & TIME |
| Place proper tools at manifolds | ☐ OK |
| Cargo and spray pumps to be Megger tested. | ☐ OK |
| Test all cargo and spray pumps. Compare local and remote pressure indications. | ☐ OK |
| The junction boxes of submerged electric pumps should be visually inspected prior to each discharge | ☐ OK |
| Check fixed gas detection units for proper function. | ☐ OK |
| Check all portable atmosphere gas instruments for proper operation as per maker’s recommendation. | ☐ OK |
| Check that GN2 supply system is working properly (Barriers/purging) | ☐ OK |
| Check N2 purging lines on deck, manifolds and at vent masts | ☐ OK |
| Check the GN2 generators work properly. Both units shall be started. | ☐ OK |
| Check cofferdams heating system | ☐ OK |
| Check all ballast pumps for proper working Compare local and remote pressure and valve position indications. | ☐ OK |
| Check that manifold and liquid domes dip trays are not filled with water. | ☐ OK |
| Check that manifold strainers are clean, of proper mesh size and ready for use | ☐ OK |
| Tightness test with soapy water have been carried out at Manifold connections between ESD valve and blind flange pressurized with Nitrogen at 5 bar. | ☐ OK |

| Pressurize offshore (seaside) manifold connections between ESD valve and blind flange with Nitrogen (not more than 5 bars). | ☐ OK |
| --- | --- |
| Check CCTV system and each camera for proper operation. Verify that washing water reservoir is full | ☐ OK |
| Check condition and proper operation of Electric motor and Cargo Machinery room fans. Test air locker in Electric motor room (as applicable). Inspect the doors. | ☐ OK |
| Check the fresh water supply on deck. Drain the FW piping when freezing temperatures are expected. | ☐ OK |
| Check the condition of scruppers on deck. Verify their tightness. | ☐ OK |
| Carry out all IAS and cargo equipment alarm and lamp tests in Cargo Control room and cargo machinery room (as applicable) | ☐ OK |
| Check that all printers related to cargo system are working properly. Verify availability of ribbons, cartridges and paper | ☐ OK |
| Check all intercommunication systems (portable walkie-talkies included) for proper function | ☐ OK |
| Check that all proper signs, labels and posters related to cargo operations are properly posted | ☐ OK |
| Check that computer for mooring lines tension monitoring system is working properly. Software for the terminal is available | ☐ OK |
| Check for any inhibited and / or disabled alarm in IAS | ☐ OK |
| Ensure that a refresh training for the participants in arms connection & disconnection procedure has been carried out at manifold platform and recorded | ☐ OK |
| Ensure that Port Contact information sheet is filled in and is posted at the CCR | ☐ OK |
| Ensure that Charterers Contacts as derived from Voyage Instructions are posted in the CCR | ☐ OK |
| Are all cargo lines and valves properly set and cross-checked by a further member of staff? (alongside) | ☐ OK |

| CARGO ENGINEER |  | CHIEF OFFICER |
| --- | --- | --- |
| (Name & Signature) |  | (Name & Signature) |