VESSEL					VOY NBR	     DATE		PORT/TERMINAL

PART 1 Ship / shore compatibility information

- Reference to ship-shore compatibility study including any special precautions required for the particular operation.

Port info database has been checked and all available information has been reviewed and understood.

The latest terminal’s operations manual is available onboard (to be confirmed with local agent).

Tick box to confirm ……………………            ☐

- UKC -Drafts – condition

| Under keel clearance limitations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max allowable draft |  | Arrival draft |  | Sailing draft |  | Water density |  |
| Hull stress, sea condition on arrival |  |  |  | Hull stress, sea condition on departure |  |  |  |
| BM: | SF: |  | GoM: | BM: | SF: |  | GoM: |
| Hull stress, sea condition on 25% of loading |  |  |  | Hull stress, sea condition on 50% of loading |  |  |  |
| BM: | SF: |  | GoM: | BM: | SF: |  | GoM: |
| Hull stress, sea condition on 75% of loading |  |  |  |  |  |  |  |
| BM: | SF: |  | GoM: |  |  |  |  |

Critical areas where cargo level will be out of sloshing limits have been identified. A spreadsheet with calculations related to required times and quantities for internal cargo transfer or additional loading is available in CCR for quick reference in case of urgent early departure Tick box to confirm …………………… ☐

| Lower Tank Sloshing Limits |  |  |  | Upper Tank Sloshing Limits |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Tanks mm |  |  |  | 70% Height of Tank Sloshing Limit |  |  |
| Cargo Tank | Capacity m3 | Sounding (mm) |  | Cargo Tank | Capacity m3 | Sounding (mm) |
| 1 |  |  |  | 1 |  |  |
| 2 |  |  |  | 2 |  |  |
| 3 |  |  |  | 3 |  |  |
| 4 |  |  |  | 4 |  |  |
| Total |  |  |  | Total |  |  |

- Ship / Shore Arms arrangement & Operating clearances

|  | SHORE | SHIP |
| --- | --- | --- |
| Number |  |  |
| Service (L or V) from Forward |  |  |
| Coupling Type |  |  |
| Nominal Diameter (in / cm) |  |  |
| Flange Standard and Type |  |  |
| Gaskets | Type : Ship ( ) or Shore supply ( ) |  |

| Operating clearances | SHORE | SHIP |  |
| --- | --- | --- | --- |
| Ship’s side to manifold flange face |  |  |  |
| Distance between manifold centers |  |  |  |
| Manifold operating clearances in mtrs | Upper: Low: | B: | L: |
| Arm Envelope counterweight operating clearance (range) in mtrs |  |  |  |
| Pre warning Alarm /ESD1 /ESD2 in mtrs | / / |  |  |

- Mooring arrangement

Berth layout description:

Mooring Arrangement:

Shore’s Gangway Operating Clearances:

| Low tide laden condition : | High tide ballast condition : |
| --- | --- |

- Ship / shore ESDS and communications compatibility (state if portable radio provided by shore and if terminal representative remains in CCR)

- Security Status

Current Security Level under which the Port is Operating:

PART 2 Preliminary preparation

1. Cargo operations / pre arrival checklist loading to be filled in and signed.

2. Pre loading meeting is to be held together with the terminal representatives. The ship/shore safety check list to be filled in and signed.

3. When the loading arms are connected to the ship’s manifold, the discharging arms are pressurized with N2 and purged to atmosphere until 02 content is below 1%. The connections are then tested for leaks using soapy water.

PART 3 Cargo operation

CARRY OUT A WARM ESD AND CUSTODY TRANSFER MEASUREMENT (CTM)

- Cooling down cargo tanks – Tanks will be considered as cooled and ready to load when tank bottom  & average tank temperature is - 130 degrees Celsius or colder. (*)

- Cooling down of liquid lines – Lines will be considered cooled when all sensors show  temperature -100 degrees Celsius or colder. (*)

(*) Attention should be given to any specific terminal’s requirement which is stated below :

| Prior arrival ☐ | Alongside ☐ |
| --- | --- |

Describe operating procedure for cooling down cargo tanks prior to arrival or alongside terminal (state estimated duration and quantity to be used) form VSL-D-048 to be filled in

Line-Up (as applicable)

LD compressor

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

Boil-off warm up heater

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

Vapour header

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

Spray line header

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

(*)Line set up has to be confirmed by a minimum of two respective officers (e.g Chief Officer, Cargo Engineer, OOW) in a priority of Seniority, starting from the lower rank and to be recorded in COB.

- Arms cool down before loading

Note: There is a number of loading terminals (usually where transshipped cargoes are loaded) where vessel is required to arrive with her deck lines cooled. Procedures and line up procedures to be included here as applicable.

Describe operating procedure for cooling the liquid lines (state estimated time)

Line-Up

Tank valves

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

HD Compressors

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

Manifold

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

(*)Line set up has to be confirmed by a minimum of two respective officers (e.g Chief Officer, Cargo Engineer, OOW) in a priority of Seniority, starting from the lower rank and to be recorded in COB.

CARRY OUT A COLD ESD TEST ON COMPLETION OF ARM(S) COOLDOWN

DESCRIBE EMERGENCY STOP PROCEDURES

DESCRIBE ACTIONS TO BE TAKEN IN THE EVENT OF A SPILL

- Loading

| Cargo data sheet to be attached | ( ) YES ( ) NO |
| --- | --- |
| Flammability, toxicity and hazards of the cargo reference |  |
| Confirm that all protective equipment as per PPE Matrix is available, in good condition and will be used accordingly. |  |

| Quantity to be loaded |  |
| --- | --- |
| Cargo density |  |
| Cargo temperature |  |
| Estimated time |  |
| Maximum loading rate according to port information and terminal’s manual |  |
| Notice of rate change |  |
| Maximum loading rate by vessel |  |
| Maximum loading manifold pressure |  |
| Allowable limits for tanks pressure range |  |
| Vapour return pressure adjustment (HD) |  |
| Final volume inside the tank will be within acceptable sloshing limits and allowance for maximum NBOG (as a minimum) during intended voyage has been taken into consideration | ( ) Tick to confirm |

|  | Upper Tank Sloshing Limits |  |  | Final sounding |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cargo Tank filling sequence | Capacity m3 | Sounding (mm) | Capacity m3 | Sounding (mm) |  |
|  | Total |  |  |  |  |  |

Describe operating procedure for loading cargo with specific reference in critical stages of operations (ramp up, ramp down, topping off, or others based on specific operation)

Line-Up

Cargo tanks

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

HD compressors

Manifolds

(*)Line set up has to be confirmed by a minimum of two respective officers (e.g Chief Officer, Cargo Engineer, OOW) in a priority of Seniority, starting from the lower rank and to be recorded in COB.

Draining and inerting manifold pipes and loading arms procedure (describe the steps throughout the operation) After the liquid draining is complete and the hydrocarbon meters read less than 1% by volume close the respective valves and proceed with arm disconnection. Arms to be disconnected one by one, the gasket must be installed and at least four bolts inserted and tightened in the blind flange before the next arm can be disconnected. Upon arms’ disconnection ensure that all flanges are fully bolted and secured. Same need to be documented within applicable PTW documents and forms and relevant entry should be also made in C.O.B. (Cargo Operation Book).

Line-Up

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

(*)Line set up has to be confirmed by a minimum of two respective officers (e.g Chief Officer, Cargo Engineer, OOW) in a priority of Seniority, starting from the lower rank and to be recorded in COB.

PART 4 De-Ballasting

| Quantity to de-ballasted |  |
| --- | --- |
| Maximum de-ballasting rate |  |
| Estimated duration |  |

- Operating procedure to de-ballast the ship (describe the steps throughout operation ballasting by gravity, by pumping, BWTS, etc.)

Line-Up

| POSITION | DESCRIPTION | VALVE |
| --- | --- | --- |

(*)Line set up has to be confirmed by a minimum of two respective officers (e.g Chief Officer, Cargo Engineer, OOW) in a priority of Seniority, starting from the lower rank and to be recorded in COB.

Confirm the awareness by signing this form

| Prepared by Chief Officer |  | Approved by MASTER |
| --- | --- | --- |

Cargo Engineer

| 2nd Officer (A) | 2nd Officer (B) | 2nd Officer (C) |
| --- | --- | --- |

PART 5 Bunkering

* In case bunkering is scheduled to be carried out alongside (not to be conducted simultaneously with Cargo Operations), otherwise to be marked as N/A

| Bunker grades & quantities |  |
| --- | --- |
| Maximum bunkering rate |  |
| Estimated duration |  |

- Bunkering procedure

Reference to be made in existing Bunkering Forms and Check lists

Confirm the awareness by signing this form otherwise to be marked as N/A

Chief Engineer

2nd Engineer

| 3rd Engineer (A) | 3rd Engineer (B) | 4th Engineer |
| --- | --- | --- |