|  | Yes | No |
| --- | --- | --- |
| When departure is confirmed, advise Chief engineer and second engineer |  |  |
| One hour before departure, ask permission from duty officer/captain to disengage turning gear for main turbine warming up (auto spinning) |  |  |
| Check if Diesel Generator is in st-by mode |  |  |
| If not in service, warm up and start TA (3 Generators should be in bus bar for bow thrusters) |  |  |
| Check alarm panels of main turbine, reset all alarms |  |  |
| 15 minutes before stand by, call bridge and wait confirmation for testing telegraph; A short time dead-slow astern on "ECR", and a short time dead-slow ahead on "wheelhouse control" and after that, keep main turbine control on "bridge control" position. |  |  |
| Take all relevant counters at official 'St by'-time |  |  |
| Take all relevant counters at official 'SSP''-time |  |  |
| After SSP, reduce electrical power consumption (fire pump, bow thrusters) with the permission of the bridge, and stop unnecessary turbo alternators |  |  |
| Shift sea suction from high to low |  |  |
| Shift MGPS valves |  |  |
| Start evaporators |  |  |
| Operate all soot blowers |  |  |
| Switch on cathodic protection system (fwd & aft) |  |  |
| After SSP, the engineer on duty has to make a serious round in engine room (check list) especially during night time |  |  |

Date:								Port:

Engineer on duty						Chief Engineer

|  | SBE | At anchor | Anchor up | SSP | ISP | RSP |
| --- | --- | --- | --- | --- | --- | --- |
| Time |  |  |  |  |  |  |
| M/T rev. counter |  |  |  |  |  |  |
| Boiler fuel counter |  |  |  |  |  |  |
| DG DO counter |  |  |  |  |  |  |
| ROB : HFO |  |  |  |  |  |  |
| MDO |  |  |  |  |  |  |
| LDO |  |  |  |  |  |  |
| Flash FWG counter |  |  |  |  |  |  |
| Sub. FWG counter |  |  |  |  |  |  |