To be completed before departure from berth or anchorage.

| Port: |  | Vessel name: |
| --- | --- | --- |
| Date: |  |  |

| # | Item | Yes | No | N/A |
| --- | --- | --- | --- | --- |
| 1 | Passage plan |  |  |  |
| a | Berth to berth passage plan for the intended passage prepared and available on the bridge with the route plotted on up to date and appropriate scale charts (official paper or electronic) |  |  |  |
| b | Passage plan checked and approved by master |  |  |  |
| c | Passage plan briefed to the bridge team including Chief Engineer prior departure from port or shifting form anchorage. C/E to confirm status of aux. machineries and propulsion system to master and bridge team to ensure state of readiness. |  |  |  |
| d | Are charts and publications for the intended voyage latest editions/corrected and up to date? Latest notice to mariner (Week No.): |  |  |  |
| e | Has checklist A and B of the passage plan been completed including Master declarations (environmental sensitive area identified, UKC, etc)? |  |  |  |
| f | Are courses, dangers, passage plan notes marked on charts for voyage? |  |  |  |
| g | Weather for the intended voyage has been obtained and this has been taken into account when preparing the passage plan |  |  |  |
| h | Is the Sat C equipment selected to the correct Navarea with respect to vessel’s geographical position? |  |  |  |
| i | Are loose items on deck, in storerooms and engine room properly lashed? |  |  |  |
| 2 | Equipment checks Has the following equipment been checked and found ready for use? |  |  |  |
| a | Anchors, cables and winches including clearing away |  |  |  |
| # | Item | Yes | No | N/A |
| b | Ancillary bridge equipment - binoculars, loud hailer, flags etc. |  |  |  |
| c | Bridge movement book/bell book |  |  |  |
| d | GMDSS log |  |  |  |
| e | Course recorder (marked with port, date and time) |  |  |  |
| f | LRIT |  |  |  |
| g | Deck machinery power |  |  |  |
| h | Echo sounder |  |  |  |
| i | Electronic navigational position fixing & monitoring aids |  |  |  |
| j | Gyro/magnetic compass and all repeaters correct and aligned including steering room repeater (if fitted) |  |  |  |
| k | Window wipers/clear view screens |  |  |  |
| l | All required flags are on board |  |  |  |
| m | Chart management tool (Chartco, Novaco Bridge Program or other similar software devices) |  |  |  |
| n | Navtex |  |  |  |
| o | Radar and associated plotting aids, including heading marker alignment |  |  |  |
| p | Speed and distance recorder |  |  |  |
| q | AIS (update information for next voyage, ETA etc.) |  |  |  |
| r | BNWAS (switch on and enter date and time in ship’s logbook) The bridge navigational watch alarm system shall be in operation whenever the ship is underway at sea or at anchor. |  |  |  |
| s | Water ingress alarm (for bulkers) Bilge alarms (engine room, pump room, fore peak) |  |  |  |
|  | IG pressure (for oil tankers only) Hospital call alarm |  |  |  |
| t | VDR/S-VDR |  |  |  |
| # | Item | Yes | No | N/A |
| u | Communications equipment – GMDSS, external, internal (particularly bridge to engine room/bridge to mooring stations) and portable Company emergency contact details - International SOS details (Primary) CIRM details (Secondary) |  |  |  |
| v | Visual checks of EPRIB, SSAS, SART |  |  |  |
| w | Bow/stern thruster |  |  |  |
| x | Bridge and engine room telegraphs, including revolution indicators including “Time” as being displayed on the telegraph printer |  |  |  |
| y | Engine movement recorder |  |  |  |
| z | Controllable pitch propeller controls and indicator |  |  |  |
| aa | RPM & ROT indicator |  |  |  |
| ab | Stabilisers |  |  |  |
| ac | Navigation lights and shapes including “Not Under Command” and “Anchor” |  |  |  |
| ae | Ship’s whistle |  |  |  |
| ad | Signalling lamps & Search light |  |  |  |
| ae | Fire detection & alarm system |  |  |  |

| # |  | Item | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
| af |  | P.A. System and Emergency Alarms Signals |  |  |  |
| ag |  | Anemometer |  |  |  |
| 3 | Steering gear Including manual/auto change-over arrangements checked as per SOLAS chapter V: safety of navigation regulation: 26 as stated below: within 12 hours before departure, the test procedure shall include, where applicable, the operation of the following: |  |  |  |
| a | Main steering gear |  |  |  |
| b | Auxiliary steering gear |  |  |  |
| c | Remote steering gear control systems |  |  |  |
| d | Steering positions located on the navigation bridge |  |  |  |
| e | Emergency power supply |  |  |  |
| f | Rudder angle indicators in relation to the actual position of the rudder |  |  |  |
| g | Remote steering gear control system power failure alarms |  |  |  |
| h | Steering gear power unit failure alarms |  |  |  |
| i | Automatic isolating arrangements and other automatic equipment |  |  |  |
| j | Full movement of the rudder according to the required capabilities of the steering gear |  |  |  |
| # |  | Item | Yes | No | N/A |
| k | Visual inspection for the steering gear and its connecting linkage |  |  |  |
| l | Operation of the means of communication between the navigation bridge and steering gear compartment |  |  |  |
| m | Rudder angle indicator including bridge wing indicators (and illumination) |  |  |  |
| 4 | Have the ship’s clocks, course and engine order recorders been synchronised? |  |  |  |
| 5 | Is the crew advised of “Stand-by time”? |  |  |  |
| 6 | Has the draft, stress and stability been checked and entered into the deck log? |  |  |  |
| 7 | Is the draft and departure condition displayed on the bridge? |  |  |  |
| 8 | Has the main engine been tested? (Ahead/astern movements) (Master must clearly specify when ME is to be tried out. e.g. After Pilot on board, loading arms disconnected, gangway, ramps, derricks, crane housed, tug fast, etc) |  |  |  |
| 9 | Last emergency engine stop test: (Date) |  |  |  |
| 10 | Has all watertight opening/watertight doors on main deck area been secured for sea? |  |  |  |
| 11 | Prior departure US port – Has the E-NOD been submitted to NVMC with copy to QI and agents? (If applicable) |  |  |  |
| 12 | ECDIS |  |  |  |
| a | Confirm primary and secondary ECDIS operational? |  |  |  |
| b | Has the ECDIS passage plan for the intended voyage been prepared? |  |  |  |
| c | Has the ECDIS passage plan preparation checklist been completed? |  |  |  |
| d | Has safety check for the ECDIS voyage route carried out and route put on monitoring? |  |  |  |
| e | Has the safety check been verified independently by 2 officers? |  |  |  |
| f | Are all charts and ENC’s for the intended voyage and other nautical publications up to date with courses laid off? |  |  |  |
| g | Are latest ECDIS charts (as applicable, namely, ENC, British Admiralty ARCS) available on both primary and secondary ECDIS and that any position input is set to applicable WGS? Ensure no offset is applied to the position? |  |  |  |
| h | Verify deep contour setting value as per company UKC policy incorporated in ECDIS? |  |  |  |
| i | Verify safety contour setting value as per company UKC policy incorporated in ECDIS |  |  |  |
| j | Verify Safety depth setting value as per company UKC policy incorporated in ECDIS |  |  |  |
| k | Verify Shallow contour setting value as per company UKC policy incorporated in ECDIS |  |  |  |
| l | Appropriate area layers selected for ENC chart in chart settings |  |  |  |
| m | All area alarm are active on the ECDIS in the alarm settings |  |  |  |
| n | Has sensor inputs to the ECDIS verified for data accuracy with respective equipment? |  |  |  |
| o | Have the latest navigational warnings and weather forecast for the area been received and same plotted on ENC? |  |  |  |
| p | On ECDIS – Have the relevant user maps been selected and displayed? |  |  |  |
| q | On ECDIS – Are all NAVTEX warnings within 15Nm either side of the track are plotted? |  |  |  |
| r | Has safety settings/configuration updated correctly in ECDIS for the passage? |  |  |  |
| # | Item | Yes | No | N/A |
| 13 | Port and pilotage |  |  |  |
| a | Master/Pilot information exchange checklist completed |  |  |  |
| b | Pilot card prepared |  |  |  |
| c | Pilot boarding time confirmed |  |  |  |
| c | Port and VTS channels monitored |  |  |  |
| d | Port VTS and Pilot advised of any special requirements |  |  |  |
| e | Preparations for pilotage complete |  |  |  |
| f | Bridge embarkation of pilot checklist completed |  |  |  |
| Additionally for offshore/DP vessels |  |  |  |  |
| g | Below point discussed with the bridge team: - Vessel shall not use Auto Pilot, while manoeuvring in Ports, Narrow Channels and when arriving and departing from port / Berth or areas with heavy traffic” - Master/Bridge team to have utmost attention to prevailing weather conditions and its effect on the vessel, in the event of an unexpected loss to propulsion or vessel position keeping capability. This should be included in the passage planning briefings and risk assessment. - DP checklist shall be completed when vessel is using the DP joystick. |  |  |  |
| 14 | Before sailing |  |  |  |
| a | All crew on board |  |  |  |
| b | Bridge team fit for duty |  |  |  |
| c | Engine room ready |  |  |  |
| d | MSI checked and communicated to bridge team |  |  |  |
| e | Pressure on fire main |  |  |  |
| f | Stowaway/security search completed |  |  |  |
| g | Anchors cleared away |  |  |  |
| h | Mooring stations manned and ready |  |  |  |
| 15 | Securing for sea |  |  |  |
| a | Cargo and cargo handling equipment secure |  |  |  |
| b | Cargo/passenger details available |  |  |  |
| c | Hull openings secure and watertight |  |  |  |
| d | Stability and draught information available |  |  |  |
| e | Watertight doors closed |  |  |  |
| f | Anchors secured and lashings verified to be tight. Extra lashings if heavy weather is expected en-route as per weather routing and Navarea warnings. |  |  |  |
| g | Pilot ladders secured and safely stowed (FOM 3.12) |  |  |  |

| Completed by: |  | Verified by Master: |  |
| --- | --- | --- | --- |
| Name: |  | Name: |  |
| Rank: |  | Location: |  |
| Date: |  | Date: |  |