To be completed when conditions exceed or are forecast to exceed Beaufort Force 6. (Compliance to be noted in deck log book at the end of each watch and checklist to be signed by Duty Officer)

| Date: | Vessel Name: |
| --- | --- |

| # |  | Item |  | Yes |  | No |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Have the following been informed: |  |  |  |  |  |
| a |  | Master |  |  |  |  |  |
| b |  | Engine Room |  |  |  |  |  |
| c |  | All crew |  |  |  |  |  |
| d |  | Marine Superintendent in charge (To be informed by the Master) |  |  |  |  |  |
| 2 |  | Have all moveable objects been secured at the following locations? |  |  |  |  |  |
| a |  | Lashings on monorail crane and provision crane rechecked? |  |  |  |  |  |
| b |  | Stores / spares, equipment, etc. stowed on deck secured? |  |  |  |  |  |
| c |  | Forecastle store checked and additional lashing applied as necessary? |  |  |  |  |  |
| d |  | Paint locker checked and all paint secured? |  |  |  |  |  |
| e |  | Steering gear room checked, and additional lashings applied as necessary? |  |  |  |  |  |
| f |  | Engine room checked and additional lashings applied as necessary |  |  |  |  |  |
| g |  | Aft rope store checked, and additional lashings applied as necessary? |  |  |  |  |  |
| h |  | Portable gangways / accommodation ladders secured? |  |  |  |  |  |
| i |  | Pilot ladders and hoists lashed and secured + additional lashings taken as required? |  |  |  |  |  |
| j |  | Chief cook notified and provision rooms (cold and dry) galley and mess rooms (crew and officers) secured? |  |  |  |  |  |
| k |  | Furniture and appliances in all common / public (smoke rooms, offices, conference rooms) areas secured? |  |  |  |  |  |
| l |  | Additional lashings on PC’s, monitors, copy machines and printers. |  |  |  |  |  |
| m |  | Bridge prepared for rough weather? |  |  |  |  |  |
| n |  | Anchor lashing checked (additional applied), spurling pipe cemented? |  |  |  |  |  |
| o |  | All mooring ropes and wires on winches are secured and covered. |  |  |  |  |  |
| p |  | Lifeboat lashings checked? |  |  |  |  |  |
| q |  | Liferaft lashings checked? |  |  |  |  |  |
| r |  | Cranes / derricks / grabs lashed and checked? |  |  |  |  |  |
| s |  | Oxygen acetylene bottles rechecked and lashed and in appropriate place. |  |  |  |  |  |
| t |  | Ventilation for bow thrusters closed |  |  |  |  |  |
| u |  | All Lube oil drums have been secured inside stores and NO drums are lashed on deck. |  |  |  |  |  |
| 3 | Change over to hand steering and switch on both steering motors |  |  |  |  |  |
| 4 | Has the need to readjust / take additional ballast been assessed |  |  |  |  |  |
| a | Loading condition verified (Trim, GM, stress, tank sloshing, propeller immersion, freeboard)? |  |  |  |  |  |
| 5 | Has the bridge manning been enhanced, and appropriate entries made in the deck log. |  |  |  |  |  |
| 6 | Have all watertight doors shut/Watertight integrity checked? |  |  |  |  |  |
| a | Tank dooms confirmed tight shut |  |  |  |  |  |
| b | Check watertight integrity of weather deck, tighten lashings and dogs of doors and hatches |  |  |  |  |  |
| c | Hatch cover locking devices rechecked? |  |  |  |  |  |
| d | Dampers for cargo hold ventilation closed? |  |  |  |  |  |
| e | All closable vents on deck and around accommodation have been checked closed and tight. Special attention to bunker tank vents and sounding pipes |  |  |  |  |  |
| 7 | Have speed and course been adjusted as necessary and manoeuvre the ship to minimise the risk of pitching, rolling and pounding? |  |  |  |  |  |
| 8 | All personnel have been specifically instructed not to go out on deck without permission of the Master. If it is unavoidable, then comply with below- |  |  |  |  |  |
| a | Specific risk assessment to be prepared and notify Marine Superintendent. Comply with HSSEQ-37 |  |  |  |  |  |
| b | Enhanced PPE (life vest / lifejacket / lifeline as applicable) to be used whenever proceeding out on deck during adverse weather. |  |  |  |  |  |
| c | Limit number of personnel going on deck |  |  |  |  |  |
| d | Alteration of course to achieve safe working conditions for crew to perform task. Good seamanship is to keep swell on opposite side of the task (e.g. port or starboard quarter if working at forward so as to reduce shipping seas). |  |  |  |  |  |
| 10 | Have instructions been issued on the following matters: |  |  |  |  |  |
| a | Hourly log entries of meteorological conditions prevailing, and vessels behaviour made? |  |  |  |  |  |
| b | In hours of darkness if vessels behaviour is causing OOW concern or if in doubt as to the severity of the weather, Master to be immediately called? |  |  |  |  |  |
| c | Transmitting weather reports to appropriate authorities or, in the case of Tropical Storms, danger messages in accordance with SOLAS 1974 Chapter V, Regulation 2(a) |  |  |  |  |  |

| # | Item | Yes | No |
| --- | --- | --- | --- |
| 11 | Following to be ensured in Machinery spaces (Engine Room) |  |  |
| a | Sump oil level for ME, AE, Air Comp and other machineries to be maintained adequate to prevent lube oil pump from losing suction in case of rolling. If required, top up the oil to maintain near higher end of the allowed range. |  |  |
| b | Closely monitor and control the ME load. If required, reduce RPM to maintain the load within allowable limits on the speed load diagram. In case torque limits are getting activated on Main Engine, cancellation should not be carried out for longer duration than recommended by maker, to prevent overloading of the engine. |  |  |
| c | Chief Engineer to review if there is need to keep generators running in parallel, and ensure that additional generator is started, if required. |  |  |
| d | Chief Engineer to review if there is need to keep Engine Room manned, and ensure that watch-keeping is maintained, if required. |  |  |
| e | Fuel Oil tanks to be drained properly and regularly to remove any water and sludge content. |  |  |
| f | Engine Room tank contents to be maintained at appropriate levels to avoid frequent alarm due to rolling (as far as practical). |  |  |
| g | Boiler water level to be maintained at appropriate levels to avoid frequent alarm due to rolling. |  |  |
| h | All sounding pipe caps to be kept closed with deadweight cocks in place and free (ensured to be in closed position). |  |  |
| i | Control Air lines to be drained properly to remove water. |  |  |
| j | All heavy spares and store items to be properly lashed, and tools in workshop well secured. |  |  |
| 12 | Have you informed your Marine Superintendent and Vessel Manager? |  |  |

| Completed by: |  | Verified by Master: |  |
| --- | --- | --- | --- |
| Name: |  | Name: |  |
| Rank: |  | Date / Time: |  |
| Date / Time: |  | Location: |  |