To be completed at the change-over of sailing navigation watches.

When the tests or checks are completed, a remark shall be made in the deck Logbook with the checklist name, time completed and signature. In such case filing of this checklist is NOT required

(N.B: CHANGEOVER SHOULD BE POSTPONED WHEN THE SHIP IS, OR IS ABOUT TO BE, ENGAGED IN A COLLISION AVOIDANCE MANOEUVRE OR A NAVIGATIONAL ALTERATION OF COURSE)

| Vessel Name: |  |  | Date: |  |
| --- | --- | --- | --- | --- |

| # | Item |  |  | Y/N/NA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| 1 | ECDIS System Confirm ECDIS system primary and back up computer running and Display not frozen. |  |  |  |  |  |  |
| 2 | Approved Passage plan loaded and both ECDIS Synchronised |  |  |  |  |  |  |

| # | Item | Y/N/NA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| 3 | ENC operating at correct scale and Profile to use |  |  |  |  |  |  |
| 4 | Brilliance settings-Optimise display. Remember that screen shades not allowed |  |  |  |  |  |  |
| 5 | Sensor status GPS 1 GPS 2 Gyro 1 Gyro 2 Speed |  |  |  |  |  |  |
| 6 | All ECDIS alarms acknowledged and investigated before takeover |  |  |  |  |  |  |
| 7 | Look Ahead -To check the look ahead settings and verify complying passage plan. -To Look ahead and confirm the ENC chart for the duration of his watch is in Order |  |  |  |  |  |  |
| # | Item | Y/N/NA |  |  |  |  |  |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
|  | -To look ahead and confirm the passage plan on ECDIS in compliance with Passage Plan form NAV 01 |  |  |  |  |  |  |
|  | x Ocean passage Look ahead distance …….m Look ahead time …….minutes Beam/Breadth …….m |  |  |  |  |  |  |
|  | Outside fairway Look ahead distance …….m Look ahead time …….minutes Beam/Breadth …….m |  |  |  |  |  |  |
|  | Inside fairway Look ahead distance …….m Look ahead time …….minutes Beam/Breadth …….m |  |  |  |  |  |  |
| 8 | DGPS 1/2 - Verify DGPS position with DGPS |  |  |  |  |  |  |
| # | Item | Y/N/NA |  |  |  |  |  |
|  | - Check datum (Always should be WGS 84) |  |  |  |  |  |  |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
|  | - Check for offsets in GPS positions in ECDIS - Check Dilution of precision (DOP - HDOP/PDOP) levels (mention the values in NAV-21 – Vessel position log) - Check DGPS stations |  |  |  |  |  |  |
| 9 | Speed Log -Verify ECDIS STW value with speed log. -Verify ECDIS SOG value with speed log. (May also be the Doppler log) |  |  |  |  |  |  |
| 10 | Gyro 1/2 -Confirm in Order -Confirm corrections applied to gyro -Compare ECDIS gyro heading with gyro compass -Check no offset applied in gyro settings in ECDIS. |  |  |  |  |  |  |
| 11 | Echo sounder -Verify ECDIS echo sounder value with echo sounder. |  |  |  |  |  |  |
| # | Item | Y/N/NA |  |  |  |  |  |
|  | -Echo sounder depth alarm on Depth alarm ………m |  |  |  |  |  |  |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| 12 | Anemometer Verify ECDIS anemometer values with anemometer |  |  |  |  |  |  |
| 13 | Radar -Confirm radar switched “ON” and overlay on ECDIS possible. -Confirm overlay align with ECDIS ENC chart for vessel in coastal waters |  |  |  |  |  |  |
| 14 | ARPA -Confirm ARPA switched “ON” and overlay on ECDIS possible. -Confirm Overlay align with ECDIS ENC chart. |  |  |  |  |  |  |
| 15 | AIS Confirm AIS switched “ON” Confirm AIS overlay possible on ECDIS |  |  |  |  |  |  |
| 16 | Navtex |  |  |  |  |  |  |
| # | Item | Y/N/NA |  |  |  |  |  |
|  | -Confirm messages received on Navtex which are within 15Nm either side of the track are manually plotted |  |  |  |  |  |  |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
|  | -Check for error rates in messages and if not displaying correctly on ECDIS and within 15Nm either side of the tack, manually correct them -Check Navtex set correctly as per Master’s standing orders - Toggle NAVTEX overlay “on” and “off”, to verify all NAVTEX messages are displaying and if within 15Nm either side of the track are manually plotted. |  |  |  |  |  |  |
| 17 | Check the following parameters on ECDIS complying with your Passage Plan as applicable: |  |  |  |  |  |  |
|  | Ocean passage Safety Contour ................. m Safety Depth ................... m Shallow Contour ................. m Deep Contour .................. m User Objects .................... on/off Manual Danger Lines .......... on/off |  |  |  |  |  |  |

| # | Item | Y/N/NA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| Outside fairway Safety Contour ................. m Safety Depth ................... m Shallow Contour ................. m Deep Contour .................. m User Objects .................... on/off Manual Danger Lines .......... on/off Inside fairway Safety Contour ................. m Safety Depth ................... m Shallow Contour ................. m Deep Contour .................. m User Objects .................... on/off Manual Danger Lines .......... on/off |  |  |  |  |  |  |  |

| # | Item | Y/N/NA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| 18 | Chart Display Settings on ECDIS Must be in compliance with the Master’s standing orders |  |  |  |  |  |  |
| 19 | Chart area alarm settings on ECDIS Must be in compliance with the Master’s standing orders Cross Track Distance appropriate? Ocean passage XTD ....... m Outside fairway XTD ....... m Inside fairway XTD ....... m |  |  |  |  |  |  |
| # | Item |  |  | Y/N/NA |  |  |  |
| Time of changeover: |  | 0000- 0400 | 0400- 0800 | 0800- 1200 | 1200- 1600 | 1600- 2000 | 2000- 0000 |
| 20 | EGC/User maps -Confirm messages received are plotted -Appropriate user maps displayed |  |  |  |  |  |  |
| 21 | Nav warnings / T&P notices - Plot all Nav warnings / T&P within 15Nm either side of the track. - If AIO available toggle “on” and “off”, to verify all T&P messages are displaying and if within 15Nm either side of the track are manually plotted. |  |  |  |  |  |  |
| Time completed |  |  |  |  |  |  |  |
| Relieved officer’s signature: |  |  |  |  |  |  |  |
| Relieving officer’s signature: |  |  |  |  |  |  |  |
| Master’s Signature |  | Date/Time | : |  |  |  |  |