A sample of Master’s Standing Orders

(Should be to the point and concise)

The watch officer is the master’s representative and is primarily responsible at all times for the safe navigation of the ship and for complying with the International Regulations for Preventing Collision at Sea, 1972.

The watch officer should keep the watch on the bridge which he should in no circumstance leave until properly relieved.

Keeping a Good Lookout

- maintaining a continuous state of vigilance by sight and hearing as well as by all other available means, with regard to any significant change in the operating environment ;

- fully appraising the situation and the risk of collision, stranding and other dangers to navigation

- detecting ships or aircraft in distress, shipwrecked persons, wrecks, debris and other hazards to safe navigation

- being aware of operations on deck

A continuous and alert lookout is the most important consideration in the avoidance of the casualties. The keeping of an efficient lookout requires to be interpreted in its fullest sense which includes the following; -

- an alert all-round visual and aural lookout to ensure a full grasp of the current situation including the presence of ships and landmarks in the vicinity;

- close observation of the movements and compass bearing of approaching ships;

- identification of ship and shore lights;

- the need to ensure that the course is steered accurately and that wheel orders are correctly executed;

- observation of the radar and echo sounder displays; and

- observation of changes in the weather, especially the visibility.

Calling the Master (notify the master)

The watch officer should notify the Master immediately under the following circumstances;

- if restricted visibility less than 3 nm is encountered or expected ([navigation light, radar]);

- if the traffic condition or the movements of other ships are causing concern;

- if difficulty is experienced in maintaining course and/ or minimum CPA of 2 nm in Open water and 1 nm in Coastal Waters

- on failure to sight land, navigation mark or to obtain sounding by the expected time;

- if unexpectedly, land or a navigation mark is sighted or a change in soundings occurs;

- on breakdown of the engines, steering gear or any essential navigational equipment;

- if the radio equipment malfunctions;

- if in any doubt the possibility of weather damage;

- if the ship meets any hazard to navigation, such as ice or a derelict; and

- in any other emergency or if in any doubt

- in case of any doubt with Cargo Tanks Pressure (on Tankers) or on going Cargo Ventilation arrangement (on Cargo ships)

- in case of any abnormal sound on deck or doubt with operations on deck

- in case of any suspicious boat/vessel approaching own ship or security of own ship is in danger

- in case of venting of cargo tanks (on tankers) with high H2S contents to release the high pressure

**	Despite the requirement to notify the Master immediately in the foregoing circumstance, the watch officer shall in addition not hesitate to take immediate actions for the safety of the ship, where circumstances so require.

**	If at any time the watch officer is to be relieved when a manoeuvre or other action to avoid any hazards is taking place, the relief of that officer shall be deferred until such action has been completed.

**	The watch officer shall continue to be responsible for the safe navigation, despite the presence of the master on the bridge, until informed specifically that the master has assumed that responsibility and this is mutually understood.

**	The watch officer notify the master immediately when in any doubt as to what action to take in the interest of safety.

**	In case of need, the watch officer shall not hesitate to use the helm, engines and sound signalling apparatus.  However, timely notice of intended variations of engine speed shall be given in accordance with the applicable procedures, if possible.

**	The watch officer shall make regular checks to ensure that :

        - the person steering the ship or the automatic pilot is steering the correct course;

        - the standard compass error is determined at least once a watch; and, the standard and gyro-compass are frequently compared;

        - the automatic pilot is tested manually at least once a watch;

        - the navigation and signal lights and other navigational equipment are functioning properly;

        - the radio equipment is functioning properly in accordance with paragraph 86, chapter VIII, part A, STCW code; and

        - the controls, alarms and indicators are functioning properly.

Changing over the watch at sea

The relieving officer of the watch should ensure that members of his watch are fully capable of performing their duties and particularly that they are adjusted to night vision.  The watch officer shall not take over the watch until his vision is fully adjusted to the light conditions and he has personally satisfied himself regarding; -

- standing orders and other special instructions relating to the navigation of the vessel;

- the position, course, speed and draft of the vessel;

- .prevailing and predicted tide, currents, weather, visibility and the effect of these factors upon course and speed;

- procedures for the use of main engines to manoeuvre when the main engines are on bridge control ; and

- the navigational situation, including but not limited to ;

        - the operational condition of all navigational and safety equipment,

        - the errors of gyro and magnetic compasses,

        - the presence and movement of ships in sight or known to be in the vicinity,

        - the conditions and hazards likely to be encountered during the watch, and

        - the possible effects of heel, trim, water density and squat on under keel clearance.

Watch keeping in port / at anchor

If the master considers it necessary, a continuous navigational watch shall be maintained at anchor.

The watch officer shall at all times be adequate to :

- ensure the safety of life, of the ship, the port and the environment, and the safe operation of all machinery related to cargo operation;

- observe international, national and local rules; and

- maintain order and the normal routine operations of the ship including cargo operations

- ensure security of the ship is not compromised at any time from any unauthorized visitors and or any boat alongside own ship

- anti-piracy watches are maintained as planned, where applicable

Taking over the port (anchor) watch

If, at the moment of handling over the watch, an important operation is being performed it shall be concluded by the officer being relieved, except when ordered otherwise by the master.

Prior to taking over the watch, the relieving officer shall be informed of the following by the watch officer in charge as to :

- the depth of the water at the berth, the draught, the level and time of high and low waters, the securing of the moorings, the arrangement of anchors and the scope of the anchor chain, and other mooring features important to the safety of the ship;  the state of main engines and their availability for emergency use ;

- all work to be performed on board the ship; the nature, amount and disposition of cargo loaded or remaining, and any residue on board after unloading the ship ;

- the level of water in bilge and ballast tanks ;

- the signals or lights being exhibited or sounded ;

- the number of crew members required to be on board and presence of any other persons on board ;

- the state of fire-fighting appliances ;

- any special port regulations ;

- the master’s standing and special orders ;

- the lines of communication available between the ship and shore personnel, including port authorities, in the event of an emergency arising or assistance being required ;

- any other circumstances of importance to the safety of the ship, it’s crew, cargo or protection of the environment from pollution ; and

- the procedures for notifying the appropriate authority of any environmental pollution resulting from ship activities

- Deck and security watch keepers details

At anchor

While at anchor, the watch officer shall :

- determine and plot the ship’s position on the appropriate chart as soon as practicable;

- when circumstance permit, check at sufficiently frequent intervals whether the ship is remaining securely at anchor by taking bearings of fixed navigation marks or identifiable shore objects;

- ensure that proper look-out is maintained;

- ensure that inspection rounds of the ship are made periodically;

- observe meteorological and tidal condition and the state of the sea;

- notify the master and undertake all necessary measures if the ship drags anchor;

- ensure that the state of readiness of the main engine and other machinery is in accordance with the master’s instructions;

- if visibility deteriorates, notify the master;

- ensure that the ship exhibits the appropriate lights and shapes and that appropriate sound signals are made in accordance with all applicable regulations ; and

- take measures to protect the environment from pollution by the ship and comply with applicable pollution regulations.

- ensure security of the ship is not compromised at any time. Any boat approaching the vessel must be as per pre-informed sources. In case of doubt Master is to be informed immediately.

- ensure that anti-piracy watches are maintained at pre-determined ports / harbours

| Date / Place | Master / MV/MT |
| --- | --- |

| C/O |  | 2/O |  | 3/O |  |
| --- | --- | --- | --- | --- | --- |

-x-